3GPP TSG-RAN WG2 Meeting #116e                            R2-2109885

e-meeting, 1st – 12th November 2021

Source: 			ZTE Corporation, Sanechips,

Title: 	Clarification on density configuration in CSI-RS based measurement

Agenda item:		6.1.4.1.2

Document for: 	Discussion and Decision

- Introduction

Based on TS 38.331, for CSI-RS based RRM measurement, in each MeasObjectNR, network can provide a “density” configuration which applies to all the CSI-RS resources of a given cell.

CSI-RS-CellMobility ::=             SEQUENCE {

cellId                              PhysCellId,

csi-rs-MeasurementBW                SEQUENCE {

nrofPRBs                            ENUMERATED { size24, size48, size96, size192, size264},

startPRB                            INTEGER(0..2169)

},

density                             ENUMERATED {d1,d3}              OPTIONAL,   -- Need R

csi-rs-ResourceList-Mobility        SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesRRM)) OF CSI-RS-Resource-Mobility

}

According to the IE definition, the value range of this parameter includes {d1, d3}, however, after checking RAN2 and 4 specifications, it is unclear if both of them can be supported by current UEs. So clarification is needed in order to avoid inter-operability issues.

- Discussion

This ‘density’ parameter is used to indicate the frequency domain density of CSI-RS resource. Value ‘d1’ means CSI-RS is transmitted on one RE in one PRB, and value ‘d3’ means CSI-RS is transmitted on three REs in one PRB. In addition, network can configure ‘frequencyDomainAllocation’ parameter to further indicate the detailed location of the CSI-RS RE.

frequencyDomainAllocation           CHOICE {

row1                                BIT STRING (SIZE (4)),

row2                                BIT STRING (SIZE (12))

},

In TS 38.331, the field description of density is defined as below:

density Frequency domain density for the 1-port CSI-RS for L3 mobility. See TS 38.211 [16], clause 7.4.1.

According to the field description, and TS 38.211, there is no restriction saying only specific value (e.g. d3) is allowed, so it is nature to interpret that both d1 and d3 can be configured by network.

Observation 1: There is no restriction on the value range of density in TS 38.331.

In addition, both density ‘d1’ and ‘d3’ are defined in TS 38.211, table 7.4.1.5.3.1. (Note: for CSI-RS based RRM measurement, UE assumes the cdm-Type is ‘noCDM’ and there is only one antenna port according to TS 38.214)

#table extracted from TS 38.211

Table 7.4.1.5.3-1: CSI-RS locations within a slot.

| Row | Ports | Density | cdm-Type |  | CDM group index |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 3 | noCDM | k0,l0, k0+4,l0, k0+8,l0 | 0,0,0 | 0 | 0 |
| 2 | 1 | 1, 0.5 | noCDM | k0,l0, | 0 | 0 | 0 |

Regarding the configuration of CSI-RS resource density, an LS [1] was sent from RAN1 at RAN2_103bis meeting, the content of the LS is pasted below:

1. Overall Description: RAN1 would to inform RAN2 that density parameter in CSI-RS-CellMobility is required for measurement of CSI-RS. RAN1 recommends that RAN2 make appropriate actions to make sure value of density is available in all cases. If a default value is to be determined by RAN2, RAN1 recommends use of d3 as a default value. 2. Actions: To RAN2: RAN1 would like kindly ask RAN2 to consider the above in their further work.

Based on the LS, density configuration is considered to be mandatorily provided in case of CSI-RS based RRM measurement, and the default value can be ‘d3’. However, it is too late to change the Need Code in RAN2, so it remains “Need R” in TS 38.331, and it is up to network implementation to ensure the configuration is valid.

So basically, both RAN1 and RAN2 specs support configuring density ‘d1’ for CSI-RS based RRM measurement.

Observation 2: For CSI-RS based RRM measurement, both RAN1 and RAN2 specs support configuring CSI-RS resource density to either ‘d1’ or ‘d3’.

Although CSI-RS based RRM measurement has been supported since Rel-15, and RAN1 and RAN2 finished the specification work in Rel-15, due to the heavy work load in RAN4, the UE requirements of CSI-RS measurement has been specified in RAN4 since Rel-16. And based on the latest version of RAN4 spec TS 38.133, we observed that only the UE requirement when density is set to ‘d3’ was defined.

Observation 3: UE requirements when density is set to ‘d1’ is not defined in RAN4 spec TS 38.133.

So far, several CSI-RS measurement related UE capabilities have been defined (see Annex). However, none of them can be used to recognize the support of different CSI-RS resource densities. So from network perspective, if the UE reports capability “csi-RSRP-AndRSRQ-MeasWithSSB” or “csi-RSRP-AndRSRQ-MeasWithoutSSB”, it can be interpreted both density ‘d1’ and ‘d3’ are supported when configuring CSI-RS based measurement.

In general, lack of UE requirement in TS 38.133 implies the corresponding scenario cannot be supported by UEs. However, since no configuration limitation is defined in TS 38.331, it is unclear how UE behaves when network configures density to ‘d1’. To avoid inter-operability issue, RAN2 is asked to clarify this issue.

Proposal 1: For a UE indicates the support of CSI-RS based RRM measurement, to clarify whether density parameter can be set to ‘d1’.

If it is hard to reach consensus in RAN2, we suggest to send LS to RAN4 for clarification. The draft LS can be found in [2].

Proposal 2: Send LS to RAN4 to clarify whether density ‘d1’ can be supported for CSI-RS based measurement. Approve the draft LS in [2]

- Conclusion and proposals

RAN2 is kindly asked to discuss and adopt the following proposal:

Observation 1: There is no restriction on the value range of density in TS 38.331.

Observation 2: For CSI-RS based RRM measurement, both RAN1 and RAN2 specs support configuring CSI-RS resource density to either ‘d1’ or ‘d3’.

Observation 3: UE requirements when density is set to ‘d1’ is not defined in RAN4 spec TS 38.133.

Proposal 1: For a UE indicates the support of CSI-RS based RRM measurement, to clarify whether density parameter can be set to ‘d1’.

Proposal 2: Send LS to RAN4 to clarify whether density ‘d1’ can be supported for CSI-RS based measurement. Approve the draft LS in [2]

- Reference

[1] R2-1813514 (R1-1809960) LS on density parameter for CSI-RS-ResourceConfigMobility  from RAN1, to RAN2.

[2] R2-2109886 LS on density configuration in CSI-RS based measurement  from RAN2, to RAN1.

- Annex

CSI-RS measurement related UE capabilities.

| csi-RSRP-AndRSRQ-MeasWithSSB Indicates whether the UE can perform CSI-RSRP and CSI-RSRQ measurement as specified in TS 38.215 [13], where CSI-RS resource is configured with an associated SS/PBCH. If this parameter is indicated for FR1 and FR2 differently, each indication corresponds to the frequency range of measured target cell. If the UE supports this feature, the UE needs to report maxNumberCSI-RS-RRM-RS-SINR. This applies only to non-shared spectrum channel access. For shared spectrum channel access, csi-RS-RLM-r16 applies. | UE | No | No | Yes |
| --- | --- | --- | --- | --- |
| csi-RSRP-AndRSRQ-MeasWithoutSSB Indicates whether the UE can perform CSI-RSRP and CSI-RSRQ measurement as specified in TS 38.215 [13], where CSI-RS resource is configured for a cell that transmits SS/PBCH block and without an associated SS/PBCH block. If this parameter is indicated for FR1 and FR2 differently, each indication corresponds to the frequency range of measured target cell. If the UE supports this feature, the UE needs to report maxNumberCSI-RS-RRM-RS-SINR. This applies only to non-shared spectrum channel access. For shared spectrum channel access, csi-RSRP-AndRSRQ-MeasWithoutSSB-r16 applies. | UE | No | No | Yes |
| csi-SINR-Meas Indicates whether the UE can perform CSI-SINR measurements based on configured CSI-RS resources as specified in TS 38.215 [13]. If this parameter is indicated for FR1 and FR2 differently, each indication corresponding to the frequency range of measured target cell. If the UE supports this feature, the UE needs to report maxNumberCSI-RS-RRM-RS-SINR. This applies only to non-shared spectrum channel access. For shared spectrum channel access, csi-SINR-Meas-r16 applies. | UE | No | No | Yes |
| increasedNumberofCSIRSPerMO-r16 Indicates support of up to 192 CSI-RS resource for L3 mobility configuration per measurement object configured with associatedSSB. | UE | No | No | Yes |
| maxNumberCSI-RS-RRM-RS-SINR Defines the maximum number of CSI-RS resources for RRM and RS-SINR measurement across all measurement frequencies per slot. If UE supports any of csi-RSRP-AndRSRQ-MeasWithSSB, csi-RSRP-AndRSRQ-MeasWithoutSSB, and csi-SINR-Meas, UE shall report this capability. | UE | CY | No | No |