3GPP TSG-RAN WG2 Meeting #116-e	R2-2110152

E-Meeting, 1 – 12, November 2021

Agenda Item	: 6.2.3 (5G_V2X_NRSL-Core)

Source	: LG Electronics Inc

Title	: Clarification on exceptional pool configuration

Document for	: Discussion and Decision

1.	Introduction

This is to discuss the clarification of the exceptional pool configuration.

2.	Discussion

Referring to the NOTE below of the current MAC specification, it is not clear whether sl-TxPoolExceptional is a resource pool in which PSFCH is configured or in a resource pool in which PSFCH is not configured.

“NOTE 2:	The MAC entity expects that PSFCH is always configured by RRC for at least one pool of resources in case that at least a logical channel configured with sl-HARQ-FeedbackEnabled is set to enabled.”

If the sl-TxPoolExceptional can be set to a resource pool in which the PSFCH is not configured, it is unclear how the Tx UE performs the transmission using the sl-TxPoolExceptional when transmitting the HARQ Feedback Enabled MAC PDU. Therefore, RAN2 should discuss the issue mentioned above.

Option 1. If sl-TxPoolExceptional may be a resource pool in which PSFCH is not configured, when the Tx UE transmits HARQ Feedback Enabled MAC PDU using sl-TxPoolExceptional, the switching to HARQ Feedback Disabled MAC PDU transmission shall be supported.

Option 2. If sl-TxPoolExceptional is always a resource pool in which PSFCH is configured, modify the NOTE of MAC specification to specify that sl-TxPoolExceptional is always a resource pool in which PSFCH is configured.

Observation 1. Referring to the NOTE of the current MAC specification, it is not clear whether sl-TxPoolExceptional is a resource pool in which PSFCH is configured or in a resource pool in which PSFCH is not configured. “NOTE 2:	The MAC entity expects that PSFCH is always configured by RRC for at least one pool of resources in case that at least a logical channel configured with sl-HARQ-FeedbackEnabled is set to enabled.”

Proposal 1. RAN2 has to clarify and decide whether sl-TxPoolExceptional is always a resource pool in which PSFCH is configured or maybe a resource pool in which PSFCH is not configured.

Proposal 2. RAN2 can discuss the following options with respect to proposal 1.

Option 1. If sl-TxPoolExceptional may be a resource pool in which PSFCH is not configured, when the Tx UE transmits HARQ Feedback Enabled MAC PDU using sl-TxPoolExceptional, the switching to HARQ Feedback Disabled MAC PDU transmission shall be supported.

Option 2. If sl-TxPoolExceptional is always a resource pool in which PSFCH is configured, modify the NOTE of MAC specification to specify that sl-TxPoolExceptional is always a resource pool in which PSFCH is configured.

“NOTE 2:	The MAC entity expects that PSFCH is always configured by RRC for at least one for each sl-TxPoolSelectedNormal and sl-TxPoolExceptional pool of resources in case that at least a logical channel configured with sl-HARQ-FeedbackEnabled is set to enabled.

3.	Conclusion

This contribution discussed clarification of the exceptional pool configuration, which can be summarized as follows:

Observation 1. Referring to the NOTE of the current MAC specification, it is not clear whether sl-TxPoolExceptional is a resource pool in which PSFCH is configured or in a resource pool in which PSFCH is not configured. “NOTE 2:	The MAC entity expects that PSFCH is always configured by RRC for at least one pool of resources in case that at least a logical channel configured with sl-HARQ-FeedbackEnabled is set to enabled.”

Proposal 1. RAN2 has to clarify and decide whether sl-TxPoolExceptional is always a resource pool in which PSFCH is configured or maybe a resource pool in which PSFCH is not configured.

Proposal 2. RAN2 can discuss the following options with respect to proposal 1.

Option 1. If sl-TxPoolExceptional may be a resource pool in which PSFCH is not configured, when the Tx UE transmits HARQ Feedback Enabled MAC PDU using sl-TxPoolExceptional, the switching to HARQ Feedback Disabled MAC PDU transmission shall be supported.

Option 2. If sl-TxPoolExceptional is always a resource pool in which PSFCH is configured, modify the NOTE of MAC specification to specify that sl-TxPoolExceptional is always a resource pool in which PSFCH is configured.

“NOTE 2:	The MAC entity expects that PSFCH is always configured by RRC for at least one for each sl-TxPoolSelectedNormal and sl-TxPoolExceptional pool of resources in case that at least a logical channel configured with sl-HARQ-FeedbackEnabled is set to enabled.

4.	Reference

[1] #115-e meeting’s chairman note, Samsung