3GPP TSG-RAN WG2 Meeting #116 electronic	R2-2110311

Online, November 1 - November 12, 2021

Agenda item:		8.10.3.3

Source:		Lenovo, Motorola Mobility

Title:		Connected mobility for NT-NTN continuity

Document for:		Discussion

- Introduction

The last RAN2#115e meeting made the following agreements regarding connected mobility for NT-NTN continuity:

- RAN2 down priorities further enhacnements for connected mode for Rel-17 for TN-NTN mobility. - RAN2 continue discussing the exact solution for TN priorization over NTN for idle mode.

This contribution further analyses the applicability of intra-NTN mobility enhancements and possible issues for TN-NTN service continuity.

- Discussion

In RAN2#114e it was agreed that:

- Same CHO trigger conditions and RRM events can be used within NTN and NTN-TN mobility provided these are supported by the UE. NTN-TN means both “from NTN to TN (hand-in)” and “from NTN to TN (hand-in) and from TN to NTN (hand-out)”. FFS for enhancements.

And during RAN2#115e there was email discussion for TN-NTN mobility, wherein the necessity of further enhancements was discussed. Some companies would like to wait for the final details if NTN CHO enhancements to see if applicable to TN-NTN mobility, while other companies consider that NTN CHO enhancements are sufficient. Based on the majority views it was agreed to down prioritize further enhancements for connected mode for Rel-17 for TN-NTN mobility.

From the mobility perspective, there could be little necessity of further enhancement between TN and NTN. However as mentioned in TS 22.261, “the 5G system shall support service continuity between 5G terrestrial access network and 5G satellite access networks owned by the same operator or owned by 2 different operators having an agreement”. From the perspective of service continuity, additional work to guarantee it may still be needed.

Take the NTN CHO enhancements as an example. For NR NTN it was agreed to introduce location-based and time-based conditions for executing CHO between NTN cells. If the location-based condition is configured, UE shall evaluate its distances to the reference points (i.e. cell centre) of serving and candidate neighbouring cells, and execute CHO if conditions are fulfilled. If the time-based condition is configured, UE shall evaluate in a given time period (T1 as start of evaluation and T2 as the end of evaluation), and execute CHO if conditions are fulfilled. The location-based or time-based conditions are introduced considering the fuzzy RSRP difference in an NTN cell and between NTN cells, allowing NTN to configure CHO conditions more flexibly and efficiently. However even with such enhancements, the data transmission between UE and network may still be interrupted during execution of handover, wherein UE needs to disconnect from the serving cell and establish connection to the target cell. The large propagation delay from NTN gNB to the UE, and possibly the larger propagation delay between NTN gNBs could still cause service interruption. This issue may not be solved by the NTN CHO enhancements.

Observation 1: The service continuity issues may not be solved by applying intra-NTN enhancements for connected mobility. Further enhancement could be necessary for TN-NTN service continuity.

To guarantee the service continuity between TN and NTN, some existing mobility mechanisms can be considered, including DAPS. If DAPS is supported in NTN, UE may maintain connection to TN and NTN at the same time during handover and thus the service continuity can be ensured regardless of the propagation delays. Besides, from technical perspective DAPS can be applied to NTN directly or possibly with only minor enhancements.

Figure 1. DAPS for TN-NTN service continuity

Observation 2: Existing mobility mechanisms including DAPS may guarantee TN-NTN service continuity.

Therefore it is proposed to consider support DAPS in NTN for the sake of TN-NTN service continuity. Other enhancements can be considered in further releases if any.

Proposal 1: RAN2 may consider support of DAPS in NR NTN for TN-NTN service continuity if time allows.

During the email discussion the idle mobility between TN and NTN was also discussed. For TN-NTN idle mobility, there has been some motivations of prioritizing TN over NTN. However as NTN and TN will possibly use different frequency bands, there is no need of further enhancement and the legacy frequency priority can do the work.

Proposal 2: The frequency-based priorities are used to prioritize TN over NTN by setting the frequency bands of TN with higher priority than that of NTN.

- Conclusion

In this contribution, we discuss the applicability of intra-NTN mobility enhancements and possible issues for TN-NTN service continuity. The following observation are given:

Observation 1: The service continuity issues may not be solved by applying intra-NTN enhancements for connected mobility. Further enhancement could be necessary for TN-NTN service continuity.

Observation 2: Existing mobility mechanisms including DAPS may guarantee TN-NTN service continuity.

And we propose:

Proposal 1: RAN2 may consider support of DAPS in NR NTN for TN-NTN service continuity if time allows.

Proposal 2: The frequency-based priorities are used to prioritize TN over NTN by setting the frequency bands of TN with higher priority than that of NTN.