3GPP TSG-RAN WG2 #116-e	R2-2110904

Electronic meeting, November 1st – 12th, 2021

Agenda Item:	8.16.4

Source:	Ericsson

Title:	Support of emergency services and PWS for SNPNs

Document for:	Discussion

# 1	Introduction

The following agreements were reached during RAN2#115-e:

Introduce a new IE/field to indicate the support of IMS emergency service for SNPN.

eCall over IMS is not supported in SNPNs in Rel-17.

PWS can be supported in SNPNs in Rel-17.

According to RAN2#115-e’s Agenda Item 8.16.4 Summary, R2-2109017 [1], the following issues need further discussion:

Cat-a-Proposal 2	For the R17 SNPN-capable UEs that are not in SNPN Access Mode and R17 Non-SNPN capable UEs:

- UE not operating in the SNPN mode should not camp on an SNPN cell supporting emergency services to obtain emergency services.

Cat-b-Proposal 1	RAN2 to discuss whether the emergency support indication for SNPN is broadcast per SNPN or per cell.

Cat-b-Proposal 2	RAN2 to further discuss whether the following kinds of UEs are allowed to camp on an SNPN cell supporting emergency services to obtain emergency services.

a) R16 SNPN-capable UEs that are in SNPN Access Mode

b)  R16 SNPN-capable UEs that are not in SNPN Access Mode

c)  R16 Non-SNPN capable UEs

d)  R15 UEs

Cat-c-Proposal 1	RAN2 to further discuss the signaling details of the emergency support indicator.

Cat-c-Proposal 2	RAN2 postpone to discuss the definition of acceptable cell and the impacts on "Any Cell Selection state" and "Camped on Any Cell state" of TS 38.304 before having an agreement on support of PWS for SNPN.

Cat-c-Proposal 3	RAN2 postpone the early implementation discussion and the impacts on support of IMS voice for SNPN.

Based on the above, in this contribution we address the previous open issues.

# 2	Discussion

## 2.1	Indicating support for IMS Emergency Services

### 2.1.1 	Common indication or indication per SNPN network?

In this section, we discuss the following proposal from RAN2#115-e’s Agenda Item 8.16.4 Summary, R2-2109017 [1]:

Cat-b-Proposal 1 RAN2 to discuss whether the emergency support indication for SNPN is broadcast per SNPN or per cell.

The legacy SIB1 ims-EmergencySupport indication is common for all PLMNs sharing the cell. However, SNPNs cover a wide range of use cases to satisfy the needs of different vertical industries. Among the different options for deploying them, there are scenarios in which they are conceived as isolated networks, as well as those in which the RAN is shared between SNPNs. For this last scenario, the networks sharing the RAN can be intended for very different services.

Figure 1. Illustration of SNPNs which do not have the same support for IMS Emergency Services.

For example, Figure 1 shows the case in which SNPN-1 corresponds to a campus and SNPN-2 to a factory. The latter, for instance, may not have any voice support as it is only intended to serve IoT devices. In this regard, emergency calls may deteriorate the performance of the network and compete for resources from IoT devices with time-critical services. Hence, when having RAN sharing cases, it should therefore be possible to disable support of emergency services in SNPNs providing time-critical services to IoT devices.

A corresponding conclusion has also been made as part of TS 23.700-07 [3], clause 6.23.4:

-	if the NG-RAN is shared by more than one network […], the broadcast indicator is related to those networks that supports Emergency Services.

Such a distinction is only possible if there is one Emergency Support indication for each listed SNPN.

The new field indicating SNPN support of IMS Emergency is signalled per SNPN. Signalling details are FFS.

## 2.2	PWS support over SNPN

As seen above, during RAN2#115-e it was agreed that PWS can be supported by SNPNs in Rel-17. The WID was thus revised and the current version (see RP-212585 [4]) now includes the following objective:

- Support of PWS for SNPN [RAN2, RAN3] - Removal of stage 2 restrictions, and any associated clarifications if needed [RAN2, RAN3]

The draft Stage 2 running CR R2-2107957 [5] has already been updated on this matter, removing the restrictions for SNPNs to support PWS.

For PLMNs the following is stated in TS 22.268 [6], clause 4.9:

The PWS offered by a PLMN may be subject to PWS regional regulatory requirements and thus, the PWS offered may differ from PLMN to PLMN as well as from region to region within a PLMN.

Similar regulatory requirements e.g., depending on the number of users served by the network, also apply for SNPNs as stated in TS 22.261, v17.8.0:

Subject to regional or national regulatory requirements for PWS (see TS 22.268 [31]), the 5G system shall be able to support PWS for non-public networks.

For PLMNs, there is no PWS support indication broadcast by PLMNs, regardless of whether different PLMNs in a network sharing scenario are subject to different requirements. Hence, we believe that legacy PWS mechanisms should also apply for SNPNs, i.e., allow SNPNs to broadcast ETWS/CMAS notifications in SIB, while not introducing any PWS support indication for network/cell selection.

Legacy PLMN mechanism apply for SNPNs which support PWS, i.e., there is no broadcast indication for PWS support.

Regarding the second part of the objective for PWS support, we found the following text in TS 38.304 which refer to PWS (ETWS/CMAS):

## 4.3 Service types in RRC_IDLE state This clause defines the level of service that may be provided by the network to a UE in RRC_IDLE state. The following three levels of services are provided while a UE is in RRC_IDLE state: - Limited service (emergency calls, ETWS and CMAS on an acceptable cell); - Normal service (for public use or non-public use on a suitable cell); - Operator service (for operators only on a reserved cell).

## 4.5 Cell Categories The cells are categorised according to which services they offer: acceptable cell: An "acceptable cell" is a cell on which the UE may camp to obtain limited service (originate emergency calls and receive ETWS and CMAS notifications). Such a cell shall fulfil the following requirements, which is the minimum set of requirements to initiate an emergency call and to receive ETWS and CMAS notification in an NR network: - The cell is not barred, see clause 5.3.1; - The cell selection criteria are fulfilled, see clause 5.2.3.2.

As nothing specific is stated for PLMNs, no clarifications are needed and the same automatically applies to SNPNs. The same can be observed for TS 38.331.

No clarifications are needed in Stage-3 specifications regarding PWS support in SNPNs.

## 2.3	Availability of emergency services and PWS for UEs

In the following, we address the below proposal from RAN2#115-e’s AI 8.16.4 Summary, R2-2109017 [1]:

Cat-a-Proposal 2 For the R17 SNPN-capable UEs that are not in SNPN Access Mode and R17 Non-SNPN capable UEs: - UE not operating in the SNPN mode should not camp on an SNPN cell supporting emergency services to obtain emergency services.

In this context, RAN2 sent an LS to CT1 and SA2 at RAN2#115-e, asking the following question (see R2-2109114 [2]):

Can CT1/SA2 to confirm that the R17 SNPN-capable UEs that are not in SNPN Access Mode and R17 Non-SNPN capable UEs cannot camp on an SNPN cell supporting emergency services to obtain emergency services via any SNPN.

And CT1 confirmed the above statement, see C1-216096 [8].

Non-SNPN capable UEs, per definition, would not select an SNPN for camping (because they simply do not support access to SNPNs), and thus, they do not (or even cannot) enter SNPN access mode for SNPN selection.

Furthermore, similar discussions were raised in RAN2#115-e’s AI 8.16.4 Summary, R2-2109017 [1] for legacy UEs:

Cat-b-Proposal 2 RAN2 to further discuss whether the following kinds of UEs are allowed to camp on an SNPN cell supporting emergency services to obtain emergency services. a) R16 SNPN-capable UEs that are in SNPN Access Mode b) R16 SNPN-capable UEs that are not in SNPN Access Mode c) R16 Non-SNPN capable UEs d) R15 UEs

For Rel-15, SNPNs have not even been specified, such that it becomes obvious that Rel-15 UEs cannot camp on SNPN cells.

Rel-15 UEs are not aware of SNPNs, and thus, would not camp on an SNPN to obtain IMS emergency services.

For Rel-16, emergency services and PWS support have been explicitly excluded, see TS 38.300 [7] clause 16.6.1:

Emergency services and ETWS /CMAS are not supported in SNPN.

Given the below RAN2#115-e agreement, Rel-16 UEs are not able to read the indication.

Introduce a new IE/field to indicate the support of IMS emergency service for SNPN.

Therefore, any Rel-16 SNPN-capable UE independent of whether it is in SNPN access mode or not, emergency services are not supported.

For Rel-16 SNPN-capable UEs, regardless if in SNPN Access Mode, emergency services are not supported.

Given that PWS support in SNPNs is explicitly excluded from Rel-16, a Rel-16 UE in limited-service state camping on an SNPN cell could be configured to skip monitoring PWS notifications as such notifications are not expected in SNPNs. Under the assumption that there is no broadcast indication for PWS support (see Section 2.2), Rel-16 UEs that are configured to constantly monitor PWS notifications regardless of whether they are camping in PLMNs or not, would potentially receive PWS warning messages. However, it cannot be expected that this is the case.

Rel-16 SNPN-capable UEs in SNPN Access Mode are not expected to monitor PWS notifications in SNPN cells.

Based on the above observations, access to IMS emergency services would only be possible for Rel-17 (and onwards) UEs in SNPN Access Mode.

Only Rel-17 (and onwards) UEs in SNPN Access Mode, can camp on acceptable SNPN cells to get IMS emergency services.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	Rel-15 UEs are not aware of SNPNs, and thus, would not camp on an SNPN to obtain IMS emergency services.

Observation 2	For Rel-16 SNPN-capable UEs, regardless if in SNPN Access Mode, emergency services are not supported.

Observation 3	Rel-16 SNPN-capable UEs in SNPN Access Mode are not expected to monitor PWS notifications in SNPN cells.

Based on the discussion in the previous sections we propose the following:

Proposal 1	The new field indicating SNPN support of IMS Emergency is signalled per SNPN. Signalling details are FFS.

Proposal 2	Legacy PLMN mechanism apply for SNPNs which support PWS, i.e., there is no broadcast indication for PWS support.

Proposal 3	No clarifications are needed in Stage-3 specifications regarding PWS support in SNPNs.

Proposal 4	Only Rel-17 (and onwards) UEs in SNPN Access Mode, can camp on acceptable SNPN cells to get IMS emergency services.

# 4	References

R2-2109017, “[Pre115-e][010][eNPN] Summary Document for AI 8.16.4”,  CMCC, RAN2#115-e, 2021-08

R2-2109114, “LS on limited service availability of an SNPN”, RAN2, RAN2#115-e, 2021-08

TR 23.700-07, “Study on enhanced support of non-public networks, TSG SA”, V17.0.0, 2021-03

RP-212585, “Revised WID: Enhancement of Private Network Support for NG-RAN”, China Telecom, RAN#93-e, 2021-09

R2-2107957, “Draft Stage 2 CR: Non-Public Network enhancements”, Nokia, Nokia Shanghai Bell, RAN2#115-e, 2021-08

TS 22.268, “Public Warning System (PWS) requirements”, V16.4.0, 2020-09

TS 38.300, “NR and NG-RAN Overall Description; Stage 2”, V16.7.0, 2021-09

C1-216096, “Reply LS on limited service availability of an SNPN”, CT1#132-e, 2021-10