3GPP TSG-RAN WG2 #116e	R2-2110951

Electronic meeting, 2021-11-01 - 2021-11-12	Revision of R2-2108452

Agenda Item:	8.10.2.2

Source:	Ericsson

Title:	On configured scheduling, DRX, LCP, HARQ and SR/BSR in NTNs

Document for:	Discussion, Decision

# Introduction

At the RAN#86 meeting, the Study Item on “Study on solutions for NR to support non-terrestrial network” was completed [1] and the corresponding work item was approved and later updated at RAN#93 [2]. The work item aims to specify the enhancements identified for NR NTN and has the following RAN2 objectives on user plane aspects:

- MAC

  - Random access:

    - Definition of an offset for the start of the ra-ResponseWindow for NTN.

    - Introduction of an offset for the start of the ra-ContentionResolutionTimer to resolve Random access contention

    - Solutions for resolving preamble ambiguity and extension of RAR window.

    - Adaptation for Msg-3 scheduling

      - Only for the case with pre-compensation of timing and frequency offset at UE side)

  - Enhancement on UL scheduling to reduce scheduling latency.

  - DRX:

    - If HARQ feedback is enabled, introduction of offset for drx-HARQ-RTT-TimerDL and drx-HARQ-RTT-TimerUL.

    - If HARQ is turned off per HARQ process, adaptions in HARQ procedure

  - Scheduling Request: Extension of the value range of sr-ProhibitTimer

- RLC

  - Status reporting: Extension of the value range of t-Reassembly

  - Sequence Numbers: extension of the SN space only for GEO scenarios

- PDCP

  - SDU discard: Extension of the value range of discardTimer.

  - Sequence Numbers: extension of the SN space for GEO scenarios.

In this paper, we discuss the remaining NR adaptations to support NTN for configured grants, SPS, DRX, logical channel prioritization, HARQ bunding and SR/BSR.

# Configured grant

For a configured grant configuration, the network may configure configuredGrantTimer or not.

If configuredGrantTimer is configured for a CG-config, it means HARQ processes will not be used for a new CG transmission while it is running, and this allows the gNB to schedule retransmissions of that HARQ process ID (HP ID), if needed. When configuredGrantTimer is configured for a CG-config and periodicity is lower than the HARQ RTT, new CG transmissions may reuse a HARQ process ID before a HARQ RTT has elapsed.

If configuredGrantTimer is not configured for a CG-config and the periodicity is less than the HARQ RTT, then the HARQ process IDs will be reused before a HARQ RTT has elapsed when the UE has UL data to transmit. If configuredGrantTimer is not configured for a CG-config and the periodicity is longer than the HARQ RTT, retransmission of a HARQ process ID is possible based on the UL decoding result.

Configured grants can be configured with or without HARQ retransmissions in Rel-15.

Configured grants can be configured with or without reusing HARQ process IDs before a HARQ RTT has elapsed in Rel-15.

There is one issue with CG, if a blind retransmission grant is received – then the configuredGrantTimer (if configured) is restarted, thus blind retransmissions mean the next new CG transmission on that HARQ process is delayed. This is not a big issue as it is always better to configure the CG with robust modulation and coding to avoid having to send new CG activations (and to avoid the need for blind retransmissions) to update the robustness, and sending blind retransmissions uses PDCCH resources which defeats the purpose of configured grants. If blind retransmissions are needed, then it may be better to send a new CG activation with more robust coding, thus configured grants are not so useful to combine with blind retransmission as no PDCCH resources are saved and a blind retransmission grant may arrive at the UE when the UE skipped transmission because of no data to send (that CS-RNTI scheduled grant will then be skipped by the UE even if new data has arrived in the UE buffer, this is not the case if a dynamic C-RNTI grant is skipped due to no UL data and a blind C-RNTI retransmission arrive – then the UE will send the new data).

As RAN1 have agreed to increase the maximum number of HARQ processes, we need to update the following parameters in ConfiguredGrantConfig:

nrofHARQ-Processes                  INTEGER(1..16),

harq-ProcID-Offset-r16                  INTEGER (0..15)            OPTIONAL,   -- Need M

harq-ProcID-Offset2-r16                 INTEGER (0..15)            OPTIONAL,   -- Need M

For example

nrofHARQ-Processes-r17                  INTEGER(1..32)             OPTIONAL,   -- Need M

harq-ProcID-Offset-r17                  INTEGER (0..31)            OPTIONAL,   -- Need M

harq-ProcID-Offset2-r17                 INTEGER (0..31)            OPTIONAL,   -- Need M

The ConfiguredGrantConfiguration shall allow for up to 32 in nrofHARQ-Processes, and up to 31 in harq-ProcID-Offset and harq-ProcID-Offset2.

In the case that the periodicity of a configured grant is short and the RTT is long in comparison to the periodicity, for example in GEO, it may not be possible to configure the configuredGrantTimer to cover a full HARQ RTT. One use case for such a configuration is to have a CG for BSRs, for example a short periodicity of 5 ms and allowedCG-List to disallow all LCHs from using this CG, then the maximum value for configuredGrantTimer of 64 times periodicity is only 320 ms which is much shorter than the HARQ RTT in GEO.

From RRC spec:

periodicity                         ENUMERATED {

sym2, sym7, sym1x14, sym2x14, sym4x14, sym5x14, sym8x14, sym10x14, sym16x14, sym20x14,

sym32x14, sym40x14, sym64x14, sym80x14, sym128x14, sym160x14, sym256x14, sym320x14, sym512x14,

sym640x14, sym1024x14, sym1280x14, sym2560x14, sym5120x14,

sym6, sym1x12, sym2x12, sym4x12, sym5x12, sym8x12, sym10x12, sym16x12, sym20x12, sym32x12,

sym40x12, sym64x12, sym80x12, sym128x12, sym160x12, sym256x12, sym320x12, sym512x12, sym640x12,

sym1280x12, sym2560x12

},

configuredGrantTimer                INTEGER (1..64)                                                         OPTIONAL,   -- Need R

configuredGrantTimer Indicates the initial value of the configured grant timer (see TS 38.321 [3]) in multiples of periodicity. When cg-RetransmissonTimer is configured, if HARQ processes are shared among different configured grants on the same BWP, configuredGrantTimer * periodicity is set to the same value for the configurations that share HARQ processes on this BWP.

periodicity Periodicity for UL transmission without UL grant for type 1 and type 2 (see TS 38.321 [3], clause 5.8.2). The following periodicities are supported depending on the configured subcarrier spacing [symbols]: 15 kHz: 2, 7, n*14, where n={1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 128, 160, 320, 640} 30 kHz: 2, 7, n*14, where n={1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 128, 160, 256, 320, 640, 1280} 60 kHz with normal CP 2, 7, n*14, where n={1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 128, 160, 256, 320, 512, 640, 1280, 2560} 60 kHz with ECP: 2, 6, n*12, where n={1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 128, 160, 256, 320, 512, 640, 1280, 2560} 120 kHz: 2, 7, n*14, where n={1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 128, 160, 256, 320, 512, 640, 1024, 1280, 2560, 5120}

periodicityExt This field is used to calculate the periodicity for UL transmission without UL grant for type 1 and type 2 (see TS 38.321 [3], clause 5,8.2). If this field is present, the field periodicity is ignored. The following periodicites are supported depending on the configured subcarrier spacing [symbols]: 15 kHz: periodicityExt*14, where periodicityExt has a value between 1 and 640. 30 kHz: periodicityExt*14, where periodicityExt has a value between 1 and 1280. 60 kHz with normal CP: periodicityExt*14, where periodicityExt has a value between 1 and 2560. 60 kHz with ECP: periodicityExt*12, where periodicityExt has a value between 1 and 2560. 120 kHz: periodicityExt*14, where periodicityExt has a value between 1 and 5120.

Extend configuredGrantTimer to cover the RTT in GEO when periodicity is short.

# Semi-persistent scheduling

When semi-persistent scheduling (SPS) is configured in NTNs, we can use the already agreed methods to enable/disable HARQ feedback per HARQ process ID. We can configure the HP IDs of configured downlink assignment were some HP IDs are with HARQ feedback and some without.

DL SPS can use the already agreed method of enabling/disabling HARQ feedback in NTNs.

In 38.321 we have:

3>	if PDCCH contents indicate SPS deactivation:

4>	clear the configured downlink assignment for this Serving Cell (if any);

4>	if the timeAlignmentTimer, associated with the TAG containing the Serving Cell on which the HARQ feedback is to be transmitted, is running:

5>	indicate a positive acknowledgement for the SPS deactivation to the physical layer.

While in 38.213 we have:

A UE is expected to provide HARQ-ACK information in response to a SPS PDSCH release after N symbols from the last symbol of a PDCCH providing the SPS PDSCH release.

When SPS is configured and HARQ feedback happens to be disabled for one or more HP IDs, there is a question on how HARQ feedback shall be interpreted in the case of SPS deactivation.

In the DCI for SPS activation/deactivation with multiple SPS configurations, the HP ID field is used for indicating the sps-ConfigIndex.

We think the correct interpretation of the SPS deactivation is that the DCI is not related to any HP ID, and thus the HARQ feedback shall always be sent from the UE (for the gNB to know the UE have received an SPS deactivation).

HARQ feedback shall always be sent for SPS deactivation, no specification impact.

As RAN1 have increased the maximum number of HARQ processes, we need to update the following parameters in SPS-Config:

nrofHARQ-Processes              INTEGER (1..8),

harq-ProcID-Offset-r16          INTEGER (0..15)                 OPTIONAL,   -- Need R

For example

nrofHARQ-Processes-r17              INTEGER (1..32)                 OPTIONAL,   -- Need R

harq-ProcID-Offset-r17          INTEGER (0..31)                 OPTIONAL,   -- Need R

The SPS-Config shall allow up to 32 for number of HARQ processes, and up to 31 in harq-ProcID-Offset.

# New DRX, LCP, and MAC CE functionality

At RAN2#115 the following was agreed

Agreements:

- Confirm the RAN2 working assumption that offset to drx-HARQ-RTT-TimerUL length is equal to UE-gNB RTT (i.e. sum on UE's TA and K_mac).

- Confirm the RAN2 working assumption that for HARQ processes with DL HARQ feedback enabled, the drx-HARQ-RTT-TimerDL length is increased by an offset equal to UE-gNB RTT (i.e. sum on UE's TA and K_mac).

- No new LCP restrictions are introduced for exisiting UL MAC CEs (if new MAC CEs will be introduced we can revisit this)

- For dynamic grants, each LCH can optionally be semi statically configured (by RRC) to be mapped to one or more HARQ processes (FFS if it's possible to map to more than one HARQ process/ process type. FFS on mapping method). If there is no RRC configuration for this, this mapping has no effect (legacy behaviour applies).

Agreements via email - from offline 101:

1a.	For at least dynamic grants, the network may optionally configure an UL HARQ retransmission state per HARQ process. Two UL HARQ retransmission states are defined in NTN: HARQ state A and HARQ state B (FFS whether "HARQ state A" and "HARQ state B" should be renamed)

1b.	HARQ state A/B are defined as follows:

-	HARQ state A: length of drx-HARQ-RTT-TimerUL is extended by UE-gNB RTT (i.e. UE PDCCH monitoring is optimized to support UL retransmission grant based on UL decoding result).

-	HARQ state B:  drx-HARQ-RTT-TimerUL is not started.

2.	Configuration of UL HARQ retransmission state is semi-static, signalled via RRC, and the decision and criteria to configure UL HARQ retransmission state is under network control.

3.	For dynamic grants, each LCH can be optionally mapped to an UL HARQ retransmission state via semi-static RRC configuration. If there is no configuration, the mapping has no effect (legacy behaviour applies).

4.	If HARQ process has not been configured with an UL HARQ retransmission state, new LCH mapping rule has no effect (i.e. UE applies legacy behaviour).

5.	The following behaviours are supported for drx-HARQ-RTT-TimerUL in NTN per HARQ process: 1) Timer length is extended by offset; 2) Timer disabled (i.e. not started)

6.	UE determines drx-HARQ-RTT-TimerUL behaviour per HARQ process based on configured UL HARQ retransmission state.

7.	For HARQ process(es) not configured with an UL HARQ retransmission state, drx-HARQ-RTT-TimerUL and drx-RetransmissionTimerUL behave as per legacy.

Agreements via email - from offline 101 second round:

- An UL HARQ retransmission state is configured per HARQ process to support new LCH mapping restriction and proper configuration of drx-HARQ-RTT-TimerUL behaviour.

2.	The network may consider delay and reliability characteristics of ongoing services when choosing to configure an UL HARQ retransmission state.

3.	Alternative naming for HARQ state A/B can be further considered during stage 3, however UE behaviour in each state should be defined in specification.

4.	RAN2 understanding is that UE behaviour in HARQ state A (i.e. extending the drx-HARQ-RTT-TimerUL by UE-gNB RTT) best supports reception of UL retransmission grant based on UL decoding result. (No RAN2 specification impact)

5.	RAN2 understanding is that UE behaviour in HARQ state B (i.e. not starting drx-HARQ-RTT-TimerUL) best supports no UL retransmission and/or blind UL retransmission. (No RAN2 specification impact)

## New functionality per HARQ process ID

The naming “UL HARQ retransmission state A/B” is FFS.

Further, the UL HARQ retransmission state is used to control two things:

- The DRX behaviour per HARQ process ID

- Facilitate LCHs to be mapped only to certain HARQ process IDs that have a wanted DRX behaviour, that is, allowing LCHs to have a new logical channel prioritization (LCP) restriction per HARQ process ID

The first purpose is about DRX behaviour, and the second purpose have nothing to do with retransmissions but all to do with new transmissions. The second purpose is actually not at all directly related to the new signalling and the behaviour controlled by it. Therefore, the terminology to describe this functionality shall not contain the word “retransmission”.

The affected behaviour of “UL HARQ retransmission state” functionality is not directly related to HARQ retransmissions.

As the new LCP restriction on LCH mapping onto certain HARQ process IDs is related to the wanted DRX behaviour of the target HARQ process ID for each LCH, it is sufficient to use the word “DRX” instead of “retransmission”.

The network may configure the DRX behaviour per HARQ process ID based on its plan to send or not send retransmission grants, but that is completely up to the network to decide.

Further the word “state” is misleading as it is not a state of DRX or a HARQ retransmission state that is controlled, it is the DRX behaviour that is controlled (for “state A” a drx timer is extended, and for “state B” the same drx timer is not started).

The “UL HARQ retransmission state” is not directly related to a HARQ state or a DRX state, but instead related to a DRX behaviour (extend drx-HARQ-RTT-TimerUL or do not start the drx-HARQ-RTT-TimerUL), therefore using “retransmission” and “state” to describe it is misleading.

Thus, in “UL HARQ retransmission state” both the “retransmission” and “state” are misleading. Therefore, we propose to use “UL HARQ-DRX mode” instead as it reflects the function that it controls, alternatively “UL DRX mode”.

The new signalling, introduced per HARQ process ID, is given a new name. Select between “UL HARQ-DRX mode A/B” and “UL DRX mode A/B” instead of the misleading “UL HARQ retransmission state A/B”.

The new name shall also be reflected in the parameter used to configure this functionality, we propose

The new “UL HARQ-DRX mode A/B”/”UL DRX mode” is controlled by a RRC parameter named “uplinkHARQ-DRX-Mode” or “uplinkDRX-Mode” and can take two values for each HARQ process ID: ModeA or ModeB.

These may be configured in the MAC-CellGroupConfig as all HARQ processes are shared in a cell group and setting it individually per serving cell by including the parameter in PUSCH-ServingCellConfig.

RAN2 to discuss whether the new RRC parameter named “uplinkHARQ-DRX-Mode” or “uplinkDRX-Mode” shall be placed in MAC-CellGroupConfig or in PUSCH-ServingCellConfig.

There is no need to have per HARQ process flexibility in selection of ModeA, ModeB or Legacy, that is, if legacy behaviour is wanted it can only be used for all HARQ process IDs.

Legacy behaviour, instead of the new DRX behaviour and new LCP restrictions, is enabled by not configuring uplinkHARQ-DRX-Mode/uplinkDRX-Mode.

The uplinkHARQ-DRX-Mode/uplinkDRX-Mode is configured for all HARQ processes at the same time, for example in a bitmap.

This will reduce the configuration signalling and the network and UE complexity of handling this configuration.

## New LCP restrictions for LCH data

For dynamic grants, RAN2 agreed that each LCH can be mapped to a UL HARQ-DRX mode.

This new mapping shall be configured in the LogicalChannelConfig to enable per LCH configuration.

The new LCP restriction per LCH is controlled by a RRC parameter named “allowedHARQ-DRX” or “allowedDRX”, in LogicalChannelConfig, and can take two values: modeA or modeB.

The field description of allowedHARQ-DRX/allowedDRX is “If present, UL MAC SDUs from this logical channel can only be mapped to HARQ processes not configured uplinkHARQ-DRX-Mode/uplinkDRX-Mode or to HARQ processes configured with uplinkHARQ-DRX-Mode/uplinkDRX-Mode indicating a value equal to the value configured by this field.

If uplinkHARQ-DRX-Mode/uplinkDRX-Mode is not configured for a HARQ process, the MAC SDUs from the LCH can use any HP ID, regardless of if allowedHARQ-DRX-Mode/allowedDRX-Mode is configured or not.

If uplinkHARQ-DRX-Mode is configured for a HP ID, Proposal 10 allows three configurations per LCH: 1) the LCH can only use ModeA HP IDs (allowedHARQ-DRX-Mode/uplinkHARQ-DRX-Mode = ModeA), 2) the LCH can only use ModeB HP IDs (allowedHARQ-DRX-Mode/uplinkHARQ-DRX-Mode = ModeB), and 3) the LCH can use ModeA and ModeB (allowedHARQ-DRX-Mode is not configured).

## New MAC CE prio in LCP

To facilitate the network to control whether the new TA report MAC CE sent during Random Access procedures shall be transmitted in the Msg3 or Msg5, we need to decide the prio of this new MAC CE in the prio list of MAC spec 5.4.3.1.3:

Logical channels shall be prioritised in accordance with the following order (highest priority listed first):

-	C-RNTI MAC CE or data from UL-CCCH;

-	Configured Grant Confirmation MAC CE or BFR MAC CE or Multiple Entry Configured Grant Confirmation MAC CE;

-	Sidelink Configured Grant Confirmation MAC CE;

-	LBT failure MAC CE;

-	MAC CE for SL-BSR prioritized according to clause 5.22.1.6;

-	MAC CE for BSR, with exception of BSR included for padding;

-	Single Entry PHR MAC CE or Multiple Entry PHR MAC CE;

-	MAC CE for the number of Desired Guard Symbols;

-	MAC CE for Pre-emptive BSR;

-	MAC CE for SL-BSR, with exception of SL-BSR prioritized according to clause 5.22.1.6 and SL-BSR included for padding;

-	data from any Logical Channel, except data from UL-CCCH;

-	MAC CE for Recommended bit rate query;

-	MAC CE for BSR included for padding;

-	MAC CE for SL-BSR included for padding.

NOTE 2:	Prioritization among Configured Grant Confirmation MAC CE, Multiple Entry Configured Grant Confirmation MAC CE, and BFR MAC CE is up to UE implementation.

If we select a prio for the new MAC CE that is lower than “C-RNTI MAC CE or data from UL-CCCH” but higher than “data from any Logical Channel, except data from UL-CCCH”, the network can select to always get the new MAC CE in Msg3/MsgA by selecting a suitable TBS for Msg3/MsgA. If the TBS of Msg3/MsgA is not large enough to include the new MAC CE, then it will be included in Msg5 (for 4-step RA) or the next UL scheduling (for 2-step RA), this is because RRCSetupRequest and RRCResumeRequest is sent on SRB0 (UL-CCCH) in Msg3/MsgA while RRCResumeComplete and RRCSetupComplete is sent on SRB1 (that is “data from any Logical Channel, except data from UL-CCCH”).

The priority of the new MAC CE in the prio list in MAC spec section 5.4.3.1.3 shall be lower than “C-RNTI MAC CE or data from UL-CCCH” but higher than “data from any Logical Channel, except data from UL-CCCH”.

Further we think the BFR MAC CE, Configured Grant Confirmation MAC CEs, BSR MAC CEs and PHR MAC CEs, that may be triggered when doing RA in CONNECTED mode, shall have higher prio than the new MAC CE. For example, a delayed PHR can prevent a factor N times higher TBS, and thus the UL data may be delayed by a factor N, while a TA report that enables a decreased Koffset only marginally decrease the UL delay.

The priority of the new MAC CE in the prio list in the MAC spec section 5.4.3.1.3 shall be between “Single Entry PHR MAC CE or Multiple Entry PHR MAC CE” and “MAC CE for the number of Desired Guard Symbols”.

# DRX HARQ RTT timers

Here we discuss the DRX timers related to HARQ RTT. Further DRX issues are discussed in section 8.

## The drx-HARQ-RTT-TimerDL and drx- RetransmissionTimerDL

In case HARQ feedback is disabled, RAN2 agreed that the drx-HARQ-RTT-TimerDL is not started. This means that drx-RetransmissionTimerDL will not be started.

Some have argued that this will not allow the gNB to send blind retransmission assignments to the UE, but when the UE receives PDCCH indicating a new transmission (DL or UL), the drx-InactivityTimer is (re)started which will allow for plenty of blind retransmissions spread out in time. The DL HARQ processes with disabled feedback are supposed to be used for HARQ processes that are reused before a RTT has passed, thus there is no need to keep the UE monitoring PDCCH longer than what it will be doing due to drx-InactivityTimer running.

For DL SPS, if HARQ feedback is disabled then neither the drx-InactivityTimer nor drx-HARQ-RTT-TimerDL will be started when the UE receives a PDU. This is a small drawback, as the gNB may not be able to send opportunistic blind retransmission assignments to the UE except when the UE wakes up for a drx-OnDurationTimer. In general link adaptation for SPS need to be more robust in order to decrease the frequency of SPS activation (with updated modulation and coding scheme) and we may even configure pdsch-AggregationFactor (bundling) either separate for the SPS configuration or for all PDSCH transmissions. Thus, SPS in no reason for changing the start of drx-RetransmissionTimerDL for HARQ processes with disabled HARQ feedback.

The start of drx-RetransmissionTimerDL is not changed in NTNs.

## The drx-HARQ-RTT-TimerUL and drx-RetransmissionTimerUL

At RAN2#114-e the following was agreed:

Agreements via email (from offline 103):

- RAN2 working assumption: Offset for drx-HARQ-RTT-TimerUL is equal to UE-gNB RTT (if RAN1 decides something that requires to change this we can revisit it).

- drx-RetransmissionTimerDL timer length is not extended in NTN

Agreements:

- The following options are supported for drx-HARQ-RTT-TimerUL in NTN per HARQ process: 1) Timer length is extended by offset; 2) Timer set to zero and/or 3) Timer disabled (i.e. not started). FFS if this is based on explicit configuration or not. We can also come back to see whether both 2 and 3 are needed.

Some companies argue for starting the drx-RetransmissionTimerUL immediately after the PUSCH transmission to allow for blind retransmissions in case the drx-HARQ-RTT-TimerUL is disabled (this would be equivalent to using the value zero for drx-HARQ-RTT-TimerUL). This is not necessary, as the UE will anyway start (or restart) drx-InactivityTimer when receiving a grant for a new transmission (as well as when receiving an assignment for new transmission) allowing plenty of opportunities for opportunistic blind retransmissions.

In case the RTT is very long as in GEO – it is not efficient to extend drx-RetransmissionTimerUL (nor drx-InactivityTimer), to allow blind retransmissions at any time (this would be similar to not configure DRX at all), as this will waste UE power. In such cases, we can utilize the drx-onDurationTimer for blind opportunistic retransmissions to reduce the energy consumption.

drx-RetransmissionTimerUL timer length is not extended in NTN.

If drx-RetransmissionTimerUL is started after each PUSCH transmission to allow for further blind retransmissions after a blind retransmission, lots of UE energy will be wasted as most of the time there is no blind retransmission.

The start of drx-RetransmissionTimerUL is not changed in NTNs.

Link adaptation for configured grants need to be more robust than for dynamic scheduling to decrease the frequency of configured grant reactivations and we may even configure the ConfiguredGrantConfig field repK larger than one (bundling). For configured grants, if drx-HARQ-RTT-TimerUL is disabled for a HARQ process, then neither the drx-InactivityTimer nor drx-HARQ-RTT-TimerUL will be started when the UE transmit PUSCH. This is a small drawback, as the gNB may not be able to send opportunistic blind retransmission grants to the UE, except when the UE is in Active Time for some other reason (like when drx-onDurationTimer is running). To use blind retransmission grants for CG transmissions defeats the purpose of CGs, and can be replaced by bundling. If a new CG transmission is skipped, due to no data to transmit, then a blind retransmission grant addressed to CS-RNTI will be ignored.

A better alternative to configured grants with opportunistic blind retransmissions is to use opportunistic dynamic grants with skip padding and opportunistic blind retransmissions. In this case, after a skipped first grant, a dynamic retransmission addressed to C-RNTI will result in a new transmission if data has arrived in the UE buffer before the retransmission grant is received.

Configured grants with opportunistic blind retransmissions is less resource efficient than dynamic grants with blind retransmissions, because when a grant is skipped due to no data then a retransmission grant with CS-RNTI will be ignored, while a dynamic retransmission grant with C-RNTI will send new data (if new data have arrived).

## Modelling the drx-HARQ-RTT-TimerDL/UL in MAC spec

When the drx HARQ RTT timers are extended for some HARQ processes and not extended for some HARQ process (in UL respectively DL), the description in MAC spec section 5.7 becomes messy as it in legacy always refer to the RRC parameters (drx-HARQ-RTT-TimerDL/UL) while the timer values will depend on the HARQ process ID. Therefore, we propose to introduce a helper variable (help timer) similar to how REPETITION_NUMBER is used in section 5.4.2.1 in the MAC spec.

Introduce one help timer HARQ_RTT_TIMER_DL for each DL HARQ process. If the HARQ process is configured to be extended, set HARQ_RTT_TIMER_DL to drx-HARQ-RTT-TimerDL plus UE-gNB RTT, else set HARQ_RTT_TIMER_DL to drx-HARQ-RTT-TimerDL.

Introduce one help timer HARQ_RTT_TIMER_UL for each UL HARQ process. If the HARQ process is configured to be extended, set HARQ_RTT_TIMER_UL to drx-HARQ-RTT-TimerUL plus UE-gNB RTT, else set HARQ_RTT_TIMER_UL to drx-HARQ-RTT-TimerUL.

When the HARQ timers are extended by UE-gNB RTT, we need to define this quantity in the spec. Preferably by including a definition in the section 3.1.

Introduce a definition of the UE-gNB RTT in section 3.1. For example: “UE-gNB RTT: The sum of Kmac and TTA, where Kmac is the broadcasted parameter FIELD_NAME and TTA is defined in TS 38.2xx clause Y.Y”

# HARQ in NTN

In this section we go through changes to HARQ handling and scheduling for support of NTNs.

## Repetition transmission based HARQ retransmissions

At RAN2#114-e the following agreement was made:

- Repetition transmission based HARQ retransmission is always allowed and is explicitly indicated per HARQ process via DCI (as in legacy).

This agreement is incorrect as it only considers part of the Rel-16 additions to bundling (bundling is called repetition in RAN1). There is also semi-static RRC configuration of bundling from Rel-15 (see repK, pdsch-AggregationFactor and pusch-AggregationFactor) and in Rel-16 for SPS (see pdsch-AggregationFactor-r16). As repetition may be very useful to achieve robustness in NTNs, all options for bundling shall be supported. Therefore, we propose to revise the previous agreement.

Repetition transmission based HARQ retransmission is always allowed and is explicitly indicated via DCI, or semi-statically with RRC signalling (for dynamic scheduling, CG and SPS) (as in legacy). Note, this revises the agreement from RAN2#114e.

## Handling HARQ failures without HARQ retransmissions

If there are no HARQ retransmissions we must rely on link adaptation and RLC AM for robustness. Here we consider how to improve the communication by increasing the frequency of RLC status reports and by enabling the UE and gNB to faster detect that there are outstanding RLC AM SDUs that may need retransmission as well as increasing the gNB knowledge of the UE uplink buffer content.

The gNB may tune the RLC parameters t-PollRetransmit, t-Reassembly, pollPDU, pollByte and t-StatusProhibit of each bearer to increase the frequency of status reports and handle the long RTT.

For the DL, the gNB may proactively poll for RLC status reports more often to increase the number of status reports so that RLC retransmissions can be faster if a HARQ transmission fails.

For the UL, the gNB may proactively send RLC status reports, for example if a HARQ transmission fails, however it is difficult for the gNB to know which LCH the failed data belongs to.

### Adding in-flight BSRs

To help the gNB in estimating what data is lost in transmissions to know which LCH to send RLC status reports for, we propose to add an “in-flight BSR”. The in-flight BSR shall report the RLC AM buffer status of RLC AM data that is waiting for RLC ACK/NACK (the data in RLC that will be retransmitted if an RLC NACK is received).

When the gNB receives the BSR with in-flight buffer indication it can compare the LCH data received with the in-flight buffer indication and deduce that data has been lost since the RLC entity has in-flight buffer but no data was received.

Add an in-flight BSR to report the amount of RLC AM data that is waiting for an RLC status report.

For example, in uplink if RLC data lost by HARQ failure do not contain an RLC poll (not the last data in the RLC buffer), RLC receiver in gNB need to detect an outstanding RLC SN (if next transmission do not fail HARQ) and start t-Reassembly, then RLC t-Reassembly need to expire and gNB can send a status report. With the new BSR, the gNB can send an RLC status report immediately when a new BSR is received (for example when gNB knows some data was failed in HARQ, if gNB knows there is some data that is waiting a HARQ retransmission it can refrain from sending an early RLC status report), which may be at the same time as the t-Reassembly would be started in this example thus cutting at least the length of the t-Reassembly from the delay of the failed data.

For example, in uplink if RLC data lost by HARQ failure contain an RLC poll (and it was the last data in the RLC buffer), the RLC transmitter in the UE would after t-PollRetransmit expires send a new poll (if UE has some new RLC data to send for the bearer, or blindly retransmit some RLC data waiting for RLC ACK/NACK if the RLC new/retransmission buffer is still empty) and the gNB would then reply with a RLC status report. With the new BSR, the gNB can send an RLC status report immediately if a new BSR is received, which may be long before t-PollRetransmit expires.

The in-flight BSR may be triggered periodically or triggered from the RLC entity (for example after a poll bit is sent then the following UL transmission can have an in-flight BSR in case the poll PDU failed in HARQ). It may be triggered after a certain number of HARQ transmissions.

### Polling for BSR

To improve the gNB knowledge of the data the UE has in the buffer, we can have the gNB poll for a BSR.

Add an indication from the gNB to the UE that it shall send a BSR or an in-flight BSR or both.

This will be beneficial if BSRs happens to fail in HARQ or gNB estimate of UE buffer content indicates the UE has data but the gNB did not receive any BSR lately.

# Efficient SR and BSR

One major hurdle for enabling faster uplink transmissions is for the UE to quickly request and receive uplink resources in order to transmit its UL message. There are currently a set of options to reduce the latency until the UE can make use of the uplink resources. One option is for the UE to make use of SR request on PUCCH resources.

## The sr-ProhibitTimer

- Extend the timer length of sr-ProhibitTimer (FFS on the details)

For the case that we rely on the SR-BSR procedure, we need to adjust the sr-ProhibitTimer to handle the large RTT of NTNs.

For prioritized services the gNB may configure the UEs to send multiple SRs during an RTT to decrease the delay in case the gNB do not detect the first SR, This legacy possibility is not an optimization for NTNs – it is a legacy function that shall be possible also in NTNs. Therefore in NTNs, this timer shall be extended with higher values in the value range including values lower than the RTT, alternatively extend the timer with a factor times an RTT offset value where the factor may take values below one.

It is not sufficient to say that RAN1 can address any reliability issues with SRs. There will always be cases with varying interference that is not fully controllable by the network (say a number of UEs in neighbouring cells starts their transmissions and create intermittent interference, this can theoretically be solved by gNBs exchanging information when scheduling different cells but that comes at a large cost in signalling and possibly delay for the scheduler to await information from neighbours).

The values added to sr-ProhibitTimer in NTN shall include values lower than the round-trip time.

# DRX enhancements for NTN

The DRX is about when the UE shall monitor PDCCH and the UE may save energy if it does not need to monitor PDCCH. Here we discuss some further issues with DRX.

## Discussion on UE Active time

When in Active time, the UE is required to monitor the PDCCH continuously and is thus reachable for the network. Part of the time when the UE is active, is controlled by timers configured by the network such as C-DRX related timers and RA related timers. A UE with a C-DRX configuration can leave active time and discontinuously monitor the PDCCH to save energy.

During the Rel-16 SI on NTN, it was acknowledged that an NTN UE will need an offset for the drx-HARQ-RTT-TimerDL and drx-HARQ-RTT-TimerUL, to allow the UE not to enter Active time too soon and to avoid monitoring the PDCCH in vain. The UE is also allowed to enter Active time triggered by certain events. See below, pasted from TS 38.321:

------------------------------------------------------------------------------------------------------------

When a DRX cycle is configured, the Active Time for Serving Cells in a DRX group includes the time while:

-	drx-onDurationTimer or drx-InactivityTimer configured for the DRX group is running; or

-	drx-RetransmissionTimerDL or drx-RetransmissionTimerUL is running on any Serving Cell in the DRX group; or

-	ra-ContentionResolutionTimer (as described in clause 5.1.5) or msgB-ResponseWindow (as described in clause 5.1.4a) is running; or

-	a Scheduling Request is sent on PUCCH and is pending (as described in clause 5.4.4); or

-	a PDCCH indicating a new transmission addressed to the C-RNTI of the MAC entity has not been received after successful reception of a Random Access Response for the Random Access Preamble not selected by the MAC entity among the contention-based Random Access Preamble (as described in clauses 5.1.4 and 5.1.4a).

------------------------------------------------------------------------------------------------------------

When a UE has a sent an SR, and it is pending, it will enter Active time and start monitoring the PDCCH as described in the yellow highlighted part above. Given the large RTT of an NTN, the UE can’t expect a response to the SR until RTT ms has elapsed.

With current DRX procedure, the UE will monitor the PDCCH in vain for RTT ms after sending an SR.

Figure 18. Example of the active time period after sending SR.

Following the proposed way of handling DRX and RA timers, it is proposed to introduce an offset to when a UE should enter Active time and start monitoring for a response to an SR. Meanwhile, the UE should make use of DRX and thus be reachable by the network while waiting for the response to the SR.

For DRX in NTN, in the case that a UE sends an SR, the UE enters Active time to monitor for a response after an offset time has elapsed.

When uplink and downlink are aligned in the gNB, we can start the monitoring of PDCCH based on the DL timing instead of using an offset after sending the SR (using for example the used Timing Advance as offset) as this gives the exact timing for the DL.

When UL and DL are aligned in the gNB, the UE shall start monitoring the PDCCH in the downlink symbol that has the same symbol number, slot number and system frame number as the first uplink symbol after the end of the Scheduling Request transmission.

If monitoring the PDCCH after Scheduling Request transmission is not as described in Proposal 25 then the UE need a timing offset that risk monitoring late or early.

When the UE performs CFRA it will not start the contention resolution timer that controls the PDCCH monitoring during CBRA. Thus, the green highlighted part above, from TS 38.321, requires the UE to enter Active time and start the monitoring of the PDCCH. As such, and similar as for sending SR, the UE will enter Active time and start monitoring the PDCCH already after receiving Msg2 (RAR). For CFRA, the UL message sent on PUSCH, according to the grant received in the RAR, is by the network seen as an acknowledgment that the UE received the RAR message, and the network will typically not schedule the UE before the granted transmission in the RAR message has been received. Given the RTT of an NTN, the UE will therefore not receive a response until at least one RTT has elapsed.

For CFRA, the network typically waits to receive msg3 granted in the RAR, as an acknowledgment that the UE has received the RAR, before it schedules the UE.

With current DRX procedure, after receiving the RAR message for CFRA, the UE will monitor the PDCCH in vain for at least RTT ms if the network does not schedule the UE in the meantime.

We propose to introduce a configurable offset to when a UE should enter Active time when doing CFRA. When the UE has sent the PUSCH message as granted in the received RAR, it would not expect a response until RTT ms have elapsed if the network does not schedule the UE in the meantime. Thus, the UE can offset the start of the active time by RTT ms.

In the case that a UE sends msg3 as response to a RAR message during CFRA, the UE enters Active time when an offset time has elapsed.

When UL and DL are aligned in the gNB and the UE sends msg3 as response to a RAR message during CFRA, the UE can be configured to start monitoring the PDCCH in the downlink symbol that has the same symbol number, slot number and system frame number as the first uplink symbol after the end of the PUSCH transmission.

Thus, if gNB intends to wait for the msg3 as an ACK that the UE have received the RAR before it schedules the UE, then gNB can configure the UE to save energy after sending msg3 by not monitoring PDCCH until the corresponding DL slot.

If monitoring the PDCCH after msg3 transmission in CFRA is not as described in Proposal 27 then the UE need a timing offset that risk monitoring late or early or it will waste energy by always monitoring.

# Conclusion

Here we first present a combined list of the observations and proposals, and after it there is a list of only the proposals.

Observation 1	Configured grants can be configured with or without HARQ retransmissions in Rel-15.

Observation 2	Configured grants can be configured with or without reusing HARQ process IDs before a HARQ RTT has elapsed in Rel-15.

Proposal 1	The ConfiguredGrantConfiguration shall allow for up to 32 in nrofHARQ-Processes, and up to 31 in harq-ProcID-Offset and harq-ProcID-Offset2.

Proposal 2	Extend configuredGrantTimer to cover the RTT in GEO when periodicity is short.

Observation 3	DL SPS can use the already agreed method of enabling/disabling HARQ feedback in NTNs.

Proposal 3	HARQ feedback shall always be sent for SPS deactivation, no specification impact.

Proposal 4	The SPS-Config shall allow up to 32 for number of HARQ processes, and up to 31 in harq-ProcID-Offset.

Observation 4	The affected behaviour of “UL HARQ retransmission state” functionality is not directly related to HARQ retransmissions.

Observation 5	The “UL HARQ retransmission state” is not directly related to a HARQ state or a DRX state, but instead related to a DRX behaviour (extend drx-HARQ-RTT-TimerUL or do not start the drx-HARQ-RTT-TimerUL), therefore using “retransmission” and “state” to describe it is misleading.

Proposal 5	The new signalling, introduced per HARQ process ID, is given a new name. Select between “UL HARQ-DRX mode A/B” and “UL DRX mode A/B” instead of the misleading “UL HARQ retransmission state A/B”.

Proposal 6	The new “UL HARQ-DRX mode A/B”/”UL DRX mode” is controlled by a RRC parameter named “uplinkHARQ-DRX-Mode” or “uplinkDRX-Mode” and can take two values for each HARQ process ID: ModeA or ModeB.

Proposal 7	RAN2 to discuss whether the new RRC parameter named “uplinkHARQ-DRX-Mode” or “uplinkDRX-Mode” shall be placed in MAC-CellGroupConfig or in PUSCH-ServingCellConfig.

Proposal 8	Legacy behaviour, instead of the new DRX behaviour and new LCP restrictions, is enabled by not configuring uplinkHARQ-DRX-Mode/uplinkDRX-Mode.

Proposal 9	The uplinkHARQ-DRX-Mode/uplinkDRX-Mode is configured for all HARQ processes at the same time, for example in a bitmap.

Proposal 10	The new LCP restriction per LCH is controlled by a RRC parameter named “allowedHARQ-DRX” or “allowedDRX”, in LogicalChannelConfig, and can take two values: modeA or modeB.

Proposal 11	The field description of allowedHARQ-DRX/allowedDRX is “If present, UL MAC SDUs from this logical channel can only be mapped to HARQ processes not configured uplinkHARQ-DRX-Mode/uplinkDRX-Mode or to HARQ processes configured with uplinkHARQ-DRX-Mode/uplinkDRX-Mode indicating a value equal to the value configured by this field.

Observation 6	If uplinkHARQ-DRX-Mode/uplinkDRX-Mode is not configured for a HARQ process, the MAC SDUs from the LCH can use any HP ID, regardless of if allowedHARQ-DRX-Mode/allowedDRX-Mode is configured or not.

Observation 7	If uplinkHARQ-DRX-Mode is configured for a HP ID, Proposal 10 allows three configurations per LCH: 1) the LCH can only use ModeA HP IDs (allowedHARQ-DRX-Mode/uplinkHARQ-DRX-Mode = ModeA), 2) the LCH can only use ModeB HP IDs (allowedHARQ-DRX-Mode/uplinkHARQ-DRX-Mode = ModeB), and 3) the LCH can use ModeA and ModeB (allowedHARQ-DRX-Mode is not configured).

Proposal 12	The priority of the new MAC CE in the prio list in MAC spec section 5.4.3.1.3 shall be lower than “C-RNTI MAC CE or data from UL-CCCH” but higher than “data from any Logical Channel, except data from UL-CCCH”.

Proposal 13	The priority of the new MAC CE in the prio list in the MAC spec section 5.4.3.1.3 shall be between “Single Entry PHR MAC CE or Multiple Entry PHR MAC CE” and “MAC CE for the number of Desired Guard Symbols”.

Proposal 14	The start of drx-RetransmissionTimerDL is not changed in NTNs.

Proposal 15	drx-RetransmissionTimerUL timer length is not extended in NTN.

Proposal 16	The start of drx-RetransmissionTimerUL is not changed in NTNs.

Observation 8	Configured grants with opportunistic blind retransmissions is less resource efficient than dynamic grants with blind retransmissions, because when a grant is skipped due to no data then a retransmission grant with CS-RNTI will be ignored, while a dynamic retransmission grant with C-RNTI will send new data (if new data have arrived).

Proposal 17	Introduce one help timer HARQ_RTT_TIMER_DL for each DL HARQ process. If the HARQ process is configured to be extended, set HARQ_RTT_TIMER_DL to drx-HARQ-RTT-TimerDL plus UE-gNB RTT, else set HARQ_RTT_TIMER_DL to drx-HARQ-RTT-TimerDL.

Proposal 18	Introduce one help timer HARQ_RTT_TIMER_UL for each UL HARQ process. If the HARQ process is configured to be extended, set HARQ_RTT_TIMER_UL to drx-HARQ-RTT-TimerUL plus UE-gNB RTT, else set HARQ_RTT_TIMER_UL to drx-HARQ-RTT-TimerUL.

Proposal 19	Introduce a definition of the UE-gNB RTT in section 3.1. For example: “UE-gNB RTT: The sum of Kmac and TTA, where Kmac is the broadcasted parameter FIELD_NAME and TTA is defined in TS 38.2xx clause Y.Y”

Proposal 20	Repetition transmission based HARQ retransmission is always allowed and is explicitly indicated via DCI, or semi-statically with RRC signalling (for dynamic scheduling, CG and SPS) (as in legacy). Note, this revises the agreement from RAN2#114e.

Proposal 21	Add an in-flight BSR to report the amount of RLC AM data that is waiting for an RLC status report.

Proposal 22	Add an indication from the gNB to the UE that it shall send a BSR or an in-flight BSR or both.

Proposal 23	The values added to sr-ProhibitTimer in NTN shall include values lower than the round-trip time.

Observation 9	With current DRX procedure, the UE will monitor the PDCCH in vain for RTT ms after sending an SR.

Proposal 24	For DRX in NTN, in the case that a UE sends an SR, the UE enters Active time to monitor for a response after an offset time has elapsed.

Proposal 25	When UL and DL are aligned in the gNB, the UE shall start monitoring the PDCCH in the downlink symbol that has the same symbol number, slot number and system frame number as the first uplink symbol after the end of the Scheduling Request transmission.

Observation 10	If monitoring the PDCCH after Scheduling Request transmission is not as described in Proposal 25 then the UE need a timing offset that risk monitoring late or early.

Observation 11	For CFRA, the network typically waits to receive msg3 granted in the RAR, as an acknowledgment that the UE has received the RAR, before it schedules the UE.

Observation 12	With current DRX procedure, after receiving the RAR message for CFRA, the UE will monitor the PDCCH in vain for at least RTT ms if the network does not schedule the UE in the meantime.

Proposal 26	In the case that a UE sends msg3 as response to a RAR message during CFRA, the UE enters Active time when an offset time has elapsed.

Proposal 27	When UL and DL are aligned in the gNB and the UE sends msg3 as response to a RAR message during CFRA, the UE can be configured to start monitoring the PDCCH in the downlink symbol that has the same symbol number, slot number and system frame number as the first uplink symbol after the end of the PUSCH transmission.

Observation 13	If monitoring the PDCCH after msg3 transmission in CFRA is not as described in Proposal 27 then the UE need a timing offset that risk monitoring late or early or it will waste energy by always monitoring.

Here is a list of the proposals made:

Proposal 1	The ConfiguredGrantConfiguration shall allow for up to 32 in nrofHARQ-Processes, and up to 31 in harq-ProcID-Offset and harq-ProcID-Offset2.

Proposal 2	Extend configuredGrantTimer to cover the RTT in GEO when periodicity is short.

Proposal 3	HARQ feedback shall always be sent for SPS deactivation, no specification impact.

Proposal 4	The SPS-Config shall allow up to 32 for number of HARQ processes, and up to 31 in harq-ProcID-Offset.

Proposal 5	The new signalling, introduced per HARQ process ID, is given a new name. Select between “UL HARQ-DRX mode A/B” and “UL DRX mode A/B” instead of the misleading “UL HARQ retransmission state A/B”.

Proposal 6	The new “UL HARQ-DRX mode A/B”/”UL DRX mode” is controlled by a RRC parameter named “uplinkHARQ-DRX-Mode” or “uplinkDRX-Mode” and can take two values for each HARQ process ID: ModeA or ModeB.

Proposal 7	RAN2 to discuss whether the new RRC parameter named “uplinkHARQ-DRX-Mode” or “uplinkDRX-Mode” shall be placed in MAC-CellGroupConfig or in PUSCH-ServingCellConfig.

Proposal 8	Legacy behaviour, instead of the new DRX behaviour and new LCP restrictions, is enabled by not configuring uplinkHARQ-DRX-Mode/uplinkDRX-Mode.

Proposal 9	The uplinkHARQ-DRX-Mode/uplinkDRX-Mode is configured for all HARQ processes at the same time, for example in a bitmap.

Proposal 10	The new LCP restriction per LCH is controlled by a RRC parameter named “allowedHARQ-DRX” or “allowedDRX”, in LogicalChannelConfig, and can take two values: modeA or modeB.

Proposal 11	The field description of allowedHARQ-DRX/allowedDRX is “If present, UL MAC SDUs from this logical channel can only be mapped to HARQ processes not configured uplinkHARQ-DRX-Mode/uplinkDRX-Mode or to HARQ processes configured with uplinkHARQ-DRX-Mode/uplinkDRX-Mode indicating a value equal to the value configured by this field.

Proposal 12	The priority of the new MAC CE in the prio list in MAC spec section 5.4.3.1.3 shall be lower than “C-RNTI MAC CE or data from UL-CCCH” but higher than “data from any Logical Channel, except data from UL-CCCH”.

Proposal 13	The priority of the new MAC CE in the prio list in the MAC spec section 5.4.3.1.3 shall be between “Single Entry PHR MAC CE or Multiple Entry PHR MAC CE” and “MAC CE for the number of Desired Guard Symbols”.

Proposal 14	The start of drx-RetransmissionTimerDL is not changed in NTNs.

Proposal 15	drx-RetransmissionTimerUL timer length is not extended in NTN.

Proposal 16	The start of drx-RetransmissionTimerUL is not changed in NTNs.

Proposal 17	Introduce one help timer HARQ_RTT_TIMER_DL for each DL HARQ process. If the HARQ process is configured to be extended, set HARQ_RTT_TIMER_DL to drx-HARQ-RTT-TimerDL plus UE-gNB RTT, else set HARQ_RTT_TIMER_DL to drx-HARQ-RTT-TimerDL.

Proposal 18	Introduce one help timer HARQ_RTT_TIMER_UL for each UL HARQ process. If the HARQ process is configured to be extended, set HARQ_RTT_TIMER_UL to drx-HARQ-RTT-TimerUL plus UE-gNB RTT, else set HARQ_RTT_TIMER_UL to drx-HARQ-RTT-TimerUL.

Proposal 19	Introduce a definition of the UE-gNB RTT in section 3.1. For example: “UE-gNB RTT: The sum of Kmac and TTA, where Kmac is the broadcasted parameter FIELD_NAME and TTA is defined in TS 38.2xx clause Y.Y”

Proposal 20	Repetition transmission based HARQ retransmission is always allowed and is explicitly indicated via DCI, or semi-statically with RRC signalling (for dynamic scheduling, CG and SPS) (as in legacy). Note, this revises the agreement from RAN2#114e.

Proposal 21	Add an in-flight BSR to report the amount of RLC AM data that is waiting for an RLC status report.

Proposal 22	Add an indication from the gNB to the UE that it shall send a BSR or an in-flight BSR or both.

Proposal 23	The values added to sr-ProhibitTimer in NTN shall include values lower than the round-trip time.

Proposal 24	For DRX in NTN, in the case that a UE sends an SR, the UE enters Active time to monitor for a response after an offset time has elapsed.

Proposal 25	When UL and DL are aligned in the gNB, the UE shall start monitoring the PDCCH in the downlink symbol that has the same symbol number, slot number and system frame number as the first uplink symbol after the end of the Scheduling Request transmission.

Proposal 26	In the case that a UE sends msg3 as response to a RAR message during CFRA, the UE enters Active time when an offset time has elapsed.

Proposal 27	When UL and DL are aligned in the gNB and the UE sends msg3 as response to a RAR message during CFRA, the UE can be configured to start monitoring the PDCCH in the downlink symbol that has the same symbol number, slot number and system frame number as the first uplink symbol after the end of the PUSCH transmission.

# References

TR 38.821 “Study on solutions for NR to support non-terrestrial networks”

RP-211784, Solutions for NR to support non-terrestrial networks (NTN)”, Thales, RAN#93, e-Meeting, September 2021