3GPP TSG-RAN2#116-e	R2-2110986

Online, 1st Nov - 12th Nov 2021

Agenda item:		8.6.5 (NR_SmallData_INACTIVE-Core)

Source:	LG Electronics Inc.

Title: 	Remaining CG-SDT issues in SDT

Document for:	Discussion and Decision

1.	Introduction

In the last meeting, RAN2 discussed open issues for CG-SDT in [1], and made following agreements.

Agreements 1. If none of the SSBs’ RSRP is above the RSRP threshold of CG-SDT criteria in the type selection phase, UE should select RA-SDT if RA-SDT criteria is met 2. MAC PDU rebuilding is not required (unless we find a case that is needed) 3. During subsequent CG transmission phase (i.e. after the UE has received response from NW) UE can initiate at least legacy RACH procedure (e.g. trigger due to no UL resources). No MAC PDU rebuilding is required. FFS if the RA-SDT RA resources can be used for subsequent data. a. At least the following conditions are agreed: (1) no qualified SSB when the evaluation is performed; (2) when TA is invalid; (3) when SR is triggered due to lack of UL resource 4. UE should release CG-SDT resource (if stored) when UE initiates RRC resume procedure from another cell which is different from the cell in which the RRCRelease is received. 5. The C-RNTI previously configured in RRC_CONNECTED state is used for UE to monitor PDCCH in CG-SDT. 6. CS-RNTI based dynamic retransmission mechanism can be reused for CG-SDT. FFS whether CS-RNTI is the same one as the one previously configured in RRC_CONNECTED or a new CS-RNTI one is provided to the UE 7. During the subsequent new CG transmission phase, for the purpose of CG resource selection, UE re-evaluates the SSB for subsequent CG transmission. FFS what happens if no SSBs are valid or if no sample is available 8. From RAN2 perspective, at least the following parameters should be included in the CG-SDT configuration. FFS whether these parameters are common for multiple CG-SDT configurations or per CG-SDT configuration. - The new TA timer in RRC_INACTIVE; - The RSRP change threshold for TA validation mechanism in SDT (details dependent on RAN1); - The SSB RSRP threshold for beam selection (i.e. UE selects the beam and associated CG resource for data transmission).

Though many issues were resolved at RAN2#115e, some FFSs (highlighted in yellow) are still left for further discussion. In this paper, we provide our view on FFS.

2.	Discussion

First, there was FFS whether the RA-SDT resources can be used for subsequent data or not during the subsequent phase of CG-SDT. Our opinion on this issue are presented in [1]. In short, the switching from CG-SDT to RA-SDT complicates the UE behaviour and we think there is no benefit.

Proposal 1: Switching from CG-SDT to RA-SDT is not supported.

Second, there was FFS about CS-RNTI and there were two options.

- Option 1. CS-RNTI is the same one as the one previously configured in RRC_CONNECTED

- Option 2. New CS-RNTI one is provided to the UE.

We think this issue can be compromised without the specification change.

[38.331 V16.6.0] PhysicalCellGroupConfig ::= SEQUENCE { harq-ACK-SpatialBundlingPUCCH ENUMERATED {true} OPTIONAL, -- Need S harq-ACK-SpatialBundlingPUSCH ENUMERATED {true} OPTIONAL, -- Need S p-NR-FR1 P-Max OPTIONAL, -- Need R pdsch-HARQ-ACK-Codebook ENUMERATED {semiStatic, dynamic}, tpc-SRS-RNTI RNTI-Value OPTIONAL, -- Need R tpc-PUCCH-RNTI RNTI-Value OPTIONAL, -- Need R tpc-PUSCH-RNTI RNTI-Value OPTIONAL, -- Need R sp-CSI-RNTI RNTI-Value OPTIONAL, -- Need R cs-RNTI SetupRelease { RNTI-Value } OPTIONAL, -- Need M ...,

Currently, in the MAC specification, CS-RNTI is not released when the UE receives RRCRelease for RRC_INACTIVE, and in the RRC specification, the need code of CS-RNTI is set to M. In other words, if the network does not provide a new CS-RNTI with the SDT configuration via RRCRelease, the UE maintains the CS-RNTI previously configured in RRC_CONNECTED and can use the CS-RNTI in SDT procedure. Therefore, we propose whether to provide new CS-RNTI or not is up to network implementation.

Proposal 2. Whether to provide new CS-RNTI or not is up to network implementation.

In RAN2#115-e, RAN2 agreed that UE re-evaluates the SSB for subsequent phase of CG-SDT. However, there are diverged view on the cases where no SSBs are valid or where no sample is available as follows.

- Option 1: Trigger the legacy RACH due to no UL resources (i.e. no CG based UL grant is provided to the HARQ process and SR/RACH will be triggered due to lack of UL resources)

- Option 2: Use any SSB (similar to RACH retransmission case)

- Option 3: Others (please explain)

We think the SDT failure handling procedure should be triggered when no SSBs are valid in the subsequent phase of CG-SDT. The SDT failure handling procedure may trigger legacy RA procedure to transmit the pending UL data.

For Option 1, we think the legacy RA procedure is not a part of SDT procedure, and thus the ongoing SDT procedure should be stopped. In addition, we think that legacy RA does not always ensure better results. For Option 2, we are not sure whether CG-SDT can succeed by using any SSB that does not meet CG-SDT threshold, i.e., TA validity and RSRP change threshold.

Therefore, we propose to trigger the SDT failure handling procedure when no SSBs are valid during the subsequent phase of CG-SDT.

Proposal 3. The UE triggers the SDT failure handling procedure when no SSBs are valid during the subsequent phase of CG-SDT.

Last, RAN2 agreed to configure the following parameters in the CG-SDT configuration, and it is remained as FFS whether these parameters are common for multiple CG-SDT configurations or per CG-SDT configuration.

- The new TA timer in RRC_INACTIVE

- The RSRP change threshold for TA validation mechanism in SDT

- The SSB RSRP threshold for beam selection

In the current specification, TAT is configured per TAG configuration. Since SDT operates on only PCell, there is no other TAG. Thus, we think the new TAT would be configured in common for multiple CG-SDT configuration. In addition, we think that there is no reason to configure these parameter in different way, i.e., single method seems sufficient. Thus, we propose to configure these parameters in common for multiple CG-SDT configuration.

Proposal 4. The following parameters are configured in common for multiple CG-SDT configuration

- The new TA timer in RRC_INACTIVE

- The RSRP change threshold for TA validation mechanism in SDT

- The SSB RSRP threshold for beam selection

3.	Conclusion

In this document, we discuss the remained FFS for CG-SDT, and made proposals as following.

Proposal 1: Switching from CG-SDT to RA-SDT is not supported.

Proposal 2. Whether to provide new CS-RNTI or not is up to network implementation.

Proposal 3. The UE triggers the SDT failure handling procedure when no SSBs are valid during the subsequent phase of CG-SDT.

Proposal 4. The following parameters are configured in common for multiple CG-SDT configuration

- The new TA timer in RRC_INACTIVE

- The RSRP change threshold for TA validation mechanism in SDT

- The SSB RSRP threshold for beam selection

4.	Reference

- R2-2110983	Switching cases of SDT and non-SDT		LG electronics Inc.