3GPP TSG-RAN WG2 Meeting #116 electronic	R2-2111xxxx

Electronic Meeting, Nov 1 – 12, 2021

Agenda item:	6.1.1

Source:	CATT

Title:	Report of ‎[AT116-e][004][NR16] CPUP split reply LS (CATT)‎

Document for:	Discussion and Decision

# Introduction

This document is for reporting the following offline discussion

- [AT116-e][004][NR16] CPUP split reply LS (CATT)

Scope: Determine agreeable parts in a first phase, if agreeable then agree on reply LS out Treat R2-2109344, R2-2111068, R2-2111069.

Intended outcome: Report, Approved LS out if applicable

Deadline: Friday W1 (Nov 5)

The participants are invited to leave their contact in the table below.

Contact list

| Company | Delegate name / Email |
| --- | --- |
| CATT | Erlin Zeng / erlin.zeng@catt.cn |
| Vodafone | chris.pudney@vodafone.com |
| Qualcomm Incorporated | mkitazoe@qti.qualcomm.com |
| LG Electronics | seungjune.yi@lge.com |
| Apple | fangli_xu@apple.com |
| vivo | kimba@vivo.com |

# Discussion

In the incoming LS from RAN3 [1], the following action is required

ACTION: RAN3 respectfully asks RAN2 whether it is possible to use the default DRB to deliver downlink packets for a QoS flow without association to any DRB yet.

In the discussion paper [2] the issue was discussed, and the following proposal was made [2]

Proposal 1 	RAN2 discuss and confirm that gNB implementation allows delivery of DL packets to UE (via default DRB or via dedicated DRB) before the new QFI to DRB mapping is configured for the QoS flow. RAN2 reply to RAN3’s LS accordingly.

In order to form a common view to reply to RAN3’s question, companies are encouraged to answer the following question.

Q1: Do you agree with the proposal 1 from [2], as shown above?

| Company name | Agree or not | Comment if any |
| --- | --- | --- |
| CATT | Yes |  |
| Vodafone | Yes | Actually, this is not “implementation allowed” BUT “mandatory for the gNB to send the DL packet on some DRB” (as the scenario in annex A.3 of TS 38.300 is that this QoS flow is known to the gNB and, in step 0 of annex A.3, the gNB seems to have informed the core network of the successful establishment of the flows that were requested by the CN!). Discarding the packet in the gNB would mean that the data volume count in the core network is inaccurate. |
| Qualcomm Incorporated | Yes |  |
| LGE | Comments | There is no default DRB in DL. The gNB should choose one DRB and send the packet through the DRB. The gNB should set the RDI bit to indicate that the UE should update the QoS flow to DRB mapping rule |
| Apple | Yes | We confirm the gNB implementation can handle this case without any spec change. And the gNB implementation shall not rely on the DL SDAP header configuration. It is because the DL SDAP header is optionally supported/configured, and the main purpose is to support reflective mechanisms. |
| vivo | Yes |  |

Then a draft LS was also provided in [3], which directly follows the proposal 1 in [2]. Rapporteur’s understanding is that the reply LS should be straightforward if there is aligned view on proposal 1. But one more question is added to this discussion just in case companies have additional comments on the reply LS to RAN3. Note that if you already made comments to Proposal 1 you don’t need to repeat it here, but please provide additional comments if not already made to the previous question.

Q2: Do you have any comments on the draft reply in [3]?

| Company name | Agree or not | Comment if any |
| --- | --- | --- |
| CATT | Yes |  |
| Vodafone | yes | LS should make it clear that is mandatory for the gNB to do this (in the case that the gNB told the CN that it had established this flow) |
| Qualcomm Incorporated | Yes |  |
| LGE | Comments | First, we agree with Vodafone that the gNB must do this. Secondly, there is no default DRB in the DL. Thus, we propose to change the conclusion as follows. gNB should deliver DL packets to UE (via dedicated DRB) even if the new QFI to DRB mapping rule is not configured for the QoS flow. |
| Apple | Yes | We can indicate in the LS that no any spec change is needed. |
| vivo | Yes | Agree with Apple |

# Conclusions

TBD

# Reference

[1] R2-2109344‎	LS on downlink unmapped QoS flows (R3-214453; contact: CATT)‎	RAN3‎	LS in	Rel-16‎	NR_CPUP_Split	To:RAN2‎

[2] R2-2111068‎	Discussion on downlink unmapped QoS flows	CATT, Huawei, HiSilicon	discussion	Rel-16‎	NR_CPUP_Split

[3] R2-2111069‎	Draft Reply LS on downlink unmapped QoS flows	CATT	LS out	Rel-16‎	NR_CPUP_Split	To:RAN3‎