3GPP TSG-RAN WG2 Meeting #116-e						R2-211xxxx

Electronic Meeting, Nov 1 – 12, 2021

Title:	[draft] Reply LS on Pre-configured MG

Response to:	R4-2115438 / R2-2109367

Release:	Release 17

Work Item:	NR_MG_enh-Core

Source:	Intel (to be RAN2)

To:	RAN4

Cc:

Contact Person:

#### Name:	Candy Yiu

E-mail Address:	candy.yiu@intel.com

1. Overall Description:

RAN2 thanks RAN4 for the LS on pre-configured MG. Following discussion RAN2 has made the following agreements:

- At least case 5 is supported for pre-configured gap. FFS for case 4.

- Case 4: NW signals the pre-configured gap (A+B in Q1) via RRC, then UE follows BWP status (B) to activates/deactivates gap upon BWP switching

- Case 5: NW signals the pre-configured gap (A in Q1) via RRC, then UE determines whether the pre-configured gap should be activated or not upon BWP switching.  For example, if it is overlapped with SSB, then pre-configured gap is deactivated, otherwise it is activated.

- RAN2 hasn't seen any usefulness of MAC-CE based activation/deactivation and prefers to not support it.

Reference to A and B are as follow:

-	A) measurement gap configuration parameters such as MGRP, MGL etc

-	B) MG status (active/de-active) per BWP

In addition, RAN2 would like to ask RAN4 for some clarifications on supporting of pre-configured MG.

Questions to RAN4:

- Q1: Can FR1 gap and FR2 gap be configured simultaneously for pre-configured gap? Is the following operation supported?

  - pre-configured FR1 gap + pre-configured FR2 gap

- Q2: Can legacy gap and pre-configured gap be configured simultaneously? For example, without considering concurrent gap, are the following operations supported?

  - One legacy per-UE gap + One pre-configured per-UE gap

  - One legacy per-UE gap + pre-configured FR1 gap and/or pre-configured FR2 gap

  - legacy FR1 gap + pre-configured FR2 gap

  - legacy FR2 gap + pre-configured FR1 gap

2. Actions:

To RAN2:

RAN2 respectfully asks RAN4 to answer the above questions.

3. Date of Next TSG-RAN2 Meetings:

TSG-RAN2 Meeting #116bis-e				17 – 25 January, 2022, Electronic Meeting