3GPP TSG-RAN WG2 Meeting #116-e                                                         R2-21xxxxx

November 1 –November 12, 2021

Title:	[Draft] LS on RAN visible QoE

Response to:

- Release:	Rel-17

- Work Item:	NR_QoE-Core

Source:	[RAN2]

To:	RAN3

CC:

Contact Person:

#### Name:	Jianhua Liu

Tel. Number:

E-mail Address:	liujianhua@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

RAN2 discussed RAN2 impact and work on RAN visible QoE and have the following initial agreements.

RAN2 will discuss RAN visible QoE configuration and reporting procedure, and leave definition of QoE metrics and what should be included in RVQOE configuration and reporting to RAN3 discuss.

For RAN3 agreements, RAN2 confirms the following procedures are feasible from RAN2 point of view.

  - It is feasible to configure RVQOE using explicit RRC IEs

  - Multiple simultaneous QoE measurements can be supported for RVQOE.  Each RVQOE measurement configuration is identified by the MeasConfigAppLayerId (or change to another generic term) corresponding to the regular QoE configuration.

  - UE RRC layer forwards the received RVQOE configuration to the concerned application layer.

  - RAN configures the required RVQOE metrics in the RVQOE configuration for UE to report.

In addition, RAN2 understand whether it is feasible to send RVQOE report to RAN using RRC explicit IEs depends on the defined metrics. RAN2 will discuss it based on defined reported metrics.

RAN2 discusses which SRB should be used to transmit RAN visible QoE report. RAN2 makes assumption that  SRB4 is applied considering no clear motivation to put higher priority to RAN visible QoE than legacy QoE from RAN2 point of view, but this can be revisited if clear requirement is identified.

RAN2 discusses RVQOE reporting pause and resume, RAN2 makes assumption that both of the RAN-visible QoE report and legacy QoE report container associated with that MeasConfigAppLayerId will be paused or resumed. But this can be revisited if clear motivation to put higher priority to RAN visible QoE than legacy QoE.

2. Actions:

To: SA4

ACTION:   RAN2 kindly asks RAN3 to consider RAN2 feedback.

3. Date of Next RAN2 Meetings:

RAN2#116-bis-e	17 January– 25 January 2022	Online

RAN2#117-e	21 February- 3 March 2022	Online