3GPP TSG-RAN WG2 Meeting #116 electronic	          R2-211xxx

Electronic meeting, 1st-12th November 2021

Agenda Item:	8.4.3

Source: 	Huawei, HiSilicon

Title:	Summary of [AT116-e][047][eIAB] Routing and re-routing continued

Document for:	Discussion and Decision

# Introduction

This paper aims at capturing the summary of email discussion.

- [AT116-e][047][eIAB] Routing and re-routing continued (Huawei)

Scope: Attempt offline agreement of remaining proposals in R2-2111266.

Intended outcome: Report

Deadline: Tuesday W2

RAN2 related agreements:

- For inter-donor-DU re-routing, support the “previous routing ID to new routing ID” BAP header rewriting. - For intra-CU cases, Support inter-donor-DU re-routing at least in the scenarios of NR-DC among donor-DUs, inter-donor-DU recovery and inter-donor-DU migration. - Support inter-CU re-routing, i.e. IAB-node re-routes the data to its original donor-CU via the alternative BAP path over the topology in target CU. - As baseline, support the 1:1 and N:1 mapping from “previous routing ID” to “new routing ID” for BAP header rewriting at the boundary node, in inter-CU routing. - As baseline, support the 1:1 and N:1 mapping from “ingress BH link + ingress BH RLC ID” to “egress BH link + egress BH RLC ID” for bearer mapping at the boundary node, in inter-CU routing.

RAN3 related agreements:

- One common inter-donor topology transport mechanism should be defined for all scenarios where traffic between a donor and an IAB DU traverses the network under another donor; FFS whether it is possible to achieve a common signaling design for all scenarios - RAN3 prefers that the boundary node processes access traffic in the same manner as the non-boundary access IAB-node. - RAN3 prefers that the boundary node performs BAP header rewriting only for traffic routed on BAP layer from a BH link in one topology to a BH link in the adjacent topology, for both UL and DL traffic. - FFS: In addition to BAP header rewriting, performs routing and bearer mapping in the same manner as the non-boundary intermediate IAB-node. - RAN3 assumes that the boundary node has only one BAP address in each topology. - RAN3 assumes that for each topology, the boundary node’s BAP address for that topology is only used to identify packets that have to be passed to upper layers. - For DL traffic, the configurations of BAP routing entry and BAP-routing-ID mapping at the boundary node need to indicate the ingress topology they refer to. For UL traffic, they need to indicate the egress topology they refer to. The indications may be implicit.

New RAN2 agreement

Inter Topology Routing - Go with B, including the following: - - If BAP address matches, deliver to upper layer; - Else: - - If routing ID matches rewriting table, perform the header rewriting; - - perform routing and mapping to BH RLC CH. - For downstream, the boundary node is able to identify/differentiate the traffic routed from inter-topology vs. the traffic routed from intra-topology, based on the ingress link. - For downstream at the boundary node, for any received data from inter-topology identified by the ingress link: - The data is delivered to upper layer, if the BAP address in the header is same as the boundary node BAP address configured in the topology of the ingress link (of this packet); otherwise, the data is determined as to be header rewritten (assumes support only of topology where decedent nodes belong to same topology). - (This requires that traffic not terminated at the boundary node should not use the BAP address in header same as the boundary node BAP address configured in the topology of the ingress link.) - Perform the header rewriting based on the configured rewriting table, and then perform routing and mapping to BH RLC CH. - For upstream at the boundary node, for any received data from lower layer: - We may keep the ingress BAP text of R16 (that is intended for donor DU but general in Stage-3), i.e. if the BAP address in header match the boundary node BAP address configured in the topology of the ingress link, deliver to upper layer. - The data is determined as to be header rewritten and perform the header rewriting accordingly, if routing ID in header matches any “previous routing ID” in the rewriting table; and then perform routing and mapping to BH RLC CH. - For Upstream, The pre-condition/criteria of “BAP header rewriting for re-routing” is that there is no available next hop found based on BAP routing ID and based on BAP address in the routing table (e.g. due to BH RLF, congestion or type2 indication, etc.), as in R16.

# Discussion

- 2.1 Issues on re-routing

Inter-CU topology redundancy        Inter-CU partial migration

Proposal 2: [Easy] [13 vs. 2] Once the “BAP header rewriting based re-routing” is triggered, BAP routes the data to the available egress link, if there is an matched entry in the Header rewriting table for re-routing. FFS egress link selection is performed before or after header rewriting (can be discussed in running CR).

Rapporteur wants to clarify one concern in the post email discussion. Based on the R2 agreement “For inter-donor-DU re-routing, support the “previous routing ID to new routing ID” BAP header rewriting”, the mapping/table for header rewriting should be the consequence.

Question 1a: Do you have concern on the above proposal or wording suggestion?

| Companies | Yes or No? | Comments |
| --- | --- | --- |
| ZTE | Yes | For the header rewriting table for re-routing, it is suggested to clarify whether it is a new table with new and old routing ID or the legacy routing table with BAP routing ID and next hop BAP address. If we design a new re-writing table for re-routing, it may cause additional specification efforts and a lot of signalling overhead. Since the CU is not aware which BAProuting ID may suffer the RLF/congestion, it has to configure the re-writing entry for all the possible UL BAP routing IDs. On the other hand, we think the legacy routing table can be used for the header rewriting purpose for re-routing. For example, when no routing entry with matched BAP routing ID and matched BAP address but a routing entry which indicate an available next hop egress link can be found, the data packet can be re-routed via this available next hop egress link and the corresponding BAP routing ID can be used to rewrite the BAP header. Actually, in Rel-16, the re-routing path is selected based on routing table instead of a new re-routing table, it is suggested to follow the Rel-16 principle and change the proposal as follows: Once the “BAP header rewriting based re-routing” is triggered, BAP routes the data to the available egress link, if there is an matched entry whose egress link corresponding to the Next Hop BAP Address is available in the routing tableHeader rewriting table for re-routing. FFS egress link selection is performed before or after header rewriting (can be discussed in running CR) |
| LG | Yes | Same understanding as ZTE. We also think that routing table can be used in case of “BAP header rewriting based re-routing” and the updated wording by ZTE is also ok to us. |
| vivo | No | The wording is ok for us. |
| Kyocera | No | We agree with Proposal 2. |
| CATT | No | We don’t think ZTE and LG’s suggested operation is correct. If there is no rewriting table, how to operate the BAP header rewriting? On the other word, the BAP layer rewriting base on what. |
| Huawei, HiSilicon | No | The agreement “For inter-donor-DU re-routing, support the “previous routing ID to new routing ID” BAP header rewriting” was clear for a table. For ZTE’s interpretation, the agreement would be “support the BAP header rewriting”. |
| Apple | No | Since it was already agreed as companies have mentioned, we can accept. |
| Intel | No concerns | As discussed during online meeting, for upstream traffic, the pre-condition/criteria of “BAP header rewriting for re-routing” is that there is no available next hop found based on BAP routing ID and based on BAP address in the routing table. For upstream traffic, the actual IAB-donor DU1’s BAP address is added in the BAP header. When the above condition is triggered, in the BH Routing Configuration (configured by IAB-donor CU1), there’s no available egress link corresponding to the original destination “IAB-donor DU1’s BAP address” (in BAP header). To allow the boundary IAB-node re-route traffic locally, BAP header rewriting is performed if an alternative “BAP routing ID” is mapped to the original destination address “IAB-donor DU1” and path ID. The alternative BAP routing ID can be: 1) for intra-donor CU inter-donor DU local rerouting: IAB-donor DU2’s real BAP address + new path ID (IAB-donor DU2 only connects to IAB-donor CU1) 2) for inter-donor CU local rerouting (donor DU2 of IAB-donor CU2 can reroute back to IAB-donor CU1): IAB-donor DU2’s pseudo BAP address + pseudo path ID. If there’s no routing ID mapped to original routing ID, there’s no egress link towards IAB-donor DU2 is established, either for intra-donor CU or inter-donor CU scenario. After BAP header rewriting, new egress link can be selected according to BH Routing Configuration, which we also try to explain inter-donor CU scenario in detail in Q1b. |
| Fujitsu | No | We agree with Proposal 2. |
| Lenovo | Yes | Same view with ZTE that not to introduce a new Header rewriting table for re-routing, and the legacy routing table can work well. When perform rerouting, once an available egress link with different BAP address is selected, then BAP entity performs BAP header rewriting with the BAP routing ID of the selected entry. And agree with the wording by ZTE. |
| Futurewei | No | No significant concerns with the wording |
| Nokia | Yes | Two concerns: 1) we should not preclude option where header rewriting is configured with the routing configuration 2) routing of earlier re-routed BAP PDUs is not covered (if the header rewriting of those BAP address based re-routed packets should be based on whole Routing ID, the configuration becomes very complex) For the first one, we support ZTE/LG/Lenovo. For the second point a solution covering also the earlier re-routed packets is presented in Ref. [10] where in the routing table for each Routing ID allowed Alternative BAP address(es) are listed and if Rel16 routing does not succeed, then routing based on an Alternative BAP address is performed. |
| Ericsson | No |  |

Question 1b: In the above proposal, do you prefer egress link selection is performed before or after header rewriting?

| Companies | egress link selection first, or header rewriting first? | Comments |
| --- | --- | --- |
| ZTE | Egress link selection first | For re-routing BAP routing ID selection, the IAB node need to first ensure the corresponding egress link is available. Once the available egress link is selected, the IAB node may perform the header rewriting and then route the packet via this available egress link. |
| LG | see comments | “Egress link selection first” may be misled, in our understanding, egress link status check first is more correct wording. If header rewriting is performed first, but the corresponding egress link is not available, we wonder whether IAB node performs one more header rewriting which is transferred to available egress link. This unnecessary rewriting step should be avoided. Thus, we think that availability of egress link of each entry for header rewriting should be check first and then select the entry for header rewriting among entries of corresponding egress link available. |
| vivo | Egress selection first | In Rel-16, an IAB-node may perform local re-routing in case of BH RLF. The IAB-node firstly checks the BH routing table to see if there is any alternative path (with the same BAP destination), and this alternative path should also be available at the same time, otherwise the path cannot be selected by the IAB-MT. In Rel-17, the only difference compared to Rel-16 is the additional procedure of BAP header rewriting, therefore we think it’s reasonable follow the similar order, that is to select the available BAP routing ID at the first place and subsequently re-write the BAP header, then to perform routing based on the re-written header. |
| Kyocera | Header rewriting first | We think the header writing is performed when the first routing is failed (i.e., when the header writing based re-routing is triggered). According to the rewritten header, the second routing is performed (i.e., the egress link is selected) as in Rel-16. |
| CATT | Header rewriting first |  |
| Huawei, HiSilicon | Header rewriting first | This should be the baseline. |
| Apple | Egress selection first | Same understanding as vivo |
| Intel | Header rewriting first | If egress link selection refers to the link towards IAB-donor DU2 (new link), we think header rewriting should be performed first. After BAP header rewriting, the boundary IAB-node can select the available egress link corresponding to the pseudo-BAP address and pseudo-path ID (for inter-donor CU re-routing). As BH Routing Configuration is configure via F1 interface, the mapping of pseudo-BAP address and egress link is added by IAB-donor CU1 in its BH Routing Configuration when path towards IAB-donor DU2 is established. To avoid conflict with other real BAP address in CU2’s topology, IAB-donor DU2’s pseudo-BAP address should be configured by IAB-donor CU2, then forward to IAB-donor CU1. With that information, IAB-donor CU1 is responsible for updating CU1’s BH Routing Configuration and BAP header rewriting table. |
| Fujitsu | Egress link selection first | This is to make sure that the selected egress link is available. |
| Lenovo | Egress link selection first | When perform rerouting, once an available egress link with different BAP address is selected, then BAP entity performs BAP header rewriting with the BAP routing ID of the selected entry. |
| Futurewei | Egress link selection first | Agree with vivo’s comment |
| Nokia | Egress link selection first | We agree with others: no header rewriting is needed if there is no available egress link or no allowed new destination BAP address. If header rewriting is performed first, the header rewriting becomes a re-routing table and configuration of the routing becomes very complex. |
| Ericsson | Header rewriting first | the egress link selection should be done after the BAP header rewriting. Because the egress link is selected from the routing table that takes as input the BAP destination and the PATH ID. Hence, before checking the routing table, the IAB node should have had already selected the new BAP destination and the new PATH ID. |

Proposal 3: [Easy][14 vs. 1] For upstream at the boundary node, it is supported to local re-route the data to CU1 topology, due to RLF/congestion/etc., which was from CU1 topology and originally to be forwarded via CU2 topology.

Question 2: Do you have concern on the above proposal or wording suggestion?

| Companies | Yes or No? | Comments |
| --- | --- | --- |
| ZTE | No | We agree with this proposal. |
| LG | No |  |
| vivo | No strong view | We wonder if we need to include ‘congestion’ as one of the reasons for upstream local re-routing since UL HbH flow control is not agreed yet. [Rapp]: Fine to remove “congestion”, if no objection from others |
| Kyocera | No | We agree with Proposal 3. |
| CATT | No |  |
| Huawei, HiSilicon | No |  |
| Apple | No |  |
| Intel | No | We agree with the proposal. However, it would be good to clarify the reason of local re-route back to CU1’s topology is due to RLF/congestion/etc in CU2’s topology. We suggest with following updates: For upstream at the boundary node, it is supported to local re-route the data back to CU1 topology, due to RLF/congestion/etc in CU2’s topology, which was from CU1 topology and originally to be forwarded via CU2 topology |
| Fujitsu | No | We agree with Proposal 3. |
| Lenovo | No | Agree with P3. |
| Futurewei | No | We agree with the intent of Intel’s proposal, but their proposed wording seems a bit awkward. Perhaps it could be more clear if phrased as follows: For upstream at the boundary node, it is supported to local re-route CU1 topology data, originally to be forwarded via CU2 topology, back to the CU1 topology in case of RLF/congestion/etc. towards CU2’s topology |
| Nokia | Yes | We would remove word “local” since it gives the impression that the re-routing is not under CU control. Also, Futurewei wording is better. |
| Ericsson | Yes | In this case we do not think it is re-routing but it is simply routing to the original path and thus there should no need of any additional procedure such as local re-routing or BAP header re-writing etc. The intention of the proposal is not clear. Is it routing or re-routing? |

Proposal 4’: [Easy][14 vs. 1] For upstream at the boundary node, it is FFS whether the BAP header rewriting operations/steps due to inter-topology routing and inter-topology re-routing are done in one step or separate steps.

Based on the new agreements for inter-topology routing, companies now may have clear understanding on how to handle the case in P3.

Question 3: Which option do you prefer on the above proposal?

Option 1: Keep it as FFS for this meeting;

Option 2: Use separate steps as baseline;

Option 3: Use one step.

| Companies | Option? | Comments |
| --- | --- | --- |
| ZTE | 1 or 2 | We prefer option 2 which presents more clear routing procedure. However, if it can not reach consensus, option 1 is also acceptable for this meeting. |
| LG | 2 | We think that BAP header rewriting due to inter-topology routing is performed before routing check-up, but BAP header rewriting due to inter-topology re-routing is performed after routing check-up. So, we prefer option 2, but option 1 is also ok to us if majority want. |
| Vivo | Option 2 | We assume this discussion is specifically related to Case 4. It would be straight-forward to perform the basic header rewriting procedure in separate steps, option 3 seems to be an optimization which can be discussed later. |
| Kyocera | Option 2 | We assume Option 2 can be a cleaner procedure. |
| CATT | See comment | For upstream, we think that the destination of BAP packet is always the donor-DU. So we don’t think inter-topology routing is needed for upstream. [Rapp] This is mainly for the “- Case 3: Data from CU1 topology originally to be forwarded to CU1 topology, but re-routed to CU2 topology due to RLF. [i.e. non-concatenated traffic’s inter-topology re-routing]”, as supported in P3. |
| Huawei, HiSilicon | Option 2 as baseline |  |
| Apple | Option 2 |  |
| Intel | Option 3 | We prefer to have single step of BAP header rewriting. As agreed during online session, the condition of BAP header rewriting is there’s no available egress link for upstream traffic. This also indicates that checking routing tables based on original BAP header is performed first. After BAP header rewriting, if there’s still no egress link available, there’s no need to rewrite the header back. |
| Fujitsu | Option 3 | We prefer option 3 which is more efficient. However, if it cannot reach consensus, option 1 is also acceptable for this meeting. |
| Lenovo | Option 3 | Option 3 is more succinct than Option 2. For Option 3, we can check the selected egress link associated with the new BAP routing ID in the inter-topology routing table, only when the selected link is available header rewriting can be performed; otherwise, local rerouting is performed firstly before header rewriting. |
| Futurewei | Option 3 | Agree with Lenovo’s understanding |
| Nokia | Option 3 | It is not clear what is meant by step here. The header rewriting should be performed only once. However, the egress link should be selected first: for routing it is based on Routing ID but for re-routing it is based only on BAP address. This has to be taken into account in the header rewriting, i.e., header rewriting cannot always be based on whole Routing ID. |
| Ericsson | Option 1 | There is only one possible routing in the upstream, i.e. the routing of ingress packets to the CU2. In the upstream, all the ingress packets are intended originally to CU1, since the descendant nodes do not know that there is a partial migration at an ancestor node. Hence, all the ingress packets in the upstream have the address of the CU1. So the boundary node can either: - Forward the packets to the next hop of the CU1 topology, as it would do normally (this can happen when no RLF, no type2- reception, no load balancing of the ingress BH RLC channel, or unavailability of the CU2 link) -> No BAP header rewriting needed; or - Route ingress packets to the CU1 (e.g in case of RLF, type-2 RLF reception, load balancing, etc). -> BAP header rewriting is needed. |

- 2.2 Issues on configurations

Proposal 10’: FFS: At least for upstream, the routing table configuration (or its entry) at the boundary node should be able to indicate whether it is for intra-topology or inter-topology (e.g. separate routing tables/entries). Namely that the boundary node must know which topology the routing ID and the next hop BAP address in the routing configuration refers to. Signaling details can be discussed later by RAN2 and RAN3. The motivation is: - The two parent nodes in different topology of the boundary node may have the same BAP address (e.g. IAB-1 and IAB-3), which makes the “Next Hop BAP Address” of routing entry ambiguous for concatenated and non-concatenated traffic, in upstream. - In upstream, the routing IDs (the one of non-concatenated traffic and the new one after header rewriting of concatenated traffic) may be same for the data to be routed to the two different egress topologies. It means there may be two entries with same routing ID, if the routing tables are merged as one. Proposal 11a’: It is FFS on whether the routing table configuration (or its entry) at the boundary node should be able to indicate whether it is for upstream toward inter-topology or downstream from inter-topology (e.g. separate routing tables/entries), after more progress in RAN2 and RAN3. - Since there is no assumption on the routing ID coordination between CUs, the routing ID used in the different topologies may be same. Also, R16 assumes the routing table configuration does not differentiate the upstream and downstream. - For concatenated traffic, one routing ID used in the 2nd topology for downstream may be same as one routing ID used in 2nd topology for upstream. There may be routing ID collision in the routing table configuration at the boundary node (i.e. one upstream routing ID may be same as one downstream routing ID). - Therefore, the routing table should be separate for upstream and downstream for concatenated traffic. Otherwise, there will be two entries with same routing IDs (i.e. one upstream routing ID may be same as one downstream routing ID.) Proposal 11b’: It is FFS on whether the header rewriting configuration (or its entry) at the boundary node should be able to indicate whether it is for upstream toward inter-topology or downstream from inter-topology (e.g. separate routing tables/entries), after more progress in RAN2 and RAN3. Namely that the boundary node must know to which topology each BAP routing ID in the header rewriting configuration refers to. The same issue occurs for the BAP header rewriting table. For concatenate traffic, one previous routing ID for upstream may be same as one previous routing ID for downstream. If the upstream and downstream have the merged one BAP header rewriting table, there may be two entries in the table with the same Previous Routing ID. Proposal 11c’: FFS: The BH RLC Channel Mapping Configuration (or its entry) at the boundary node should be able to indicate the egress/ingress topology, i.e. whether it is for upstream toward inter-topology or downstream from inter-topology (e.g. separate tables/entries). Signaling details can be discussed later by RAN2 and RAN3. Namely that the boundary node must know to which topology each BH RLC channel in the BH RLC Channel Mapping Configuration refers to. The similar issue occurs for the bearer mapping table. The parent node of the boundary node in CU2’s topology (IAB-3) may have the same BAP address with the child node of the boundary node (IAB-4). In that case, one entry of the BH RLC CH mapping table can be “BAP add1+ BH RLC ID1=>BAP add1 + BH RLC ID2”. It is ambiguous on whether it is “RLC1 of IAB-3=>RLC2 of IAB-4” as downstream or “RLC1 of IAB-4=>RLC2 of IAB-3” as upstream.

Instead of keeping all those proposals as FFS, rapporteur would like to propose to agree on the high-level principle and leave the configuration details to later RAN2/3 meeting.

Proposal 12: The boundary node must know which topology the below information refers to:

1: each routing ID and each next hop BAP address in the routing configuration;

2: each BAP routing ID in the header rewriting configuration;

3: each BH RLC channel in the BH RLC Channel Mapping Configuration;

Question 4: Do you agree on the above proposal 12?

| Companies | Yes or No? | Comments |
| --- | --- | --- |
| ZTE | See comments | For the routing configuration at boundary node, it is suggested to have a separate routing table specific for UL concatenated traffic forwarding in CU2 topology. For the other DL concatenated traffic, DL non concatenated traffic and UL non concatenated traffic, another routing table may be configured for these traffic. For BAP routing ID in the header rewriting configuration, it only need to indicate whether it is for DL or UL. For the BH RLC channel in the BH RLC channel mapping configuration at boundary node, we think it is necessary to indicate whether it is for concatenated traffic or non-concatenated traffic. [Rapp]: The configuration and table can be discussed later. The proposal is only the first step to setup our common intention/understanding. |
| LG | see comments | All FFSes above considers all fields of configuration, but now the question asks only about a specific field of configuration. We think that if routing table at the boundary node is able to indicate whether it is for concatenated or non-concatenated traffic, all information in the routing table can be separately identified. With this understanding, what is the intention of the question? The rapporteur wants that only a part of information in configuration can be separately identified as concatenated or non-concatenated traffic, and others not. We basically think that separate routing and rewriting table for concatenated or non-concatenated traffic is sufficient, but no need to have separate routing/rewriting table for upstream and downstream. However, BH RLC Channel Mapping Configuration at the boundary node may need to be able to indicate whether it is for upstream or downstream traffic. [Rapp]: whether we want the whole table to be topology specific or only per entry to be topology specific can be discussed later. The proposal is only to achieve high level intention. |
| Vivo | Yes |  |
| Kyocera | Yes, but… | For the routing configuration, we assume there are separate routing configurations managed by CU1 and CU2 respectively. One managed by CU2 is applied for UL concatenated traffic, while another managed by CU1 is applied for other traffic, i.e., DL concatenated traffic, DL non-concatenated traffic and UL non-concatenated traffic. [Rapp]: That means whole table is topology specific, which is covered by the proposal. The stage3 details can be discussed in the future. For the BH RLC channel mapping configuration, we assume there are two separate configurations as well, and we wonder if these can be associated with the routing configurations, i.e., the BH RLC channel mapping configuration is determined based on the selected routing configuration. |
| CATT | Yes |  |
| Huawei, HiSilicon | Yes |  |
| Apple | Yes |  |
| Intel | Yes to 1). No to 2). No to 3). | For 1), Yes. In case the pseudo routing ID configured by IAB-donor CU2 is conflict with any real routing ID of CU1’s topology, in the BH Routing Configuration, IAB-donor CU1 should configure the boundary IAB-node with which topology one routing ID belongs to. However, for next hop BAP address, it should be ok the pseudo next hop BAP address is conflict with other CU1’s real BAP address, as this entry will only be selected when the boundary IAB-node decides to perform rerouting. For egress link in that entry, it can be left to IAB-donor CU1’s implementation to set different egress link (value) to real/pseudo routing IDs from CU1 or CU2, respectively. For 2), No need. As we discussed in Q1, for upstream traffic, BAP header rewriting is performed for re-route only when there’s no available link matching to its original routing ID (in CU1’s topology). Even real-routing ID in CU1’s topology is conflict with pseudo-routing ID configured by IAB-donor CU2, after header rewriting (although header is not changed), the boundary IAB-node can still find the new egress link towards CU2’s topology corresponding to the pseudo routing ID (CU2’s topology). Additionally, from our understanding, as only IAB-donor CU1 knows the real BAP address and routing ID of its own IAB-nodes, the BAP rewriting table is configured by IAB-donor CU1 with two entries “real routing ID” and “pseudo routing ID”. For upstream, if BAP header rewriting condition is met, perform BAP header rewriting if routing ID in the BAP header matches “real routing ID”. For downstream, after deciding whether to deliver to upper layer, other traffic from CU2’s topology performs BAP header rewriting by matching BAP header with “pseudo routing ID” in the BAP header rewriting table. For 3), No need. Recalling BH RLC Channel Mapping configuration is also configured by F1-terminated IAB-donor CU (IAB-donor CU1), IAB-donor CU1 can set different egress link (upstream)/ingress link (downstream) towards original topology and new topology. In this case, different BH RLC channel IDs are mapped to different egress/ingress links by IAB-donor CU1’s implementation. There’s no need to differentiate the topology. |
| Fujitsu | See comment | We don’t think item 1 is necessary. For upstream at the boundary node, if it selects egress link by looking up the BAP header rewriting configuration, there will be no routing ID ambiguity or next hop BAP address ambiguity. It will avoid adding one new routing table or new indication in routing table. No need to change R16 F1AP signaling for this aspect. For item 2, there should be separate BAP header rewriting configurations for UL concatenated traffic and for DL concatenated traffic. The boundary node should first determine whether it is concatenated traffic for UL or DL. We think it’s clearer to identify each BAP rewriting configuration entity by DL or UL than using topology. We are fine with item 3. |
| Lenovo | Yes |  |
| Futurewei | Yes |  |
| Nokia | No | Next hop BAP address is relevant only for selection the egress link, i.e., it is a local parameter, it can also be a pseudo BAP address. Therefore, it is not topology dependent. It seems that the selected Approach B requires multiple routing tables (up/downstream, intra/inter-topology), multiple header rewriting tables (up/downstream, intra/inter-topology) and multiple BH RLC mapping configurations. (Approach C only required update of the single routing table, nothing else.) |
| Ericsson |  | We can leave it FFS for this meeting. These are stage 3 details which can be discussed later. |

# Conclusion and proposals

Based on the above summary, following proposals are given.

# Reference

- R2-2107251	Summary of [Post114-e][075][eIAB] Open Issues on Re-routing	Huawei, HiSilicon

- R2-2107170	BAP-layer traffic processing at the boundary node	Qualcomm

- R2-2107253	Inter-donor routing, local rerouting and RLF indication for R17-IAB	Huawei,

- R2-2107445	Inter-donor CU Topology migration	 Intel

- R2-2107648	Open issues on (re-)routing	Fujitsu

- R2-2108026	Topology adaptation enhancements	Samsung

- R2-2108141	Discussion on inter-donor topology redundancy	ZTE, Sanechips

- R2-2108422	Boundary IAB node behaviour for partial and full inter-donor migration	Ericsson

- R2-2108482	Solutions for Inter-Donor Routing and Bearer Mapping 	Futurewei Technologies