3GPP TSG-RAN WG2 Meeting #116 electronic	          _R2-2111356

Electronic meeting, 1st-12th November 2021

Agenda Item:	8.12.2.2

Source: 	Huawei, HiSilicon

Title:	Summary of [AT116-e][110][RedCap] Identification and access restriction

Document for:	Discussion and Decision

# Introduction

This paper aims at capturing the summary of email discussion.

- [AT116-e][110][RedCap] Identification and access restriction (Huawei)

Initial scope: Continue the discussion on remaining aspects of RedCap identification (msg1/msg3/msgA) and access restriction (cell barring/UAC), e.g. based on the proposals in R2-2109577

Initial intended outcome: Summary of the offline discussion with e.g.:

    - List of proposals for agreement (if any)

    - List of proposals that require online discussions

    - List of proposals that should not be pursued (if any)

Initial deadline (for companies' feedback): Friday 2021-11-05 0900 UTC

Initial deadline (for rapporteur's summary in R2-2111344): Friday 2021-11-05 1200 UTC

Proposals marked "for agreement" in R2-2111344  not challenged until Monday 2021-11-08 1000 UTC will be declared as agreed via email by the session chair (for the rest the discussion will further continue offline until the CB session in Week2).

Contact Table

| Companies | Name | Email |
| --- | --- | --- |
| OPPO | Haitao Li | lihaitao@oppo.com |
| Lenovo | Hyung-Nam Choi | hchoi5@lenovo.com |
| NEC | Hisashi Futaki | hisashi.futaki@ nec.com |
| Nokia | Samuli Turtinen | samuli.turtinen@nokia.com |
| Sequans | Noam Cayron | noam.cayron@sequans.com |
| Apple | Naveen Palle | naveen.palle@apple.com |
| MediaTek | Pradeep Jose | pradeep[dot]jose[at]mediatek[dot]com |
| Sierra Wireless | Serkan Dost | sdost@sierrawireless.com |
| Futurewei | Yunsong Yang | yyang1@futurewei.com |
| Ericsson | Tuomas Tirronen | tuomas.tirronen@ericsson.com |
| Qualcomm | Linhai He | linhaihe@qti.qualcomm.com |
| Intel | Yi Guo | Yi.guo@intel.com |
| Spreadtrum | Lifeng Han | Lifeng.Han@unisoc.com |
| Sharp | LIU Lei | lei.liu@cn.sharp-world.com |
| Samsung | Seungbeom Jeong | s90.jeong@samsung.com |
| vivo | Chenli | chenli5g@vivo.com |
| DENSO | Hideaki Takahashi | hideaki.takahashi.j6e@jp.denso.com |
| Huawei, HiSilicon | Yulong Shi | Shiyulong5@huawei.com |
| BT | Salva Diaz | salva.diazsendra@bt.com |
| ZTE | LiuJing | liu.jing30@zte.com.cn |
| CATT | Xiangdong zhang | zhangxiangdong@catt.cn |
| Xiaomi | Yanhua li | Liyanhua1@xiaomi.com |
| T-Mobile USA | John Humbert | John.Humbert2@T-Mobile.com |

- Discussion

  - Msg1 early identification

The proposal is to discuss when the RedCap UE should use Msg1 early identification in MAC layer perspective, if the Msg1 early identification is configured by NW. The motivation is that MAC does not need to differentiate the RRC message to be transmitted, and use Msg1 early identification as long as the RA procedure is CBRA. This is because any CBRA requires the NW to identify the UE type when scheduling RAR.

Proposal 1: In MAC perspective, RedCap UE use Msg1 early identification whenever transmitting preamble for CBRA, as long as the Msg1 early identification is configured by NW.

Question 1: Do you agree on the above proposal 1?

| Companies | Yes or No? | Comments |
| --- | --- | --- |
| OPPO | Yes |  |
| Fujitsu | Yes |  |
| Lenovo | Yes |  |
| NEC | Yes | This should be under network control, so UE shall do this when configured by NW. |
| Nokia | Depends | We think the RedCap UE could also use the common RACH even though Msg1 identification is also configured, e.g., in case 2RX RedCap UE is in good coverage. This way the RedCap specific PRACH would not be congested that easily. |
| Sequans | Yes | This should be at least the baseline. It may not be so simple to define a rule as suggested by Nokia, but we are OK to consider it if time allows. |
| MediaTek | Yes |  |
| Sierra Wireless | With minor updates | We agree with Nokia. For some RedCap UEs, on some networks, Msg1 early identification may not be needed whereas for other RedCap UEs it might be so the configuration may not globally apply to all Redcap UEs. Reducing the number of UEs using the RedCap PRACH resources would reduce the resources dedicated for RedCap only and improve spectral efficiency and providing MNOs with more control. With the following minor change this can be acceptable: In MAC perspective, a RedCap UE uses Msg1 early identification whenever transmitting preamble for CBRA, as long as the Msg1 early identification is configured for the RedCap UE by NW. |
| Futurewei | - | From NW’s PoV, the NW may wish the RedCap UEs to use the dedicated ROs/preambles that the NW has configured, because the NW may apply extra coverage boosting in the DL/UL scheduling for the subsequent Msg2/Msg3/Msg4 when responding to such Msg1s, comparing to those Msg1s that use the regular ROs/preambles. From UE’s PoV, for RedCap UEs with 1 Rx antenna or poor RSRP measurement, they will risk their Msg1 being a failure if they don’t use the dedicated ROs/preambles that the NW configures. On the other hand, for RedCap UEs with 2 Rx antennas or good RSRP measurement, they may not care so much. For these later UEs, if the initial PRACH fails to solicit any response from the NW, should the UE be allowed to use the regular ROs/preambles for the retry, for suspicion that there has been a collision or congestion condition going on in the dedicated RACH resources? |
| Ericsson | Yes | This should be at least the baseline. |
| Qualcomm | Yes |  |
| Intel | Yes |  |
| Spreadtrum | Yes | If the network configure the dedicated PRACH resource for Msg1 early indication, the UE should use it for CBRA RACH procedure. |
| Sharp | Yes |  |
| Samsung | Yes | No strong view but the proposal can be reworded (merely for the readability) e.g. 'A RedCap UE always uses dedicated RACH resources for RedCap UEs, if configured.' |
| vivo | Yse | The RedCap UE will apply Msg1 early identification if the Msg1 early identification is configured by NW. |
| DENSO | Yes |  |
| Huawei, HiSilicon | Yes |  |
| ZTE | Yes |  |
| CATT | Yes |  |
| Xiaomi | Yes |  |

Summary:

Clear majority is fine with the proposal.

Nokia think RedCap UE can choose not to use Msg1 early identification even configured by NW. Rapporteur understanding this may not work: If NW configures Msg1 early identification, it will assume all RedCap UE should identify itself in Msg1. Otherwise, the RedCap UE, as proposed by Nokia, pretending to be legacy UE, gNB may schedule Msg2 in larger bandwidth supported by RedCap UE. This will cause RACH failure.

Futurewei’s question should be discussed by RAN1. Rapporteur understand RedCap UE should not fall back to legacy Msg1, if dedicated Msg1 fails.

Rapporteur make the update using the wording from Sierra Wireless, but see no difference.

Proposal 1’:[Easy] In MAC perspective, a RedCap UE uses Msg1 early identification whenever transmitting preamble for CBRA, as long as the Msg1 early identification is configured for RedCap by NW.

Agreement

Confirm the following working assumption with the modifications in red:

- For 4-step RACH, support the early indication of RedCap UEs at least in Msg1.

  - The early indication in Msg1 can be configured to be enabled/disabled via SIB

    - FFS how to support enable/disable the early indication

  - FFS details e.g.: From RAN1 perspective, the following methods can be used for early indication both for shared initial UL BWP and separate initial UL BWP (if supported)

    - separate PRACH resource

    - PRACH preamble partitioning

This proposal is just to confirm the RAN1 agreement, which then can be further considered by the RACH partitioning session.

Proposal 2: For Msg1 early identification, RAN2 confirm both dedicated ROs and dedicated PRACH preamble should be supported.

Question 2: Do you agree on the above proposal 2?

| Companies | Yes or No? | Comments |
| --- | --- | --- |
| OPPO | Yes |  |
| Fujitsu | Yes |  |
| Lenovo | Yes |  |
| NEC | Yes | RAN2 can follow RAN1 agreements. |
| Nokia | Yes |  |
| Sequans | Yes |  |
| Apple | Ok |  |
| MediaTek |  | This should be left to the RACH partitioning session |
| Sierra Wireless | Yes |  |
| Futurewei | Yes |  |
| Ericsson | Yes |  |
| Qualcomm | Yes |  |
| Intel | Yes | But should not this be discussed in common RACH discussion in section 8.18 RACH indication and partitioning? |
| Spreadtrum | Yes |  |
| Sharp | Yes |  |
| Samsung | Yes but | Technically both are feasible, but I am not sure it would be good to have such flexibility. Perhaps to have separate PRACH resource would be sufficient? |
| vivo | Yes | RAN1 has agreed dedicated ROs and dedicated PRACH preamble can be used for early indication both for shared initial UL BWP and separate initial UL BWP (if supported). Hence both methods for Msg1 early identification can be supported, and in our view using one of the two methods is enough for Msg1 based early identification. |
| DENSO | Yes |  |
| Huawei, HiSilicon | Yes |  |
| BT | Yes |  |
| ZTE | Yes |  |
| CATT | Yes |  |
| Xiaomi | Yes |  |

Summary:

No clear objection. Rapporteur will assume this agreeable, which will be considered further by the RACH partitioning session.

Proposal 2:[Easy] For Msg1 early identification, RAN2 confirm both dedicated ROs and dedicated PRACH preamble should be supported.

A straightforward method is to use the dedicate RACH configuration for Msg1 early identification to implicitly indicate whether the early indication in Msg1 is configured. When the dedicate RACH configuration for Msg1 early identification is present, it means the Msg1 early identification is enabled; otherwise, it means the Msg1 early identification is disabled.

Proposal 3: Msg1 early identification is enabled/disabled implicitly by the presence of dedicate RACH configuration for Msg1 early identification.

Question 3: Do you agree on the above proposal 3?

| Companies | Yes or No? | Comments |
| --- | --- | --- |
| OPPO | Yes |  |
| Fujitsu | Yes |  |
| Lenovo | Yes but | If Redcap is the only R17 feature for which separate RACH resources need to be configured in the cell, then proposal is fine with us. Otherwise, we may need to come back on this issue depending on outcome of the common R17 PRACH partitioning session. |
| NEC | Yes |  |
| Nokia | Yes |  |
| Sequans | Yes | Assuming the different configuration for different features are well defined |
| Apple | Yes | But further revising might be needed (based on outcome of unified RACH) |
| MediaTek |  | This will need to be decided as part of the RACH partitioning session |
| Sierra Wireless | No | Agree with Lenovo that PRACH partitioning session may affect this proposal. Also as mentioned in Q1 , Msg 1 early indication may not apply to all Redcap UEs so this is acceptable only when Msg1 applies to all Redcap UEs. This can be clarified with the following proposed text change: Msg1 early identification is enabled/disabled implicitly by the presence of dedicate RACH configuration for Msg1 early identification when Msg1 early indication applies to all Redcap UEs. |
| Futurewei | Yes |  |
| Ericsson | Yes | This is stage-3 detail which can be sorted out when implementing the CR but in general OK, there should not be need to have a separate indication beyond the configuration. However, this relates also to the other discussion on RACH fragmentation and indication of features. |
| Qualcomm | Yes |  |
| Intel | Yes |  |
| Spreadtrum | Yes | The implicit way can be used for enable/disable Msg1 early indication and explicit indication is not needed. |
| Sharp | Yes |  |
| Samsung | Yes |  |
| vivo | Yes | This is the most straightforward approach. We could agree it as the baseline at least. |
| DENSO | Yes | Separate indication is not needed. |
| Huawei, HiSilicon | Yes |  |
| BT | yes |  |
| ZTE | Yes |  |
| CATT | Yes |  |
| Xiaomi | Yes |  |

Summary:

Clear majority is fine with the proposal.

For Lenovo comments, rapporteur understand the proposal is only for RedCap case. It is clarified by the update.

Proposal 3’: [Easy] For RedCap, Msg1 early identification is enabled/disabled implicitly by the presence of dedicate RACH configuration for Msg1 early identification.

  - Msg3 early identification

The proposal is to discuss whether we need the Msg3 early identification for the RRC resume case, where two LCIDs are needed, i.e. for CCCH and CCCH1.

While for CCCH1, it is only used for RRCResumeRequest1, normally, when the gNB recieves the RRCResumeRequest1, it can fetch the UE capability from the stored UE context and recognize the UE is RedCap UE. But in some cases, the RRC resume procedure may fail and fallback to RRC connection establishment procedure, the target gNB is not able to know/fetch the UE context by its Msg3, and can not identify it is RedCap UE.

Proposal 4: Two reserved LCIDs are used for CCCH and CCCH1 cases respectively for Msg3 early identification.

Question 4: Do you agree on the above proposal 4?

| Companies | Yes or No? | Comments |
| --- | --- | --- |
| OPPO | No | If fallback to RRC connection establishment procedure, Msg3 will use the eLCID for CCCH, by then NW will identify the RedCap UE. |
| Fujitsu | No |  |
| Lenovo | Yes | We understood the question is about the resume case. |
| NEC | Yes | This seems straightforward |
| Nokia | Yes | eLCID cannot be used as we cannot increase the Msg3 size, hence, this is the only option. |
| Sequans | Yes | Agree with Nokia |
| MediaTek | Yes |  |
| Sierra Wireless | Yes |  |
| Futurewei | Yes |  |
| Ericsson | Yes, but | We assume the new CCCH LCID would be used for the RRCSetupRequest as well? |
| Qualcomm | Yes |  |
| Intel | Yes | We see the need to support CCCH1 for resume. |
| Spreadtrum | Yes |  |
| Sharp | Yes |  |
| Samsung | No | Currently CCCH of size 64 bits is used only for RRCResumeRequest1 which uses full size of resumeIdentity and is used from RRC_INACTIVE. Furthermore, a UE does not have to include early indication when it transits from RRC_INACTIVE to RRC_CONNECTED, as network always retrieves UE's capability from the last serving gNB during RRC resume procedure. Therefore, RAN2 does not have to introduce a dedicated LCID for CCCH of size 64 bits. |
| vivo | Yes | If fallback to RRC connection establishment, the RRC resume request message may also be used for Msg3 early identification. Since RRCResumeRequest1 use CCCH1, 2 reserved LCIDs are required for CCCH and CCCH1 cases. |
| DENSO | No | Agree with Samsung and OPPO. A new LCID is enough for CCCH. |
| Huawei, HiSilicon | Yes |  |
| ZTE | Yes |  |
| CATT | No | The left reserved LCID resources is not so abundant now, we don’t suggest to occupy two reserved LCID values for only Redcap topic. A separate MAC subheader can be used to carry the LCID for Msg3 early identification. |
| Xiaomi | Yes | The new CCCH1 for resume. If NW cannot acquire UE’ context, it still need early indication in CCCH1. |

Summary:

Clear majority support the proposal.

OPPO/Fujistu/Samsung/DENSO/CATT have different view. For OPPO’s comment, rapporteur understand the fall back case will not transmit Msg3 again.

Rapporteur assume the answer to Ericsson’s question is Yes, which will be the same new LCID for other Msg3.

Proposal 4: [Easy] Two reserved LCIDs are used for CCCH and CCCH1 cases respectively for Msg3 early identification.

The proposal is to discuss whether RedCap UE should always use Msg3 early identification, or only in some cases. The motivation is that it comes for free since we anyway has to define the dedicated LCID for RedCap. There is no difference form UE complexity to use either legacy LCID or the dedicated LCID. Since NW may disable the Msg1 early identification, UE has to support the Msg3 early identification in that case. So, there should be no reason make it as optional capability.

Proposal 5: In MAC perspective, RedCap UE uses the dedicated LCID for Msg3 early identification, whenever the Msg3 includes the CCCH data.

Question 5: Do you agree on the above proposal 5?

| Companies | Yes or No? | Comments |
| --- | --- | --- |
| OPPO | Yes |  |
| Fujitsu | Yes |  |
| Lenovo | Yes |  |
| NEC | Yes |  |
| Nokia | Yes |  |
| Sequans | Yes |  |
| Apple | No | This is not the agreement. |
| MediaTek | No | Similar to Q1, the RedCap UE should only use the dedicated LCID if the NW configures the UE with msg3 early identification. |
| Sierra Wireless | Yes | Msg3 early identification comes for free (as indicated above) so it should always be used by RedCap UEs. This would reduce the need for dedicated PRACH resources for RedCap UEs. |
| Futurewei | - | If P5 is agreed: from UE’s PoV, it is simpler; from NW’s PoV, even if the gNB has enabled Msg1 early identification, the gNB is still required to understand the new LCID in order to route the message correctly, which probably is no big deal for NW implementations. On the other hand, some contributions to RAN2 #116-e propose that the NW can enable/disable Msg3 early identification. If RAN2 adopt such NW controlled approach, the RedCap UE cannot use Msg3 early identification if the NW has disabled it. Maybe we should first decide whether the NW can enable/disable Msg3 early identification or not. |
| Ericsson | Yes | We think the UE should always use the LCID to indicate it is a RedCap UE. We don’t see any drawbacks in this, and this would also simplify the testing (cf. the concern brought up in some contributions). |
| Qualcomm | Yes |  |
| Intel | No | Based on agreements, the UE shall only use MSG3 based early identification if MSG1 is disabled. |
| Spreadtrum | Yes | We think early indication function is used for the network to have differential handling for RedCap and non-RedCap UE during initial access. If the UE does not support it or use it while the network wants to, this purpose will not be achieved. Also the network can enable/disable Msg3 early identification by an explicit indication in SIB. |
| Sharp | Yes |  |
| Samsung | Yes |  |
| vivo | No | Firstly, if the dedicated LCIDs are introduced, then using the dedicated LCIDs bring no extra overheads, the RedCap UE can use the dedicated LCID for early identification. If network doesn’t configure Msg.1 based early identification, then the dedicated LCID for Msg.3 based early identification could be used. Otherwise, Msg.3 based early identification isn’t necessary, maybe the dedicated LCID could be used to bring additional information. Besides, from UE point of view, there is no need to support or implement two duplicated functionalities. In this way, it is not fair for UE to support both msg.1 and msg.3 based early identification mandatorily. |
| DENSO | Yes | There is no issue to include the LCID for RedCap anytime in Msg.3. |
| Huawei, HiSilicon | Yes |  |
| BT |  | I wonder to know if it will be possible to introduce an additional point for discussion that have been bring to this meeting by two different companies, BT and CMCC in R2-2110659 and R2-2110535 respectively. Proposal to discuss: Msg3 identification can be configured to be enabled or disabled by the network |
| ZTE | Yes |  |
| CATT | No | For the case of Msg3-based on-demand SI, it is not necessary to use dedicated LCID, even the Msg3 includes CCCH data of rrcSystemInfoReqeust singling, for we have not defined redcap-specific SIB. On the other hand, we have not achieved any agreement on the detail of LCID based Msg3 early identification. And we also agree with the proposal of BT, we actually have not had an agreement on whether the early identification can be disabled, entirely. |
| Xiaomi | No | On the condition that the NW configures the UE with msg3 early identification. |

Summary:

Yes: 15

No: 6?

MTK/ Futurewei/BT/ CATT, Xiaomi: Msg3 early identification should be controlled by NW;

Intel/vivo: UE use Msg3 early identification, only if Msg1 early identification is disabled.

Proposal 5: [To discuss] In MAC perspective, RedCap UE uses the dedicated LCID for Msg3 early identification:

Option 1: whenever the Msg3 includes the CCCH data [15];

Option 2: whenever the Msg3 includes the CCCH data and Msg1 early identification is not configured [2];

Option 3: whenever the Msg3 includes the CCCH data and Msg3 early identification is enabled by NW [4].

  - MsgA early identification

For MsgA early identification, it is assumed the only MAC PDU part is RAN2 scope. So, below is proposed.

Proposal 6: At least the dedicated LCID (i.e. the Msg3 early identification solution) can be supported for MsgA early identification. It is up to RAN1 on the need of dedicated preamble and/or dedicated PUSCH resource configuration.

Question 6: Do you agree on the above proposal 6?

| Companies | Yes or No? | Comments |
| --- | --- | --- |
| OPPO | Yes |  |
| Fujitsu | Yes |  |
| Lenovo | Yes |  |
| NEC | Yes |  |
| Nokia | Yes |  |
| Sequans | Yes |  |
| Apple | Yes |  |
| MediaTek | Yes |  |
| Sierra Wireless | Yes |  |
| Futurewei | Yes | If RAN2 decide that the NW can enable/disable Msg3 early identification in 4-step RACH, the NW should also be able to exercise the same control for the dedicated LCID based MsgA early identification. |
| Ericsson | Yes |  |
| Qualcomm | Yes |  |
| Intel | Yes |  |
| Spreadtrum | Yes | We think it is better to follow the agreement on 4-step RACH early indication, considering UE may fall back to 4-step RACH. Then in RAN2 we can at least support the dedicated LCID for MsgA early identification. |
| Sharp | Yes |  |
| Samsung | Yes | The wording can be similar to P5 above: 'In MAC perspective, RedCap UE uses the dedicated LCID for MsgA early identification, whenever the MsgA includes the CCCH data.' |
| vivo | Yes |  |
| DENSO | Yes |  |
| Huawei, HiSilicon | Yes |  |
| BT | Yes |  |
| ZTE | Yes |  |
| CATT | Yes |  |
| Xiaomi | Yes |  |

Summary:

All companies support the proposal.

Proposal 6: [Easy] At least the dedicated LCID (i.e. the Msg3 early identification solution) can be supported for MsgA early identification. It is up to RAN1 on the need of dedicated preamble and/or dedicated PUSCH resource configuration.

  - Cell barring and RedCap supporting

This is one leftover from the running CR of 38.3331, i.e. whether the RedCap specific cell barring is optional IE or mandatory IE for gNB supporting RedCap.

Option 1: mandatory IEs with value {barred, notBarred} (if gNB supports RedCap);

Option 2: optional IEs with value {barred};

Option 3: optional IEs with value {notBarred};

Proposal 7: Introduce two mandatory IEs in SIB1 with {barred, notBarred} values for 1RX and 2RX RedCap UE respectively.

Question 7: Do you agree on the above proposal 7? (You may also give your preferred option)

| Companies | Yes or No? | Comments |
| --- | --- | --- |
| OPPO | Yes, but | In ASN.1, those two IEs have to be optional, right? Because some gNBs may not support RedCap and do not need to broadcast them. |
| Fujitsu | Yes | These two IEs are mandatory for RedCap UEs, but is optional present in ASN.1. |
| Lenovo | Yes | From ASN.1 encoding point of view Option 2 and 3 require 2 extra bits (“optionality bits). |
| NEC | No | RAN2 agreed to make RedCap specific IFRI in SIB1 is mandatory and if it is absent, the RedCap UE considers the cell does not support RedCap. So, there is already a way to inform RedCap UE is NOT barred. What we need further is to indicate RedCap UE is barred. Then, we think firstly RAN2 should clarify whether we should support the case where only 2 Rx RedCap is barred? We do not think this is valid or useful case. The main reason why 1 Rx is to be barred is those RedCap UEs may need special treatment (e.g. larger repetition, lower data rate), but there seems to be no reason to bar only 2 Rx RedCap UEs. Therefore, we think two optional IEs with value {barred} and {barred-1Rx} should be sufficient. The former is applied for both 2 Rx and 1 Rx RedCap UEs. If this is not agreeable, then the alternative option may be to have two optional IEs with value {barred-2Rx} and {barred-1Rx}, i.e. Option 2 |
| Nokia | No | They need to be OPTIONAL for ASN.1 |
| Sequans | No | These must be optional for cells not supporting RedCap, so there is no benefit in mandating RedCap-supporting cells to always set them |
| Apple | Yes | SIB-IFRI can provide the support of RedCap by the cell, then for the cells that support RedCap, the gNB should provide bar/not-barred for each of the 1Rx/2Rx. In ASN.1, we can signal such that the entire sequency (1Rx/2Rx) can be absent for gNBs not supporting RedCap, but for gNBs supporting RedCap, the barring(or not) should be there… like in MIB. |
| MediaTek | No | These surely must be Optional from ASN.1 perspective, for non-RedCap cells. |
| Sierra Wireless | Yes but depends on Q8 | The Proposal of “Option 1” is agreeable as long as no IE means Redcap UE is barred. |
| Futurewei | Yes | If barring 1Rx/2Rx UEs is based on cell loading, having the IEs disappeared and re-appeared in SIB1 from time to time may be awkward. It may be simpler to keep the format the same but allow the values to be switched. |
| Ericsson | No | The IEs cannot be mandatory but optional as brought up in the comments above. Furthermore, we don’t see use case for allowing 1 Rx but no 2 Rx RedCap UEs in a cell. Thus, we shouldn’t need more than one barring bit for 1 Rx UE: This can be simply cellBarredRedCap1Rx with value ENUMERATED{true}, which is optional. RedCap UE with 2 Rx chains can simply follow the presence of RedCap-specific IFRI in SIB1. If it is not present, no RedCap UEs are allowed in the cell (for time being). |
| Qualcomm | Yes | As long as a cell supports RedCap, the IE {barred, notBarred} shall present. Otherwise, RedCap UEs can’t determine whether the cell supports RedCap or not. |
| Intel | No | It should be optional IEs. option 2 should be sufficient. |
| Spreadtrum | Yes,but | If the network supports Redcap, it shall indicate its status for Redcap access through the IE. But in ASN.1 it should be OPTIONAL. |
| Sharp | No | They are optional. |
| Samsung |  | We think option 2 is sufficient. Absence of this IE if cell supports redcap (i.e. IFRI bit present) means not barred. |
| vivo | Yes, but | We think these two IEs should be optional. When gNB supports RedCap UE, two IEs must be included in SIB1. |
| DENSO | No | As already commented, the field cannot be mandatory present from the ASN.1 viewpoint, since the NCE can be used for the other features which do not require the RedCap specific field. Option 2 is a proper choice from our point of view. |
| BT | No | They should be mandatory supported by RedCap UEs but they should be optional from a ASN.1 point of view to cover non-RedCap cells. It is our proposal to have Msg3 enable/disable in the same way it is agreed for Msg1. That fact will reinforce the need to be this optional. |
| ZTE | No | From ASN.1 point of view, we cannot add mandatory fields in SIB1. We prefer to have two fields to control 1Rx and 2Rx UEs separately, but the fields should be optional, and option 2 is preferred. |
| CATT | No | Option 2 may be ok. For Option 2, the absent of the IEs mean notBarred implicitly. |
| Xiaomi | NO | Only for gNB supporting RedCap, these two IEs are mandatory. |

Summary:

Option 1: mandatory IEs with value {barred, notBarred} (conditional mandatory if gNB supports RedCap);

- 10 companies: OPPO, Fujitsu, Lenovo, Apple, Sierra Wireless, Futurewei, QC, vivo, Huawei, Xiaomi

Option 2: optional IEs with value {barred};

- 6 companies: NEC, Intel, Samsung, DENSO, ZTE, CATT

Option 3: optional IEs with value {notBarred};

For OPPO, Nokia, Sequans, MTK, vivo comments: Even in option 1, this is optional, for gNB not supporting RedCap case. Option1 means the IE is mandatory for gNB supporting RedCap.

Ericsson propose new option: “This can be simply cellBarredRedCap1Rx with value ENUMERATED{true}, which is optional. RedCap UE with 2 Rx chains can simply follow the presence of RedCap-specific IFRI in SIB1. If it is not present, no RedCap UEs are allowed in the cell (for time being).”

Even though option 1 has clear majority view. It is clear that the new IE will be absent anyway for legacy cell. So, it is good only to discuss the case for gNB supporing RedCap UE. Then the discussion point focus on whether we use one value or two values. Rapporteur propose RAN2 to discuss.

Proposal 7: [To discuss] For the gNB supporting RedCap UE case, introduce below for 1RX and 2RX RedCap UE respectively,

Option 1: two mandatory IEs in SIB1 with {barred, notBarred} [9]

Option 2: two optional IEs in SIB1 with {barred}[4]

In any case above, the RedCap specific cell barring IEs will be absent in legacy gNB. This proposal is to discuss how to handle this the absence from RedCap UE side.

Proposal 8: RAN2 to discuss whether RedCap UE uses the absence of RedCap specific cellBarred or RedCap specific IFRI, to consider the gNB as not supporting RedCap.

Option 1: consider the gNB as not supporting RedCap

Option 2: consider the gNB as allowed to camp, and differentiate whether gNB supporting RedCap by the presence of RedCap specific IFRI.

Question 8: Which option do you prefer for RedCap UE, in case the RedCap specific cell barring IE(s) is absent?

| Companies | Option? | Comments |
| --- | --- | --- |
| OPPO | Option 1 |  |
| Fujitsu | Option 1 |  |
| Lenovo |  | We wonder about this question. We thought RAN2 already agreed in RAN2#115-e that the Redcap specific IFRI is used to indicate whether a gNB supports or does not support Redcap, see below. “If RedCap-specific IFRI is absent from broadcast SI, the UE considers the cell does not support RedCap.” |
| NEC |  | This Q8 is valid only when RedCap specific cell barring IEs is mandatory in Q7. As commented to Q7, there is no need to have two similar functions, i.e. mandatory RedCap specific IFRI in SIB1 is sufficient. |
| Nokia |  | UE can determine only on cell basis, not gNB basis. |
| Sequans |  | Agree with NEC |
| Apple | Not sure how to choose here, but: | SIB1-IFRI – absence (gNB does not support RedCap, and so does not transmit RedCap specific IEs in SIB1 (including the redCap specific cellBarred). SIB1-IFRI – present, then UE looks at mandatory fields 1RX/2RX for their respective barring info. |
| MediaTek |  | Agree with NEC |
| Sierra Wireless | Option 1 | If the IEs are not present, then the UE can interpret that the gNB does not support RedCap and thus barred. |
| Futurewei | Option 1 | In addition, the RedCap UE should further consider that the cell status “barred” is indicated, at least as long as the UE continues to operate as a RedCap UE (i.e., FFS if the UE falls back to legacy operation). We suggest that the cell status “barred” is indicated, not that the cell status is treated as if “barred” like in the case of being unable to acquire SIB1, because knowing that the cell doesn’t support RedCap, the RedCap UE should not come back to the same cell very soon, e.g., coming back to the same cell after merely 300 seconds may not be long enough. |
| Ericsson |  | We have earlier agreement on this, as brough up by Lenovo. We don’t think a new agreement is needed (see also our reply to Q7). |
| Qualcomm | Option 1 |  |
| Intel |  | From UE side, what’s difference between “network does not support RedCap” and “network does not allow the camp”? The UE should treat both cases as “not allowed to camp”. Or is this question dedicated for question 9? |
| Spreadtrum | Option 1 |  |
| Sharp |  | Agree with NEC |
| Samsung |  | Agree with NEC |
| vivo |  | We think in the case that RedCap UE can’t find a suitable cell which support RedCap, it should be allowed for RedCap UE to camp cells not supporting RedCap to receive the ETWS/CMAS notifications and ETWS and CMAS SIBs. |
| DENSO |  | Agree with NEC and Lenovo. It has already been ironed out. |
| BT | Option 1 | This was already agreed in RAN2#115-e. No need to reopen the discussion. |
| ZTE |  | Agree with NEC and Lenovo. |
| CATT |  | Agree with Lenovo and NEC. |
| Xiaomi | Option1 | Yes, we only agreed RedCap-specific IFRI case. But for RedCap-specific Cellbar, we can treat the same way. |

Summary：

This question is pending on P7, we can discuss this later.

In case that the RedCap specific IFRI/cellBarred is absent, the UE considers that the cell does not support RedCap and cannot camp on this cell. It is, however, not clear how the UE performs intra-frequency reselection in this case since there is no RedCap specific IFRI to follow.

So, in case the cell is barred due to not supporting RedCap, how to consider the intra-frequency cell reselection for RedCap UE:

Option 1: consider IFRI as “allowed”;

Option 2: follow the legacy IFRI in MIB;

Proposal 9: Intra-frequency cell reselection is considered as “allowed” by RedCap UEs, in case the cell is barred due to not supporting RedCap.

Question 9: Do you agree on the above proposal 9? (You may also give your preferred option)

| Companies | Yes or No? | Comments |
| --- | --- | --- |
| OPPO | Yes | This is to reduce the missing opportunities for RedCap UEs to reselect to another intra-F RedCap-supporting cell. |
| Fujitsu | Option2 | We prefer that RedCap UEs should follow the legacy IFRI in MIB in case that the RedCap specific IFRI is absent. |
| Lenovo | None | We think Redcap UE shall ignore the IFRI bit setting from MIB. Why should a Redcap UE follow IFRI from a cell that does not support Redcap? |
| NEC | Yes | Unfortunately, the legacy gNB does not take into account support of RedCap in neighbour cells by different gNBs. There might be the case where neighbour cell in the same frequency supports the RedCap. |
| Nokia | No | Option 2 seems more accurate. |
| Sequans | Yes | Mostly we don’t see a good reason to prevent in spec the UE from trying and leave it to implementation |
| Apple | Op2 |  |
| MediaTek | Do not specify | When a UE finds a cell that doesn’t support RedCap, further actions in Idle mode should be left to UE implementation |
| Sierra Wireless | Yes |  |
| Futurewei | It is a combination of both. Please see comments | Given RedCap-only cell is not supported any more, we think that the IFRI for RedCap UE should be decided by two factors, namely the IFRI in MIB and the RedCap-specific IFRI in SIB1, just like the cell status for RedCap UE is decided by both the cellBarred in MIB and the corresponding RedCap-specific barring indication in SIB1. More specifically, we think the cell status for RedCap UE should be decided as the “OR” of “barred” between the cellBarred in MIB and the corresponding RedCap-specific barring indication in SIB1, i.e., as long as one of them is ‘barred” or treated as if “barred”, the cell status for the RedCap UE is “barred”; otherwise, it is “notBarred”. Similarly, the IFRI for RedCap UE should be decided as the “OR” of “notAllowed” between the IFRI in MIB and the RedCap-specific IFRI in SIB1, i.e., as long as one of them is “notAllowed”, the IFRI for the RedCap UE is “notAllowed”; otherwise, it is “allowed”. Note that missing SIB1 or missing IFRIRedCap in SIB1 should be treated by RedCap UEs as if “allowed”, just as missing MIB today. |
| Ericsson | No | We think option 2 is correct, i.e. in the case RedCap-specific IFRI is not present, the UE should follow the IFRI in MIB. |
| Qualcomm | Yes | RedCap UEs should not read/use legacy IFRI. RedCap may have different access control policy from legacy. |
| Intel | No | Normally the network will be upgraded simultaneously for the same frequency. We consider the UE can follow the legacy IFRI in MIB, i.e. option 2. |
| Spreadtrum | Yes |  |
| Sharp | No | Option 2 |
| Samsung | Yes |  |
| vivo | Yes | It is not logical for RedCap UEs to follow legacy IFRI, as it may not valid for RedCap in real deployment. |
| DENSO | No | Support Option 2, since NW can still apply the same reselection policy as in the legacy. |
| Huawei, HiSilicon | Yes |  |
| BT | No | Option 2 is preferable |
| ZTE | No | In real deployment, it is most likely intra-frequency neighbor cells do not support RedCap either. For sake of UE power consumption, UE can consider IFRI as “not allowed”. And we don’t think it is proper to follow IFRI in MIB which is configured for non-RedCap UE. |
| CATT |  | Option 1 maybe. But this depends on the deployment strategy of the operator, that is, whether the operator wants to deploy two cells in the same area, one supports Recap UE, and one not supports. |
| Xiaomi | - | Option2 is not preferred. Maybe depends on operator’s requirement to allow reselection or not. |

Summary:

Option 1: OPPO, NEC, Sequans, Sierra Wireless, Spreadtrum, Samsung, vivo, Huawei, CATT

Option 2: Fujitsu, Nokia, Apple, Ericsson, Intel, Sharp, DENSO, BT

Option 3: Not to define. Lenovo, MTK,

Option 4: “not allowed”. ZTE

Rapporteur understand both options work. It is proposed to have a quick decision online.

Proposal 9: [To discuss] In case the cell is barred due to not supporting RedCap, intra-frequency cell reselection is considered by RedCap UE as:

Option 1: “allowed”; [9]

Option 2: follow the legacy IFRI in MIB; [8]

Option3: Not to specify (i.e. UE implementation) [2]

  - Cell selection

Considering there may be bad coverage of UL for RedCap UE, it is better to introduce the specific cell selection parameters for RedCap UEs to allow them to select a suitable cell.

Proposal 11: Support the RedCap specific cell selection parameters.

Question 10: Do you agree on the above proposal 11? (You may also want to discuss other RedCap specific parameters, please answer the question first and the indicate others if any.)

| Companies | Yes or No? | Comments |
| --- | --- | --- |
| OPPO | No for this release | We can consider this when we discuss low power class RedCap in Rel-18. There is no strong motivation in Rel-17. |
| Fujitsu | Yes | We support to introduce the specific cell selection parameters for RedCap UEs with e.g. 1 RX. |
| Lenovo | No | If there are UL coverage issues in a cell then it affects all UEs (normal and Redcap UEs). NW will consider this by setting appropriate values of the cell selection parameters which are broadcast in the cell. We wonder why we need to treat Redcap UEs differently compared to normal UEs. |
| NEC |  | Maybe, but we think RAN2 should wait for the outcome of Coverage enhancement WI. |
| Nokia | No |  |
| Sequans | No | We don’t currently see a strong reason for this. We are fine to not agree anything for now as well. |
| MediaTek | No | We do not see a strong reason for this to be considered in Rel-17 |
| Sierra Wireless | No |  |
| Futurewei | Yes |  |
| Ericsson | No | Have similar thinking as OPPO that this might be more relevant e.g. if a lower power class is introduced in a later release. |
| Qualcomm | Yes | Especially for RedCap UEs with 1Rx |
| Intel | Yes | Rx specific threshold is needed for cell (re)selection considering that the coverage could be different for UEs with different Rx. |
| Spreadtrum | No |  |
| Sharp | No |  |
| Samsung | No | We agree with its necessity, but not sure RAN2 can discuss it in Rel-17. |
| vivo | Yes | Besides, we also think the black list could be included in cell selection parameters for RedCap UEs. |
| DENSO | Yes | Single Rx UE specific parameters are necessary to compensate the coverage. |
| Huawei, HiSilicon | Yes |  |
| BT | No | There is no need to consider RedCap in a different way than normal UEs. If RedCap devices have a different map of coverage, the cost to deploy RedCap in the network will increase due to all the extra validations required. |
| ZTE | No | For cell selection related thresholds, our answer is no. But we think RAN2 need to discuss whether separate reselection priorities can be configured for RedCap, may relate to the newly added P13 below. |
| CATT | Yes |  |
| Xiaomi | No | There is CE that can be applied to Redcap and non-Redcap UEs. |

Summary:

There seems majority want to postpone this discussion. Note that we have touch this discussion several times in previous meeting with no conclusion.

Proposal 11: [Easy] RAN2 deprioritize/postpone the discussion on the RedCap specific cell (re)selection related parameters.

One late comment provided by Nokia via email

“In the previous meeting neighboring cell acceptance of RedCap UE access in system information was discussed in the e-mail discussion, but it was never discussed online. We propose to continue the discussion in this meeting. Maybe the whole discussion can be just referred in this e-mail discussion and it can be then discussed online? Or alternatively new discussion point can be added?

- Proposal 11 [To discuss] Whether information on neighboring cell acceptance of RedCap UE access is provided in system information.

[Rapporteur]: P11 was not discussed online so far.”

Rapporteur would like to refer to the last meeting offline summary R2-2108892 to give one low priority proposal for online discussion.

Proposal 13: Discuss whether system information should provide information on which cells accept RedCap UE access, and if, what this information should include (e¸g. support, barring?) and in which form (e.g. NCell, allow-list, exclude-list)

  - UAC

SA1 [2] has concluded that no new UAC Access Category or Access Identity is required, since current UAC can apply also to RedCap UEs, and UAC Access Categories and Access Identities are not intended to differentiate UEs based on device radio capabilities.

In order to provide flexible and separate control for RedCap UEs, as the network may want to control RedCap UEs separately from non-RedCap UEs, it is better to broadcast a specific set of UAC parameters for RedCap UEs, which is similar to NB-IoT and eMTC. However, it will increase signalling overheads if a complete set of UAC parameters (i.e. UAC-BarringInfoSet which contains uac-BarringFactor, uac-BarringTime and uac-BarringForAccessIdentity) specific to Redcap UEs is broadcast. Therefore, how to design the RedCap specific UAC parameters needs to be further considered in order to reduce the signalling overhead and enable a flexible configuration. For example, based on the CT1 LS [3] discussed in RAN2#115-e meeting, there is a solution provided by CT1 that an offset value can be introduced to the unified access control barring information. By this way, a specific UAC parameters can be provided for RedCap UEs with only limited signalling overhead.

Proposal 12: Support the RedCap specific UAC parameters, with consideration of signalling overhead reduction.

Question 11: Do you agree on the above proposal 12?

| Companies | Yes or No? | Comments |
| --- | --- | --- |
| OPPO | No | Given that SA1’s LS indicates no requirement for separate UAC control for RedCap UEs, we should not introduce this in Rel-17. |
| Fujitsu | No |  |
| Lenovo | No | Acc. to SA1 response a Redcap UE applies the existing UAC framework. Therefore, we see no need to apply UAC parameters specific for Redcap UEs. Furthermore, the concerned CT1 LS is about the MINT feature where disaster roaming UE applies a new Access Identity 3 for access attempts in a cell that offers disaster roaming service. For such UEs either a specific uac-BarringFactor or uac-BarringFactorOffset will be configured (which option to apply has not been decided yet in RAN2). Therefore, the use case is completely different from Redcap and cannot be applied for Redcap UEs. |
| NEC | No | There are already many ways to differentiate RedCap UEs from legacy or non-RedCap UEs, e.g. separate ROs or preambles (for Msg1 early identification) and specific LCIDs (for Msg3 early identification). The latter is actually intended to reject the access only from RedCap UEs. On top of these, we do not see strong motivation to introduce something more which cause SIB1 overhead. |
| Nokia | No |  |
| Sequans | No |  |
| Apple | No |  |
| MediaTek | No |  |
| Sierra Wireless | No |  |
| Futurewei | Yes | But we also need CT1’s inputs. |
| Ericsson | No |  |
| Qualcomm | No | We should respect SA1’s decision. |
| Intel | No | SA1 did not identify the requirement on separate UAC. Do not see the reason why RAN2 needs to discuss this. |
| Spreadtrum | No |  |
| Sharp | No |  |
| Samsung | No | Prefer to accept SA1's view. |
| vivo | Yes | To provide same quality of service, RedCap UE will require more radio resource than non-RedCap due to RedCap UE’s low capability. Hence, it’s natural for the operator to reject more access attempt from RedCap than that from non-RedCap, in order to serve more UEs when congestion occurs. Without RedCap specific UAC parameters, the operator can either bar the RedCap UE via SIB1 or reject an access request from RedCap after UAC is performed, e.g. during RRC connection setup procedure. In our understanding, the former is overkilled, while the latter leads to radio resource waste comparing to block UE via UAC. Hence, RedCap specific UAC parameters bring benefit when radio resource is exhausted. Our understanding on the intention for CT1 LS is “UAC Access Categories and Access Identities are not intended to differentiate UEs based on device radio capabilities”, while from the access control perspective the traffic (voice, data) from a RedCap UE would not be treated differently to the traffic (voice, data) of the same type from a non-Redcap UE, which should be the principle we need to consider. If there is no RedCap specific UAC parameters, RedCap UEs which have same service as non-RedCap UEs will be treated differently from non-RedCap UEs when cell barring. |
| DENSO | No | Agree to respect SA1’s decision. |
| Huawei, HiSilicon | Yes |  |
| BT | No |  |
| ZTE | No | Agree to respect to SA1’s decision. |
| CATT | No |  |
| Xiaomi | No |  |

Summary:

There seems clear majority not supporting this.

Proposal 12: [Easy] Do not support the RedCap specific UAC parameters.

- Phase II discussion

Following proposals are selected to make further progress. NOTE that rapporteur will be a little aggressive on the updated questions, to see if we can move forward a bit.

Proposal 4: [To discuss] [16 vs. 5] Two reserved LCIDs are used for CCCH and CCCH1 cases respectively for Msg3 early identification.

Rapporteur propose to make a working assumption based on the clear majority views. The use of WA means that if RAN2 found any issue to occupy two reserved LCIDs in the future, we can just delete/return the one for CCCH1 back to be reserved.

Proposal 4_update: Working assumption: Two reserved LCIDs are used for CCCH and CCCH1 cases respectively for Msg3 early identification.

Question II-1: Do you have concern on the above updated proposal 4?

| Companies | Yes or No? | Comments |
| --- | --- | --- |
| MediaTek | No | We are ok with such a working assumption |
| Futurewei | No | We support P4 and are OK with it being a working assumption. |
| Qualcomm | No |  |
| InterDigital | No |  |
| Samsung | Yes | As indicated to the email reflector, it is about wasting few remaining LCID space, and also about future-proof of NR technology. Since eLCID space cannot be used for this case, the use of (few remaining) LCID needs to be technically justified, instead of just wasting the space. |
| DENSO | Yes | Agree with Samsung |
| Sharp | No |  |
| NEC | No | no problem as working assumption |
| Spreadtrum | No | We are fine with this WA. |

Proposal 5: [To discuss] In MAC perspective, RedCap UE uses the dedicated LCID for Msg3 early identification:

Option 1: whenever the Msg3 includes the CCCH data [15];

Option 2: whenever the Msg3 includes the CCCH data and Msg1 early identification is not configured [2];

Option 3: whenever the Msg3 includes the CCCH data and Msg3 early identification is enabled by NW [5].

Rapporteur understand option 2/3 will cause lots of online discussion, so it is better to keep it as FFS if time does not allow. But, the common part among options is that MAC will treat all the data from RRC layer equally without differentiating the RRC message, as long as it is for CCCH data.

Proposal 5_updated: In MAC perspective, RedCap UE uses the dedicated LCID for Msg3 early identification, whenever the Msg3 includes the CCCH data. FFS on whether it requires no other precondition, or precondition as “when Msg1 early identification is not configured”, or precondition as “when Msg3 early identification is enabled by NW”.

Question II-2: Do you have concern on the above updated proposal 5?

| Companies | Yes or No? | Comments |
| --- | --- | --- |
| MediaTek | No | We are ok with the proposal |
| Futurewei | - | We can agree on P5 for now. If RAN2 decides that the NW can disable Msg3 early identification (which we think RAN2 should discuss), the agreement can be updated accordingly. |
| Qualcomm | No |  |
| InterDigital | No |  |
| Samsung | No but | Can we directly try to agree Option 1 to avoid future discussion? We do not see any technical reason to have such additional conditions in Options 2 and 3. |
| DENSO | No |  |
| Sharp | No |  |
| Spreadtrum | No, but | We think it is better that network can make decision whether Msg3 early identification is enable or not. |

Proposal 7: [To discuss] For the gNB supporting RedCap UE case, introduce below for 1RX and 2RX RedCap UE respectively,

Option 1: two mandatory IEs in SIB1 with {barred, notBarred} present for gNB supporting RedCap [10]

Option 2: two optional Ies in SIB1 with {barred} [6]

As an example of ASN.1 mentioned by companies, the difference among option ½ is clear. There seems no significant difference among option ½

SIB1-v17xy-Ies ::=               SEQUENCE {

cellAccessRelatedInfoRedCap-r17     CellAccessRelatedInfoRedCap-r17              OPTIONAL,   -- Need R

nonCriticalExtension                SEQUENCE {}                                             OPTIONAL

}

CellAccessRelatedInfoRedCap-r17 ::=               SEQUENCE {

Option 1:

cellBarredRedCap1Rx-r17            ENUMERATED {barred, notBarred},

cellBarredRedCap2Rx-r17            ENUMERATED {barred, notBarred}

Option 2:

cellBarredRedCap1Rx-r17            ENUMERATED {barred}                            OPTIONAL,  -- Need R

cellBarredRedCap2Rx-r17            ENUMERATED {barred}                            OPTIONAL  -- Need R

}

Question II-3: Which option do you prefer?

It is good if you can also indicate whether it is acceptable for the option you are not preferring. Also, it will be good if you can compromise to only select among option ½.

| Companies | Option ½? | Comments |
| --- | --- | --- |
| MediaTek | See response | We are ok with the example ASN.1 structure shown above for Option 1. However, we are not ok with the text describing option 1, i.e. two mandatory Ies in SIB1 with {barred, notBarred} present for gNB supporting RedCap The text above seems to imply that the barring information is mandatory for a gNB supporting RedCap. If a gNB that supports RedCap does not need to bar based on number of antennae, it should be free to skip transmission of cellAccessRelatedInfoRedCap-r17 IE. |
| Futurewei | Option 1 |  |
| Qualcomm | Option 1 only | They think RedCap-specific barring fields should be mandatorily present in SIB1. When one of them is absent, say, cellBarredRedCap1Rx-r17, it means that the cell does not support 1Rx, so that 1Rx RedCap Ues do not need to consider it in its future cell re-/selection. This Rx specific supporting information cannot be implemented through IFRI. |
| IntrerDigital | Option 1 | Either way works fine but Option 1 follows the legacy signalling and so looks more straight forward. Regarding to the MediaTek’s concern, for your scenario, the gNB should signal cellBarredRedCap1Rx-r17="barred" and cellBarredRedCap2Rx-r17="notBarred"? So we don’t see any problem with the current description. RedCap IFRI can be mandatory present with enumeration values “allowed” and “notAllowed” in the IE "CellAccessRelatedInfoRedCap-r17". |
| Samsung | Option 2 | Prefer option 2 but ok to support option 1 as well. |
| DENSO | Option 1 | Given that the parent field is optional, Option 1 makes sense to save bits from ASN.1 viewpoint. |
| T-Mobile USA | Option 2 | We are concerned about the level of development needed to support REDCAP UE’s. There needs to be an option for operators to deploy REDCAP UE’s without making any modifications to the network. If IE cellAccessRelatedInfoRedCap-r17 isn’t present the UE needs to be allowed to access the network. |
| Sharp | Option 1 | OK with option 1 considering CellAccessRelatedInfoRedCap-r17 is optional. |
| NEC | Option 2 | Prefer Option 2, as most of time these bits are not required in normal situation. Also, it is unclear what the previous agreement on IFRI in SIB1 (i.e. mandatory present in a cell supporting RedCap) mean together with the Option1? |
| Spreadtrum | Option 1 | We are OK with option 1, as CellAccessRelatedInfoRedCap-r17 is OPTIONAL. |

Proposal 9: [To discuss] In case the cell is barred due to not supporting RedCap, intra-frequency cell reselection is considered by RedCap UE as:

Option 1: “allowed”; [9]

Option 2: follow the legacy IFRI in MIB; [8]

Option3: Not to specify (i.e. UE implementation) [2]

Option4: “not allowed” [1]

Rapporteur understand option ½ has the clear majority view. It is proposed to exclude option ¾ for this meeting, and we make the final decision next meeting on option ½.

Proposal 9_updated: In case the cell is barred due to not supporting RedCap, it is FFS that intra-frequency cell reselection is considered by RedCap UE as “allowed” or follows the legacy IFRI in MIB.

Question II-4: Do you have concern on the above updated proposal 9?

| Companies | Yes or No? | Comments |
| --- | --- | --- |
| MediaTek | Yes | We should only specify something if there a clear justification. The default option is Option 3, i.e. do not specify anything. What is the justification to specify UE behavior in this case? Without a clear justification, neither Option 1 not Option 2 should be considered. |
| Futurewei | Yes | The UE should follow the legacy IFRI in MIB in this case. Given RAN2 has agreed that when cell status “barred” is indicated in MIB, it applies to RedCap Ues, when the legacy IFRI in MIB is “notAllowed”, it should also apply to RedCap Ues. Essentially, a RedCap UE should use the “OR” of “notAllowed” between the IFRIs in MIB and SIB1 to determine the final IFRI, just as it uses the “OR” of “barred” indication between MIB and SIB1. Given we have ruled out RedCap-only cell, is it a valid NW configuration where the legacy IFRI in MIB is “notAllowed” but the IFRIRedCap in SIB1 is “allowed”? |
| Qualcomm | No | It seems to us that Option 1 and 3 practically are the same, because generally if something is not specified in the spec, it is allowed. |
| Interdigital | YES | Option 1 makes sense as the NW RedCap capability is supposed to be per cell capability. If RedCap UE follows the legacy IFRI in MIB, then that may impact the existing deployment as operator needs to ensure that the IFRI in MIB is set to an appropriate value even for the non-RedCap capable cells. |
| Samsung | No | No concern on proposal |
| DENSO | No | O.K to keep it as FFS for the time being. |
| T-Mobile USA | Yes | This adds unnecessary complexity to REDCAP UE’s |
| Sharp | No | OK with the proposal. |
| NEC | No | Considering split views, we are OK to have one more meeting, but the issue should be decided in the next meeting. |
| Spreadtrum | No | We are fine with it. |

- Conclusion and proposals

Based on the above summary, following proposals are given.

Proposal 4: TBD

Proposal 5: TBD

Proposal 7: TBD

Proposal 9: TBD

Proposal 13: [To discuss] Discuss whether system information should provide information on which cells accept RedCap UE access, and if, what this information should include (e¸g. support, barring?) and in which form (e.g. NCell, allow-list, exclude-list)

Proposal 11: [To discuss] RAN2 deprioritize/postpone the discussion on the RedCap specific cell selection related parameters.

- Reference

- R2-2109577	Identification and access restriction of RedCap UE	Huawei, HiSilicon

- S1-211363, Reply LS on Unified Access Control (UAC) for RedCap, SA1.

- C1-213527 LS on UAC enhancements for minimization of service interruption when disaster condition applies, CT1.

- R2-2108892	[offline 104] Identification, access and camping	Ericsson