3GPP TSG RAN WG2 Meeting #116-e	                                        R2-2111305

- Electronic, 1st – 12th November 2021

Agenda item:		4.5, 7.4

Source:			Lenovo

Title:		Report from email discussion [AT116-e][205][LTE] Miscellaneous LTE CRs (Lenovo)

Document for:	 	Discussion and decision

# Introduction

This report summarizes the email discussion below that took place during RAN2#116-e meeting:

- [AT116-e][205][LTE] Miscellaneous LTE CRs (Lenovo)

Scope:

    - Discuss LTE CRs marked for this discussion (if needed)

Intended outcome:

    - Discussion report in R2-2111305

    - Agreeable CRs (if any)

Deadline for providing comments, for rapporteur inputs, conclusions and CR finalization:

    - Initial deadline (for company feedback):  1st week Thu, UTC 0900

    - Initial deadline (for rapporteur summary):  1st week Thu, UTC 1700

    - Deadline for CR finalization: 2nd week Wed, UTC 0900

# Reference

The following documents are treated in this email discussion:

[1]	R2-2109828	Addition of missing TEI15 features	Lenovo, Motorola Mobility (Rapporteur)	CR	Rel-15	36.306	15.10.0	1825	-	F	TEI15

[2]	R2-2109829	Addition of missing TEI15 features and other corrections	Lenovo, Motorola Mobility (Rapporteur)	CR	Rel-16	36.306	16.6.0	1826	-	F	TEI15, 5G_V2X_NRSL-Core, LTE_NR_DC_CA_enh-Core

[3]	R2-2109830	Corrections to HSDN cell reselection enhancement	Lenovo, Motorola Mobility	CR	Rel-15	36.331	15.15.0	4726	-	F	TEI15

[4]	R2-2109831	Corrections to HSDN cell reselection enhancement	Lenovo, Motorola Mobility	CR	Rel-16	36.331	16.6.0	4727	-	A	TEI15

[5]	R2-2111148	Correction to application layer measurement and reporting	Google Inc.	CR	Rel-15	36.331	15.15.0	4746	-	F	LTE_QMC_Streaming-Core

[6]	R2-2111149	Correction to application layer measurement and reporting	Google Inc.	CR	Rel-16	36.331	16.6.0	4747	-	A	LTE_QMC_Streaming-Core

[7]	R2-2110805	Miscellaneous corrections	Nokia (rapporteur), Qualcomm Incorporated	CR	Rel-16	36.300	16.6.0	1350	-	F	LTE_feMTC-Core, LTE_1024QAM_DL-Core, TEI16

[8]	R2-2111136	Correction on cellsTriggeredList 	Samsung R&D Institute UK	CR	Rel-16	36.331	16.6.0	4745	-	F	LTE_Aerial-Core

[9]	R2-2111178	Conditional Handover with Two Trigger Events	MediaTek Inc.	CR	Rel-16	36.306	16.6.0	1832	-	F	LTE_feMob-Core

[10]	R2-2109803	Discard of received segments of RRC messages	Samsung	CR	Rel-16	36.331	16.6.0	4725	-	F	TEI16

# Contact information

| Company | Contact Name, Email |
| --- | --- |
| Lenovo | Hyung-Nam Choi, hchoi5@lenovo.com |
| Huawei, HiSilicon | jun.chen@huawei.com |
| Samsung | seungri.jin@samsung.com |
| Google | frankwu@google.com |

# Discussion

## Addition of missing TEI15 features and other corrections

In the CRs [1], [2] the following changes to TS 36.306 for R15 and R16 are proposed:

- For both R15 and R16:

- To add description of the UE support for the two missing TEI15 features below:

- MDT measurement logging in any cell selection state: The support of this feature has been defined as conditionally mandatory for UEs which support logged measurements in RRC_IDLE upon request from the network.

- MBMS reception using Receive Only Mode: The support of this feature has been defined as an optional feature without capability signaling for UEs supporting MBMS.

- For R16 only:

- To align the description of the capability v2x-SupportedBandCombinationListEUTRA-NR-r16 with its description in TS 36.331.

- To clarify in the description of eutra-IdleInactiveMeasurements-r16 that a UE indicating support of RRC connection suspension can report the CRS-based RRM measurements performed in RRC_IDLE when the UE is in RRC_CONNECTED.

- It should be noted that this change was already discussed in RAN2#114-e meeting with the conclusion that the change is correct and could be merged into 36.306 rapporteur miscellaneous corrections CR once such is proposed.

Question 1: Do companies agree on the proposed changes to TS 36.306 for R15 and R16?

| Company | Yes/No | Additional comments |
| --- | --- | --- |
| Lenovo | Yes | Proponent |
| Huawei, HiSilicon | Yes | For the following change, we think the description should be aligned with TS 36.331 definition, i.e. for anyCellSelectionDetected, it states that this field is used to indicate the detection of any cell selection state, as defined in TS 36.304 [4]. So we proposed the following change ### 7.10.x MDT measurement logging in any cell selection state It is mandatory to support the detection of any cell selection stateMDT measurement logging in any cell selection state in RRC_IDLE for UEs which are supporting logged measurements in RRC_IDLE upon request from the network as specified in TS 36.331 [5] and TS 36.304 [14]. |
| Samsung | Yes |  |
| Google | Yes | The change proposed by Huawei is confusing. It makes the capability bit indicate support of the detection of any cell selection state. This capability bit should only indicate that that the UE supports MDT measurement logging in any cell selection state. Therefore, we are fine with the changes proposed by Lenovo. “-“ is missing in the “rev” field on the coversheet for both CRs. |

## Corrections to HSDN cell reselection enhancement

In the CRs [3], [4] the following changes to TS 36.331 for R15 and R16 are proposed:

- In SIB3: to specify the presence of field cellReselectionInfoHSDN-r15 (containing the field cellEquivalentSize-r15) condition to the presence of field speedStateReselectionPars (containing the parameters to determine UE mobility state). Otherwise, there may be the risk that the network may incorrectly configure the field cellReselectionInfoHSDN-r15 without the speed dependent reselection parameters in the field speedStateReselectionPars.

- In SIB5: to dummify the redundant field hsdn-Indication-r15 in IE InterFreqCarrierFreqInfo-v1530 and to remove the concerned field description since the presence of interFreqNeighHSDN-CellList-r15 in IE InterFreqCarrierFreqInfo-v1530 already indicates the presence of HSDN cells on the concerned carrier frequency.

Rapporteur’s comment: The proposed changes are motivated from the outcome of the email discussion [Post115-e][090][TEI17] Mobility-state-based cell reselection for NR High Speed railway Dedicated Network (CMCC) wherein the CRs for introducing the HSDN cell reselection enhancements to NR were discussed. Amongst other for the CR to TS 38.331 it was agreed to specify the presence of field cellEquivalentSize-r17 in SIB2 condition to the presence of field speedStateReselectionPars (containing the parameters to determine UE mobility state), and not to introduce the redundant fields hsdn-Indication-r17 in SIB4 (for inter-frequency cell reselection) and hsdn-EUTRA-Indication-r17 in SIB5 (for inter-RAT cell reselection).

Question 2: Do companies agree on the proposed changes to TS 36.331 for R15 and R16?

| Company | Yes/No | Additional comments |
| --- | --- | --- |
| Lenovo | Yes | Proponent |
| Huawei, HiSilicon | Yes |  |
| Samsung | Yes |  |
| Google | Yes |  |

## Correction to application layer measurement and reporting

In the CRs [5], [6] the following change to TS 36.331 for R15 and R16 is proposed:

- In subclause 5.3.5.8 to specify in case measConfigAppLayer is released as result of full configuration, the UE performs the actions as specified in 5.3.10.9 (Other configuration), i.e.

- RRC informs upper layers that the measConfigAppLayer is released, and

- the UE discards received application layer measurement report information from the upper layers, and

- the UE considers itself not to be configured to send application layer measurement reports.

Otherwise, the UE may still perform application layer measurement and reporting, which is not expected by the network.

- 5.3.5.8 Radio Configuration involving full configuration option The UE shall: 1> if the UE is connected to EPC: 2> release/ clear all current dedicated radio configurations except for the following: - the MCG C-RNTI, - the MCG security configuration, - the PDCP, RLC, logical channel configurations for the RBs, - the logged measurement configuration; 1> else if the UE is connected to 5GC: 2> release/ clear all current dedicated radio configurations except for the following: - the MCG C-RNTI, - the MCG security configuration, - the configurations (SDAP if configured, PDCP, RLC and logical channel) for the RBs; NOTE 1: Radio configuration is not just the resource configuration but includes other configurations like MeasConfig and OtherConfig. In case (NG)EN-DC is configured, this also includes the entire NR SCG configuration. Such NR SCG configuration does not include the DRB configuration as configured by nr-RadioBearerConfig1 and nr-RadioBearerConfig2). - if the measConfigAppLayer is released as a result of the full configuration: 2> perform actions as if the measConfigAppLayer is received and set to release, as specified in 5.3.10.9; 1> if the RRCConnectionReconfiguration message includes the mobilityControlInfo: 2> release/ clear all current common radio configurations; 2> use the default values specified in 9.2.5 for timer T310, T311 and constant N310, N311; 1> else: …

Question 3: Do companies agree on the proposed change to TS 36.331 for R15 and R16?

| Company | Yes/No | Additional comments |
| --- | --- | --- |
| Lenovo | No | If measConfigAppLayer needs to be released due to full configuration we think that it will be done via otherConfig IE. So, there is no need for the additional specification in 5.3.5.8. |
| Huawei, HiSilicon | No | Share similar views as Lenovo. |
| Samsung | No | Share similar views as Lenovo. |
| Google | Yes | Proponent. In full configuration, the UE (i.e., RRC) releases the otherConfig IE as specified in section 5.3.5.8. However, upper layers are not aware of the full configuration so the upper layers does not release the measConfigAppLayer. The UE should not be required to do the application layer measurement after the full configuration. Besides, in the full configuration case, the UE should discard received application layer measurement report information received from the upper layers. Currenly, RRC does not release application layer measurement report information received from the upper layers. |

## Miscellaneous stage 2 corrections

In the CR [7] the following changes to TS 36.300 for R16 are proposed:

- In subclause 5.1.2 to add "1024QAM" to the list of supported DL modulations since 1024QAM was introduced in Rel-15.

- In subclause 23.7a to delete the below text which was introduced in Rel-14 by CR0972r2 in R2-1702405. This is to avoid duplication of description in TS 36.306 and misinterpretation in that the larger UL maximum TBS size for Category M1 BL UE is same as the larger UL maximum TBS size for Category M2 BL UE.

A Category M2 BL UE supports a larger DL and UL maximum TBS size for unicast compared to a Category M1 BL UE. A Category M1 BL UE may support a larger UL maximum TBS size indicated by a separate UE capability.

Question 4: Do companies agree on the proposed changes to TS 36.300 for R16?

| Company | Yes/No | Additional comments |
| --- | --- | --- |
| Lenovo | Yes (for 1st change), No (for 2nd change) | On 1st change we agree but we wonder whether it should be added already from R15 since it is an essential information. On 2nd change: We think that the two sentences are not wrong, so they can be kept. Furthermore, to our understanding the intention of the 2nd sentence below was motivated from the fact that cat M1 was specified to support the TBS size of 1000 bits or 2984 bits. So, if a cat M1 UE wants to indicate that it supports 2984 bits in CE mode A then it has to indicate value “m1” in ue-CategoryUL and ce-PUSCH-NB-MaxTBS-r14 in the UE capability signaling. “A Category M1 BL UE may support a larger UL maximum TBS size indicated by a separate UE capability.” |
| Huawei, HiSilicon | Yes for both changes |  |
| Google | Yes |  |

## Corrections on measurement report triggering for aerial vehicles

In the CR [8] the following changes to TS 36.331 for R16 are proposed:

- In 5.5.4.1 to change “larger than or equal to” into “smaller than” in the first condition for comparing the number of cell(s) in the cellsTriggeredList with the number of cells given by numberOfTriggeringCells.

- In 5.5.4.1 to remove the second condition for comparing the number of cell(s) in the cellsTriggeredList with the number of cells given by numberOfTriggeringCells, so that only a larger or equal number of cells can trigger a measurement report.

Reason for the changes is that without the changes measurement reporting is not triggered even when the configured events are fulfilled.

Rapporteur’s comment: Support for Aerial Vehicles was introduced in R15, however the changes are proposed for R16. For the discussion the proponent is asked to explain the reason to fix this issue only in R16 and not from R15.

- 5.5.4 Measurement report triggering - 5.5.4.1 General If security has been activated successfully, the UE shall: 1> for each measId included in the measIdList within VarMeasConfig: … 2> if the triggerType is set to event and if the corresponding reportConfig includes numberOfTriggeringCells, and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more applicable cells for all measurements after layer 3 filtering taken during timeToTrigger defined for this event within the VarMeasConfig: 3> If the VarMeasReportList does not include a measurement reporting entry for this measId (a first cell triggers the event): 4> include a measurement reporting entry within the VarMeasReportList for this measId; 3> If the number of cell(s) in the cellsTriggeredList is larger than or equalsmaller than to numberOfTriggeringCells: 4> include the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId; 3> else: 4> include the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId; 4> If the number of cell(s) in the cellsTriggeredList is larger than or equal to numberOfTriggeringCells: 54> set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0; 54> initiate the measurement reporting procedure, as specified in 5.5.5; …

Question 5: Do companies agree on the proposed changes to TS 36.331 for R16?

| Company | Yes/No | Additional comments |
| --- | --- | --- |
| Huawei, HiSilicon |  | Wait for more explanations based on Rapp’s comment. |
| Samsung | Yes | Thanks for rapporteur to point out that this aerial vehicle has been adopted from Rel-15. We didn’t catch that by mistake, this correction should be done from Rel-15 spec to Rel-16 one as you pointed. |
| Google | Yes |  |

## Conditional Handover with Two Trigger Events

In the CR [9] the following change to TS 36.306 for R16 is proposed:

- To change the conditionally mandatory support of cho-TwoTriggerEvents-r16 to optional if the UE supports conditional handover. The reason is that some UEs may not be able to support evaluation of two measIds and CHO execution when both events are satisfied.

Rapporteur’s comment: There is a companion CR R2-2111173 to TS 38.306 proposing the same for NR, see AI 6.1.4.1.1. This CR will be discussed in “[008][NR16]” and to avoid duplicated discussion rapporteur suggests not to discuss the LTE CR in this email discussion. Hence, interested companies are asked to provide comments to this CR along with the companion NR CR in “[008][NR16]”.

## Discard of received segments of RRC messages

In the CR [10] the following changes to TS 36.331 for R16 are proposed:

- To specify in subclause 5.3.12 that upon leaving RRC_CONNECTED or RRC_INACTIVE the UE shall discard any segments of segmented RRC messages received as shown below. Otherwise, the UE may not discard any segments of RRC message upon transition to RRC_IDLE and at next RRC connection, they may be wrongly considered while assembling for the RRC message.

### 5.3.12 UE actions upon leaving RRC_CONNECTED or RRC_INACTIVE Upon leaving RRC_CONNECTED or RRC_INACTIVE, the UE shall: - reset MAC; 1> if leaving RRC_INACTIVE was not triggered by the reception of RRCConnectionRelease including idleModeMobilityControlInfo or altFreqPriorities: … - if leaving RRC_CONNECTED was triggered by suspension of the RRC: … 1> else: 2> release the wlan-OffloadConfigDedicated, if received; NOTE 2: BL UEs or UEs in CE verifies validity of SI when released to RRC_IDLE. 1> discard any segments of segmented RRC messages received; 1> release the LWA configuration, if configured, as described in 5.6.14.3; 1> release the LWIP configuration, if configured, as described in 5.6.17.3;

- To add “of the message” in subclause 5.6.25.3 as shown below. This is to clarify that these segments are pertaining to the assembled RRC message only.

- 5.6.25.3 Reception of DLDedicatedMessageSegment by the UE Upon receiving DLDedicatedMessageSegment message, the UE shall: 1> store the segment; 1> if all segments of the message have been received: 2> assemble the message from the received segments and process the message according to 5.3.5 for the RRCConnectionReconfiguration message or 5.3.3.4a for the RRCConnectionResume message; 2> discard all segments of the message.

Rapporteur’s comment: The first change aligns with what has been specified in TS 38.331. In RAN2#112-e (November 2020) the CR2151 in R2-2009985 from MediaTek was agreed with which the condition below was added in subclause 5.3.11 (UE actions upon going to RRC_IDLE):

1>	discard any segments of segmented RRC messages stored according to 5.7.6.3;

And on CR cover page the following reason for change was given:

In certain cases, the UE can transition to RRC_IDLE while awaiting segments of a downlink RRC message, e.g.:

- If upper layers indicate release of the RRC connection, which may occur for several reasons, including T3540 expiry or UE inability to send a follow-on request for an unsupported feature after receiving REGISTRATION ACCEPT from the network (section 5.3.1.3 of TS 24.501, last paragraph).

- If T319 expires while waiting for segments of the RRCResume message.

- If cell re/selection occurs while T319 is running.

Question 6: Do companies agree on the proposed changes to TS 36.331 for R16?

| Company | Yes/No | Additional comments |
| --- | --- | --- |
| Lenovo | Yes (for 1st change), No (for 2nd change) | On the 1st change: we agree but we suggest to update CR cover page to clarify the cases in which the UE discards received segments of RRC messages (similar as it was done in the NR CR) and that it aligns with the agreed NR CR. On the 2nd change: due to the condition “1> if all segments of the message have been received” it should be clear that these segments pertain to the assembled RRC. |
| Huawei, HiSilicon | Yes for 1st change | The second change is not needed because the top condition is about the DLDedicatedMessageSegment message, and then all sub-level descriptions are only about the message. |
| Samsung | Yes for 1st change | Proponent. Also agree to suggestion from Lenovo to update CR cover page (i.e. aligning with NR CR) |
|  | Yes for 2nd change | Proponent. Just to respond to Huawei's comment, it should be noted that all instances of "message" in description relate to complete RRC message (i.e. RRCConnectionReconfiguration message or RRCConnectionResume message), whereas top-line refers to a segmented "message" (i.e. DLDedicatedMessageSegment message). Therefore, there remains an ambiguity. Intention of the proposed change is to clarify that discard is for all the segments of the assembled RRC message only and not for all stored segments. This clarification is needed as there is a possibility that UE may have segments corresponding to more than one RRC message. |
| Google | Yes for 1st change |  |

# Conclusion

Based on company’s feedback the following proposals are made:

<To be updated>