3GPP TSG RAN WG2 Meeting #116-e	                                        R2-2111406

- Electronic, 1st – 12th November 2021

Agenda item:		4.2

Source:			Lenovo

Title:		[AT116-e][401][eMTC R15] Scheduling restrictions of positioning SI messages (Lenovo)

Document for:	 	Discussion and decision

# Introduction

This report summarizes the email discussion below that took place during RAN2#116-e meeting:

- [AT116-e][401][eMTC R15] Scheduling restrictions of positioning SI messages (Lenovo)

Scope: Check whether the intention is agreeable and there is sufficient support
in principle; collect initial comments regarding the wording etc.

Intended outcome: Report in R2-2111406

Deadline: Wednesday 2021-11-03 12:00 UTC

# Reference

The following documents are treated in this email discussion:

[1]	R2-2109514	Summary of discussion on missing scheduling restrictions of positioning SI messages for eMTC	Lenovo, Motorola Mobility	discussion	Rel-15	LCS_LTE_acc_enh-Core

[2]	R2-2109515	Addition of scheduling restrictions of positioning SI messages for eMTC	Lenovo, Motorola Mobility	CR	Rel-15	36.331	15.15.0	4691	-	F	LCS_LTE_acc_enh-Core	R2-2107261

[3]	R2-2109516	Addition of scheduling restrictions of positioning SI messages for eMTC	Lenovo, Motorola Mobility	CR	Rel-16	36.331	16.6.0	4692	-	A	LCS_LTE_acc_enh-Core	R2-2107262

# Contact information

| Company | Contact Name, Email |
| --- | --- |
| Qualcomm | mdhanda@qti.qualcomm.com |

# Discussion

## Issue on missing scheduling restrictions of positioning SI messages for eMTC

In [1] a summary of discussion on the issue of missing scheduling restrictions of positioning SI messages for eMTC is provided which took place in the Positioning session at RAN2#115-e:

- If positioning assistance data are broadcast to BL/CE UEs which are camped on an LTE cell that supports this type of UEs then majority of companies confirmed that

- Positioning SIBs (SystemInformationBlockPos) in PosSystemInformation-r15-IEs are transmitted in the BR version of SI message (SystemInformation-BR-r13).

- Scheduling information of the positioning SI messages (posSchedulingInfoList-r15) is sent in the BR version of SIB1 (SystemInformationBlockType1-BR-r13).

- Scheduling restrictions given by schedulingInfoList-BR-r13 are currently missing for scheduling of positioning SI messages to BL/CE UEs.

- In order to fix this issue, the solution to introduce a new field posSchedulingInfoList-BR-r15 (containing the missing scheduling restrictions for scheduling of positioning SI messages to BL/CE UEs) in SIB1-BR was agreed. However, on the actual changes a slight majority of companies preferred to have them reviewed in the eMTC session. Therefore, the corresponding CRs to TS 36.331 for introducing the solution were only endorsed.

Question 1: Do companies have comments on the outcome of discussion from the Positioning session on the issue of missing scheduling restrictions of positioning SI messages for BL/CE UEs?

| Company | Yes/No | Additional comments |
| --- | --- | --- |
| Qualcomm | No |  |

## CRs for adding the missing scheduling restrictions of positioning SI messages for eMTC

The endorsed CRs [2], [3] introduce the agreed solution for adding the missing scheduling restrictions of positioning SI messages for BL/CE UEs). In [1] some clarifications to the proposed changes in the CRs are given:

- New field posSchedulingInfoList-BR-r15 has been introduced in the late NCE branch of SIB1 as Rel-15 NCE of field bandwidthReducedAccessRelatedInfo. This was done since in Rel-16 the regular NCE branch of SIB1 is already taken by SystemInformationBlockType1-v1610-IEs.

- New field posSchedulingInfoList-BR-r15 indicates the list of additional scheduling information (in terms of si-Narrowband-r13 and si-TBS-r13) of positioning SI messages for BL UEs and UEs in CE. It includes the same number of entries, and listed in the same order, as in posSchedulingInfoList-r15.

- New condition “SI-Pos-BR” on the presence of field posSchedulingInfoList-BR-r15 has been introduced to clarify that the field is mandatory present if posSchedulingInfoList-r15 is included in SIB1-BR. Otherwise the field is not present.

Question 2: Do companies agree on the changes in the CRs or are further changes needed?

| Company | Yes/No | Additional comments |
| --- | --- | --- |
| Qualcomm | No | The intent of the changes is correct but ASN.1 can be more compact if element in bandwidthReducedAccessRelatedInfo-v15xy us made mandatory as follows: SystemInformationBlockType1-v15xy-IEs ::= SEQUENCE { bandwidthReducedAccessRelatedInfo-v15xy SEQUENCE { posSchedulingInfoList-BR-r15 PosSchedulingInfoList-BR-r15 OPTIONAL -- Cond SI-Pos-BR } OPTIONAL, -- Cond BW-reduced nonCriticalExtension SEQUENCE {} OPTIONAL } Delete description of SI-Pos-BR from the table. Update field description as follows: posSchedulingInfoList-BR Indicates additional scheduling information of positioning SI messages for BL UEs and UEs in CE. E-UTRAN always includes this field if posSchedulingInfoList-r15 is included in SystemInformationBlockType1-BR, and iIt includes the same number of entries, and listed in the same order, as in posSchedulingInfoList-r15. Draft revised CR with these changes uploaded to the folder. |

# Conclusion

Based on company’s feedback the following proposals are made:

<To be updated>