3GPP TSG-RAN WG2 Meeting #116-e															R2-2111364

Electronic, November 1 – 12, 2021

Agenda item:	8.11.7

Source: 	Qualcomm Incorporated (Moderator)

Title: 	Summary of [AT116-e][615][POS] PRUs

Document for:		Discussion and Decision

# 1.	Introduction

This document summarizes the following email discussion:

- [AT116-e][615][POS] PRUs (Qualcomm)

Scope: Discuss the handling of the PRU topic taking the related contributions into account, and determine a way forward.

Intended outcome: Report to positioning session in R2-2111364, and LS out if necessary

Deadline:  Monday 2021-11-08 1000 UTC (report available)

At RAN2#115-e the following agreements were made on positioning positioning reference units [11]:

Agreements:

Proposal 1 (modified): For purposes of RAN2 discussion, the PRU functionality as described in the RAN1 LS can be considered as UE with known location (to some degree of accuracy) at least (16/17).

PRU modelled as a gNB can be discussed in RAN3 (no RAN2 action).

Agreement:

RAN2 confirm that the PRU considered as a UE supports the normal LPP procedures for assistance data transfer and location information transfer.

Companies are asked to provide their views on the stated topics and questions.

Contact Points

Respondents to the email discussion are kindly asked to fill in the following table.

| Company | Name | Email Address |
| --- | --- | --- |
| Qualcomm (moderator) | Sven Fischer | sfischer@qti.qualcomm.com |
| Huawei, HiSilicon | Yinghao Guo | yinghaoguo@huawei.com |
| vivo | Annie Zhong | tingting.zhong@vivo.com |
| ZTE | Yu Pan | pan.yu24@zte.com.cn |
| CATT | Jianxiang Li | lijianxiang@datangmobile.cn |
| Xiaomi | Xiaolong Li | lixiaolong1@xiaomi.com |
| Intel | Yi Guo | Yi.guo@intel.com |
| Apple | Sasha Sirotkin | ssirotkin@apple.com |
| Lenovo, Motorola Mobility | Robin Thomas | rthomas7@lenovo.com |
| Samsung | June Hwang | June77.hwang@samsung.com |
| Ericsson | Ritesh Shreevastav, Fredrik Gunnarsson | ritesh.shreevastav@ericsson.com, fredrik.gunnarsson@ericsson.com |
| InterDigital | Jaya Rao, Fumihiro Hasegawa | jaya.rao@interdigital.com, fumihiro.hasegawa@interdigital.com |
| OPPO | Xin YOU | youxin@oppo.com |
| Fraunhofer | Birendra Ghimire | Birendra.ghimire@iis.fraunhofer.de |
| Nokia | Mani Thyagarajan | mani.thyagarajan@nokia.com |

# 2.	Input Contributions

The following PRU related contributions have been submitted in AI 8.11.7:

[1]	R2-2109489, "Discussion on Positioning Reference Units (PRUs)", CATT, ZTE Coroporation, Intel Coroporation.

[2]	R2-2109827, "Support of Positioning Reference Units", Lenovo, Motorola Mobility.

[3]	R2-2109919, "On the Positioning Reference Units aspects", Ericsson.

[4]	R2-2109983, "Discussion on support for positioning reference unit", vivo.

[5]	R2-2110039, "Stage-3 impacts of PRU support", Apple.

[6]	R2-2110177, "Discussion on PRU", Huawei, HiSilicon.

[7]	R2-2110826, "Remaining issues for Positioning Reference Units", Qualcomm Incorporated.

[8]	R2-2110827, "[draft] Response LS on Positioning Reference Units (PRUs) for enhancing positioning performance", Qualcomm Incorporated.

[9]	R2-2110934, "Discussion on supporting Positioning Reference Units", InterDigital.

[10]	R2-2111109, "Discussion on how to manage PRU", Xiaomi.

Other References:

[11]		R2-2108835, "Report from session on positioning and sidelink relay", Session Chair (MediaTek).

[12]		R2-2106920 (R1-2106326), "LS on Positioning Reference Units (PRUs) for enhancing positioning 					performance", RAN1.

# 3.	Stage 2 Aspects

## 3.1	General PRU Functions and Capabilities

| CATT et al. [1] | Proposal 1: RAN2 to confirm PRUs may be requested to provide its own known location coordinate information, antenna orientation information if known, the positioning measurements to the LMF, and Transmit the UL SRS signals according to the LS from RAN1 [2]. Proposal 2: RAN2 to confirm with RAN1 that all NR RAT-Dependent positioning methods may be supported by PRUs. Proposal 3: RAN2 to agree to support both stationary PRU and moving PRU in Rel-17. |
| --- | --- |
| Lenovo [2] | Proposal 4: Support reporting of the known location information source by a PRU UE (e.g., RAT-independent methods, manual/offline/preconfigured location, etc.) to the LMF. Proposal 5: PRU UE to support change/update of the location information to the LMF. FFS the signalling (e.g., solicited/unsolicited request) and any relevant event-triggered criteria. |
| vivo [4] | Proposal 3: The PRU should report PRU location information (e.g., known location coordinate, movement velocity, movement direction) to LMF. |
| Qualcomm [7] | Proposal 2: All LPP positioning methods can be supported by a PRU, dependent on PRU capabilities. |

According to the RAN1 LS in [12], at least the following PRU functionality may be supported:

"PRU may support, at least, some of the Rel-16 positioning functionalities of UE, if agreed, which is up to RAN2. The positioning functionalities may include, but not limited to, the following:

-	Provide the positioning measurements (e.g., RSTD, RSRP, Rx-Tx time differences)

-	Transmit the UL SRS signals for positioning

PRU may be requested by the LMF to provide its own known location coordinate information to the LMF. If the antenna orientation information of the PRU is known, the information may also be requested by the LMF."

Question 1:	Which of the following general functions and capabilities should be supported by a PRU:

(1)	Provide PRU location coordinates to an LMF.

(2)	Provide the location source to an LMF (e.g., location method used, pre-configured, etc.)

(3)	Provide PRU antenna information to an LMF.

(4)	Transmit UL-SRS.

(5) Provide location measurements to an LMF. The PRU location measurements can be provided for:

(5a)	All NR RAT-dependent positioning methods only.

(5b)	All LPP positioning methods.

(6)	PRUs may be stationary or moving.

(7)	Provide PRU movement information to an LMF (e.g., location updates, velocity, etc.).

(8)	Other – please specify.

| Company |  | Item(s) # |  | Comments |
| --- | --- | --- | --- | --- |
| Huawei, HiSIlicon |  | (1), (2), (3), (4), (5a), (6), |  | We think that the UE’s known location should be transferred from the UE to the LMF rather than configured by OAM. This can be reflected by the following in the LS from RAN1. PRU may be requested by the LMF to provide its own known location coordinate information to the LMF. For the positioning method that the PRU should support, we think that we should bare in mind the functionality of the PRU, which is to help LMF adjust the timing for the network entities. Hence, we think a PRU should only support the positioning methods that can enable this functionality. Hence, we think at least RAT-independent positioning methods should be excluded. We should further investigate whether all the RAT_depedent positioning methods should be supported, e.g., whether E-CID is needed. While for (7), it leaves us the impression that the UE shall trigger the update and autonomously send the updated location and velocity to the LMF. While, we think this is not needed. The PRU’s service should be triggered by the LMF that the it should only be triggered when the LMF needs it. |
| vivo |  | (1), (3), (4),(5a),(6), (7) |  | We think no need to support (2). Because PRU location information(e.g., (1),(3),(7)) is enough, which is used by LMF to calculate the correction information. PRUs are used to enhance RAT-Dependent positioning methods, so we think all NR RAT-dependent positioning methods only are enough. |
| ZTE | (1)(3)(4)(5)(6)(7)(8) |  | 1. Support (1) (3) and (4) to align with RAN1’s LS. 2. support 5a and 5b: No matter how can PRU get its precise location, it just reports the location to LMF. In addition, RAN1 assumes PRU can be seen as a normal UE with known location. So, PRU should not be limited to only support RAT-dependent positioning methods. 3.(6) and (7) are coupled, If PRU can move, PRU should be able to update its precise location. 4.for (8), PRU should report at least its identifier to tell network it is different from other UE. |  |
| CATT | (1), (3), (4), (5a), (6),(8) |  | (5a) doesn’t mean the PUR should have capabilities which support all RAT-D positioning methods. As for (1) PRU location coordinates can be configured to LMF because the location is known. On the other hand, PRU location coordinates also can be provided to LMF because RAN1 says PRU may be requested by the LMF to provide its own known location coordinate information to the LMF. From RAN2’s perspective, there is no limitation on how to get the PRU’s known location. So it’s better to confirm with RAN1 that if the known location can be the location which is calculated by PRU. (8) Other – the uncertainty of calculated location coordinates which needs confirmation with RAN1. We think (2)&(7)& (8) needs confirmation from RAN1, because: It is not clear why the location source is required by an LMF (e.g., location method used, pre-configured, etc.). RAN2 may send an LS to RAN1 to confirm if this information is required. As for (7), it seems an enhancement which needs further evaluation and confirmation with RAN1 as well. As for (8), the uncertainty of calculated location coordinates may be required to report together with location coordinates for performance enhancement. |  |
| Xiaomi | - (3) (4) (5a) (6) |  | For (2) and (7), we think it should be confirmed by RAN1 first. |  |
| Intel |  | (1), (2), (3), (4), (5a), (6) |  | Regarding 5, RAN1 only mentioned RAT dependent positioning methods. Therefore we should focus on 5a first. Regarding (7), it should be mainly for moving PRU. MO-LR should be sufficient. |
| Apple |  | (4), (5), (8) |  | (1) is misleading – is the question about e.g. UE-based location estimate or known location information? We don’t think PRUs need to provide their location to LMF – the PRU location information should already be available through OAM. Regarding (8), PRU capability probably need to be provided to LMF. |
| Qualcomm |  | (1), (2), (3), (4), (5b), (6), (7) |  | On (5) we don't see a need to restrict the positioning methods for PRUs, and if LPP is going to be used, how this would be possible in practice. E.g., a PRU may support RTK to determine the PRU location, etc. |
| Lenovo, Motorola Mobility |  | (1), (2), (3), (4), (5a), (6), (7) (8) |  | (2) – Location source represents additional assistance information to the LMF to help ascertain the quality of the available known location. (7) – In the case that the PRU UE is moving, it is also important the corresponding location updates are provided to the LMF, so that the reference measurements made by the PRU UE are not outdated/invalid. (8) – Additionally, the PRU UE should be able to provide a positioning QoS (if available), e.g., accuracy, confidence interval associated with the provided known location, which assists the LMF in selecting the PRU UE. |
| Samsung |  | (1),(2),(3),(5b), (6), (7), (8) |  | (7) is only for moving indicated in (6). (8) Further we think confidence (or uncertainty) level for its identified location information should be able to report to LMF. (8) And indicator for identifying PRU at LMF side is necessary at least for requiring the location/and other info associated LMF. |
| Ericsson |  | (1), (2), (4), (5b), (6), (7) |  | Comments: - – already supported for any UE via LPP LocationCoordinates IE, which does not need to be modified - – already supported for any UE via LPP LocationSource-r13 IE, which does not need to be modified - – not already supported in LPP, and should not be added unless this has been studied and general applicability considered, which is not within scope of this WI - – already supported for any UE - – all LPP positioning methods are already supported for general UEs - – general UEs may be stationary or moving, not a specific PRU aspect - – this is already supported by the LPP LocationCoordinates and Velocity IEs Everything that is needed in order to configure this behaviour is already available in LPP for a general UE in the CommonIEsRequestLocationInformation IE, except that the locationInformationType needs to be extended to allow reporting of both a position estimate and positioning measurement, i.e. LocationInformationType ::= ENUMERATED { locationEstimateRequired, locationMeasurementsRequired, locationEstimatePreferred, locationMeasurementsPreferred, …, [[ locationEstimateAndMeasurementsRequired ]] .} The only needed capability is therefore associated to the support of this new LocationInformationType |
| InterDigital |  | (1), (2), (3), (4), (5b), (6) and (7) |  | We think (2) is a useful information to provide to LMF for enabling the LMF to determine whether the location method used by PRU is similar and compatible with that used by a target UE. On (5), we do not think the PRU operation should be limited to only RAT-dependent positioning methods. We think (6) and (7), related to the PRU movement/mobility, should be other information that is useful to provide to LMF for deciding whether and how the PRU can be used for reporting measurements. |
| OPPO |  | 1), (2), (4), (5b) and (6) for stationary PRU |  | We wonder how moving PRU obtain its location. If it is based on its own measurement, how to guarantee the reliability of the provided location coordinates? |
| Fraunhofer |  | (1), (2), (3),(4), (5a),(6),(7) , (8) |  | On (2), the UE can provide a location source. In our opinion, the PRUs could evaluate different positioning methods / measurements, and indicate the best positioning method for the given location at the reported time, so that the LMF can subsequently configure the UEs in the vicinity with the positioning method taking into account the UE feedback. Question (3) follows the RAN1 agreements Furthermore, the PRUs could also support monitoring the local environment, monitoring impairing factors like multipath, interference (inter-system and jamming), spoofing etc. |
| Nokia |  | (1), (2), (3), (4). See comments for (5) and (6) |  | (1): In Rel-17, the PRU can support just the MO-LR procedure to determine the PRU location coordinates but some signalling enhancements may be needed for the PRU to provide the so determined location coordinates and location source to the LMF. (3): RAN1 specifically asked for antenna orientation information but the question seems to mention a generic antenna information. If information other than antenna orientation is needed, it is fine as long as it is an information already specified in specification. Otherwise, we need to confirm with RAN1 what specific information is required. (4): This is a requirement from RAN1, and it looks like RAN1 assumes that PRU will provide support for UL positioning also. However, this depends on LCS client residing at LMF and the LMF being able to instigate UL positioning procedure in the gNB. In Rel-17, we could support just the DL methods using MO-LR procedure. (5): This is a requirement from RAN1. The PRU must provide location measurements that the LMF requests for timing corrections. However, full support for PRU to provide location measurements to an LMF in the context of UE-assisted positioning for any positioning method for the purpose of determining the PRU position is not required in Rel-17. (6): It is our understanding that in Rel-17, PRU mobility support is not needed. This can be confirmed with RAN1 (7): Mobility support for PRU and movement information from PRU were not requested in the RAN1 LS. We feel support for mobility may extend the scope of work for PRU in Rel-17, which should be avoided (8): We don’t support additional enhancements at this stage in Rel-17 than the absolute minimum requested by RAN1. So, we feel we should not brainstorm on other functions or information exchange for PRU at this time. |

Moderator's Summary:

-	15 companies replied to the question.

-	For none of the proposed seven general PRU functions and capabilities there is a full consensus among all interested companies:

- 	(1) Provide PRU location coordinates to an LMF:		Supported by 14 of 15 companies

1 company (Apple) thinks the PRU location information should be available at LMF through OAM.

-	(2)	Provide the location source to an LMF:				Supported by 10 of 15 companies.

5 companies think the location source may not be needed (vivo, ZTE, CATT, Xiaomi, Apple) (e.g., location uncertainty may be enough, or should be confirmed by RAN1 first, etc.)

-	(3)	Provide PRU antenna information to an LMF:	Supported by 12 of 15 companies

1 company (Apple) thinks the PRU location information should be available at LMF through OAM, which could then also be the case for any antenna information.

1 company (Ericsson) thinks this requires further study and is out of scope of the WI.

-	(4)	Transmit UL-SRS:										Supported by 14 of 15 companies

1 company (Nokia) suggests to focus on DL-only methods in this Release (e.g., since this depends on LCS client residing at LMF and the LMF being able to instigate UL positioning procedure in the gNB).

-	(5)	Provide location measurements to an LMF:

(5a) NR RAT-dependent positioning methods only:	Supported by 7 of 15 companies

(5b) All LPP positioning methods:						Supported by 6 of 15 companies

2 companies (Apple, Nokia) did not specifically distinguish between 5a/5b and/or depends on RAN1 requirements

-	(6)	PRUs may be stationary or moving:					Supported by 12 of 15 companies

1 company (Nokia) thinks PRU mobility support is not needed and may require confirmation from RAN1.

-	(7)	Provide PRU movement information to an LMF:	Supported by 8 of 15 companies

This depends on support for (6).

-	(8)	Other:

Items mentioned include PRU identifier (ZTE, Samsung), location uncertainty (CATT, Lenovo), PRU capabilities (Apple), additional measurements (Fraunhofer)

Given the incoming RAN1 LS which already lists some basic PRU functionality, but also mentions "up to RAN2", it is proposed to confirm at least support for the RAN1 items and continue the discussion on additional PRU functionality.

Proposal 1:	RAN2 confirms that a PRU can support at least the following functionality (as described in the RAN1 LS), dependent on PRU capability:

- Provide the positioning measurements (e.g., RSTD, RSRP, Rx-Tx time differences) to an LMF.

- Transmit the UL SRS signals for positioning.

- Provide its own known location coordinate information to an LMF.

- Provide its antenna orientation information to an LMF.

## 3.2		PRU registration at LMF/Management of PRUs

| CATT et al. [1] | Proposal 4: RAN2 to agree that LMF can initiate a MT-LR without impact on SA2, by invoking the GMLC service. Proposal 5: RAN2 to agree that PRUs info is configured to LMF via O&M before step 0 in the procedure of PRU Positioning Operations to support an MT-LR. Proposal 6: RAN2 to discuss PRU Positioning Operations to support an MT-LR to be added in the clause X.Y of TS38.305 and capture the TP as baseline in the annex. |
| --- | --- |
| Lenovo [2] | Proposal 1: LMF is responsible for the management of the PRU (e.g., configuration) via existing LPP procedures. |
| vivo [4] | Proposal 1: LS to SA2 to study how to enable the LMF to be aware of PRUs in the network. |
| Apple [5] | Proposal 1: the known locations of the PRUs is made available to the LMF via OAM. Proposal 3: regular UEs should not be used as PRUs, which may require appropriate capability signalling. |
| Huawei [6] | Proposal 4: The LMF can request the location and/or positioning measurements from the PRUs by its implementations and no LCS requests for the PRUs are needed to trigger the LMF. |
| Qualcomm [7] | Proposal 1: Enable an LMF to act as an "LCS Client" for PRUs, i.e., enable an LMF to instigate location procedures (e.g., LPP, NRPPa procedures) with PRUs without location requests from an AMF. |
| InterDigital [9] | Proposal 1: Support assignment and usage of identifiers related to PRU operation (e.g. PRU ID, LPP session ID for PRU) when transferring any LPP signalling/messages between LMF and PRU |
| Xiaomi [10] | Proposal 1: The PRU can send MO-LR request to the LMF to indicate that there is an available PRU in the network. Proposal 2: The LMF can acquire the PRU information based on OAM configuration. |

From the Proposals above and general PRU purpose, it appears obvious that the consumer of the PRU location information is an LMF. From an LCS point of view, this seems to imply that the LMF effectively becomes the "LCS Client" for PRUs.

Question 2:	Do you agree that the consumer of the PRU location information is an LMF, and therefore, an LMF needs to be enabled to instigate location procedures with PRUs in the network?

| Company |  | YES / NO |  | Comments |
| --- | --- | --- | --- | --- |
| Huawei, HiSIlicon |  | Yes for the former part; NO for the latter part, but this is up to SA2 to decide |  | For the first part of the question, “Do you agree that the consumer of the PRU location information is an LMF,” we agree that for the use of PRU, this is different from the legacy types of positioning requests (e.g., MT-LR, MO-LR..) that this is not triggered by a LCS client or external AF. The positioning procedure should be triggered when the LMF thinks there is a need for the LMF to perform timing adjudgment for the network entities. For the latter part of the question an LMF needs to be enabled to instigate location procedures with PRUs in the network? We think it is better for the AMF and LMF to take up the role of the PRU management. The following procedure has been proposed in [6] for the procedure Hence, the PRU information is managed in the UDM. When LMF wants to perform positioning, the LMF can first request to the AMF. Then the AMF can query the UDM and the UDM can returns the UE ids of the PRUs. Then, like the other types of LCS requests, the LMF can reuse the service operation Nlmf_location_determine to get the location and measurements of the PRU However, we think the latter question is not for RAN2 to answer and should be determined by SA2, thus we think we should send an LS to SA2 to trigger the discussion in SA2. |
| vivo |  | Yes |  | Because the benefit of PRUs with the known location is enhancing the positioning performance, which is different from the positioning procedure for normal UEs. |
| ZTE |  |  | We agree it is LMF that needs to know the PRU location information at LMF-based positioning, however whether AMF and other core network element should be involved in the structure should be determined in SA2 |  |
| CATT |  | Yes |  | We prefer that LMF initiates a MT-LR without impact on SA2. There is no time budget in SA2 for PRU in Rel-17. So we propose that LMF may initiate a MT-LR to GMLC and GMLC may trigger a location request to target AMF within the SUPI. To request PRU location, an LMF is proposed to send Ngmlc_Location_ProvideLocation Request to GMLC based on the PRUs info configured to LMF via O&M, e.g. UE Identification (SUPI), Client Type and serving AMF ID. According to the SA2 specification, the description in clause 4.3.2 in TS 23.273 mentioned: “AFs and NFs may access LCS services from a GMLC in the same trust domain (e.g. in the same PLMN) using the Ngmlc interface or Event Exposure with location information from an AMF in the same trust domain using the Namf interface.” So the LMF acts as an NF accesses LCS services by invoking the GMLC service, and AMF will response the request following the existing procedure. There won't be impact on SA2 when an LMF needs to be enabled to instigate location procedures with PRUs within the above procedure. |
| Xiaomi |  |  | We agree that the LMF should know the PRU information, but we don’t think the LMF must initiate the location procedure. The PRU can send MO-LR request to the LMF to indicate there is an available PRU in the network, and the following LPP procedures can be triggered by LMF. |  |
| Intel |  | Yes |  | We would prefer the solution without the impact on AMF. |
| Apple |  | Yes |  |  |
| Qualcomm |  | Yes |  | The PRU location measurements are needed by an LMF only. An LMF would not receive a location request for PRUs from an AMF. |
| Lenovo, Motorola Mobility |  | Yes |  | LMF is the end consumer of the UE location information, however we also share the view that CN impacts may need SA2 feedback as well, although our first preference is to avoid any impacts to SA2 in this release. |
| Samsung |  | Yes |  | Basic purpose of using PRU is to calibrate the location estimate procedure using the reported meas results from PRU and its reported location. Regarding AMF involvement, we prefer the solution without AMF involvement. |
| Ericsson |  | SA2 needs to be consulted, RAN2 cannot decide |  | As of now AMF makes request for positioning to LMF and includes a correlation Id that is used for UE associated signaling. Further LMF is not aware of the AMF instance serving the UE. |
| InterDigital |  | Yes |  | We agree that the location information of PRU is for the benefit of LMF. As such, unlike the legacy procedures (e.g. MO-LR, NI-LR/MT-LR) with AMF involvement, LMF should be able to instigate the location procedures for PRU. |
| OPPO |  |  |  | Agree with Huawei and Ericsson. Prefer to involve SA2 for the final decision. |
| Fraunhofer |  | Yes |  | We agree that the LMF is the consumer of information generated by the PRU. |
| Nokia |  | Yes and No. See comments. |  | The solution and use case from RAN1 seem to require the LMF to know the location of the PRU. So, the LMF has to be a consumer of the PRU location information. However, the question of whether LMF can instigate location procedures with PRU, without receiving a Location Request from the AMF may require SA2 input. For Rel-17, ONE of the following options can be considered: - In Rel-17, we can leave the details of how LMF obtains the location of the PRU to LMF implementation (or OAM configuration of static PRU location in LMF). If this option is chosen, then items (1) and (2) under question 1 are left to implementation but item (5) support is needed for LMF to get measurements for the purpose of timing corrections. - Restrict PRU to use only MO-LR procedure (UE can be the LCS client) but enhance LPP to allow PRU to report it’s location to the LMF. |

Moderator's Summary:

-	It seems there is consensus that the consumer of the PRU location information is an LMF (e.g., reference location measurements etc. are needed by an LMF to determine "correction information").

-	However, there are different views about the consequences of this fact; i.e., whether an LMF needs to be enabled to instigate location procedures with PRUs in the network or not. Multiple companies also suggested already solutions (which is the discussion of Question 3), although, it appears there is no clear consensus about what the problem actually is (or whether there is a problem).

-	1 company (Huawei) thinks an AMF can still instigate location procedures with PRUs in the network. However, it is also said that "when LMF wants to perform positioning, the LMF can first request to the AMF", which appers contradicting. However, it is also acknowldged that this requires SA2 expertise.

-	1 company (CATT) thinks an LMF can already initiate a MT-LR, and there is no impact to SA2.

-	1 company (Xiaomi) agrees that the LMF should know the PRU information, but at the same time don’t think the LMF must initiate the location procedure. Instead, the location procedure can be initiated by the PRU itself via MO-LR.

-	1 company (Nokia) thinks that in this Release, a solution could be left to implementation (OAM, etc.), and/or could be PRU triggered (existing MO-LR).

-	Multiple companies also think that this requires SA2 expertise. However, it is not always clear to the moderator whether this refers to the actual problem statement (i.e., "an LMF needs to be enabled to instigate location procedures with PRUs in the network") or to the actual solution for this problem, or both.

To instigate location procedures by an LMF with PRUs in the network, the availability of PRUs (e.g., PRU IDs etc.) need to be known by an LMF.

Question 3:	If your answer to Question 2 was 'YES', which of the following general options can/should be used to make an LMF aware of PRUs in a network:

(1)	PRU information is configured in an LMF via OAM. The PRU information may for example include PRU IDs, location coordinates, etc. (TBD)).

(2)	Left to implementation.

(3)	LMF can initiate a MT-LR without impact on SA2, by invoking the GMLC service.

(4)	The PRU can send MO-LR request to the LMF to indicate that there is an available PRU in the network.

(5)	Should be studied and decided by SA2.

(6)	Other – please specify.

| Company |  | Item(s) # |  | Comments |
| --- | --- | --- | --- | --- |
| Huawei, HiSIlicon |  | (5) |  | Whether the PRU should be registered and the information within the registration should be decided by SA2. Our thinking is that the type of LCS is not MT-LR or MO-LR or any other types of LCS. But this again, should be decided by SA2 |
| vivo |  | (5) |  | We think at least LS to SA2 first is more appropriate. Because the whole procedure for PRU also involves SA2. |
| ZTE | (1) (5) |  | The identifier of PRU should be told to LMF at least. Further, the registration of PRUs should be decided by SA2 |  |
| CATT |  | (1),(3),(4) |  | LMF can initiate a MT-LR without impact on SA2, by invoking the GMLC service with OAM configuration. There is no time budget in SA2 for PRU in Rel-17. PRU also can send MO-LR request to the LMF to indicate that there is an available PRU in the network. |
| Xiaomi | - (4) |  | LMF can acquire the PRU information by OAM configuration and MO-LR request from PRU, then the LMF can trigger the LPP procedure to acquire the relate information from PRU. |  |
| Intel |  | (1) (3) (4) |  | It can be resolved by RAN2. |
| Apple |  | - (5) |  | LCS aspects need to be discussed in SA2, the rest (e.g. OAM) can be decided in RAN2. |
| Qualcomm |  | (5), (6) |  | On (1) and (2): We prefer an open standard. (3) and (4) seem not feasible (see 23.273 for the specification of MT-LR and MO-LR). However, a variant of (4) seem possible where the "MO-LR" is replaced by a new LCS message, which can then also carry all PRU specific information. This could also avoid specifying PRU specific information in LPP (which would create overhead for regular UEs). |
| Lenovo, Motorola Mobility |  | (1), (3), (4) |  | (3) and (4) are more flexible options which are under RAN2 scope. SA2 can be involved in later releases. |
| Samsung |  | (1), (3), (4) |  | We wonder if RAN2’s only solution cannot make any feasible solution for this issue. Assuming RAN2’s area, 1,3,4 can be considered. |
| Ericsson |  | (1), (2), (5) |  | These are the only options that fits within the scope of the WI, where for example (1) would correspond to a list of UEs identified by its MSISDN would be sufficient, and then existing procedures with only the new location information type would be needed. (2) is also possible. (5) Consent from SA2 is needed. |
| InterDigital |  | (3), (4), (5) |  | For (3), similar to MT-LR, an approach where the LMF requests the GMLC (instead of LCS client) can be considered to inquire on the availability of PRU. For (4), we think an approach based on UE-initiated request to network, similar to MO-LR, for a UE to indicate its capability and availability to operate as a PRU can be considered. We also think supporting any of such approaches should involve SA2, and hence (5). |
| Fraunhofer |  | (5) |  |  |
| Nokia |  | One of (1), (2), or (4) |  | In Rel-17, one of the options from (1), (2) and (4) can be used. For Rel-18, we support (5) |

Moderator's Summary:

-	14 companies replied to the question.

-	For none of the proposed six items there is a full consensus among all interested companies:

-	(1) PRU information is configured in an LMF via OAM:		Supported by 9 of 14 companies.

-	(2)	Left to implementation:										Supported by 2 of 14 companies.

-	(3)	LMF can initiate a MT-LR without impact on SA2:		Supported by 5 of 14 companies.

- 5 companies (CATT, Intel, Lenovo, Samsung, InterDigital) think an LMF can already initiate a MT-LR and there is no SA2 impact (i.e., existing functionality). Lenovo and Samsung also think that MT-LR is under RAN2's scope/responsibility.

-	(4)	The PRU can send MO-LR request to the LMF:			Supported by 7 of 14 companies.

- 7 companies (CATT, Xiaomi, Intel, Lenovo, Samsung, InterDigital, Nokia) think a PRU should initiate a MO-LR, which however, seem to imply (among others) that an LMF can obtain reference location measurements only after a PRU initiated a MO-LR. Lenovo and Samsung also think that MO-LR is under RAN2's scope/responsibility.

-	(5)	Should be studied and decided by SA2:						Supported by 8 of 14 companies.

- 8 companies (Huawei, vivo, ZTE, Apple, Qualcomm, Ericsson, InterDigital, Fraunhofer think this is a SA2 issue. 1 company (Nokia) think SA2 should only be involved in Rel-18.

MT-LR or MO-LR procedures are defined in TS 23.273, with Stage 3 support in e.g., TS 24.501, 24.571, 24.080. Clearly, these are not RAN2 owned specifications. Therefore, whether there are SA2/CT specification impacts or not can not really be decided by RAN2.

However, moderator believes that the OAM configuration and/or configuration left to implementation could be further discussed in RAN2/3. E.g., how this information is shared between different LMFs and AMFs in a network, etc.

Proposal 2:	RAN2 to discuss further whether PRU specific information can be configured in an LMF via proprietary means (e.g., OAM), and if so, which PRU specific information this includes.

# 4.	Stage 3 Aspects

## 4.1	LPP Signalling

| CATT et al. [1] | Proposal 7: RAN2 to agree PRU capability in LPP Provide Capabilities message can be sent to LMF, so PRU can be identified by LMF in MO-LR and then perform PRU Positioning Operations in Figure 7.3.3-1. Proposal 8: RAN2 to send the reply LS to RAN1 to check if the measurement corrections report from PRU to the LMF is needed for enhancement. |
| --- | --- |
| Lenovo [2] | Proposal 2: Separate UE capabilities for PRU are to be defined. RAN1 input is required. Proposal 3: The PRU UE can report its known available location information to the LMF via: 1) LPP signalling; 2) RRC signalling (e.g. using CommonLocationInfo message) via gNB. Proposal 3: The PRU UE can report its location information to the LMF via 1) LPP signalling; 2) combination NRPPa and RRC signalling via gNB; 3) offline/pre-configured location calibration. Proposal 6: PRU UEs can include positioning QoS information as part of its location estimate report to determine the quality/uncertainty of the location estimate. FFS whether an existing IE may be reused, or any new information is needed (e.g., confidence levels). |
| vivo [4] | Proposal 2: The current LPP and NRPPa messages can be reused with enhanced IEs to support UE-typed PRU positioning. Proposal 4: Enhance the current ProvideLocationInformation message to include PRU location information. |
| Apple [5] | Proposal 2: not to define LPP signalling for PRU to send its known location to the LMF. |
| Huawei [6] | Proposal 1: Enhance the LPP Request/Provide location information message to support the transfer of PRU antenna orientation information. Proposal 2: Enhance the LPP capability transfer procedure to support the transfer of PRU antenna orientation information. Proposal 3: Re-use the existing LPP procedure for the PURs to provide the positioning measurements and the locations. |
| InterDigital [9] | Proposal 2: LPP capability transfer procedure is used to transfer of new capability information related to PRU operation (e.g. supported positioning methods, known location of PRU, accuracy/uncertainty of location information) between UE (i.e. PRU) and LMF. Proposal 3: LPP assistance data transfer procedure is used to transfer new assistance information related to PRU operation (e.g. PRS configurations, validity conditions, evaluation conditions) between UE (i.e. PRU) and LMF Proposal 4: LPP location information transfer procedure is used for requesting/sending of new location information related to PRU operation (e.g. UE-assisted or UE-assisted+UE-based mode, measurement report and/or location estimates) between UE (i.e. PRU) and LMF |
| Xiaomi [10] | Proposal 3: The LPP capabilities transfer procedure can be used by LMF to acquire PRU capabilities and the PRU capabilities may include the known location, antenna orientation and mobility state of the PRU. Proposal 4: The LMF should indicate UE to report its known location and/or location based on PRS measurement when the LMF acquires the PRU location by LPP request location information message. |

As summarized in section 1 above, RAN2 already confirmed that a PRU considered as a UE supports the normal LPP procedures for assistance data transfer and location information transfer. Extensions may be needed as usual, dependent on agreed PRU functionality and other LCS aspects. However, the current agreement does not include the LPP Capability Exchange mentioned in various Proposals above.

Question 4:	Do you agree with the following: 
RAN2 confirm that the PRU considered as a UE supports the normal LPP procedures for capability transfer.

| Company |  | YES / NO |  | Comments |
| --- | --- | --- | --- | --- |
| Huawei, HiSIlicon |  | No |  | We think the only difference between PRU and the normal UE is that the PRU can send antenna orientation information and the known location to the LMF. For this LPP UE capability needs to be defined. |
| vivo |  | Yes |  | Normal LPP procedures for capability transfer is supported by PRU, which can be used to indicate PRU’s capability(e.g., supported positioning method). |
| ZTE | Yes |  | the stage 2 description of capability transfer, assistance data transfer and location information transfer should be the same between PRU and normal UE. The stage 3 PRU capability will need a FFS |  |
| CATT | Yes |  | PRU considered as a UE supports the normal LPP procedures for capability transfer, in addition to more capabilities as PRU. |  |
| Xiaomi |  | Yes |  | The PRU can support normal LPP procedures, but the detailed IE need to be enhanced such as introducing PRU capability. |
| Intel |  | Yes |  | We need to define PRU capability in LPP, and PRU should supports the normal LPP procedure for capability transfer. |
| Apple |  | Yes |  | PRU capability should be defined |
| Qualcomm |  | Yes |  | LMF need to know the location methods supported by a PRU. |
| Lenovo, Motorola Mobility |  | Yes |  | Both dynamic and static capability information should be supported as in the case of a normal UE supporting LPP. |
| Samsung |  | Yes |  | LPP procedure for capability transfer can be used for the PRU specific capability information. |
| Ericsson |  | Yes |  | Normal LPP is sufficient. A possible capability that goes with the support for the new location information type “both position estimate and position measurements required” |
| InterDigital |  | Yes |  | The existing LPP capability transfer procedure can be used for transferring any capability information related to PRU functionality |
| OPPO |  | Yes |  | As agreed in last meeting, PRU can support normal LPP procedure. |
| Fraunhofer |  | Yes |  | The PRU needs to support extra signalling, such as the antenna orientation, but other than that it shall support the normal LPP procedures like a normal UE. |
| Nokia |  | See comments |  | We should first decide on the way forward for Rel-17 work on PRU. We proposed some way forward options for Rel-17 work on PRU in our answers to Q2, Q3 and Q7. This should be decided first before we look in to LPP capabilities and detailed stage 3 signalling aspects. |

Moderator's Summary:

-	15 companies replied to the question.

-	13 companies think that LPP capability procedures can also be used for PRUs.

-	1 company (Huawei) thinks that a new LPP capability needs to be defined. However, it seems they also agree that this should be done in LPP.

-	1 company (Nokia) thinks we should first decide on the general PRU support/way forward before discussing Stage 3 details.

Given that RAN2 already started to discuss Stage 3 details and confirmed that PRU considered as a UE supports the normal LPP procedures for assistance data transfer and location information transfer, it appears obvious that the same can also apply to the capability transfer.

Proposal 3:	RAN2 confirm that the PRU considered as a UE supports the normal LPP procedures for PRU capability transfer.

## 4.2	Assistance Data

| CATT et al. [1] | Proposal 9: RAN2 to send the reply LS to RAN1 to ask what the exact correction information from LMF to UE for UE-based positioning (commercial UE) is. |
| --- | --- |
| Lenovo [2] | Proposal 7: LMF may provide DL-PRS differential correction information via a new posSIB to assist UEs in compensating differential errors for UE-based positioning. Send LS to RAN1. |
| vivo [4] | Proposal 5: RAN2 to confirm with RAN1 if it is valuable to provide the correction information from LMF to UE for UE-based positioning. Proposal 6: NR-PositionCalculationAssistance IE in ProvideAssistanceData message can be enhanced to provide the correction information from LMF to UE for UE-based positioning. |

An LMF may determine correction terms from PRU mesurements to assist/improve the location measurements for target UEs. For UE-based mode, the correction terms would be needed at the UE.

Question 5:	Do you agree that the "correction information" determined by an LMF from PRU measurements should be provided to a target UE for UE-based mode?

| Company |  | YES / NO |  | Comments |
| --- | --- | --- | --- | --- |
| Huawei, HiSIlicon |  | Yes, but |  | Yes, but this is part of the current AD for the UE for the UE-based mode. We wonder any specification change is needed? |
| vivo |  | Yes |  | For UE-based positioning, UE will calculate the positioning result and the correction information can be beneficial for UE to improve the positioning accuracy. In addition, the impacted RAN2 spec is limited. |
| ZTE | Yes |  | From RAN1 perspective, PRU is aimed for cancelling timing error information. For UE based, LMF needs to tell UE the correct information, or to tell UE the real-time PRU location estimate. |  |
| CATT | Yes |  | We need to confirm with RAN1 that what the correction information from LMF to UE for UE-based mode. To HW: RSTD may already be supported. But For other corrections, further enhancement may be clarified by RAN1. |  |
| Xiaomi |  | Yes |  | We can send LS to RAN1 to ask whether the correction information is needed for UE based positioning and what the correction information is. |
| Intel |  | Yes |  | The detailed correction information should be decided by RAN1. |
| Apple |  | Yes, but |  | RAN1 should agree on the correction information first |
| Qualcomm |  | Yes |  | At least for the NR positioning methods, the details of the "correction information" seem primarily a RAN1 issue. |
| Lenovo, Motorola Mobility |  | Yes |  | RAN2 could agree on the need to provide “correction information” to also support UE-based methods, while the details of what type of “correction information” can be left up to RAN1. |
| Samsung |  | Yes |  | We also have the same view with Huawei that this is in the current AD for UEB mode. And of course, the correction information details should be concluded in RAN1. |
| Ericsson |  | Yes |  | This is the same for any UE requesting AD – nothing specific for this discussion |
| InterDigital |  | Yes |  | Same view as others; any correction information is to be decided by RAN1 |
| OPPO |  | Yes |  | But this will not introduce any spec impact. |
| Fraunhofer |  | Yes |  | Correction information needs to be discussed in RAN1. The PRUs could be source for monitoring the local environment effects (such as multipath, jamming spoofing) and the information collected from the PRUs could be a valuable assistance data to provide to nearby UEs. Having said that, we believe that a normal UE should also be able to report such events and not just PRUs. |
| Nokia |  | Conditional Yes |  | This depends on whether RAN1 agreed solution requires the target UE to do UE-based positioning and use the correction information from LMF to apply corrections to the UE estimated position of itself. We should check with RAN1 on this. Restricting the Rel-17 work to UE-assisted DL positioning and have LMF use the correction information would help introduce a baseline functionality in Rel-17 which can be expanded in Rel-18 with additional functionalities. |

Moderator's Summary:

-	There seems to be consensus that the "correction information" determined by an LMF from PRU measurements should be provided to a target UE for UE-based mode.

-	However, some companies think that this is already supported today with LPP (Huawei, Samsung, Ericsson, OPPO).

-	Multiple companies (CATT, Xiaomi, Intel, Apple, Qualcomm, Lenovo, InterDigital, Fraunhofer, Nokia) point out that we do not know what kind of "correction information" are considered in Rel-17, and this requires further RAN1 input.

The specific "correction information" may depend on the RAT-dependent/RAT-independent positioning method supported. Companies proposed to send an LS to RAN1 for the specific "correction information"

Question 6:	If your answer to Question 5 was 'YES', do you agree that an LS should be send to RAN1 asking RAN1 which "correction information" should be provided to UEs for UE-based mode?

| Company |  | YES / NO |  | Comments |
| --- | --- | --- | --- | --- |
| Huawei, HiSIlicon |  | No |  | No change is needed. The correction can be reflected by the AD provided |
| vivo |  | Yes |  | We think the specifically required correction information is in the RAN1 scope. |
| ZTE | Yes |  | Suggest to ask RAN1 what concrete solution can the PRU help to improve positioning accuracy. That decides what stage 3 assistance data will be needed: the correct information, or the real-time PRU location estimate. |  |
| CATT |  | Yes |  | RAN2 had no idea what the correction information is for the performance enhancement. RAN1 is responsible for defining the correction information. |
| Xiaomi | Yes |  | We can ask RAN1 whether the correction information is needed for UE based positioning and what the correction information is. |  |
| Intel |  | Yes |  |  |
| Apple |  | Yes |  |  |
| Qualcomm |  | Yes |  |  |
| Lenovo, Motorola Mobility |  | Yes |  |  |
| Samsung |  | Yes |  |  |
| Ericsson |  | No |  | No specific AD for this discussion, no change needed. |
| InterDigital |  | Yes |  |  |
| OPPO |  | No |  |  |
| Fraunhofer |  | Yes |  |  |
| Nokia |  | Yes |  | See our answer to Q5. We should also ask for a high-level description of the solution agreed in RAN1. |

Moderator's Summary:

-	There seems to be consensus that the "correction information" requires RAN1 input.

-	Huawei and Ericsson think no additional "correction information" is needed. However, it is not clear which parts of the current assistance data are considered as "correction information".

Proposal 4:	Send an LS to RAN1 asking RAN1 whether the "correction information" derived by an LMF from PRU measurements should be provided to target UEs for UE-based mode of operation, and if so, what these "correction information" includes.

# 5.	Way Forward related Proposals

| Ericsson [3] | Proposal 1 PRU discussion is postponed to Rel-18 when SA2 has discussed the PRU first. |
| --- | --- |
| Huawei [6] | Proposal 5: Send a LS to SA2 regarding how to manage the information of PRUs by the network. RAN2 can discuss the specification impacts after the procedures of PUR information management being defined by SA2. Proposal 6: Discuss the UE capability of supporting PRU after the issue of how to manage the PRU information has been determined. |
| Qualcomm [7] | Proposal 3: Since the specification details for Proposal 1 are in the realm of SA2, agree on the LS to SA2 in R2-2110827 [7]. Proposal 4: Before discussing specification changes for PRUs in RAN2, RAN2 should wait for the SA2 LCS Stage 2 support of PRUs. |

Dependent on how PRU registration and management will be supported (see Questions 2/3), the detailed RAN2 specification changes may differ, incl. the possibility of no RAN2 specification impacts at all (e.g., if LCS procedures are used to convey PRU specific information).

The Proposals above essentially suggest that the discussion on PRU details in RAN2 should be postponed until the PRU registration and management has been defined by SA2. However, the general PRU functions and capabilities (e.g., Question 1) could still be further discussed and decided in RAN2 in parallel to any SA2 work.

Based on this, moderator would suggest the following general way forward for discussion (note, some proposed items below depend on the outcome of the Questions asked above – no need to repeat individual comments which already have been made in the answers to the questions above).

Question 7:	Do you agree with the following Way Forward on how to handle the PRU topic:

(1)	Send a LS to SA2 asking SA2 to add location service support for PRUs such that an LMF can instigate location procedures with PRUs in the network.

(2)	Postpone the discussion of RAN2 specification details until the location service support for PRUs has been completed by SA2.

(3)	RAN2 continues to discuss the general PRU functionality and capabilities (e.g., like in Question 1).

(4)	Send an LS to RAN1, asking RAN1 which "correction information" determined from PRU measurements should be provided to target UEs for UE-based mode.

Please provide any comments on e.g., which items above can not be agreed, any proposed changes to the suggested items, or any additional items necessary at this stage.

| Company | Comments |
| --- | --- |
| Huawei, HiSIlicon | We agree with the following: (1), (3) For (2), we think RAN2 needs to progress on the required LPP changes, since this is already agreed by RAN1 as part of the R17 pos For (4), this is clarified by our answer above that the adjustment can be reflected by the AD and no additional information is needed. |
| vivo | Agree with (1)-(4). |
| ZTE | Support (1) and (4). (1) is to decide the whole skeleton and (4) is for stage 3 details |
| CATT | We agree with (3), (4), supporting PRU(s) in RAN level without impact on SA2 in Rel-17. One more question to RAN1 in (4), can the known location be the location which is calculated by PRU? For (1), there is no time budget in SA2 to support PRUs in Rel-17. RAN can support PRU in Rel-17 without impact on SA2. For (2), there is a solution in RAN level without SA2 impact to support PRU. RAN2 may make progress in Rel-17 based on the solution without impact on SA2. |
| Xiaomi | Agree with (3) and (4). For (1), if PRU can send MO-LR request to the LMF, we think there is no impact on SA2. |
| Intel | (3) (4) RAN2 can continue the work on PRU with necessary coordination with RAN1. |
| Apple | All of the above |
| Qualcomm | All |
| Qualcomm-2 | I meant to say: We support all items (1)-(4) in the proposed WF |
| Lenovo, Motorola Mobility | WF Options (3)(4), can be prioritised due to the limited time available. |
| Samsung | (3),(4) with same view as those supporting. |
| Ericsson | (1), (3) RAN2 cannot make the decision on its own. End to end responsibility is SA2 and hence SA2 needs to be consulted. RAN2 can make only one little change – introduce a new location information type to allow LMF to indicate position estimate and position measurement required. That is all that is needed. Provided SA2 provides their consent. |
| InterDigital | Agree with (1)-(4), with priority given to (3) and (4) |
| OPPO | Agree with (1)-(3). |
| Fraunhofer | All above. |
| Nokia | We support introducing a baseline PRU functionality in Rel-17 and make minimal enhancements on the RAN side to have a basic PRU functionality in Rel-17. Further enhancements can be considered in Rel-18 with support from SA2. We proposed some way forward options for Rel-17 work on PRU in our answers to Q2 and Q3. - In Rel-17, we can leave the details of how LMF obtains the location of the PRU to LMF implementation (or OAM configuration of static PRU location in LMF), OR, we can restrict PRU to use only MO-LR procedure (UE can be the LCS client) but enhance LPP to allow PRU to report it’s location to the LMF. - In Rel-17, provide support for use of corrections only for UE-assisted DL positioning techniques. |

Moderator's Summary:

-	16 companies replied to the question.

-	For none of the proposed four items there is a full consensus among all interested companies:

-	(1)	Send a LS to SA2:													Supported by 9 of 16 companies.

-	(2) Postpone the discussion of RAN2 specification details
		until SA2 specified support:										Supported by 6 of 16 companies.

-	(3) RAN2 continues to discuss the general PRU functionality:	Supported by 13 of 16 companies

-	(4) Send an LS to RAN1 on details of "correction information": Supported by 11 of 16 companies.

-	1 company (Nokia) suggests leaving the PRU support in Rel-17 up to implementation (e.g., OAM, etc.) or restrict PRU to use only MO-LR procedure (PRU can be the LCS client). In addition, it is suggested to support "correction information" for DL-only methods in Rel-17.

The companies which do not support sending an LS to SA2 believe that PRUs can already be supported with existing SA2/CT specifications and therefore, no consultation with SA2 is required. However, if PRUs can indeed be supported in the current LCS framework, SA2 could still confirm this.

Similar, the companies which do not support sending an LS to RAN1 believe that no additional "correction information" is needed beyond what is currently supported in the LPP assistance data. However, if this should indeed be the case, RAN1 could still confirm this intention.

Therefore, the following Way Forward is proposed at this stage:

Proposal 5:	Regarding the handling of the PRU topic, agree the following way forward:

(1) Send an LS to SA2 asking SA2 whether the MT-LR or MO-LR location procedures as currently specified in TS 23.273 can be used to enable an LMF obtaining location measurements from PRUs (via LPP) and to trigger SRS transmission of PRUs (via NRPPa), or whether an LMF needs to be enabled to instigate location procedures for a PRU (e.g., LPP, NRPPa procedures) without receiving a location request for the PRU from an AMF (i.e., in the absence of an MT-LR or MO-LR for the PRU), and if so, whether support can be provided as part of Release 17.

(2) Send an LS to RAN1 asking RAN1 whether the LMF determined "correction information" obtained from PRU measurements need to be provided to target UEs for UE-based mode of operation, and if so, ask RAN1 to provide further details on the specific "correction information" which need to be provided to target UEs. In addition, ask RAN1 to provide further details on the "PRU antenna orientation information" which should be provided to an LMF.

(3)	RAN2 continues to discuss the general PRU functionality and capabilities.

(4)	Revisit the handling of the PRU topic once a response LS from SA2 has been received.

A draft LS to SA2 and RAN1 (cc: RAN3) is proposed in Section 7 below.

# 6.	Other

For any other comments, companies are requested to use the following table.

| Company | Comments |
| --- | --- |

# 7.	Proposed Draft LS

3GPP TSG-RAN WG2 Meeting #116-e		R2-21xxxxx

Electronic Meeting, November 1 – 12, 2021

Title:	[draft] Response LS on Positioning Reference Units (PRUs) for enhancing positioning performance

Response to:	R2-2106920 (R1-2106326), LS on Positioning Reference Units (PRUs) for enhancing positioning performance

Release:	Rel-17

Work Item:	NR_pos_enh

Source:	Qualcomm Incorporated (to be RAN2)

To:	SA2, RAN1

Cc:	RAN3

Contact Person:

Name:                   	Sven Fischer

E-mail Address:   	sfischer@qti.qualcomm.com

1. Overall Description:

RAN2 thanks RAN1 for their LS on Positioning Reference Units (PRUs) for enhancing positioning performance.

RAN2 discussed support of PRUs as described in the RAN1 LS R2-2106920 (R1-2106326, S2-2105263). Based on the information provided by RAN1, RAN2 understands that a PRU is a device with a priori known location and which performs location measurements (at this known location). In addition, the PRU can transmit SRS to enable TRPs to measure and report UL positioning measurements from PRUs at known location. The PRU or TRP measurements can then be compared by a location server with the measurements expected to determine "correction information" for other nearby target devices. The location measurements for other target devices may then be corrected based on the previously determined "correction information".

RAN2 discussed the modelling of PRUs and agreed that the PRU can be considered (at least) as a UE (from LMF perspective). However, since the PRU location measurements are needed by an LMF itself different views exist in RAN2 on how this can be enabled in the current LCS architecture.

Some companies in RAN2 believe that PRU support has no impact on SA2 and CT specifications and think that the MT-LR procedures as currently specified in TS 23.273 can also be instigated by an LMF (e.g., an LMF may initiate a location request to a GMLC) or existing MO-LR procedures as specified in TS 23.273 can be insitgated by a PRU for enabling an LMF to obtain PRU location measurements or transmit UL-SRS.

Other companies in RAN2 believe that an LMF needs to be enabled to instigate location procedures for a PRU (e.g., LPP, NRPPa procedures) without receiving a location request for the PRU from an AMF (i.e., in the absence of an MT-LR or MO-LR for the PRU). These companies also believe that this effectively means that the LMF should act as an "LCS Client" for PRUs.

However, since the LCS procedures and overall architecture are in the realm of SA2, RAN2 would like to confirm with SA2 whether PRU support as described by RAN1 can be provided with the current LCS framework as specified by SA2 (e.g., TS 23.273) or whether any changes would be required.

In addition, RAN2 discussed whether the LMF determined "correction information" obtained from PRU measurements need to be provided to target UEs for UE base mode of operation. As mentioned by RAN1 in the LS, a PRU may provide "antenna orientation information" to an LMF and RAN2 would like to ask RAN1 to provide further details of the "PRU antenna orientation information".

2. Actions:

To SA2 group.

ACTION: 	RAN2 kindly asks SA2 whether the MT-LR or MO-LR location procedures as currently specified in TS 23.273 can be used to enable an LMF obtaining location measurements from PRUs (via LPP) and to trigger SRS transmission of PRUs (via NRPPa), or whether an LMF needs to be enabled to instigate location procedures for a PRU (e.g., LPP, NRPPa procedures) without receiving a location request for the PRU from an AMF (i.e., in the absence of an MT-LR or MO-LR for the PRU), and if so, whether support can be provided as part of Release 17.

To RAN1 group.

ACTION: 	RAN2 kindly asks RAN1 whether the LMF determined "correction information" obtained from PRU measurements need to be provided to target UEs for UE-based mode of operation, and if so, kindly asks RAN1 to provide further details on the specific "correction information" which need to be provided to target UEs.
RAN2 also kindly asks RAN1 to provide further details on the "PRU antenna orientation information" which should be provided to an LMF.

3. Date of Next TSG-RAN2 Meetings:

TSG-RAN2 Meeting #116-bis-e		17-25 January 2022				Electronic Meeting

TSG-RAN2 Meeting #117-e		21 February – 3 March 2022		Electronic Meeting

# 8.	Summary of Proposals

Proposal 5:	Regarding the handling of the PRU topic, agree the following way forward:

(1) Send an LS to SA2 asking SA2 whether the MT-LR or MO-LR location procedures as currently specified in TS 23.273 can be used to enable an LMF obtaining location measurements from PRUs (via LPP) and to trigger SRS transmission of PRUs (via NRPPa), or whether an LMF needs to be enabled to instigate location procedures for a PRU (e.g., LPP, NRPPa procedures) without receiving a location request for the PRU from an AMF (i.e., in the absence of an MT-LR or MO-LR for the PRU), and if so, whether support can be provided as part of Release 17.

(2) Send an LS to RAN1 asking RAN1 whether the LMF determined "correction information" obtained from PRU measurements need to be provided to target UEs for UE-based mode of operation, and if so, ask RAN1 to provide further details on the specific "correction information" which need to be provided to target UEs. In addition, ask RAN1 to provide further details on the "PRU antenna orientation information" which should be provided to an LMF.

(3)	RAN2 continues to discuss the general PRU functionality and capabilities.

(4)	Revisit the handling of the PRU topic once a response LS from SA2 has been received.

A draft LS to SA2 and RAN1 (cc: RAN3) is proposed in Section 7 below.

Proposal 3:	RAN2 confirm that the PRU considered as a UE supports the normal LPP procedures for PRU capability transfer.

Proposal 1:	RAN2 confirms that a PRU can support at least the following functionality (as described in the RAN1 LS), dependent on PRU capability:

- Provide the positioning measurements (e.g., RSTD, RSRP, Rx-Tx time differences) to an LMF.

- Transmit the UL SRS signals for positioning.

- Provide its own known location coordinate information to an LMF.

- Provide its antenna orientation information to an LMF.

Proposal 2:	RAN2 to discuss further whether PRU specific information can be configured in an LMF via proprietary means (e.g., OAM), and if so, which PRU specific information this includes.

Already included in Proposal 5:

Proposal 4:	Send an LS to RAN1 asking RAN1 whether the "correction information" derived by an LMF from PRU measurements should be provided to target UEs for UE-based mode of operation, and if so, what these "correction information" includes.