3GPP TSG-RAN WG2 #116-e											R2-2111427

Online, 1 – 12 November 2021

Agenda item:	6.2.3

Source: 	LG Electronics Inc. (Rapporteur)

Title: 	Summary [AT116-e][708][V2X/SL] Miscellaneous CR on 38.321

Document for:	Discussion and decision

# Introduction

This document summarizes the offline discussion as:

- [AT116-e][708][V2X/SL] Miscellaneous CR on 38.321 (LG)

Scope: Discuss CRs in R2-2110159, R2-2109597, R2-2110058, R2-2110829, R2-2109534, R2-2111138, and R2-2110832, and merge the agreeable changes. Note agreements from discussion in R2-2109417, R2-2109418/R2-2109598, and R2-2110152 are also captured.

Intended outcome: 38.321 CR in R2-2111426 and discussion summary in R2-211427 (if need)

Deadline: 11/9, 10:00am UTC

| Company | Name | E-mail |
| --- | --- | --- |
| LG | Giwon Park | giwon.park@lge.com |
| Apple | Zhibin Wu | zhibin_wu@apple.com |
| MediaTek | Guanyu Lin | guanyu.lin@mediatek.com |
| OPPO | Qianxi Lu | qQianxi.lu@oppo.com |

# Correction CRs

Table 1: Correction CRs: R2-2111138, R2-2100159

| Tdoc | Title | Company | Summary of changes/proposals | Rapporteur’s view |
| --- | --- | --- | --- | --- |
| R2-2111138 /R2-2100159 | Corrections on Parameter Description of the Formula for Computing CG slots | CATT | - Add ‘logical’ in definition of T'max. - Change to Change to Proposed change: | 1st change (add ‘logical’): Rapporteur doesn't think there is a need for correction because the sentence in the clause is referring to the RAN1 specification. 2nd change is an editorial change. |

Q1: On the above CR in R2-2111138/R2-2100159 in Table 1, do you agree on the change as proposed?

| Company | Agree/disagree on the proposed change | Wording change (suggestion) if needed | Further comments |
| --- | --- | --- | --- |
| ZTE | Disagree 1st change Agree 2nd change. |  | Agree rapporteur that 1st change is not needed. Slots that belongs to the associated resource pool means that this slot is a logical slot. |
| APPLE | 2nd chnge is fine |  |  |
| MediaTek | Agree 1st and 2nd change |  | For the first change, we think there is no harm to emphasize “logical”. |
| OPPO | 2nd change is fine |  |  |
| LG | 2nd change is fine |  |  |

Table 2: Correction CRs: R2-2109597

| Tdoc | Title | Company | Summary of changes/proposals | Rapporteur’s view |
| --- | --- | --- | --- | --- |
| R2-2109597 | Correction on the dynamic sidelink grants | Huawei, HiSilicon | Proposal. Add the descriptions that a sidelink grant addressed to SL-RNTI is considered as dynamic sidelink grant. | It is missing and needs to be added. |

Q2: On the above CR in R2-2109597 in Table 2, do you agree on the change as proposed?

| Company | Agree/disagree on the proposed change | Wording change (suggestion) if needed | Further comments |
| --- | --- | --- | --- |
| ZTE | Disagree |  | We do not see the necessary to capture it explicitly. What we can do with this sentence? |
| Apple | No | I think the intention of the sentence is listed a special case for dynamic grant. There is no need to add the normal case in the same sentence. If it is not clear, we can say “is also considered as a dynamical Sidelink grant” |  |
| MediaTek | Agree |  | Fine to us for this sentence to cover all cases of dynamic sidelink grant. |
| OPPO | Agree |  | We have raised this proposal previously but rejected by R2.. yet still good to see there are also interest from others to correct this – if there is no usage for this, we can remove this sentence (actually there is), otherwise, good to cover all cases. |
| LG | Agree |  |  |

Table 3: Correction CRs: R2-2110058

| Tdoc | Title | Company | Summary of changes/proposals | Rapporteur’s view |
| --- | --- | --- | --- | --- |
| R2-2110058 | Correction on the usage of sl-ReselectAfter | Apple, OPPO, Qualcomm Incorporated, Huawei, HiSilicon |  | Proposed change may be merged in Rapporteur CR as a reflection of the result of the RAN1's discussion (RAN1 LS R1-2108438). |

Q3: On the above CR in R2-2100058 in Table 3, do you agree on the change as proposed?

| Company | Agree/disagree on the proposed change | Wording change (suggestion) if needed | Further comments |
| --- | --- | --- | --- |
| ZTE | Agree |  |  |
| Apple | Agree (Proponent) |  |  |
| MediaTek | Agree |  |  |
| OPPO | Agree (Proponent) |  |  |
| LG | Agree |  |  |

Table 4: Correction CRs: R2-2110829

| Tdoc | Title | Company | Summary of changes/proposals | Rapporteur’s view |
| --- | --- | --- | --- | --- |
| R2-2110829 | Correction on TX parameters selection | ZTE Corporation, Sanechips | Reason for change: In 38.331, both sl-PSSCH-TxConfigList and sl-CBR-PriorityTxConfigList are optional IE, however, how to handle the case that sl-PSSCH-TxConfigList is not configured is missing in the MAC specification. Proposed change: | Proposed change is reasonable. |

Q4: On the above CR in R2-2110829 in Table 4, do you agree on the change as proposed?

| Company | Agree/disagree on the proposed change | Wording change (suggestion) if needed | Further comments |
| --- | --- | --- | --- |
| ZTE | Agree |  |  |
| Apple | Agree |  |  |
| MediaTek | Agree |  |  |
| OPPO | Agree |  |  |
| LG | Agree |  |  |

Table 5: Correction CRs: R2-2109534

| Tdoc | Title | Company | Summary of changes/proposals | Rapporteur’s view |
| --- | --- | --- | --- | --- |
| R2-2109534 | Corrections to Sidelink BWP operation | Samsung | Proposal. Added the following: - transmission/reception for S-PSS and S-SSS and BSR triggering only if SL BWP is active. - Cancellation of SL CSI reporting, SL BSR reporting and SR reporting upon BWP deactivation. 1st change 2nd change | The intention of the proposal is understandable. However, from the Rapporteur point of view, we are not sure whether these changes are really critical at this late timing of the maintenance phase, but we can check other companies’ views. |

Q5: On the above CR in R2-2109534 in Table 5, do you agree on the change as proposed?

| Company | Agree/disagree on the proposed change | Wording change (suggestion) if needed | Further comments |
| --- | --- | --- | --- |
| ZTE | Agree for canceling SR and BSR. Disagree for others |  | Considering that S-PSS and S-SSS are transmitted together with SL-BCH(i.e. sidelink MIB message), and current spec captures that SL-BCH is not transmitted if BWP is deactived , we think PHY layer will handle this case. For canceling CSI report procedure, we do not see the necessary to do this. |
| Apple | 1st change is fine. 2nd change is not needed. |  | We have concern that if we agree with 2nd chang, then there are many maore places need to be fixed by add “if SL BWP is active” |
| MediaTek | Fine with the first change. Second change is not needed. |  |  |
| OPPO | Fine with the first change. Second change is not needed. |  |  |
| LG | Disagree |  | If the numerology (e.g., sub-carrier spacing, CP length) of the activated UL BWP does not match the numerology of the SL BWP in the legacy procedure, the SL BWP is deactivated. Uu's UL BWP can be dynamically changed (e.g., activation) through DCI, and if the above conditions are satisfied, SL BWP can be momentarily deactivated. Therefore, we do not think it is critical to introduce additional optimization in the maintenance phase now. |

Table 6: Correction CRs: R2-2110832

| Tdoc | Title | Company | Summary of changes/proposals | Rapporteur’s view |
| --- | --- | --- | --- | --- |
| R2-2110832 | Correction on HARQ information indication | ZTE Corporation, Sanechips | This CR is an addition of description according to the RAN1’s agreement (RAN1 CR R1-2108376). We can hear the opinions of other companies whether it can be captured like the suggestion below: Proposed change: | It would be enough for this parameter to be in the RAN1 specification. |

Q6: On the above CR in R2-2110832 in Table 6, do you agree on the change as proposed?

| Company | Agree/disagree on the proposed change | Wording change (suggestion) if needed | Further comments |
| --- | --- | --- | --- |
| ZTE | Agree |  | For the necessity of the note, it can be seen that current spec also describes it’s up to UE implementation to select the positive-negative or negative-only acknowledgement, we think it’s better to clarify how UE set the RV value. Additionally, according to RAN1’s modification, the RV value is indicated to PHY layer together with other HARQ information, like HARQ process number, NDI field,etc. And according to MAC spec, RV is one of HARQ information that will be indicated from MAC to PHY as shown in following: Sidelink transmission information: Sidelink transmission information included in a SCI for a SL-SCH transmission as specified in clause 8.3 and 8.4 of TS 38.212 [9] consists of Sidelink HARQ information including NDI, RV, Sidelink process ID, HARQ feedback enabled/disabled indicator, Sidelink identification information including cast type indicator, Source Layer-1 ID and Destination Layer-1 ID, and Sidelink other information including CSI request, a priority, a communication range requirement and Zone ID.. There, the RV value is indicated from MAC to PHY. |
| Apple | Agree with the intention, but the NOTE is not needed. |  |  |
| MediaTek | Agree |  |  |
| OPPO | Agree with the intention, but the NOTE is not needed. |  |  |

Table 7: Correction CRs: R2-2109417

| Tdoc | Title | Company | Summary of changes/proposals | Rapporteur’s view |
| --- | --- | --- | --- | --- |
| R2-2109417 | Left issue on maxTransNum | OPPO, Apple, Ericsson, Lenovo, Motorola Mobility | RAN2 agreements in this e-meeting - Proposal 1 RAN2 confirm the revised WA that “UE assumes that next retransmission(s) of the MAC PDU is required when FB is disabled (and PUCCH is configured), for CG, if sl-CG-MaxTransNumList is configured with a value not larger than the number of CG resources available, when sl-CG-MaxTransNum is not reached”.  Agreed. Will be captured as normative text and wordings will be discussed in [AT116-e][708]. - Proposal 2 When FB is disabled (and PUCCH is configured), if sl-CG-MaxTransNumList is configured with a value larger than the number of CG resources available, when CG resource is exhausted and sl-CG-MaxTransNum is not reached, UE assumes that next retransmission(s) of the MAC PDU is required and thus reporting NACK. Agreed. Will be captured as normative text and wordings will be discussed in [AT116-e][708]. | Rapporteur thinks that the condition that sl-MaxTransNum is set smaller/larger than the number of CG resources does not need to be specified in the specification. Thus, I made a text that can be commonly applied to P1/P2 as below. Rapporteur suggestion on correction CR ##### 5.22.1.3.1a Sidelink process 1> if sl-MaxTransNum corresponding to the highest priority of the logical channel(s) in the MAC PDU has been configured in sl-CG-MaxTransNumList for the sidelink grant by RRC and the number of transmissions of the MAC PDU has been reached to sl-MaxTransNum; or 1> if a positive acknowledgement to this transmission of the MAC PDU was received according to clause 5.22.1.3.2; or 1> if negative-only acknowledgement was enabled in the SCI and no negative acknowledgement was received for this transmission of the MAC PDU according to clause 5.22.1.3.2: 2> flush the HARQ buffer of the associated Sidelink process. - if HARQ feedback has been disabled for the MAC PDU and sl-MaxTransNum corresponding to the highest priority of the logical channel(s) in the MAC PDU has been configured in sl-CG-MaxTransNumList for the sidelink grant by RRC and the number of transmissions of the MAC PDU has not reached sl-MaxTransNum after all PSSCH duration(s) in a sl-PeriodCG for the sidelink grant: - instruct the physical layer to signal a negative acknowledgement on the PUCCH according to clause 16.5 of TS 38.213 [6]. |

Q7: On the above proposal in R2-2109417 in Table 7, do you agree on the Rapporteur’s suggestion for CR correction?

| Company | Agree/disagree on the proposed change | Wording change (suggestion) if needed | Further comments |
| --- | --- | --- | --- |
| ZTE | Agree | sl-CG-MaxTransNumList,sl-MaxTransNum should be in italics |  |
| Apple | Agree with the intention , but we have comments | “if sl-PUCCH-Config is configured” is missing as one of the conditions. But in overlal, we think thte change is better done in 5.22.1.3.2 along with other PUCCH transmissions, insead of in 5.22.1.3.1a |  |
| MediaTek | Agree |  |  |
| OPPO | Agree with the intention, but the change is to be rephased | Firstly, we should change on 5.22.1.3.2, where the pucch reporting is described, instead of creating a new paragragh in 5.22.1.3.1a. |  |
| LG | Agree | Agree with Apple/OPPO’s suggestion. |  |

Table 8: Correction CRs: R2-2109418, R2-2109598

| Tdoc | Title | Company | Summary of changes/proposals | Rapporteur’s view |
| --- | --- | --- | --- | --- |
| R2-2109418 | Correction on UL-SL prioritization | OPPO, Apple, MediaTek, CATT | #116-e agreement: - Cross reference issue needs to be removed. - Detailed wording/update will be further discussed in [AT116-e][708]. | At this meeting, it was agreed that only the cross-reference would be removed, so Rapporteur doesn't think any other modifications other than the ones (R2-2109418) below are necessary. |
| R2-2109598 | Clarification on the UL and NR SL prioritization | Huawei, HiSilicon |  |  |

Q8: On the above CR in R2-2109418, R2-2109598 in Table 8, do you agree on the change as proposed?

| Company | Agree/disagree on the proposed change | Wording change (suggestion) if needed | Further comments |
| --- | --- | --- | --- |
| ZTE | Agree |  |  |
| Apple | Agree with R2-2109418 |  |  |
| MediaTek | Agree with R2-2109418 |  |  |
| OPPO | Agree (proponent) |  |  |
| LG | Agree |  |  |

Table 9: Correction CRs: R2-2110152

| Tdoc | Title | Company | Summary of changes/proposals | Rapporteur’s view |
| --- | --- | --- | --- | --- |
| R2-2110152 | Clarification on exceptional pool configuration | LG | #116-e agreement - At least one sl-TxPoolExceptional pool is always a resource pool in which PSFCH is configured - Will be captured as the note in MAC. Detailed wordings will be discussed in [AT116-e][708]. Proposed change: NOTE 2: The MAC entity expects that PSFCH is always configured by RRC for at least one pool of resources in sl-TxPoolSelectedNormal and for the resource pool in sl-TxPoolExceptional in case that at least a logical channel configured with sl-HARQ-FeedbackEnabled is set to enabled. |  |

Q9: On the above proposal in R2-2110152 in Table 9, do you agree on the suggestion for CR correction?

| Company | Agree/disagree on the proposed change | Wording change (suggestion) if needed | Further comments |
| --- | --- | --- | --- |
| ZTE | Agree |  |  |
| Apple | Agree |  |  |
| MediaTek | Agree |  |  |
| OPPO | Agree with wording suggestion | NOTE 2: The MAC entity expects that PSFCH is always configured by RRC for at least one resource pool of resources in sl-TxPoolSelectedNormal and for the resource pool in sl-TxPoolExceptional in case that at least a logical channel configured with sl-HARQ-FeedbackEnabled is set to enabled. |  |
| LG | Agree |  |  |

# Conclusion