3GPP TSG RAN WG2#116-e	R2-2109324

e-Meeting, 1st - 12th November, 2021

3GPP TSG-RAN WG1 Meeting #106-e	R1-2108622

e-Meeting, August 16 – 27, 2021

Title:	Reply LS on time gap information in SCI

Response to:	R1-2106413 (R2-2106623)

Release:	Rel-17

Work Item:	NR_SL_enh-Core

Source:	RAN1

To:	RAN2

Cc:	-

Contact Person:

#### Name:	Kevin Lin

E-mail Address:	<Kevin.Lin@oppo.com>

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

1. Overall Description:

RAN1 thank RAN2’s LS in R2-2106623 and would like to provide our response to the following question.

Q1): For R17 SL DRX design, from RAN1 perspective, whether it is feasible for the Rx-UE to determine the time location of the next retransmission resource(s) of the TX UE (assuming that resource is not reselected by the TX UE) based on the “Time resource assignment” field in SCI?

Answer: Regarding RAN2’s question, in RAN1’s opinion it is feasible, other than in the following exceptional cases:

- SL transmission dropping due to prioritization or congestion control

- Due to re-evaluation, a re-selected resource is earlier than a reserved resource by UE implementation in Mode 2

- If (pre-)configured with many-to-one mapping between Tx and Rx resource pools in some cases (e.g., when PSFCH is not configured)

2. Actions:

To RAN WG2

RAN1 respectfully requests RAN2 to take the above response into account in the future work.

3. Dates of Next TSG-RAN WG1 Meetings:

TSG RAN WG1 Meeting #106bis-e	11 – 19 October 2021	eMeeting

TSG RAN WG1 Meeting #107-e	11 – 19 November 2021	eMeeting