3GPP TSG-RAN WG2 Meeting #116-electronic	 R2-2109438

Online, November 1st – November 12th, 2021                 Revision of R2-2107055

Source: 	vivo

Title:  	Handling of non-SDT Data Arrival via BSR

Agenda Item:	8.6.3

Document for:	Discussion and Decision

Introduction

In past RAN2 meetings, the following agreements regarding the handling of non-SDT DRB with data arrival had been achieved [1] - [2]:

RAN2#113e Agreement [1]: - The UE behaviour for handling of non-SDT data arrival after sending the first UL data packet is fully specified (i.e. not left to UE implementation). - FFS RAN2 will consider the additional option of using DCCH message to indicate arrival of non-SDT data (details to be discussed). Discussion will continue on all three options. RAN2#113bis-e Agreements [2]: - Non-SDT radio bearers are only resumed upon receiving RRCResume (same as today) - Down-scope to two solutions (CCCH or DCCH) and ask SA3 about security issues (explain that CCCH message will be repeated in same cell and ask if there is a question).

Therefore, in this contribution, we would like to provide our understanding on the handling of non-SDT data arrival. Hopefully, RAN2 can select a simple but efficient solution to handle the issue.

Discussion

After triggering SDT, there may be arriving data mapping to non-SDT RB(s) from upper layers. In some special cases, the non-SDT data may just arrive following the SDT data but before the first available UL transmission resource for the very first UL data transmission, as shown in the following Figure 1.

Considering we have already agreed that the arrival of non-SDT data should be reported to the network when this data comes after the very first UL data transmission in the SDT procedure, we think this design logic can be extended to the case where non-SDT data arrives prior to the very first UL data transmission in the SDT procedure. In other words, the UE should continue the ongoing SDT procedure in this scenario.

Figure 1: Arrival of non-SDT data

Proposal 1: If non-SDT data arrives before sending the first UL data packet but after triggering the SDT procedure, UE continues the ongoing SDT procedure and indicates the arrival of non-SDT data.

In RAN2#113bis-e meeting, the BSR reporting mechanism for indicating the arrival of non-SDT data was excluded. The reason is that most companies hold a view that BSR reporting cannot indicate the detailed data volume of an RB that is suspended. However, in the RAN2#114-e meeting UP session, a warm discussion on whether BSR calculation can be used for an RB that is suspended had been launched [3]. Further, in the RAN2#115-e meeting UP session, it was finally confirmed that the suspended RBs shall be considered for BSR calculation (i.e. no change to the specifications). In other words, BSR can be used to indicate the pending data volume of an RB that is suspended.

In our understanding, from the UE implementation point of view, there is no technical difference between an RB that is suspended due to RRC re-establishment and that due to RRC release with suspend configuration. In this sense, we think now it is clear that BSR calculation can be also used for RB(s) that is suspended in legacy NR, then it seems spontaneous that this solution can be inherently adopted for SDT procedure with data arrival from non-SDT RB(s) that is suspended. So we suggest reconsidering the BSR solution for indicating new data arrival from non-SDT RBs. This is because the BSR reporting mechanism might be the simplest (i.e. no new RRC behavior and new RRC message is needed, meanwhile, no AS security issue is found) and most effective solution (data volume can be reported to the network, compared to those two RRC based solutions) for the handling of non-SDT RBs. Otherwise, if BSR reporting for RB that is suspended is supported and we have also agreed on either the CCCH-based solution or DCCH-based solution for SDT, RAN2 has to further discuss the selection between the existing NR mechanism and the SDT-specific mechanism for the handling of non-SDT data arrival.

Proposal 2: RAN2 to reconsider the BSR solution for indicating new data arrival from non-SDT RBs.

Conclusions

In this contribution, we have provided our understanding on the handling of non-SDT data arrival. The following proposals are made:

Proposal 1: If non-SDT data arrives before sending the first UL data packet but after triggering the SDT procedure, UE continues the ongoing SDT procedure and indicates the arrival of non-SDT data.

Proposal 2: RAN2 to reconsider the BSR solution for indicating new data arrival from non-SDT RBs.

- 4. Reference

[1] RAN2#113-e Session Chair Notes (Small data transmission).

[2] RAN2#113bis-e Session Chair Notes (Small data transmission).

[3] R2-2106642 Report of [AT114-e][002][NR15] User Plane.

[4] Draft R2-115-e Meeting Report v1.