3GPP TSG-RAN WG2 #116-e	Tdoc R2-2109589

Electronic meeting, 2021-11-01 - 2021-11-12

Agenda Item:	8.1.2.1

Source:	Ericsson

Title:	Multicast Service Continuity Aspects

Document for:	Discussion, Decision

# 1	Introduction

In this document, we will discuss remaining aspects for Multicast Service Continuity regarding the RRC-based MRB bearer type change both for HO and non-HO scenarios. Mobility of RRC_CONNECTED UE to a non-MBS supporting NG-RAN node is also considered.

Relevant agreements from recent RAN2-115e meetings are as follows.

  - In RRC signalling, one MRB can be configured with PTM only or PTP only or both PTM and PTP. PTM, PTM+PTP or PTP-only can be changed from one to other via RRC signaling.

  - In RRC signalling, Support DL only UM RLC configuration for PTM, both DL and UL AM RLC configuration for PTP, DL only UM RLC configuration for PTP, FFS both DL and UL UM RLC configuration for PTP.

  - FFS whether PDCP SR can be triggered due to bearer type change in RRC signaling and FFS how to trigger PDCP SR if needed.

# 2	Discussion

## 2.1	General aspects of MRB configuration and RRC-based bearer type change

RAN2 already agreed MRB bearer can be configured with PTP-only, PTM-only, and split MRB. As discussed earlier in [1], without HO, the reliability requirement of potential MBS services can already be fulfilled by using PTM transmission with HARQ mechanism. Thus, to maximize benefit of multicast transmission, it is natural that either PTM-only MRB or split MRB is configured in the system. Typically, PTM-only configuration is used. When HO takes place, to minimize the potential data loss, PTM-only MRB is reconfigured to be split MRB for the use of PTP AM leg with PDCP SR and PDCP retransmission.

While PTP-only configuration is not typical, the split MRB is considered as means to perform dynamic switch between PTP and PTM transmission without RRC signaling, i.e., by having the MBS service scheduled through the PTP leg. That means the bearer type change between PTP-only to PTM-only MRB is not useful. We also note that RAN3 has agreed the switch between PTP to PTM should be handled at DU level [2]. Configuring PTP-only MRB would unnecessarily require F1 signaling between CU and DU, i.e., RRCReconfiguration.

PTM-only and split MRB are two typical configurations of an MRB.

Regarding PDCP operation upon MRB bearer type change, there is an open aspect being discussed in UP email discussion [3], i.e., whether PDCP re-establishment or PDCP data recovery needs to be performed for RRC-based MRB type change. It is worth considering two closely related cases: bearer type change for HO, and bearer type change without HO.

For the HO-related MRB bearer type change, as the PDCP anchor point changes, like legacy operation, either PDCP re-establishment or PDCP data recovery takes place depending on whether there is change in AS security key or not, respectively. Note that even though AS security is not used for protecting MBS data (as agreed in SA3#104-e, i.e., conclusion for the key issue#2 [4]), it is still needed for protection of control signaling related to multicast operation, and thus AS security key change in mobility is still applicable, as in legacy.

For MRB bearer type change due to HO, legacy PDCP procedures, i.e., PDCP re-establishment and PDCP data recovery are used as in legacy.

In case of bearer type change not due to HO, the PDCP anchor point does not change. The same common PDCP entity is used for both current and new type of MRB bearer. Moreover, in this case, there is no need to reset the ROHC protocol given that this bearer type change scenario does not impact the compression operation. There is additionally a need to have a new way of handling initialization of PDCP state variables. However, this does not justify having a PDCP re-establishment procedure which can be handled separately. In short, re-establishment due to bearer type change is unnecessary. In addition, PDCP data recovery is specified for UL transmission direction, and it is up to network implementation for providing a DL direction. It is proposed that no PDCP re-establishment or PDCP data recovery is needed for MBR bearer type change not connected to a mobility event/HO.

For MBR bearer type change not due to HO, no PDCP re-establishment nor PDCP data recovery is needed.

Another aspect is how to implement the bearer type change in RRC specification. This is captured in current running CR for TS38.331 [5]. This is assumed to be discussed in RAN2. In our understanding, the bearer type change can be performed as follows:

- Network reconfigures the type of bearer via in RRCReconfiguration message of which the radioBearerConfig IE is extended to contain mrb-ToAddModList and mrb-ToReleaseList to separate actions for configuring MRBs from DRBs and SRBs.

- Among many parameters, mrb-ToAddModList includes mrb-Identity, reestablishPDCP flag, recovePDCP flag, and pdcp-Config that indicates parameters for PDCP entity. pdcp-Config has a statusReportRequired flag, that is supposed to be set when either the reestablishPDCP flag or recovePDCP flag is set. As discussed above, the two flags for re-establishment and data recovery are only set in HO scenario.

- In the RRCReconfiguration, there is IE to reconfigure RLC, i.e., CellGroupConfig  rlc-BearerToAddModList  RLC-BearerConfig. RLC-BearerConfig contains logical channel ID (LCID) and servedRadioBearer as identity of associated radio bearer and thus linking the RLC entity with an PDCP entity. Similarly, there is also CellGroupConfig  rlc-BearerToReleaseList  LogicalChannelIdentity to indicate which RLC entity and corresponding logical channels to be released upon reconfiguration. By using these two RLC-related IEs simultaneously or separetely, network can realize the inclusion or removal of an RLC entity and corresponding logical channel from an MRB bearer. For example, two RLC bearers can refer to the same MRB bearer , i.e., mrb-Identity in servedRadioBearer to realize the split bearer setting.

- In short, when the bearer type change takes place, the radioBearerConfig  mrb-ToAddModList and/or mrb-ToReleaseList specify configuration for MRB and the associated common PDCP entity, while CellGroupConfig  rlc-BearerToAddModList and/or RLC-BearerToReleaseList specify configuration for linked RLC entities.

For MRB bearer type change, radioBearerConfig IE is extended to include mrb-ToAddModList and mrb-ToReleaseList IEs for configuration of MRB and its PDCP entity, while RLC-BearerConfig IE is extended to link an RLC entity to the common PDCP entity, i.e., via mrb-Identity as servedRadioBearer.

## PDCP SR for RRC-based MRB bearer type change

In the rapporteur report for UP email discussion [2], there are two relevant proposals below.

Proposal 4 (17/23):

-	In order to minimize the loss during MRB bearer type change, NW may configure UE to send a PDCP status report for the MRB bearer type change;

-	For MRB configured by upper layers to send a PDCP status report in the uplink (field statusReportRequired in PDCP-Config IE in RRC), the receiving PDCP entity shall trigger a PDCP status report in case of MRB type change;

- 	NW is required to configure a bidirectional PTP leg (e.g. either PTP-only MRB or split MRB) if statusReportRequired is provided. It is up to network in which case statusReportRequired is configured.

Proposal 5: If PDCP SR is supported for RRC based MRB bearer type change, it is FFS on whether the legacy triggers of PDCP SR (as ‘upper layer requests a PDCP data recovery’ or ‘upper layer requires a PDCP entity re-establishment’) are reused or new trigger(s) of PDCP status report should be defined.

As detailed above, we think no PDCP re-establishment or PDCP data recovery is needed for MBR bearer type change not connected to a mobility event/HO. Typically, PTM-only is used and reliability requirement can already be fulfilled. Even if the change of bearer type from PTM-only to either PTP-only or split MRB would be needed in future use cases, we think it is not necessary to try to minimize data loss for such a bearer type change considering that PTM transmission with UM RLC mode is not lossless by nature. It is therefore proposed to not minimize data loss for the MRB bearer type change not due to HO.

For MRB bearer type change not due to HO, no need to minimize data loss.

In addition, as analyzed in [1], we do not see the need to specify any new UE assistance information for the network to trigger UE to send PDCP SR. Legacy operation for PDCP SR applies for MRB bearer type change upon HO. That is, network may trigger UE to provide PDCP SR by setting reestablishPDCP or recoverPDCP and statusReportRequired flags as true for a MRB (re)configured with a bidirectional AM RLC entity.

No new UE-based trigger for transmission of PDCP status report are introduced.

For MRB bearer type change due to HO, network may trigger UE to send PDCP SR by setting reestablishPDCP or recoverPDCP and statusReportRequired flags as true for MRB configured with a bidirectional AM RLC entity.

In case there is not ongoing MBS at the target, a question is what type of MRB the target node should configure the UE via RCReconfiguration/Handover command? We think this decision is up to network, i.e., target node. However, if network starts configuring PTP-only MRB, it will require another round of RRCReconfiguration to benefit from PTM transmission. Whereas the split MRB with PTP AM RLC entity allows for the reliable HO as well as benefiting from PTM transmission without the need of an additional RRCReconfiguration.

For HO to an MBS-supporting node without the ongoing MBS session, it is up to network to configure the type of MRB bearer. Split MRB allows for reliable HO as well as benefits from PTM transmission without additional RRCReconfiguration.

## 2.3 HO to non-MBS supporting nodes

Regarding HO to non-MBS supporting node, recent RAN2-115e meeting has agreed:

- RAN2 assumes that from RAN2 perspective, mobility from the source gNB supporting MBS to target gNB not supporting MBS can be achieved by switching the traffic from delivery via MRB to delivery via DRB either before or during the handover. Whether and how this can be done without data losses has to be further investigated and requires progress and input from other WGs, i.e., RAN3 and SA2.

In addition, there is an Editor’s note in current TS38.300 running CR [6]:

- FFS whether the switching the traffic from delivery via MRB to delivery via DRB either before or during the handover.

Companies in the CP email discussion [7] think that the source gNB provides multicast data via DRB shortly before the handover to minimize the loss due to time needed for full configuration at target cell. We think that where/when this change takes place, i.e., if the change is in supporting source node or during HO, i.e., target establishes PDU and decides configuration in RRCReconfiguration to send to UE, has no RAN2 impact. This topic should be discussed in RAN3 first. In case the switch from delivery via MRB to via DRB is made at the source node before the handover, for example, to minimize the possible data loss, RAN2 need to discuss how to reconfigure/convert from MRB to DRB transmission. It is proposed that RAN2 consult RAN3 in an LS before considering further this aspect.

Whether switching from delivery via MRB to delivery via DRB before or during the handover to non-MBS supporting node has no RAN2 impact.

LS to RAN3 and SA2 to consult whether to convert delivery via MRB to delivery via DRB to support mobility from an MBS supporting node to non-MBS supporting node before or during the handover.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	PTM-only and split MRB are two typical configurations of an MRB.

Observation 2	For MRB bearer type change due to HO, legacy PDCP procedures, i.e., PDCP re-establishment and PDCP data recovery are used as in legacy.

Observation 3	For HO to an MBS-supporting node without the ongoing MBS session, it is up to network to configure the type of MRB bearer. Split MRB allows for reliable HO as well as benefits from PTM transmission without additional RRCReconfiguration.

Observation 4	Whether switching from delivery via MRB to delivery via DRB before or during the handover to non-MBS supporting node has no RAN2 impact.

Based on the discussion in the previous sections we propose the following:

Proposal 1	For MBR bearer type change not due to HO, no PDCP re-establishment nor PDCP data recovery is needed.

Proposal 2	For MRB bearer type change, radioBearerConfig IE is extended to include mrb-ToAddModList and mrb-ToReleaseList IEs for configuration of MRB and its PDCP entity, while RLC-BearerConfig IE is extended to link an RLC entity to the common PDCP entity, i.e., via mrb-Identity as servedRadioBearer.

Proposal 3	For MRB bearer type change not due to HO, no need to minimize data loss.

Proposal 4	No new UE-based trigger for transmission of PDCP status report are introduced.

Proposal 5	For MRB bearer type change due to HO, network may trigger UE to send PDCP SR by setting reestablishPDCP or recoverPDCP and statusReportRequired flags as true for MRB configured with a bidirectional AM RLC entity.

Proposal 6	LS to RAN3 and SA2 to consult whether to convert delivery via MRB to delivery via DRB to support mobility from an MBS supporting node to non-MBS supporting node before or during the handover.

# 4	References

R2-2108080, “Reliability for Multicast and for Multicast Service Continuity”, Ericsson, RAN2#115e, August 2021.

R3-21xxxx, “Disc and TP Bearer Management F1E1”, Ericsson, RAN3#114e, November 2021.

R2-21xxxxx, “Report of [Post115-e][092][MBS] UP issues”, Lenovo, RAN2#116e, November 2021.

Meeting Report for SA3_104-e, SA3#104-e, August 2021.

TS38.331 Running CR for MBS in NR (Huawei, HiSilicon).

TR38.300 Running CR for MBS in NR (CMCC).

R2-21xxxxx, “Report of [Post115-e][091][MBS] CP issues”, Huawei, RAN2#116e, November 2021.