3GPP TSG RAN WG2#116-e	R2-2109890

e-Meeting, 1st - 12th November, 2021

Title:	[Draft] Reply LS on specification impact for methods on efficient utilization of licensed spectrum that is not aligned with existing NR channel bandwidths

Response to:	R2-2109353/R4-2114751

Release:	Rel-17

Study Item:	FS_NR_eff_BW_util

Source:	ZTE Corporation [to be RAN2]

To:	RAN4, RAN1

Cc:

Contact Person:

Name: 	Jing Liu

E-mail Address: 	liu.jing30@zte.com.cn

Attachments:	N/A

1. Overall Description:

RAN2 would like to thank RAN4 for the LS on specification impact for methods on efficient utilization of licensed spectrum that is not aligned with existing NR channel bandwidths.

RAN1 discussed the questions, and would like to provide the following response to RAN4 questions:

- For the wider CBW:

  - clarify if there is any limitation for the UL carrier positions (not just BWP positions) legacy UEs support for uplinkChannelBW-PerSCS-List and scs-SpecificCarrierList in symmetric operating bands with a fixed duplex distance and asymmetric UL/DL channel bandwidth.

  - RAN2 response: In RAN2 specification, there is no limitation on the UL carrier position for UE specific channel bandwidth in symmetric operating bands with a fixed duplex distance and asymmetric UL/DL channel bandwidth. RAN2 specification only specifies for TDD operation, the BWP pair (UL BWP and DL BWP with the same bwp-Id) must have the same center frequency.

  - confirm UE behaviour if it is possible to configure a carrier that is not fully contained in the NR band, i.e. the carrier can extend beyond the low edge of the band and/or the high edge of the band?

  - RAN2 response: From RAN2 perspective, configuring a carrier that is not fully contained in the NR band can be regarded as an invalid configuration, and the UE behaviour in such case is unclear.

- For the overlapping CBWs from network perspective (one cell approach):

  - clarify whether a single SSB and CORESET (e.g. for cases where irregular BWs >10 MHz where a 4.28 MHz wide initial BWP can be in the common frequency range), can be used to configure UEs with different channel BWs on different parts of the BS channel.

  - RAN2 response: From RAN2 perspective, RRC_CONNECTED UEs will be configured with UE specific channel bandwidth through dedicated signalling, so in case single SSB and CORESET are deployed, it is feasible to configure UEs with different channel BWs on the different parts of the BS channel.

  - clarify whether two time staggered SSBs and CORESET#0 on the same frequency (when the frequency separation is not enough to send them simultaneously at the same time and thus time staggering is needed) are supported in RAN1/2 specifications so that UEs configured with left and right channels of the next smaller regular size can track their own time staggered SSB and CORESET#0.

  - RAN2 response: RAN2 specification does not prohibit network to transmit two time staggered SSBs and CORESET#0, but the two SSBs and CORESET#0 should be associated to separate SIB1 (with different NCGI), so from UE perspective, they are two different cells..

- For the overlapping CBWs from UE perspective (two cell approach / CA approach):

  - if two different Bandwidth Parts for the UE are overlapping, and both contain a subset of CSI-RS resources that are mapped to the same subset of overlapping RBs for the same UE, please clarify how does UE report CSI for the overlapped part, e.g. does UE report CSI for each cell separately, or just once for the overlapping part, or something else?

  - clarify how PDCCH reception in overlapped CA when PCell and SCell PDCCH resources partially overlap and whether there are any impacts to cross-carrier scheduling

  - RAN2 response: RAN2 leaves the physical layer operation related questions to RAN1 to answer, and RAN2 confirms that no UE capability was designed for indicating the support of overlapping CA.

- For the overlapping CBWs from UE perspective (one cell approach):

  - Is it possible to configure the UE with a dedicated carrierBandwidth in the ServingCellConfig that is wider than/partially outside the carrierBandwidth configured in SIB1?

  - RAN2 response: From RAN2 perspective, this configuration is invalid, network needs to ensure the dedicated carrierBandwidth configured in ServingCellConfig is completely within the carrierBandwidth configured in SIB1.

  - Clarify for equalization purposes in the DL, does the BS need to know the split between the subset of PRBs from a main RF carrier versus PRBs from an additional RF carrier are received on different channel/antenna before combining. If pre-coding assumes all PRBs experience the same channel/antenna, is signalling required so that BS pre-coding can account for the path differences of main carrier PRBs and additional carrier PRBs.

  - RAN2 response: From RAN2 perspective, the RAN2 defined functions do not require the BS to know the split of PRBs between main RF carrier and additional RF carrier.

2. Actions:

To RAN4:

ACTION: RAN2 respectfully asks RAN4 to take above responses into account in their further work.

3. Date of Next TSG WG RAN2 Meetings:

3GPP  RAN2#117-e 	         21th  – 25th February 2022		   	Electronic Meeting