3GPP TSG-RAN WG2 Meeting #116 electronic	R2-2110402

Online, 1st -12th November 2021

Source:	CATT

Title:	Further Consideration on Paging Subgrouping

Agenda Item:	8.9.2.1

Document for:	Discussion and Decision

Introduction

A post email discussion [Post115-e][089] on paging subgrouping [1] was triggered after RAN2#115-e to progress this topic. In this contribution, we would like to give our consideration based on RAN1’s progress.

Discussion

The following agreements on UE subgroup indication were agreed in RAN1.

RAN1#105-e Agreement: For UE subgroups indication in physical layer, maximum of 8 subgroups per PO is supported. RAN1#106b-e Agreement: For NR Rel-17, paging indications to UE subgroups are carried only in PEI.

And on the draft RRC parameters list for R17 power saving in RAN1, a new parameter, subgroupsNumPerPO, was introduced to indicate the “Number of subgroups supported per PO in the cell, for UE to read the subgroup indication from physical layer signaling”:

| WI code | Sub-feature group | RAN1 specification | Section | RAN2 Parent IE | RAN2 ASN.1 name | Parameter name in the spec | New or existing? | Parameter name in the text | Description | Value range | Default value aspect | Per (UE, cell, TRP, …) | UE-specific or Cell-specific | Specification | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8.7.1.1 Paging Enh. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Paging enhancement | 38.213 |  |  |  | subgroupsNumPerPO | new |  | Number of subgroups per Paging Occasion (PO) for UE to read subgroups indication from physical-layer singling | TBD with maximum of 8 |  | per cell | cell-specifc |  | Agreement: For UE subgroups indication in physical layer, maximum of 8 subgroups per PO is supported. |

It can be noted that RAN1 did not distinguish the number of subgroups for CN-assigned, Nsg-CN and UEID-based, Nsg-UEID, but considered one single value. Therefore subgroupsNumPerPO denotes the total number of supported subgroups per PO, Ntot, irrespective of whether:

- Ntot = Nsg-CN or Nsg-UEID if the cell supports only one type of subgroups, CN-assigned or UEID-based, or

- Ntot = Nsg-CN + Nsg-UEID if the cell supports both types of subgroups with non-overlapping sets

- Ntot = Nsg-UEID if the cell supports both types of subgroups with overlapping sets and CN-assigned subgroup ID remapping.

Observation 1: RAN1 agreed that the total number of supported subgroups per PO in PEI, subgroupsNumPerPO, is cell-specific and RRC configured.

And since this parameter is cell-specific it means it is decided by RAN, not CN.

Observation 2: The total number of supported subgroups per PO in PEI, subgroupsNumPerPO, is decided by RAN.

Proposal 1: Based on RAN1 agreements, RAN2 confirms the total number of subgroups supported in a cell is configurable, decided by RAN, and can be different in different cells.

No matter whether and how CN-assigned subgrouping co-exists with UEID-based subgrouping, the CN-assigned subgroup is carried in PEI. Thus, considering RAN1’s agreements, the number of CN-assigned subgroups supported in a cell is constrained by RAN, and can be different in different cells.

Proposal 2: Based on RAN1 agreement, RAN2 confirms the number of CN-assigned subgroups supported in a cell is constrained by RAN, and can be different in different cells.

In last RAN2#115 e-meeting, we agreed:

R2 assumes that All the cells within the registration area supports the same number of CN assigned subgroups, i.e. no remapping of CN assigned group ID to RAN subgroup ID (will revisit only if serious issues are found).

We indeed find serious issues with this agreement:

- Registration area is a concept from UE side and can be different across UEs of the same cell. But the number of CN-assigned subgroups for a cell commonly applies to all UEs in that cell. Therefore it is not accurate (impossible?) to say all the cells within the registration area support the same number of CN-assigned subgroups.

- Since the number of CN-assigned subgroups supported in a cell is constrained by the total number of subgroups supported in that cell (which is decided by RAN), and AMF connects with a large number of gNBs, while a gNB, in turn, connects with multiple AMFs, it is inefficient to exchange the number of CN-assigned subgroups supported between AMF and gNBs. Moreover, mandating that all the cells with the same tracking area support the same number of CN-assigned subgroups is useless as it does not bring any benefit, complexity-wise, on CN, gNB, or UE. Then it is possible that the number of CN-assigned subgroups in CN is larger than the total number of subgroups supported in a cell. In order to support flexible paging subgroup mechanism, remapping of CN-assigned subgroup ID by CN to subgroup indication in PEI needs to be re-considered.

Observation 3: Serious issues are found with the RAN2 agreement mandating a same number of CN assigned subgroups across cells from same RA.

Proposal 3: RAN2 reconsiders supporting remapping of  subgroup ID assigned by AMF onto the subgroup set supported in PEI by RAN.

Conclusion

In this contribution, we discuss the total number of subgroups supported in a cell based on RAN1’s agreement. We observe and propose:

Observation 1: RAN1 agreed that the total number of supported subgroups per PO in PEI, subgroupsNumPerPO, is cell-specific and RRC configured.

Observation 2: The total number of supported subgroups per PO in PEI, subgroupsNumPerPO, is decided by RAN.

Proposal 1: Based on RAN1 agreements, RAN2 confirms the total number of subgroups supported in a cell is configurable, decided by RAN, and can be different in different cells.

Proposal 2: Based on RAN1 agreement, RAN2 confirms the number of CN-assigned subgroups supported in a cell is constrained by RAN, and can be different in different cells.

Observation 3: Serious issues are found with the RAN2 agreement mandating a same number of CN assigned subgroups across cells from same RA.

Proposal 3: RAN2 reconsiders supporting remapping of  subgroup ID assigned by AMF onto the subgroup set supported in PEI by RAN.

Reference

- R2-2109647, Summary of [Post115-e][089][ePowSav] Paging Subgrouping, Xiaomi, RAN2#116 e-meeting