3GPP TSG-RAN WG2 Meeting #116 electronic					                          R2-2110524

# Online, 1 – 12 November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 2836 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on Timing Reference of PSCell SMTC Configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2021-10-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | For the case of NR SA to EN-DC HO with PSCell addition, it’s unclear whether the UE applies the SMTC configuration based on the timing reference of source NR PCell or target EUTRA PCell according to current RRC spec. The reason is that current TS 36.331 only refers to the case of EN-DC handover with PSCell addition or SN change and TS 38.331 only refers to the case of EN-DC handover with PSCell addition or SN change. Furthermore, the network has a way to determine the SMTC of the target NR PSCell based on the timing of the source NR PCell since the NR PCell can know the timing of the target NR PSCell. Therefore, it should be clarified that for the case of NR SA to EN-DC handover with PSCell addition, the source PCell provides the SMTC configuration of the target NR PSCell and the SMTC configuration is based on the timing reference of source PCell. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that during the NR SA to EN-DC handover with PSCell addition, the PSCell SMTC configuration can be provided by source NR PCell based on source PCell timing, and UE should apply the source NR PCell timing as the reference. Impact analysis Impacted functionality: SMTC Impacted architecture options: NR SA, EN-DC Inter-operability: - If the UE implements the CR, but the network does not. If the network provides the PSCell SMTC configuration based on target EUTRA PCell timing, but the UE applies the configuration based on source NR PCell, the UE could not acquire the PSCell DL timing soon. - If the network implements the CR, but the UE does not. If the network provides the PSCell SMTC configuration based on source NR PCell timing, but the UE applies the configuration based on target EUTRA PCell, the UE could not acquire the PSCell DL timing soon. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Which cell will be the timing reference for the PSCell SMTC configuration is ambiguous for the case of NR SA to EN-DC handover with PSCell addition. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of changes

### 6.2.2	Message definitions

#### –	MobilityFromNRCommand

The MobilityFromNRCommand message is used to command handover from NR to E-UTRA/EPC, E-UTRA/5GC or UTRA-FDD.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

MobilityFromNRCommand message

-- ASN1START

-- TAG-MOBILITYFROMNRCOMMAND-START

MobilityFromNRCommand ::=           SEQUENCE {

rrc-TransactionIdentifier           RRC-TransactionIdentifier,

criticalExtensions                  CHOICE {

mobilityFromNRCommand           MobilityFromNRCommand-IEs,

criticalExtensionsFuture        SEQUENCE {}

}

}

MobilityFromNRCommand-IEs ::=       SEQUENCE {

targetRAT-Type                      ENUMERATED { eutra, utra-fdd-v1610, spare2, spare1, ...},

targetRAT-MessageContainer          OCTET STRING,

nas-SecurityParamFromNR             OCTET STRING                                                OPTIONAL,   -- Cond HO-ToEPCUTRAN

lateNonCriticalExtension            OCTET STRING                                                OPTIONAL,

nonCriticalExtension                MobilityFromNRCommand-v1610-IEs                             OPTIONAL

}

MobilityFromNRCommand-v1610-IEs ::=     SEQUENCE {

voiceFallbackIndication-r16             ENUMERATED {true}                                       OPTIONAL,   -- Need N

nonCriticalExtension                    MobilityFromNRCommand-v1660-IEs                                             OPTIONAL

}

MobilityFromNRCommand-v1660-IEs::=     SEQUENCE {

smtc-r16						MTC-SSB-NR-r16                                               OPTIONAL, -- Need S

criticalExtensionsFuture               SEQUENCE {}

}

-- TAG-MOBILITYFROMNRCOMMAND-STOP

-- ASN1STOP

MobilityFromNRCommand-IEs field descriptions

nas-SecurityParamFromNR If targetRAT-Type is eutra, this field is used to deliver the key synchronisation and Key freshness for the NR to LTE/EPC handovers and a part of the downlink NAS COUNT as specified in TS 33.501 [11]. If targetRAT-Type is utra-fdd, this field is used to deliver the key synchronisation and Key freshness for the NR to FDD UTRAN handover and a part of the downlink NAS COUNT as specified in TS 33.501 [11].

smtc-r16 The SSB periodicity/offset/duration configuration of target cell for NR PSCell addition. This field is only present in the case of NR SA to EN-DC handover with PSCell addition. When UE receives this field, UE applies the configuration based on the timing reference of source NR PCell for PSCell addition. If both this field and the smtc-r15 in targetRAT-MessageContainer specified in TS 36.331 are absent, the UE uses the SMTC in the measObjectNR having the same SSB frequency and subcarrier spacing, as configured before the reception of the RRC message.

targetRAT-MessageContainer The field contains a message specified in another standard, as indicated by the targetRAT-Type, and carries information about the target cell identifier(s) and radio parameters relevant for the target radio access technology. A complete message is included, as specified in the other standard. See NOTE 1

targetRAT-Type Indicates the target RAT type.

voiceFallbackIndication Indicates the handover is triggered by EPS fallback for IMS voice as specified in TS 23.502 [43].

NOTE 1:	The correspondence between the value of the targetRAT-Type, the standard to apply, and the message contained within the targetRAT-MessageContainer is shown in the table below:

| targetRAT-Type | Standard to apply | targetRAT-MessageContainer |
| --- | --- | --- |
| eutra | TS 36.331 [10] (clause 5.4.2) | DL-DCCH-Message including the RRCConnectionReconfiguration |
| utra-fdd | TS 25.331 [45] (clause 10.2.16a) | Handover TO UTRAN command |

| Conditional Presence | Explanation |
| --- | --- |
| HO-ToEPCUTRAN | This field is mandatory present in case of inter system handover to "EPC" or "FDD UTRAN". Otherwise it is absent. |

End of changes