3GPP TSG-RAN WG2 Meeting #116-e 	R2-2110609

# Online, 1 – 12 November 2021

Title:	Draft reply LS on QoE configuration and reporting related issues

Response to:	S5-214520, S4-211291

Release:	Rel-16

Work Item:	NR_QoE-Core

Source:	Huawei (to be RAN2)

To:	SA5, SA4

Cc:	RAN3

Contact Person:

#### Name:	Jun Chen

E-mail:	jun.chen@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

1. Overall Description:

RAN2 would like to thank SA4 and SA5 for their LS(es) in S4-211291 and S5-214520 on QoE configuration and reporting related issues. RAN2 would like to inform about further progress that RAN2 made with respect to those.

To support multiple QoE measurement configurations for a single service type, RAN2 agreed that the QoE configuration will be identified on Uu interface using an RRC identifier, called MeasConfigAppLayerId currently. In order to be able to identify to which of the QoE configurations a report provided by an application layer belongs to, RAN2 agreed the following:

- The UE RRC layer forwards the MeasConfigAppLayerId together with the QoE configuration to the application layer

- The UE application layer sends the MeasConfigAppLayerId together with the QoE report to the UE RRC layer

In relation to the maximum size of QoE configuration and QoE report, RAN2 would like to indicate that it is possible to lift the size limitations by using RRC message segmentation. However, since RRC segmentation is an optional UE capability, RAN2 agreed the following:

- The maximum size of the QoE configuration container should be 8 kBytes, to ensure all UEs are able to receive it

- QoE report container will be specified as OCTET STRING with no maximum size in RRC signalling

- The UE will inform application layer whether the UE supports sending QoE reports with the size exceeding the previous limitation of 8 kBytes (depending on whether UE supports RRC segmentation).

2. Actions:

To SA5, SA4:

ACTION: 	RAN2 respectfully asks SA4 and SA5 to take the above information into account, introduce the necessary modifications in their specifications and provide feedback, as needed.

3. Date of Next TSG-RAN WG2 Meetings:

RAN2#116-bis-e	17 – 25 January 2022			Online

RAN2#117-e 	21 February – 3 March 2022			Online