3GPP TSG-RAN WG2 Meeting #116 electronic	R2-2110880

Online, Nov 1 – Nov 12, 2021

Agenda item:	8.12.2.2

Source: 	Sierra Wireless S.A.

Title: 	Early identification and camping restrictions for RedCap UE

Document for:	Discussion/Decision

# Introduction

This document discusses the following RAN2 lead objectives that were included in the RedCap WI RP-211574[1]:

- Specify functionality that will enable RedCap UEs to be explicitly identifiable to networks through an early indication in Msg1 and/or Msg3, and Msg A if supported, including the ability for the early indication to be configurable by the network. [RAN2, RAN1]

- Specify a system information indication to indicate whether a RedCap UE can camp on the cell/frequency or not; it shall be possible for the indication to be specific to the number of Rx branches of the UE. [RAN2, RAN1]

# Discussion

## Early Identification Configuration

The RedCap WID includes the objective:

- Specify functionality that will enable RedCap UEs to be explicitly identifiable to networks through an early indication in Msg1 and/or Msg3, and Msg A if supported, including the ability for the early indication to be configurable by the network. [RAN2, RAN1]

Since both Msg1 and Msg3 early indication has been agreed to be supported for RedCap devices, it would be beneficial for the gNB to be able to select, none, one or both, for the Redcap UE to use in SIB. This would allow operators to be able to flexibly configure what type of early indication is used by what type of Redcap UE based on the network deployment configuration. The need for early indication and how early, depends on the network deployment configuration (e.g., carrier bandwidth, frequency range, ISD (inter-site distances), etc.) and Redcap capabilities (e.g., HD-FDD, 256QAM, 1Rx/2Rx, etc.).

The following related agreement has been made by RAN1 [2]:

Agreements: Confirm the following working assumption with the modifications in red: - For 4-step RACH, support the early indication of RedCap UEs at least in Msg1. - The early indication in Msg1 can be configured to be enabled/disabled via SIB - FFS how to support enable/disable the early indication - FFS details e.g.: From RAN1 perspective, the following methods can be used for early indication both for shared initial UL BWP and separate initial UL BWP (if supported) - separate PRACH resource - PRACH preamble partitioning - FFS: whether/how to address RA-RNTI overlapping issue - FFS the possibility of supporting Msg3 for the early indication Whether/how to support early indication of RedCap UEs in Msg3 in Rel-17 is up to RAN2.

RAN1 agreement: The early indication in Msg1 can be configured to be enabled/disabled via SIB

Given this, a similar agreement for Msg3 is proposed:

Msg3 early RedCap UE indication shall be configured to be enabled/disabled via SIB

The main reason to support Msg1 early indication is for Msg2 PDCCH/PDSCH coverage recovery for RedCap devices. Msg1 early indication of RedCap devices will require allocating significant new physical layer resources (e.g., PRACH resources) which increases overhead and lowers system efficiency. Allocating multiple PRACH resources to identify the number of antennas would exacerbate the issue which is why the following agreement was made in RAN1[3]:

Conclusion: - No consensus to support early identification of the number of Rx branches in Msg1/Msg3/MsgA for Redcap UE in Rel-17

RAN1 conclusion:  No consensus to support early identification of the number of Rx branches in Msg1/Msg3/MsgA for Redcap UE in Rel-17

The amount of dedicated PRACH resources needed for MSG1 early indication will need to be scaled according to the number of RedCap UEs that are configured to use it, thus reducing the number RedCap UEs that use Msg1 for early indication will reduce the amount of dedicated PRACH resources that is needed.

Reducing the number of RedCap UEs that use Msg1 Early Indication will reduce the amount of dedicated PRACH resources.

For example, some network deployments at lower frequencies and/or smaller ISD (inter-site distance) may not require coverage recovery for Msg2, whereas other networks will. In some cases, the need for coverage recover may depend on whether the RedCap UE has a high-quality antenna or the number of antennas. If MSG1 early indication is only enabled for a reduce set of RedCap UEs (i.e., those which need Msg2 coverage recovery), then the amount of PRACH resources can be reduced and system efficiency is improved. Given this type of commercial deployment configuration is common (i.e., lower frequency and/or smaller ISD) and with more gNBs being deployed in the future, this network deployment configuration is important to consider and thus the following proposal is made:

Msg1 early indication can be enabled/disabled for a reduced set of Redcap devices

FFS: how to define a reduced set of Redcap devices e.g., via UE capabilities

## Msg3 Early Indication Message

It is possible that more than one Msg3 LCID could be allocated for RedCap UEs. This could be used by the UE to inform the gNB more information about its capabilities at an earlier stage. For example, in some deployments identifying early if a RedCap UE is a half or full duplex would be preferred as it would allow the gNB to treat FD-FDD UEs differently early on. Considering that HD-FDD is an optional RedCap feature, this may be a benefit to more RedCap devices.

If more than one LCID is available for signaling of RedCap, then it can be used to inform the network of other RedCap capabilities, such as FD-HDD.

## Camping restrictions

The RedCap WID includes the objective:

- Specify a system information indication to indicate whether a RedCap UE can camp on the cell/frequency or not; it shall be possible for the indication to be specific to the number of Rx branches of the UE. [RAN2, RAN1]

One consideration for operators would be access control for roaming RedCap UEs. Since RedCap devices may require more resources per bit and thus cost more per bit to service, a network operator will likely want to bar some or all roaming RedCap UEs but allow “Home” subscribers. Alternatively, it may want to permit only certain roaming RedCap UEs.

The system information shall indicate whether a Roaming RedCap UE is permitted to access a cell/frequency.

# Summary

- RAN1 agreement: The early indication in Msg1 can be configured to be enabled/disabled via SIB

- Msg3 early RedCap UE indication shall be configured to be enabled/disabled via SIB

RAN1 conclusion:  No consensus to support early identification of the number of Rx branches in Msg1/Msg3/MsgA for Redcap UE in Rel-17

Reducing the number of RedCap UEs that use Msg1 Early Indication will reduce the amount of dedicated PRACH resources.

Msg1 early indication can be enabled/disabled for a reduced set of Redcap devices

FFS: how to define a reduced set of Redcap devices e.g., via UE capabilities

If more than one LCID is available for signaling of RedCap, then it can be used to inform the network of other RedCap capabilities, such as FD-HDD.

The system information shall indicate whether a Roaming RedCap UE is permitted to access a cell/frequency.

# References

- 3GPP RP-211574 Revised WID on support of reduced capability NR devices

- 3GPP RAN1 Chairman’s Notes, RAN1#106e

- 3GPP RAN1 Chairman’s Notes, RAN1#105e