3GPP TSG-RAN WG2 Meeting #116 electronic	R2-2111001

Online, November 1 – 12, 2021	(Revision of R2-2108709)

Agenda Item:	8.3.3

Source: 	ASUSTeK

Title:  	Interaction between NAS and AS for network switching

Document for:	Discussion and Decision

- Introduction

Conclusions for coordinated leaving for Multi-USIM device are agreed in SA2 as below [1]:

[…] - When the UE connection with the network is resumed, the previous assistance info, if any, to temporarily restrict/filter MT data/signalling handling is revoked: - When the UE performs the BUSY procedure (refer to clause 8.1) the UE may include assistance information in the SERVICE REQUEST message for MT data/signalling handling (e.g. paging restriction). - When the UE needs to remove the previously requested assistance information from Idle state, the UE includes a release request indication in the SERVICE REQUEST message. If no assistance information is provided, the existing paging filtering is removed. This release request indication is needed so that the network does not unnecessarily keep the UE in Connected state. - If the UE returns to the PLMN A from PLMN B to perform a TAU due to mobility or periodic Update reasons, the TAU may include a release request indication and optionally assistance information. If no assistance information is provided, the existing paging filtering is removed. The release request indication is needed so that the network does not unnecessarily keep the UE in Connected state. - In the NAS SERVICE REQUEST or TAU message indicating leave request (EPS case), the UE provides leaving indication to the CN and the UE may provide assistance information to the network in the leaving procedure regarding MT data/signalling handling. - The assistance information may include: - Information to temporarily restrict/filter MT data/signalling handling: - An indication that the UE should only be paged for voice (MMTel voice or CS domain voice (for EPS)); or - An indication that the UE should not be paged at all; or - PDN connection(s) for MT notification/paging restriction. NOTE 3: Whether there is a need of an expected leaving time/duration information will be discussed in normative phase.

This document discusses the necessary interaction between NAS and AS for network switching.

- Discussion

Based on the conclusions in SA2, the UE could provide the assistance information for different levels of paging restriction to assist the network to handle the MT data/signaling and decide whether to page to the UE during the UE leaving. When the paging restriction is provided, the network would only page the UE for certain PDN connection(s), for voice, or even not page the UE at all.

If the paging restriction for the network is set to “All paging is restricted” [2][3], the UE would not be paged by the network at all. Hence, the UE does not need to monitor paging from the network. Since paging monitoring is controlled by AS, NAS should inform AS if the paging restriction is set to “All paging is restricted” to let AS determine whether to perform paging monitoring.

Proposal 1:	NAS should inform AS if paging restriction is set to “All paging is restricted”.

Proposal 2:	UE does not monitor paging from a network if paging restriction for the network is set to “All paging is restricted”.

SA2 also agrees to remove the paging restriction based on UE request, e.g. SERVICE REQUEST. If the paging restriction was set to “All paging is restricted” and is revoked, the UE should resume paging monitoring. Therefore, NAS should also inform AS if the paging restriction is revoked.

Proposal 3:	NAS should inform AS if the paging restriction is revoked.

- Conclusion

Interaction between NAS and AS for network switching is proposed as below:

Proposal 1:	NAS should inform AS if paging restriction is set to “All paging is restricted”.

Proposal 2:	UE does not monitor paging from a network if paging restriction for the network is set to “All paging is restricted”.

Proposal 3:	NAS should inform AS if the paging restriction is revoked.

- Reference

- TR 23.761 V17.0.0, Study on system enablers for devices having multiple USIM.

- TS 24.301 V17.3.0, NAS protocol for EPS.

- TS 24.501 V17.3.1, NAS protocol for 5GS.