3GPP TSG-RAN WG2 Meeting #116 electronic	                                       R2-2111044

Online, November 1 – November 12, 2021

Agenda Item:	8.10.2.2

Source:	CMCC

Title:	Remaining Issue on LCP Restrictions and CG Impact in NTN

Document for:	Discussion, Decision

Introduction

In previous RAN2 meeting, there were some extensively discussions about enabling/disabling HARQ feedback [1] [2], and had achieved some conclusions:

- For dynamic grants, each LCH can optionally be semi statically configured (by RRC) to be mapped to one or more HARQ processes (FFS if it's possible to map to more than one HARQ process/ process type. FFS on mapping method). If there is no RRC configuration for this, this mapping has no effect (legacy behaviour applies).

However, there are still some key issues left. In this contribution, we will elaborate the above left issues, and provide our proposals.

Discussion

LCP Impact caused by disabling HARQ UL retransmission

Currently in the running CR TS 38.321, it is specified as below:

RRC additionally controls the LCP procedure by configuring mapping restrictions for each logical channel: /*****text omitted*****/ - allowedDRX-LCPmode which sets the allowed DRX-LCP mode(s) of a dynamic grant for transmission. - allowedHARQ-state which sets the UL HARQ retransmission state allowed for transmission. /*****text omitted*****/ - select the logical channels for each UL grant that satisfy all the following conditions: /*****text omitted*****/ 2> allowedDRX-LCPmode, if configured, includes the DRX-LCP mode for the HARQ process associated to the dynamic UL grant. Does not apply if the HARQ process associated to dynamic UL grant is not configured with a DRX-LCP mode. 2> allowedHARQ-state, if configured, is the same as the HARQ retransmission state, according to the allowedHARQ-state, of the HARQ process identified for the dynamic UL grant. Does not apply if the MAC entity is not configured with allowedHARQ-state.

Per running CR TS 38.331, uplinkHARQ-DRX-LCP-Mode is defined under MAC-CellGroupConfig per cell group per UE, used to set the DRX-LCP mode per HARQ process ID. Meanwhile, allowedHARQ-DRX-LCP is defined under LogicalChannelConfig. Actually, the two terms are both defined for indication the HARQ retransmission states, however, usage of different terms introduce the confusion and difficulties in description of UE behaviour of LCP proceure. Therefore, it is proposed to rename the parameter names to align the IE under MAC-CellGroupConfig and LogicalChannelConfig for same purposes.

In last RAN2 meeting, it is agreed that for dynamic grants, each LCH can optionally be semi statically configured (by RRC) to be mapped to one or more HARQ processes and a left FFS is if it's possible to map to more than one HARQ process/ process type. If it is restricted th Proposal 1: it is proposed to rename the parameter names to align the IE under MAC-CellGroupConfig and LogicalChannelConfig for same purpose of indication the HARQ retransmission states.

at each LCH can only be map to one HARQ process, the buffer jam may occur once there is no available HARQ process allocated for arriving data. Considering the long RTT propagation delay in NTN, we tend to prefer that each LCH can be mapped to one or more HARQ processes. However, we don’t think more than one HARQ process types are needed, which just introduce the complexity and confusion during logical channel mapping procedure.

Proposal 2: It is proposed that each LCH can be mapped to one or more HARQ processes with only one HARQ process type.

SPS/CG Impact caused by disabling HARQ UL retransmission/DL Feedback

Currently, only the granularity of HARQ process for enabling/disabling HARQ feedback had been regarded as potential consensus of participants during email discussion. Moreover, the aspect of the necessity of the enabling / disabling of HARQ feedback can be configurable on a per Configured CG/SPS (Semi-Persistent Scheduling) basis via RRC signalling haven’t been touched in both online and email discussion yet, which had been identified and captured as reasonable issue in the TR 38.821 after the email and online discussion in SI phase, as follows:

- Semi-Persistent Scheduling should to be supported for HARQ processes with enabled and disabled HARQ feedback. Details can be decided in the WI phase.

As we know, in NTN, the maximum round trip delay is 541.46ms for GEO and 25.77ms for LEO. Hence, configured grant/SPS can reduce the PDCCH load for DCI scheduling and decrease the latency for uplink resource application. One of the main benefits for NTN with configured grant is due to the large delays associated with UE getting uplink resources when the propagation delays are high. In a normal case, the UE has to go through the SR->Grant(for BSR)->BSR->Grant(for UL data) procedure in order to get sufficient uplink resources.

Observation 1: Configured grant/SPS can avoid DCI scheduling and uplink resource application, which will largely reduce the scheduling delay.

Proposal 3: RAN2 need to consider enabling / disabling the HARQ feedback of SPS configurations via RRC signaling.

There are two different types of uplink grants: type 1 is the method where the configured grant parameters, such as periodicity, MCS, the start offset in time domain and other scheduling information are configured via RRC only. And for type 2, which is similar as DL SPS, the information such as periodicity and HARQ-processes are configured by RRC and the start offset in time domain and other is indicated via DCI scrambled by CS-RNTI.

At the other hand, for configured grant, the HARQ-processes ID is calculated based on the following formula:

HARQ Process ID = [floor(CURRENT_symbol/periodicity)] modulo nrofHARQ-Processes

In the downlink, for Semi-persistent, the HARQ-processes ID is calculated based on the following formula:

HARQ Process ID = [floor (CURRENT_slot × 10 / (numberOfSlotsPerFrame × periodicity))] modulo nrofHARQ-Processes

According to above formula, we can observe that the HARQ-processes ID of both CG and SPS are calculated from parameters of radio resource allocation in time domain, e.g. configured periodicity, start off in time domain, configured HARQ process number. This is different from dynamic scheduling, where there is no relationship between HARQ ID and radio resource allocation in time domain. On the other hand, in Rel-16 industrial IoT/URLLC WID, multiple SPS/configured grant configurations per cell can be configured to a UE has already been agreed in RAN1 and RAN2.

Observation 2: The HARQ-processes ID of both CG and SPS are calculated from parameters of radio resource allocation in time domain, e.g. configured periodicity, start off in time domain, configured HARQ process number. This is different from dynamic scheduling, where there is no relationship between HARQ ID and radio resource allocation in time domain.

Observation 3: in Rel-16 industrial IoT/URLLC WID, multiple SPS/configured grant configurations per cell can be configured to a UE has already been agreed in RAN1 and RAN2.

Therefore, for UL CG, since in Rel-15, HARQ feedback has been ruled out instead of automatically re-transmission in configured repetition number, this means the configured start offset of CG cannot be selected randomly, which must be in the scope of configured disabled HARQ process number. Or an additional note in the specification is needed to clarify that the configured disabled/enabled HARQ process number has no impact on the HARQ number used in UL CG.

Observation 4: The configured start offset of CG cannot be selected randomly, which must be in the scope of configured disabled HARQ process number. Or an additional note in the specification is needed to clarify that the configured disabled/enabled HARQ process number has no impact on the HARQ number used in UL CG.

However in NR-U, HARQ feedback has been supported for CG data transmission, therefore in this scenario, mechanism of HARQ feedback enabling/disabling still needs to be taken into consideration.

Observation 5: In NR-U scenario, HARQ feedback enabling/disabling mechanism still needs to be considered in case of UL CG configured.

Proposal 4: RAN2 need to consider enabling/disabling the HARQ feedback of UL CG configurations via RRC signaling in case of NR-U scenario.

Regarding DL SPS, since the HARQ feedback does still work, the situation is more awkward. Hence, if the HARQ ID is been reserved by RRC, this means the start offset of SPS cannot be selected randomly. Hence, the enable / disable of HARQ feedback on a SPS configurations cannot be implemented by enable / disable of HARQ feedback on a per HARQ process basis via RRC signaling. Furthermore, as the bottleneck is the lack of buffer size for HARQ soft combining in UE side, the application of DL SPS seems more important in NTN system for the reduction of the PDCCH load for DCI scheduling overhead and the latency for data transmission.

Observation 6: If the HARQ ID is been reserved by RRC, this means the start offset of SPS cannot be selected randomly. Hence, the enable / disable of HARQ feedback on a SPS configurations cannot be implemented by enable / disable of HARQ feedback on a per HARQ process basis via RRC signalling.

Proposal 5: RAN2 need to consider the conflict between RRC configuration of enabling / disabling of HARQ feedback and SPS/configured grant configurations via RRC signaling.

Conclusion

Based on the above discussion, we provide the following observations:

Observation 1: Configured grant/SPS can avoid DCI scheduling and uplink resource application, which will largely reduce the scheduling delay.

Observation 2: The HARQ-processes ID of both CG and SPS are calculated from parameters of radio resource allocation in time domain, e.g. configured periodicity, start off in time domain, configured HARQ process number. This is different from dynamic scheduling, where there is no relationship between HARQ ID and radio resource allocation in time domain.

Observation 3: in Rel-16 industrial IoT/URLLC WID, multiple SPS/configured grant configurations per cell can be configured to a UE has already been agreed in RAN1 and RAN2.

Observation 4: The configured start offset of CG cannot be selected randomly, which must be in the scope of configured disabled HARQ process number. Or an additional note in the specification is needed to clarify that the configured disabled/enabled HARQ process number has no impact on the HARQ number used in UL CG.

Observation 5: In NR-U scenario, HARQ feedback enabling/disabling mechanism still needs to be considered in case of UL CG configured.

Observation 6: If the HARQ ID is been reserved by RRC, this means the start offset of SPS cannot be selected randomly. Hence, the enable / disable of HARQ feedback on a SPS configurations cannot be implemented by enable / disable of HARQ feedback on a per HARQ process basis via RRC signalling.

Based on the discussion we propose the following:

Proposal 1: it is proposed to rename the parameter names to align the IE under MAC-CellGroupConfig and LogicalChannelConfig for same purpose of indication the HARQ retransmission states.

Proposal 2: It is proposed that each LCH can be mapped to one or more HARQ processes with only one HARQ process type.

Proposal 3: RAN2 need to consider enabling / disabling the HARQ feedback of SPS configurations via RRC signaling.

Proposal 4: RAN2 need to consider enabling/disabling the HARQ feedback of UL CG configurations via RRC signaling in case of NR-U scenario.

Proposal 5: RAN2 need to consider the conflict between RRC configuration of enabling / disabling of HARQ feedback and SPS/configured grant configurations via RRC signaling.

Reference

- R2-2008188 Summary of [AT111][107][NTN] Pre-compensation and other MAC issues

- R2-2008214 Summary of [AT111][107][NTN] Pre-compensation MAC issues Phase 2

- RAN2-111e - R16 eMIMO-CLI-PRN-RACS - R17 NTN-REDCAP (Sergio)_2020-08-28_final