3GPP TSG-RAN WG2 Meeting #116bis electronic                      R2-22x

Online, January 17-25, 2022

Agenda Item:	8.12.2.2

Source: 	Huawei, HiSilicon

Title:	Summary of [AT116bis-e][103][RedCap] Identification and access restriction (Huawei)

Document for:	Discussion and Decision

# Introduction

This paper aims at capturing the summary of email discussion.

- [AT116bis-e][103][RedCap] Identification and access restriction (Huawei)

Initial scope: Discuss identification and access restriction aspects based on submitted contributions

Initial intended outcome: Summary of the offline discussion with e.g.:

    - List of proposals for agreement (if any)

    - List of proposals that require online discussions

    - List of proposals that should not be pursued (if any)

Initial deadline (for companies' feedback): Wednesday 2022-01-19 1300 UTC

Initial deadline (for rapporteur's summary in R2-2201734): Wednesday 2022-01-19 1500 UTC

Proposals marked "for agreement" in R2-2201734 not challenged until Thursday 2022-01-20 0300 UTC will be declared as agreed via email by the session chair (for the rest the discussion will continue in the GTW session).

Contact Table

| Companies | Name | Email |
| --- | --- | --- |
| Samsung | Jaehyuk JANG | jack.jang@samsung.com |

- Discussion

  - Msg3 early identification

We have the following FFS from last meeting:

In MAC perspective, RedCap UE uses the dedicated LCID for Msg3 early identification, when the Msg3 includes the CCCH data. FFS on whether it requires no other precondition, or precondition as “when Msg1 early identification is not configured”, or precondition as “when Msg3 early identification is enabled by NW.

In addition, some contributions from operators are listed below for your consideration:

R2-2200861	Discussion on access restrictions and early identification	CMCC

R2-2201623	Support and network behaviour for RedCap early indication messages	BT Plc, Deutsche Telekom AG, Telecom Italia S.p.A., TurkCell, CMCC, NTT DOCOMO INC., Orange, Vodafone, KDDI

In MAC perspective, RedCap UE uses the dedicated LCID for Msg3 early identification, when the Msg3 includes the CCCH data.

Option 1: it requires no other precondition;

Option 2: it requires precondition as “when Msg1 early identification is not configured”

Option 3: it requires precondition as “when Msg3 early identification is enabled by NW”

Question 1: Which option do you prefer?

| Companies | Option? | Comments |
| --- | --- | --- |
| Samsung | Option 1 | We do not see any reason to add (unnecessary) precondition. |

Another issue is whether RedCap should always support this Msg3 early identification. In case this is optionally supported, and there is no UE capability signaling, it means some RedCap UE may not indicate its type in Msg3, even if NW side configures/enables Msg3 early identification. Namely that an UE, not using dedicated LCID in Msg3, may still be one RedCap UE.

Question 2: Do you think Msg3 early identification is optionally supported by RedCap UE?

| Companies | Yes or No? | Comments |
| --- | --- | --- |
| Samsung | No | Since RAN2 assumes a dedicated gNB for RedCap UEs, both network and UE should support this identification. |

  - MsgA early identification

We have several straight forward proposals from R2-2200797, which use the similar principle as Msg1 early identification.

- Proposal 2	In MAC perspective, a RedCap UE uses MsgA PRACH early identification when it transmits preamble for CBRA if MsgA PRACH early identification is configured for RedCap by NW.

- Proposal 3	For MsgA PRACH early identification, RAN2 confirms both dedicated ROs and dedicated PRACH preamble can be supported from signalling point of view.

- Proposal 4	For RedCap, MsgA PRACH early identification is enabled/disabled implicitly by the presence of dedicated RACH configuration for MsgA PRACH early identification.

Question 3: Do you agree the above proposals 2/3/4 in R2-2200797?

| Companies | Yes? | Comments |
| --- | --- | --- |
| Samsung | Yes | For the completeness, CCCH in MsgA should also contain dedicated LCID for RedCap UEs. |

  - IFRI

We have the FFS points as below:

In case the cell is barred due to not supporting RedCap, intra-frequency cell reselection is considered by RedCap UE as:

Option 1: “allowed”;

Option 2: “not allowed”;

Option 3: follow the IFRI in MIB;

Option 4: UE implementation.

Question 4: Which option do you prefer?

| Companies | Option? | Comments |
| --- | --- | --- |
| Samsung | - | RAN2 should have a consensus on the deployment scenario to make a decision: if RAN2 assumes homogeneous deployment per frequency i.e. if network supports redcap on a frequency, Option 2 should be chosen. Option 1 should be used if RAN2 assumes that deployment can be non-homogeneous per frequency i.e. network can support redcap on some but not all cells of the frequency. |

Question 5: Do you agree that, as in legacy, in case the cell is barred due to being unable to acquire the MIB, intra-frequency cell reselection is considered by RedCap UE as “allowed”?

| Companies | Yes or No? | Comments |
| --- | --- | --- |
| Samsung | Yes | Same as in legacy. |

If the cellBarred field in MIB is set to barred, RedCap UE should:

Option 1: follow the legacy IFRI in MIB.

Option 2: continue to read SIB1 of the barred cell and follow the intraFreqReselectionRedCap indicated in SIB1.

Question 6: which option do you prefer?

| Companies | Option? | Comments |
| --- | --- | --- |
| Samsung | Option 2 | Since RAN2 decided to introduce IFRI for RedCap UEs, UE should check the IFRIs in SIB1. |

Question 7: Do you agree that, in case the cell is barred due to being unable to acquire the SIB1, intra-frequency cell reselection is considered by RedCap UE as “allowed”?

| Companies | Yes or No? | Comments |
| --- | --- | --- |
| Samsung | Yes | In case of failure to acquire SIB1 from the cell, UE has no information about the IFRI bit in SIB1. Note that even if IFRI bit in MIB is set to not allowed, IFRI bit in SIB1 can be set to allowed. It is possible that UE can find another cell which supports redcap on the frequency. Hence, the RedCap UE should bar the cell only, if it fails to acquire SIB1. |

  - ASN.1 for cell barring in SIB1

Another leftover on the signalling design for cellBarredRedCap1Rx/2Rx is the decision between following options:

Option 1: use two mandatory sub-IEs with {barred, notBarred} values included in one optional parent IE cellBarredRedCap-r17.

cellBarredRedCap-r17        SEQUENCE {

cellBarredRedCap1Rx-r17        ENUMERATED {barred, notBarred},

cellBarredRedCap2Rx-r17        ENUMERATED {barred, notBarred}

}                                                                                 OPTIONAL,  -- Need R

Option 2: use two optional IEs with {barred} values

cellBarredRedCap1Rx-r17             ENUMERATED{barred}                       OPTIONAL,  -- Need R

cellBarredRedCap2Rx-r17             ENUMERATED{barred}                       OPTIONAL,  -- Need R

Question 8: Which option do you prefer?

| Companies | Option? | Comments |
| --- | --- | --- |
| Samsung | Option 1 | We slightly prefer Option 1, which is cleaner approach than Option 2. |

One issue on the presence of intraFreqReselectionRedCap for RedCap cell is discussed in below contributions.

R2-2200797: Proposal 8	Revert the agreement “If RedCap-specific IFRI is absent from broadcast SI, the UE considers the cell does not support RedCap.”

R2-2200861: Proposal 1: Both RedCap-specific cellbarred and RedCap specific IFRI should be taken into consideration for a UE to decide whether RedCap is supported or not in the cell, for example, if one of the two IEs is absent means RedCap is not supported.

This is somehow depends on the ASN.1 design above for cellBarredRedCap1Rx/2RX (i.e. whether somehow the presence cellBarredRedCap1Rx/2Rx can aleady indicate the supporting of RedCap ).

Question 9: Do you agree that the cell supporting RedCap should always present the intraFreqReselectionRedCap in SIB1? (i.e. the previous agreement is still valid)

| Companies | Yes or No? | Comments |
| --- | --- | --- |
| Samsung | Yes | - |

  - Cell (re)selection parameters

Another leftover is on the need of configuring some RedCap specific parameters/priority for cell (re)selection. The below question is to achive the high-level intention first.

Question 10: Do you agree to support the RedCap specific cell (re)selection parameter?

| Companies | Yes or No? | Comments |
| --- | --- | --- |
| Samsung | Yes | A RedCap UE with 1 Rx branch would require higher 'minimum required signal strength/quality level' (i.e. Qrxlevmin/Qqualmin from the cell selection criterion S). So, the network should provide a separate 'minimum required signal strength/quality level' for a RedCap UE especially with 1 RX branch. We also support the different priority for cell reselection in SIB2&4: Since most companies in RAN2 assume that RedCap UE can connect to a gNB that supports RedCap UEs, so the legacy priority for cell reselection would not be applicable in certain deployment scenario (which is not a corner case, we think). |

  - Neighbour cell supporting

We see many contributions proposing to introduce the SI indication on whether neighbor cells supports/allows RedCap access.

Question 11: Do you agree that system information should provide information on which cells/frequencies accept RedCap UE access (e.g. by considering whether supporting RedCap)?

| Companies | Yes or No? | Comments |
| --- | --- | --- |
| Samsung | - | It depends on the conclusion from Question 4: if RAN2 assumes homogeneous deployment per frequency, then different priority in Question 10 would be sufficient. Otherwise, 'allowed-cell-list' for RedCap UEs would be beneficial. |

  - UE capability for paging

This issue is discussed in below contributions:

R2-2200597, Proposal 9: Paging to RedCap UE should be only sent in cells allowing the target RedCap UE camping. To enable one gNB to determine whether a target UE is RedCap or not, UERadioPagingInformation is extended to include RedCap related radio capability (e.g. 1rx RedCap or 2rx RedCap).

R2-2200554, Proposal 8: The Rx branches capability should be included in the UERadioPagingInformation inter-node message.

Some motivations are cited below:

In NR, there exists UERadioPagingInformation inter-node message, which includes UE’s capability like supported NR frequency bands. If received, the NG-RAN node may use it to apply specific paging schemes, e.g., performs paging only on UE supported NR frequency bands. UE’s Rx capability can also be utilized for specific paging scheme. It is agreed that SIB1 indicates cell barring for 1 Rx branch and 2 Rx branches separately for RedCap UEs. With cellBarredRedCap1Rx (or cellBarredRedCap2Rx) indicated in SIB1, RedCap UEs with 1 Rx branch (or 2 Rx branches) are not allowed to camp in the cell. Correspondingly, the gNB may not perform paging in the cell if paging messages for UEs with above Rx capability are received. Thus, as assistance information for specific paging scheme, the Rx branches of UE should be included in above UERadioPagingInformation message.

Observation 1: Based on the RX branches capability, gNB can only send paging message to the specific UEs (e.g. if the paging message is for 1RX UE but the cell bars all 1RX RedCap UEs, gNB can choose not to send paging message.).

Question 12: Do you agree the Rx branches capability should be included in the UERadioPagingInformation inter-node message?

| Companies | Yes or No? | Comments |
| --- | --- | --- |
| Samsung | No | We think that it is not an essential feature, but an optimization which might be useful in certain scenarios (as provided in the example above). |

- Conclusion and proposals

Based on the above summary, following proposals are given.

- Reference

- R2-2200287	Open issues on Early identification, camping restrictions and NCD-SSB	Intel Corporation

- R2-2200554	Identification and access restriction of RedCap UE, and NCD-SSB related issues	Huawei, HiSilicon

- R2-2200597	Remaining issues on NCD SSB, identification and access for RedCap	vivo, Guangdong Genius

- R2-2201113	RedCap UE power-saving aspects at cell re-selection	Apple	discussion	NR_redcap-Core

- R2-2200208	Cell barring aspects	Samsung Electronics Co., Ltd

- R2-2200249	Discussion on RedCap UE's identification and camping restrictions	OPPO

- R2-2200332	Cell (re)selection details for RedCap UEs	Samsung Electronics

- R2-2200343	System Information and supporting for RedCap UEs	KDDI Corporation

- R2-2200468	Discussion on UE access restrictions for Redcap devices	Beijing Xiaomi Mobile Softwar

- R2-2200469	Discussion on early Identification for Redcap devices	Beijing Xiaomi Mobile Softwar

- R2-2200568	Camping restrictions of RedCap UE	Fujitsu

- R2-2200609	On Access and Camping Restrictions	ZTE Corporation, Sanechips

- R2-2200616	Further considerations on access restrictions	NEC

- R2-2200639	Discussion on the open issues of identification and access restrictions for RedCap UE	Spreadtrum Communications

- R2-2200686	Discussion on the remaining issues of early identification and IFRI	CATT	discussion

- R2-2200725	Corrections for cellBarred in MIB handling for RedCap UE	InterDigital, Europe, Ltd.

- R2-2200797	Early indication & access restriction for RedCap UEs	Ericsson

- R2-2200836	NR-REDCAP access restriction/allowance indication to ease mobility	THALES

- R2-2200861	Discussion on access restrictions and early identification	CMCC

- R2-2201207	Discussion on identification and access restrictions for RedCap UEs	LG Electronics UK

- R2-2201232	Early identification and camping restrictions for RedCap UE	Sierra Wireless. S.A.

- R2-2201237	Neighbour cell information and cell (re)selection for RedCap UE	DENSO CORPORATION

- R2-2201587	Further details of identification, access, and camping restrictions	Nokia, Nokia Shanghai Bell

- R2-2201623	Support and network behaviour for RedCap early indication messages	BT Plc, Deutsche Telekom AG, Telecom Italia S.p.A., TurkCell, CMCC, NTT DOCOMO INC., Orange, Vodafone, KDDI