3GPP TSG-RAN WG2 Meeting #116bis-e                                                 R2-22xxxxx

E-Meeting: 17 – 25 January 2022

Agenda item:	9.1.2

Source:	Qualcomm Incorporated

Title:	Report of [AT116bis-e][302][NBIOT/eMTC R17] RLF Measurements (Qualcomm)

Document for:	Discussion and Decision

# Introduction

During online session at RAN2#116bis-e, an offline is initiated to progress on the following and other open issues.

Agreements [Online]: - Confirm that early RLF for NB-IoT is not supported in Release 17. - Value for TSearchDeltaP is [15s, 30s, 45s, 60s] - Neighbour cell monitoring in RRC_CONNECTED has no impact on neigbour cell monitoring in RRC_IDLE state and vice-versa - If upon transition to RRC_CONNECTED state, UE is not in relaxed neighbour cell monitoring state in RRC_IDLE, then timer TsearchDeltaP restarted with the RRC_CONNECTED state timer value. - If upon transition to RRC_CONNECTED state, UE is in relaxed neighbour cell monitoring state in RRC_IDLE, then timer TsearchDeltaP is not started. - In RRC_CONNECTED state UE exits relaxed neighbour cell monitoring state then timer TsearchDeltaP is started with the RRC_CONNECTED timer value (i.e., when NRSRP - Srxlev >= sSearchDeltaP (FFS update variable names offline). - FFS: whether the RRC_CONNECTED state reference level is derived by taking the RRC_IDLE state reference level and adjusted by nrs-PowerOffsetNonAnchor for the connected mode carrier. - FFS: whether UE reports to NW when the criteria is met.

- [AT116bis-e][302][NBIOT/eMTC R17] RLF Measurements (Qualcomm)

Scope: address the FFS above and other open issues.

Intended outcome: report in R2-2201793, agreements offline.

Deadline: Monday 24th January 1200 UTC.

# Discussion

## How to set the reference level for RRC_CONNECTED state

It has been agreed that the RRC_IDLE state neighbour cell relaxed monitoring state can be continued in RRC_CONNECTED state until UE determines the state needs to change and for this a reference level is needed to determine whether the serving cell has degraded by more than sSearchDeltaP. In RRC_IDLE mode, the initial reference level upon cell selection/reselection is set to the first measured Srxlev because UE is required to perform neighbour cell measurements for at least a period of TsearchDeltaP.

Following are two options briefly discussed online to set the reference level upon entry to RRC_CONNECTED state so that the RRC_IDLE state neighbour cell measurement state can be continued:

      - Set the RRC_CONNECTED state reference level to RRC_IDLE state Srxlevref

      - Set the RRC_CONNECTED state reference level to the last serving cell measurement, Srxlev, obtained before entering RRC_CONNECTED state.

Question 1: Which option do you support?

| Company | Option 1 or 2 | Comment |
| --- | --- | --- |
| Huawei, HiSilicon | Option 2 | It is better to initialise the criterion with the most recent value |
| Qualcomm | Option 2 | Either of the two options are fine. |

Response summary:  TBD.

- TBD

## Assistance information from UE

RAN4 is defining performance requirements for RRC_CONNECTED state neighbour cell measurements. Depending on the requirements and the specific NPDCCH configuration it may not be possible for UE to perform inter-frequency measurements. For this reason, some companies propose an indication from UE to network can be supported so that network can alter the NPDSCH/NPUSCH scheduling to allow UE to perform measurements.

Question 2: Do you support indication from UE to NW that indicates UE needs to perform inter-frequency measurements?

| Company | Yes/No | Comment |
| --- | --- | --- |
| Huawei, HiSilicon | No | as RANP has agreed on ‘no measurement gaps’, it is clear to us that the measurements are best effort. Even if we had an indication, it would still be effort because we cannot assume that the eNB will do any. On the other hand, the NW can via observations guess that the UE may need to perform measurements. |
| Qualcomm | Yes | We don’t see best-effort without scheduling gaps works in most realistic configurations of NPDCCH. By the time network realises ‘UE may need scheduling’ gaps, it may be too late because by that time UE may need Rmax repetitions to be able to decode the NDCCH, which would leave insufficient time between NPDCCH occasions to do any measurements. Also considering the case a mobile UE is actually stationary but in poor steady condition hence UE will refrain from initiating neighbour cell measurements but network may skip scheduling thinking UE needs to perform inter-frequency measurements hence unnecessarily lowering data throughput and prolonging the connection. In summary, leaving it to best effort will degrade UE performance rather then bring benefit hence pointless defining inter-frequency measurements. A simple MAC indication combined with C-DRX can greatly make inter-frequency measurement realisable without have too much negative impact on UE performance. |

Response summary: TBD

- TBD

## Dedicated signalling to enable/disable neighbour cell measurement

Rapportuer reviewed the other company contributions in [4][5][6]& [7] and concluded the new proposals are either covered by the online discussion/agreements/FFSes or are aspects for CR drafting. Rapportuer did identify following proposal from one contribution [5] that needs offline discussion:

“The network can enable/disable the neighbor cell measurement for UE in RRC_CONNECTED by dedicated signaling.”

Question 3: Do you support dedicated signalling to enable/disable neighbour cell measurement for UE in RRC_CONNECTED state?

| Company | Yes/No | Comment |
| --- | --- | --- |
| Huawei, HiSilicon | No |  |
| Qualcomm | No |  |

Response summary: TBD

- TBD

## Any other suggestions/open issues

Question 4: Any other suggetion/comments?

| Company | Suggestion/open issue/Comments |
| --- | --- |

Response summary: TBD

- TBD

# Conclusion

TBD

Proposal 1	TBD

Proposal 2	TBD

Proposal 3	TBD

Proposal 4	TBD

# References

- R2-201306, 17Additional enhancements for NB-IoT and LTE-MTC, RAN#88e, Reno, June 2020.

- R4-2120340, WF on RRM requirements for Rel-17 NB-IoT and LTE-MTC, RAN4#100-e, Nov 2021.

- 3GPP TS 36.304, User Equipment (UE) procedures in idle mode, Release 16.

- R2-2200675, On remaining issues for connected mode measurements for RLF, Nokia, RAN2#116bis-e, Jan 2022.

- R2-2200681, Remaining FFSs on RLF measurements, ZTE, RAN2#116bis-e, Jan 2022.

- R2-2201020, Consideration on open issues for neighbour cell measurement in RRC connected stat, QC, RAN2#116bis-e, Jan 2022.

- R2-2201077, Discussion on connected mode measurement in NB-IoT, E///, RAN2#116bis-e, Jan 2022.

# Participants

| Company | Name | e-mail address |
| --- | --- | --- |
| Huawei, HiSilicon | Odile Rollinger | odile.rollinger@huawei.com |
| Qualcomm | Mungal Dhanda | mdhadna@qti.qualcomm.com |