3GPP TSG-RAN WG2 Meeting #116bis-e	R2-2200346

E-meeting, 17st – 25th Jan 2022

Agenda Item:	8.1.4

Source: 	Huawei, HiSilicon

Title: 	Discussion on user plane open issues

Document for:	Discussion and decision

1	Introduction

In this contribution, based on the agreements in previous meetings as described in the Annex, we will discuss the remaining UP issues, including DRX operation for multicast, identity of MTCH for multicast, need of PDCP polling mechanism for multicast, initialization of PDCP variables and RLC variables for multicast, initialization of PDCP variables and RLC variables for broadcast, COUNT wrap-around issue for broadcast, and maximum number of MBS sessions.

- Discussion

  - DRX operation for multicast

1) PTM initial transmission for multicast:

According to the current specification, in unicast DRX, when UE receives a MAC PDU, UE starts the drx-HARQ-RTT-TimerDL for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback, and UE only starts drx-RetransmissionTimerDL for the corresponding HARQ process in the first symbol after the expiry of drx-HARQ-RTT-TimerDL if the MAC PDU was not successfully decoded.

In NR MBS, multiple UEs interested in the same MBS service will receive the same DCI and MBS MAC PDU, which is different from unicast. If the DRX mechanism for unicast is directly reused in DRX for PTM transmission, Active Time of different UEs receiving the same MBS service may be not aligned in following case:

- According to the DRX mechanism for unicast, only UEs that failed to receive the MBS data 1 will start Retransmission Timer. As a result, Active Time of the UEs that successfully received the MBS data 1 is not aligned with that of others UEs which failed to receive the MBS data 1. In this case, if network sends a new transmission MBS data 2 by PTM transmission while Retransmission Timer is running of some UEs, the UEs that successfully receive MBS data 1 may be in the sleep state and cannot receive the MBS data2.

Considering the differences between unicast and MBS, it is not possible to directly reuse unicast DRX mechanism for multicast DRX. Otherwise, for PTM transmission, some UEs may fail to receive new transmission transmitted during the running of Retransmission Timer of other UEs.

Observation 1: Some UEs may fail to receive new transmission transmitted during the running of Retransmission Timer of other UEs.

In order to avoid some UEs missing new PTM transmissions during the running of Retransmission Timer, for multicast PTM transmission, if neither onDuration Timer nor Inactivity Timer is running, and Retransmission Timer of only some UEs is running, the network should not schedule new transmissions.

Proposal 1: For DRX operation for multicast PTM, if neither onDuration Timer nor Inactivity Timer is running, and Retransmission Timer of only some UEs is running, the network should not schedule new transmissions.

2) Support for short DRX of multicast:

In unicast, short DRX cycle is beneficial for burst services which is sporadic. For multicast, it is unclear whether burst services exist. And, considering that the support of short DRX cycle will introduce additional specification complexity, at least in R17 MBS, we propose that short DRX for multicast is not supported.

Proposal 2: Short DRX for multicast is not supported in R17 MBS.

3) Support for DRX command MAC CE of multicast:

During last meeting, whether to support DRX command MAC CE for multicast was discussed. And if DRX command MAC CE for multicast is supported, there are two options that needs to be considered.

- Option 1: the existing DRX command MAC CE and its corresponding LCID for unicast DRX is reused for multicast.

- Option 2: a new LCID corresponding to DRX command MAC CE is defined for multicast.

However, for option1, some UEs may fail to receive the initial transmission by PTM transmission for a MAC PDU corresponding to DRX command MAC CE for multicast, and the network may perform the retransmission of the MAC PDU by PTP transmission. In this case, UE will incorrectly determine that the DRX command MAC CE is for unicast DRX and will incorrectly stop drx-onDurationTimer and drx-InactivityTimer for unicast DRX. While for option2, we have to design a MAC CE with new format to realize per G-RNTI control and use a new LCID for the MAC CE.

Considering the potential issue for option1 and the specification effort for option2, we propose to not support DRX command MAC CE for multicast without clear benefit.

Proposal 3: DRX command MAC CE for multicast is not supported in R17 MBS.

4) Coordination between DRX for unicast and DRX for PTM transmission/MBS SPS transmission of multicast:

According to RAN1#104 e-meeting agreement, when ACK/NACK based HARQ-ACK feedback is enabled for multicast, after the network performs initial transmission by PTM transmission for a MAC PDU, the network may perform retransmission of the MAC PDU by PTP transmission for a specific UE or perform retransmission of the MAC PDU by PTM transmission. Furthermore, according to RAN1#104bis e-meeting agreement, after the network performs initial transmission by MBS SPS transmission for a MAC PDU, the network may perform retransmission of the MAC PDU by PTP transmission for a specific UE or perform retransmission of the MAC PDU by PTM transmission.

Hence, when ACK/NACK based HARQ-ACK feedback is enabled for multicast, in order to ensure that UE can receive the DCI scheduling retransmission by PTP transmission, after UE fails to receive PDSCH for PTM scheduling/MBS SPS transmission, DRX for unicast should also allow the UE to monitor the retransmission scheduled by PTP. Currently there are two options to be selected from:

- Option 1: the UE monitors UE specific PDCCH/C-RNTI only when drx-RetransmissionTimer-DL-PTM is running and PTP retransmission is expected.

- Option 2: the UE monitors UE specific PDCCH/C-RNTI only during unicast DRX’s active time. Unicast DRX’s RTT timer can be started when PTP retransmission is expected.

First, we think Option2 makes more sense as it aligns with the principle that unicast DRX timers control unicast DRX behaviour. Second, it is not very clear about the saying of “when PTP retransmission is expected”. In other words, when does UE expect PTP retransmission and when does it not?  Therefore it is better for the gNB to control this and send an explicit indication to the UE to notify whether PTP retransmission for PTM is enabled.

Proposal 4: For multicast, when the UE fails to receive PDSCH for PTM transmission, unicast DRX’s RTT timer is started if PTP retransmission for PTM is enabled by network.

5)  PTM DRX in HARQ-ACK feedback disabled for multicast:

According to RAN1 agreements, there are two methods for enabling/disabling HARQ-ACK feedback.

- Method 1: RRC signalling configures the presence of the field “enabling/disabling HARQ-ACK feedback indication” in the group-common DCI and the configuration is per G-RNTI/G-CS-RNTI.

- Method 2: RRC signalling configures directly whether the HARQ-ACK feedback is enabled or disabled and the configuration is per G-RNTI/G-CS-RNTI.

For method 1, since group-common DCI may indicate that the HARQ-ACK feedback is enabled, network needs to configure the RTT Timer and Retransmission Timer to UE. In this case, if a group-common DCI corresponding to a MAC PDU indicates that the HARQ-ACK feedback is disabled, network will not perform retransmission for the MAC PDU, and UE does not need to start RTT Timer and Retransmission Timer for the MAC PDU.

For method 2, if network configures directly the HARQ-ACK feedback is disabled by RRC signalling, network does not need to configure the RTT Timer and Retransmission Timer to UE, and UE will not start RTT Timer and Retransmission Timer.

Proposal 5: For multicast, if HARQ-ACK feedback is disabled for a G-RNTI/G-CS-RNTI, UE does not start the corresponding RTT Timer and Retransmission Timer.

  - Identity of MTCHs associated with different G-RNTIs for multicast

According to RAN2#116 e-meeting agreement, common LCID space is shared by DTCHs and MTCHs for multicast. The left issue is that whether the same LCID can be configured  for different MTCHs corresponding to different G-RNTIs within one UE. If the same LCID (e.g., LCID 1) is configured to one UE for two MTCHs (e.g., MTCH 1 and MTCH 2) corresponding to two G-RNTIs (e.g., G-RNTI 1 and G-RNTI 2), there will be an issue. If UE did not receive the DCI scrambled by G-RNTI 1or G-RNTI 2 for the initial PTM transmission, after receiving PTP retransmission of the initial PTM transmission , the UE cannot understand whether LCID 1 in the MAC PDU corresponds to MTCH 1 of G-RNTI 1 or MTCH 2 of G-RNTI 2.

To avoid the issue, there are three options.

- Option 1: the same LCID cannot be configured for different MTCHs corresponding to different G-RNTI within one UE, and only LCID 1~32 can be used for MTCH (i.e., eLCID cannot be used for MTCH).

Within one UE, only LCID can uniquely identify an MTCH. If two G-RNTIs are not configured to one UE simultaneously, the network can configure the same LCID for the two G-RNTIs. And when a new UE joins a multicast, network may need to reconfigure the LCID of many UEs to avoid LCID collision. Take the following figure as an example:

- At first, there is no UE that is configured with both G-RNTI 2 and G-RNTI 5. UE1 is configured with G-RNTI 1 and G-RNTI 2. And the network configures the same LCID for G-RNTI 2 and G-RNTI 5.

- Later, UE1 is configured with G-RNTI 5 besides G-RNTI 1 and G-RNTI 2. In this case, the network needs to adjust the LCID configuration to avoid LCID collision with UE1, because LCID corresponds to two G-RNTIs will be the same and UE1 won’t be able to differentiate.

With option1, it will be complex for network to manage the LCIDs of MTCH for multicast, and the number of G-RNTIs/MRBs that can be supported in a cell will be limited.

Figure 1. LCIDs corresponding to different G-RNTIs cannot be the same within one UE

- Option 2: the same LCID can be configured for different MTCHs corresponding to different G-RNTIs within one UE, i.e. G-RNTI+LCH ID is unique in one UE. But the PTP retransmission of PTM transmission for these G-RNTIs cannot be enabled simultaneously by network.

For example, assume that UE1 is configured with G-RNTI 1 and G-RNTI 2 both of which correspond to the same LCID. In this case, PTP retransmission of PTM transmission for at most one G-RNTI of G-RNTI1 and G-RNTI2 can be enabled for UE1.

Figure 2. LCIDs corresponding to different G-RNTIs can be the same within one UE under certain conditions

To avoid complex gNB handing and the limitation on the maximum number of G-RNTIs/MRBs supported by a cell, we propose to support option 2.

Proposal 6a: The same LCID can be configured for different MTCHs corresponding to different G-RNTIs within one UE, i.e. G-RNTI+LCH ID is unique in one UE to identify an RLC entity.

Proposal 6b: To avoid ambiguity, the network implementation should not simultaneously configure PTP based retransmission for two G-RNTIs of the same LCID.

  - PDCP polling mechanism for multicast

It has been agreed that PTM RLC can only be configured as UM. As a result, data loss may happen in normal PTM transmission. Of course, to improve reliability, the network can switch to PTP leg for subsequent packet delivery. But the packets lost during PTM transmission may not have a chance to be retransmitted via PTP unless some reliable feedback is introduce to PDCP. Therefore, a kind of polling mechanism which is similar to what was agreed or LTE LWA can be introduced in PDCP layer. This can be designed as simple as possible and will not bring much specs effort as any trigger of the polling can be left to NW implementation.

Proposal 7: Introduce PDCP polling for MBS similar as LTE LWA and leave the trigger to the NW implementation.

  - Initialization of PDCP variables and RLC variables for multicast

As discussed in the previous meetings, HFN synchronization is not needed during multicast transmission. And for the FMC included in PDCP status report, gNB can simply ignore the HFN value part and deduce the right PDCP PDUs for retransmission. Besides, the PDCP SR in LTE does not contain HFN value and the reason that NR uses FMC instead of FMS is just to unify the PDCP SR format. So HFN synchronization between UE and network is not necessary in any case. It is better not to design extra signalling and procedure for a purpose that is not useful.

Proposal 8: For multicast, it is up to UE implementation to select the initial value of HFN part of RX_NEXT and RX_DELIV.

In the PDCP running CR, regarding with the initialization of RX_DELIV, there is an editor’s note about what the initial value of RX_DELIV will be set. During previous discussion, it was agreed to set RX_DELIV to a value before the RX_NEXT. A possible value was given as (x – 0.5 × 2[PDCP-SN-Size–1]) modulo (2[PDCP-SN-Size]) like in sidelink groupcast and broadcast. However there is still concern about this value. Therefore we propose to leave the exact value to up to UE implementation.

Proposal 9a: For multicast, it is up to UE implementation to set the initial value of RX_DELIV to a value before RX_NEXT.

And, according to RAN2#116 e-meeting agreement, for multicast, the initial value of RX_Next_Reassembly is set to a value before the RX_Next_Highest. Similar to PDCP, we propose to leave the exact value to up to UE implementation.

Proposal 9b: For multicast, it is up to UE implementation to set the initial value of RX_Next_Reassembly to a value before RX_Next_Highest.

  - Initialization of PDCP variables and RLC variables for broadcast

1) Initialization of PDCP variables

Since both AS security and PDCP status report are not supported for broadcast, HFN between UE and network does not need to be aligned.

So, we propose that the initial value of HFN part of PDCP variables (i.e., RX_NEXT and RX_DELIV) is set by UE itself (similar to sidelink groupcast and broadcast)

Proposal 10: For broadcast, it is up to UE implementation to select the initial value of HFN part of RX_NEXT and RX_DELIV.

According to RAN2#115 and RAN2#116 e-meeting agreement, for multicast, the initial value of SN part of RX_NEXT and RX_DELIV is set according to the SN of the first received packet. But there was no agreement for broadcast.

For broadcast, the initial value of SN part of RX_NEXT and RX_DELIV also can be set according to the SN of the first received packet.

- RX_NEXT: obviously, the initial value of SN part of RX_NEXT is (x +1) modulo (2[PDCP-SN-Size]), where x is the SN of the first received PDCP Data PDU.

- RX_DELIV: based on the latest RAN1 progress, there is no out-of-order reception case. Therefore we propose to set the initial value of RX_DELIV equal to RX_NEXT for broadcast unless out-of-order reception case is identified by RAN1.

Proposal 11a: For broadcast, the initial value of SN part of RX_NEXT is (x +1) modulo (2[PDCP-SN-Size]), where x is the SN of the first received PDCP Data PDU.

Proposal 11b: For broadcast, the initial value of SN part of RX_DELIV is (x +1) modulo (2[PDCP-SN-Size]), where x is the SN of the first received PDCP Data PDU, i.e. same as RX_NEXT.

2) Initialization of RLC variables

According to RAN2#115 and RAN2#116 e-meeting agreement, for multicast, the initial value of RX_Next_Highest and RX_Next_Reassembly is set according to the SN of the first received UMD PDU containing an SN. But there was not agreement for broadcast.

For broadcast, the initial value of RX_Next_Highest and RX_Next_Reassembly also is set according to the SN of the first received UMD PDU containing an SN.

- RX_Next_Highest: obviously, the initial value of RX_Next_Highest is set to the SN of the first received UMD PDU containing an SN.

- RX_Next_Reassembly: based on the latest RAN1 progress, there is no out-of-order reception case. Therefore we propose to set the initial value of RX_Next_Reassembly equal to RX_Next_Highest for broadcast unless out-of-order reception case is identified by RAN1.

Proposal 12a: For broadcast, the initial value of RX_Next_Highest is set to the SN of the first received UMD PDU containing an SN.

Proposal 12b: For broadcast, the initial value of RX_Next_Reassembly is set to the SN of the first received UMD PDU containing an SN, i.e. same as RX_Next_Highest.

  - COUNT wrap-around issue for broadcast

For unicast, network can make sure that COUNT does not wrap around by releasing the old DRB and add a new DRB before COUNT wrap-around happens. During this procedure, UE will not detect COUNT wrap-around at all. But for broadcast, there are two differences:

- Difference 1: broadcast MRB establishment and broadcast MRB release are up to UE implementation, and the network cannot sense all UEs receiving the broadcast service.

So, before COUNT wrap-around happens, network cannot release the old MRB and add a new MRB for all UEs receiving the broadcast service.

- Difference 2: HFN is set by UE itself, and HFN between UE and network is not aligned.

In this case, COUNT wrap-around may happen for UE and UE may consider it abnormal. It is unclear what the UE behaviour will be.

To solve the above issues, there are three solutions to consider.

- Solution 1: COUNT wrap-around is allowed for broadcast and RAN2 further discusses UE behaviour when it happens.

- Solution 2: Introduce an indication in the MCCH message which indicates UE to release the old MRB and establish a new MRB.

- Solution 3: How to prevent COUNT wrap-around for broadcast is left to UE implementation. Before UE detects COUNT wrap-around, UE itself may perform broadcast MRB release and broadcast MRB establishment.

Solution 1 needs further study and solution 2 will cause service interruption, so we propose to adopt solution 3.

Proposal 13: How to prevent COUNT wrap-around for broadcast is left to UE implementation.

  - Maximum number of MBS sessions

For the following LS from SA2, we need to discuss the maximum number of MBS sessions that can be associated to a PDU session.

To SA4, SA6, RAN2:

ACTION: 	SA2 kindly asks SA4, SA6 and RAN2 to provide feedback on the maximum number of MBS sessions that can be associated to a PDU session.

If the maximum number of MBS sessions that can be associated to a PDU session is smaller than the maximum number of MBS sessions that can be supported by UE, additional PDU session(s) need to be established for MBS session, which is unreasonable. So we propose that the maximum number of MBS sessions that can be associated to a PDU session is the maximum number of MBS sessions that can be supported by UE.

Proposal 14: The maximum number of MBS sessions that can be associated to a PDU session is the maximum number of MBS sessions that can be supported by UE.

3	Conclusions

In this contribution, we discussed the general considerations for remaining UP issues. Based on our discussion, we conclude with the following observations and proposals:

Observation 1: Some UEs may fail to receive new transmission transmitted during the running of Retransmission Timer of other UEs.

Proposal 1: For DRX operation for multicast PTM, if neither onDuration Timer nor Inactivity Timer is running, and Retransmission Timer of only some UEs is running, the network should not schedule new transmissions.

Proposal 2: Short DRX for multicast is not supported in R17 MBS.

Proposal 3: DRX command MAC CE for multicast is not supported in R17 MBS.

Proposal 4: For multicast, when the UE fails to receive PDSCH for PTM transmission, unicast DRX’s RTT timer is started if PTP retransmission for PTM is enabled by network.

Proposal 5: For multicast, if HARQ-ACK feedback is disabled for a G-RNTI/G-CS-RNTI, UE does not start the corresponding RTT Timer and Retransmission Timer.

Proposal 6a: The same LCID can be configured for different MTCHs corresponding to different G-RNTIs within one UE, i.e. G-RNTI+LCH ID is unique in one UE to identify an RLC entity.

Proposal 6b: To avoid ambiguity, the network implementation should not simultaneously configure PTP based retransmission for two G-RNTIs of the same LCID.

Proposal 7: Introduce PDCP polling for MBS similar as LTE LWA and leave the trigger to the NW implementation.

Proposal 8: For multicast, it is up to UE implementation to select the initial value of HFN part of RX_NEXT and RX_DELIV.

Proposal 9a: For multicast, it is up to UE implementation to set the initial value of RX_DELIV to a value before RX_NEXT.

Proposal 9b: For multicast, it is up to UE implementation to set the initial value of RX_Next_Reassembly to a value before RX_Next_Highest.

Proposal 10: For broadcast, it is up to UE implementation to select the initial value of HFN part of RX_NEXT and RX_DELIV.

Proposal 11a: For broadcast, the initial value of SN part of RX_NEXT is (x +1) modulo (2[PDCP-SN-Size]), where x is the SN of the first received PDCP Data PDU.

Proposal 11b: For broadcast, the initial value of SN part of RX_DELIV is (x +1) modulo (2[PDCP-SN-Size]), where x is the SN of the first received PDCP Data PDU, i.e. same as RX_NEXT.

Proposal 12a: For broadcast, the initial value of RX_Next_Highest is set to the SN of the first received UMD PDU containing an SN.

Proposal 12b: For broadcast, the initial value of RX_Next_Reassembly is set to the SN of the first received UMD PDU containing an SN, i.e. same as RX_Next_Highest.

Proposal 13: How to prevent COUNT wrap-around for broadcast is left to UE implementation.

Proposal 14: The maximum number of MBS sessions that can be associated to a PDU session is the maximum number of MBS sessions that can be supported by UE.

4	References

- S2-2109171, Reply LS on maximum number of MBS sessions that can be associated to a PDU session.

5	Annex: Agreements in previous meetings

In RAN2#114 and 115 e-meeting, the following agreements for LCH for multicast were made:

RAN2#114 e-meeting: MTCH is specified for PTM transmission of NR MBS. MTCH is mapped to the DL-SCH. DTCH is reused for PTP transmission of NR MBS. FFS if there is a need to have specific LCID spaces for the used channels. Multiplexing/de-multiplexing of different logical channels associated with the same G-RNTI is supported for NR MBS. FFS if Multiplexing/de-multiplexing of different logical channels associated with the same G-CS-RNTI is supported for NR MBS. Multiplexing/de-multiplexing of different logical channels associated with the C-RNTI is supported for NR MBS. RAN2#115 e-meeting: - FFS whether to share common LCID space for Multicast PTM and Unicast DTCH. FFS How many PTM LCIDs to be reserved if separate space is used. - Multicast PTP and Unicast DTCH/DRB share common LCID space. - Multiplexing/de-multiplexing of different logical channels associated with the same G-CS-RNTI is supported for NR MBS.

In RAN1#103 e-meeting, the following agreements for terminology related to MBS transmission scheme were made:

Agreements: For convenience of discussion, consider the following clarification as RAN1 common understanding. - PTP transmission: For RRC_CONNECTED UEs, use UE-specific PDCCH with CRC scrambled by UE-specific RNTI (e.g., C-RNTI) to schedule UE-specific PDSCH which is scrambled with the same UE-specific RNTI. - PTM transmission scheme 1: For RRC_CONNECTED UEs in the same MBS group, use group-common PDCCH with CRC scrambled by group-common RNTI to schedule group-common PDSCH which is scrambled with the same group-common RNTI. This scheme can also be called group-common PDCCH based group scheduling scheme. - PTM transmission scheme 2: For RRC_CONNECTED UEs in the same MBS group, use UE-specific PDCCH with CRC scrambled by UE-specific RNTI (e.g., C-RNTI) to schedule group-common PDSCH which is scrambled with group-common RNTI. This scheme can also be called UE-specific PDCCH based group scheduling scheme. - Note: The ‘UE-specific PDCCH / PDSCH’ here means the PDCCH / PDSCH can only be identified by the target UE but cannot be identified by the other UEs in the same MBS group with the target UE. - Note: The ‘group-common PDCCH / PDSCH’ here means the PDCCH / PDSCH are transmitted in the same time/frequency resources and can be identified by all the UEs in the same MBS group. - FFS whether or not to have additional definition of transmission scheme(s) Agreements: Support SPS group-common PDSCH for MBS for RRC_CONNECTED UEs - FFS: use group-common PDCCH or UE-specific PDCCH for SPS group-common PDSCH activation/deactivation - FFS: whether to support more than one SPS group-common PDSCH configuration per UE - FFS: whether and how uplink feedback could be configured - FFS: retransmission of SPS group-common PDSCH

In RAN1#104 e-meeting, the following agreements for MBS HARQ were made:

Agreement: For RRC_CONNECTED UEs, if ACK/NACK based HARQ-ACK feedback is supported for PTM scheme 1, and if initial transmission for multicast is based on PTM transmission scheme 1, support retransmission(s) using PTP transmission. - The HARQ process ID and NDI indicated in DCI is used to associate the PTM scheme 1 and PTP transmitting the same TB. Agreement: For RRC_CONNECTED UEs receiving multicast, support the following: - ACK/NACK based HARQ-ACK feedback for multicast, - It is up to network to configure orthogonal PUCCH resources among UEs within the same group. - FFS: NACK-only based HARQ-ACK feedback for multicast, - It is up to network to configure the PUCCH resources and the PUCCH resources can be shared among UEs within the same group. - FFS details. Agreement: For the cases of HARQ-ACK feedback (at least for ACK/NACK based feedback) is available for multicast and unicast for a given UE receiving multicast, for determining the PUCCH resource, - Support multiplexing for the same priority and prioritizing for different priorities at least when the corresponding PUCCH resources overlap in time in a slot. - FFS whether it is subject to UE capability. - FFS the case of non-overlapping PUCCHs resources for HARQ-ACK in the same slot. - FFS whether sub-slot based PUCCH transmission for HARQ-ACK is supported. - FFS the case of HARQ-ACK feedback for multicast and other UCI for unicast. Agreement: For RRC_CONNECTED UEs, more than one SPS group-common PDSCH configuration for MBS can be configured per UE subject to UE capability - The total number of SPS configurations supported by a UE currently defined for unicast is not increased due to additionally supporting MBS. - FFS: How to allocate the total SPS configurations between MBS and unicast. Agreement: For RRC_CONNECTED UEs, support HARQ-ACK feedback for SPS group-common PDSCH for MBS - FFS: The retransmission scheme(s) - FFS: The HARQ-ACK details for SPS PDSCH and activation/deactivation, which can be discussed in AI 8.12.2

In RAN1#104bis e-meeting, the following agreements for MBS HARQ were made:

Agreement: The retransmission scheme for a given SPS group-common PDSCH can be either PTM scheme 1 or PTP. FFS: Whether PTM scheme 1 retransmission and PTP retransmission can be used simultaneously for different UEs in the same MBS group Agreement: The same HARQ process ID and NDI are used for PTM scheme 1 (re)transmissions and PTP retransmissions of the same TB.

In RAN2#115 and 116 e-meeting, the following agreements for MBS DRX were made:

RAN2#115 e-meeting: For multicast PTM transmission, Multicast DRX pattern is configured on a per G-RNTI basis (i.e. independent of legacy UE-specific DRX for unicast transmission). Legacy UE-specific DRX pattern for unicast is reused for PTP transmission of NR MBS, which means the UE specific DRX pattern are for both unicast services and the MBS PTP bearer of UE Multicast long DRX support is baseline for PTM. FFS whether to support optional short DRX or not. The Multicast Long DRX operation has to support the following parameters which are similar to the UE-specific DRX for unicast, where the last two parameters are needed if the HARQ- feedback is enabled: - drx-onDurationTimerPTM - drx-InactivityTimerPTM - drx-LongCycleStartOffsetPTM - drx-SlotOffsetPTM - drx-HARQ-RTT-TimerDLPTM - drx-RetransmissionTimerDLPTM RAN2#116 e-meeting: [050] it is up to network implementation on how to configure DL RTT and Re-transmission timer of multicast DRX in case of multicast HARQ ACK/NACK feedback using UE specific PUCCH resources. FFS for case of disabled HARQ FB. [050] For group common PTM Multicast HARQ PUCCH resources (NACK only feedback), the same group of UEs have aligned HRAQ RTT and DL Re-Tx timer configuration. HARQ RTT timer counting starts from end of common PUCCH resource based NACK transmission (i.e. same as Unicast DRX behaviour). FFS for case of disabled HARQ FB. [050] FFS whether short DRX cycle is supported for multicast DRX. [050] FFS how UE monitors UE specific PDCCH/C-RNTI for possible PTP transmission for PTM HARQ retransmission in active time of multicast DRX, the following alternatives are on the table (one to be selected): - Option 2: the UE monitors UE specific PDCCH/C-RNTI only when drx-RetransmissionTimerDLPTM is running and PTP retransmission is expected. - Option 3: the UE monitors UE specific PDCCH/C-RNTI only during unicast DRX’s active time. Unicast DRX’s RTT timer can be started when PTP retransmission is expected. [050] FFS For DRX command MAC CE for multicast DRX, the following alternatives are on the table (one to be selected): - Option 2b: use a DRX command MAC CE per multicast DRX operation (i.e. per G-RNTI basis) - Option 3: neither legacy DRX command MAC CE nor new DRX command MAC CE is used for multicast DRX, i.e. no DRX command MAC CE for multicast DRX.

In RAN1#106, 106bis and 107 e-meeting, the following agreements for enabling/disabling HARQ-ACK feedback were made:

RAN1#106 e-meeting: Agreement: Update the WA made in RAN1#105-e meeting regarding enabling/disabling HARQ-ACK feedback as follows: Working assumption: For enabling/disabling ACK/NACK-based HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast via dynamic group-common PDSCH: - RRC signaling configures the enabling/ disabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback. - If RRC signaling configures the function of group-common DCI based indication, group-common DCI indicates (explicitly or implicitly) whether ACK/NACK based HARQ-ACK feedback is enabled/disabled - Otherwise, enabling/disabling ACK/NACK based HARQ-ACK feedback is configured by RRC signaling. - FFS details on RRC signaling and group-common DCI indicating. - FFS whether/how this option is extended to apply to NACK-only based feedback and multiple G-RNTI cases. - FFS the relation to the HARQ-ACK codebook types and HARQ-ACK codebook construction. - FFS the relation to the enabling/disabling ACK/NACK based HARQ-ACK feedback for retransmission. - FFS whether/how to allow UE not to react to the DCI signaling, but instead follow UE-specific RRC configuration for HARQ feedback. - FFS whether/how to apply it to SPS group-common PDSCH. - UE capability for enabling/ disabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback is introduced and FFS details. Note: It is up to network implementation to avoid any potential HARQ ACK mismatch between different UEs in the same multicast group RAN1#106bis e-meeting: Agreement: For group-common DCI indicating whether ACK/NACK based HARQ-ACK feedback is enabled/disabled, down-select from the following alternatives: - Alt1: Reuse one existing field in the group-common DCI. - Alt2: Introduce a new field in the group-common DCI. RAN1#107 e-meeting: Agreement - For PTM retransmission, - if UE is configured to enable/disable HARQ-ACK per group-common DCI indication for initial transmission, whether HARQ-ACK is enabled/disabled for PTM retransmission also follows the indication in the group-common DCI scheduling the PTM retransmission. - if UE is configured directly whether the HARQ-ACK is enabled/disabled, it applies to both PTM initial transmission and retransmission. - For PTP retransmission, the HARQ-ACK is always enabled. Agreement Support enabling/disabling HARQ-ACK for NACK-only based feedback. - The relevant agreements made for ACK/NACK based feedback can be extended for the support of NACK-only, including: - RRC signalling configures the presence of the field “enabling/disabling HARQ-ACK feedback indication” in the group-common DCI and the configuration is per G-RNTI. - RRC signalling configures directly whether the HARQ-ACK feedback is enabled or disabled and the configuration is per G-RNTI. Agreement For group-common DCI indicating whether ACK/NACK based HARQ-ACK feedback is enabled/disabled, the “enabling/disabling HARQ-ACK feedback indication” is included in DCI format 1_1 scrambled by G-RNTI - For DCI format 1_1 scrambled by G-CS-RNTI, it is discussed separately. Agreement For the DCI format including the field of “enabling/disabling HARQ-ACK feedback indication” for multicast scheduling, the field is a new field with 1 bit. Agreement For multicast SPS PDSCH without PDCCH scheduling, support the following: - RRC signalling configures the presence of the field “enabling/disabling HARQ-ACK feedback indication” in the group-common DCI for multicast SPS activation. - The configuration is per G-CS-RNTI. - Separate UE capability is needed from that for dynamic scheduling for multicast. - RRC signalling configures directly whether the HARQ-ACK feedback is enabled or disabled. The configuration is per G-CS-RNTI.

In RAN2#115 and 116 e-meeting, the following agreements for initialization of PDCP variables and RLC variables for multicast were made:

RAN2#115 e-meeting: For PTM PDCP state variables setting while configured, the SN part of COUNT values of these variables are set according to the SN of the first received packet (by the UE) and the HFN indicated by the gNB, if needed. Initialize the PTM RLC entity for an MRB configuration, the value of RX_Next_Highest and RX_Next_Reassembly are set according to the SN of the first received packet containing an SN. RAN2#116bis e-meeting: for multicast MRB, the initial value of the SN part of RX_NEXT is (x +1) modulo (2[PDCP-SN-Size]), where x is the SN of the first received PDCP Data PDU. the initial value of RX_DELIV is set to a value before RX_NEXT, e.g. the initial value of the SN part of RX_DELIV is (x – 0.5 × 2[PDCP-SN-Size–1]) modulo (2[PDCP-SN-Size]), where x is the SN of the first received PDCP Data PDU. If HFN is needed (FFS), the initial value of HFN (maybe + related PDCP SN to avoid ambiguity of HFN FFS) is indicated by the gNB by RRC (e.g. during RRC based MRB bearer type change). for multicast PTM, the RX_Next_Highest is initially set to the SN of the first received UMD PDU containing an SN for multicast PTM, the initial value of RX_Next_Reassembly is set to a value before the RX_Next_Highest.