3GPP TSG-RAN WG2 #117	R2-220xxxx

- Online, February 21 – March 3, 2022

Agenda item:	8.6.3

Source:	Rapporteur (ZTE)

Title:	Summary of [AT117-e] [501] [ Sdata] CP additional open issues – Phase2

Document for:	Discussion & Decision

# Introduction

This document summarizes open issues after the initial discussion regarding CP open issues based on the summary in R2-2203732.

Deadline for companies’ inputs: Wed, March 2nd, 9:00 UTC

# Capabilities

## Relation between CG-SDT and RA-SDT capabilities

[CB] Proposal 3 (17 out of 22): UE supporting CG-SDT shall also support 4-step RA-SDT

17 out of 22 companies supported the above proposal. However, this seems controversial. So, the rapporteur would try to understand if there will be any objections if we go the opposite way. Then we have all the views and hopefully we can make a quick decision.

Q1: Can companies accept the following proposal (or do you have strong objections)?

Proposal: UE supporting CG-SDT need not support 4-step RA-SDT (i.e. these capabilities are independent).

| Company | Acceptable / Strong concern | Comments (please explain why there is a strong concern) |
| --- | --- | --- |
| LGE | - Acceptable | We still don’t see the need to couple independent features. During online discussion, some companies argue that the UE has to support 4-step RA. We agree that 4-step RA is mandatory for all UEs, but what we are talking about is 4-step RA-SDT not the 4-step RA. Each SDT feature (i.e. CG-SDT, 2-step RA-SDT, and 4-step RA-SDT) is independent, and 4-step RA is supported regardless of SDT feature. If what companies want is make 4-step RA-SDT mandatory, then we can accept it. But, in this case, UE supporting 2-step RA-SDT should also support 4-step RA-SDT. In this case, we can say that 4-step RA-SDT is mandatory, and CG-SDT and 2-step RA-SDT are optional. |

## Separte capability for SRB

For the proposal to have separate capability for SRB for SDT, the responses were very mixed (Support 10, Not Support 7, No strong view 5).

As a quick way forward, the rapporteur wold like to check if companies would have strong concerns (objections) if we go with the following proposal

Q2: Can companies accept the following proposal (or do you have strong objections)?

Proposal: Separate capability is needed for SRB (i.e. for NAS messages)

| Company | Acceptable / Strong concern | Comments (please explain why there is a strong concern) |
| --- | --- | --- |
| LGE | - Acceptable | The SRB SDT is mainly introduced for transmitting Positioning data in SRB2. As Positioning is an optional feature, we think it is logical that SRB SDT is also an optional feature. |

# Parameter value ranges

## DVT values

For data volume thresholds, there were a number of good inputs. The rapporteur view is that this issue is not controversial. The following proposal is made to accommodate the company views expressed.

- Have one small value to cover the TCP/IP header size (i.e. to cover the keep alive traffic) – byte 32 covers this

- Reduce the granularity at lower sizes a bit to avoid too many small packet sizes (removed everything under 100 bytes apart from the 32 byte packet above)

- Include at least one packet size to cover the positioning packet (the size at and above 9000 are now included)

- Cover a few more values at higher range (some more values aboe 9000 are also included).

So, the following proposal is made as a result of the above comments seen during the discussion.

Proposal: DVT is configured as follows:

ENUMERATED {byte32, byte100, byte200, byte400, byte600, byte800, byte1000, byte2000, byte4000, byte8000, byte9000, byte10000, byte12000, byte24000, byte48000, byte96000}

Q3: Can companies accept the above proposal (or do you have strong objections)?

| Company | Acceptable / Strong concern | Comments (please explain why there is a strong concern) |
| --- | --- | --- |
| LGE | - Acceptable | But, we still think finer granularity is not so needed. Maybe we can remove some values and add spare values. E.g., ENUMERATED {byte32, byte100, byte200, byte400, byte600, byte1000, byte2000, byte4000, byte9000, byte12000, byte24000, byte48000, byte96000, spare1, spare2, spare3} |

## SDT error detection timer values

UE vendors expressed concern over long SDT error detection timers. On the one hand the timer should be not too long but it should also allow the network to send any response message in DL (otherwise UE will be paged and hence the purpose of SDT is is not seved).

Keeping in mind the comments expressed, the compromise proposal below is provided (to avoid the largest – i.e. the 10 sec value), but to extend this up to 6 seconds.

Considering that the longest value is only a possible configuration and anyway the network may use smaller value and in fact the network can send the UE back to inactive as soon as the DL acknowledgment is received, the following proposal is made:

Proposal: SDT error detection timer (t3xx) is configured as follows:

t3XX    ENUMERATED {ms100, ms200, ms300, ms400, ms600, ms1000, ms2000, ms3000, ms6000, spare7, spare6, spare5, spare4, spare3, spare2, spare1}

Q4: Can companies accept the above proposal (or do you have strong objections)?

| Company | Acceptable / Strong concern | Comments (please explain why there is a strong concern) |
| --- | --- | --- |
| LGE | - Acceptable |  |

## CG-SDT periodicities

Long periodicities for CG-SDT would allow for better resource utilization at the network. However, it seems majority view is not to support this. Rapporteur notes that if we want to support long periodicities, we have to support also delayed start of the SDT error detection timer (since companies don’t seem to like the long SDT error detection timer values as noted in section 3.2).  Based on this, the rapporteur’s feeling is that it is hard to support the longer periodicities now for CG-SDT period. There will also be a minor impact to RAN1 specs if we support longere periodicities.

Based on the above considerations, the following proposal can be tried (as it is).

Proposal: Do not support long CG-SDT periodicities:

Q5: Can companies accept the above proposal (or do you have strong objections)?

| Company | Acceptable / Strong concern | Comments (please explain why there is a strong concern) |
| --- | --- | --- |
| LGE | - Acceptable |  |

# Other issues

## RACH failure handling

The majority view seems to be that we can stick to existing mechanism (i.e. RACH error indication from MAC results in no action in RRC and we rely on the error detection timer). Now that the error detection timer value is not too long per above proposal, may be this is perhaps an acceptable way forward? So, we could check the proposal as it is

Proposal: For handling RACH failure (i.e. that Max RACH preamble transmission is reached) during SDT procedure, MAC indicates RACH problem indication to RRC. RRC does not any action for this indication similar to legacy operation in RRC_INACTIVE/RRC_IDLE. RA procedure is continued.

Q6: Can companies accept the above proposal (or do you have strong objections)?

| Company | Acceptable / Strong concern | Comments (please explain why there is a strong concern) |
| --- | --- | --- |
| LGE | - Concern | We think the SDT error detection timer (t3xx) is typically configured with longer value than T319. If we just follow the legacy procedure, interference level will be much increased considering the longer value of t3xx. |

## UL non-SDT data arrival indication

The view expressed between UAI and new message is pretty evenly split. However, it seems some companies expressed view that RRCResumeRequest is reused (seems this is not feasible). Rapporteur’s expectation is that when all views from companies are concerned, UAI might have slight majority. So, it is proposed to try this way forward. It is anticipated that there will be no objections to this since this is mainly a matter of taste anyway.

Proposal: For the non-SDT data arrival indication, use UAI as the baseline.

Q7: Can companies accept the above proposal (or do you have strong objections)?

| Company | Acceptable / Strong concern | Comments (please explain why there is a strong concern) |
| --- | --- | --- |
| LGE | - Acceptable |  |

Further, based on the CT1 reply LS, it seems that it is up to implementation whether a new resumeCause can be sent or not by upper layers (as CT1 couldn’t reach a consensus on this issue). Based on this, rapporteur thinks we can include the resumeCause as an optional IE (to cover all implementation possibilities on the UE side).

So, we can check the following proposal:

Proposal: ResumeCause value is included in UAI as an optional IE (and hence is provided to the network if upper layers provide it to the AS).

Q8: Can companies accept the above proposal (or do you have strong objections)?

| Company | Acceptable / Strong concern | Comments (please explain why there is a strong concern) |
| --- | --- | --- |
| LGE | - Acceptable |  |

## Unlicensed spectrum

Rapporteur believes that there is no impact to the running CR for this discussion. So, no proposal is made and no further discussion is needed. If companies spot anything specific they can comment directly to the CR for this. So, no further discussion on this aspect is deemed necessary in phase2.

## Handling of pending data after RRCReject

During the phase 1 discussion only 8 out of 21 compnaies think that this is an essential issue. The rapporteur also agrees with the majority companies that this issue is not essential (i.e. can be left to network/UE implementation). Hence no further discussion will be needed on this in phase2.

# Contact Points

Respondents to the email discussion are kindly asked to fill in the following table.

| Company | Name | Email Address |
| --- | --- | --- |
| LG Electronics | SeungJune Yi | seungjune.yi@lge.com |

# References

[1] R2-2202275	Discussion on control plane issues of SDT	OPPO

[2] R2-2202556	Control plane aspects of SDT	Apple

[3] R2-2202590	Analysis on CP open issue of SDT	Lenovo, Motorola Mobility

[4] R2-2202674	Additional discussion on identified open CP issues	Intel Corporation

[5] R2-2202736	Remaining issues of control plane aspects of SDT	China Telecom

[6] R2-2202805	Handling of DL non-SDT during SDT	CATT

[7] R2-2202846	Remaining issue on CS-RNTI configuration for CG-SDT	ASUSTeK

[8] R2-2202960	Remaining issues on CP aspects of SDT	Qualcomm Incorporated

[9] R2-2203009	Remaining control plane aspects of SDT	NEC

[10] R2-2203155	CP aspects for SDT	Ericsson

[11] R2-2203337	Control plane common aspects for SDT	Huawei, HiSilicon

[12] R2-2203353	SDT control plane aspects	Nokia, Nokia Shanghai Bell

[13] R2-2202982	Further Consideration on the Handling of non-SDT Data Arrival	vivo

[14] R2-2202610, Discussion on remaining issues for SDT user plane, Huawei, HiSilicon