3GPP TSG RAN WG2 Meeting #117-e	R2- 2202921

Electronic, 21th February – 3rd March, 2022

Agenda Item:	8.20.3 (Extending NR operation to 71GHz)

Source: 	Samsung

Title: 	Discussion on L2 buffer size

Document for:	Discussion & decision

Introduction

In 3GPP RAN#91 plenary e-meeting, the updated WID RP-210862 [1] was approved as the outcome of the study item on supporting NR above 52.6GHz [2]. The WID is aimed to extend NR operation up to 71 GHz considering the operation in both licensed and unlicensed band.

The WID also indicates that RAN1 specify new SCS, 480 kHz and 960 kHz, for operation in this frequency range over 52.6GHz and RAN2 implement the design aspect related to the operation.

In the previous meeting RAN2 116-e, there was the discussion on the need of UE capability to address concern on too high L2 buffer size requirement. However, RAN2 postponed the decision on that issue and companies are asked to bring further analysis.

In this paper, we first try estimating the maximum possible L2 buffer size to check how serious the problem can be with some extreme case of using asymmetric SCS in band combination including FR2-2 band, and discuss the need to introduce new UE capability to handle this problem.

Discussion

L2 buffer size estimation

In this paper, we first try estimating the required total L2 buffer size with the use of asymmetric SCS based on the agreements given by RAN1/2 at this point. In particular, we will focus on the maximum possible total L2 buffer size with some extreme case to see how serious the L2 buffer problem can be.

In TS 38.306 [3], the total layer 2 buffer size is defined as the sum of the number of bytes that the UE is capable of storing in the RLC transmission windows and RLC reception and reassembly windows and also in PDCP reordering windows for all radio bearers. Thus, the required total layer 2 buffer size is defined as follow:

The required total layer 2 buffer size in MR-DC and NR-DC is the maximum value of the calculated values based on the following equations: - MaxULDataRate_MN * RLCRTT_MN + MaxULDataRate_SN * RLCRTT_SN + MaxDLDataRate_SN * RLCRTT_SN + MaxDLDataRate_MN * (RLCRTT_SN + X2/Xn delay + Queuing in SN) - MaxULDataRate_MN * RLCRTT_MN + MaxULDataRate_SN * RLCRTT_SN + MaxDLDataRate_MN * RLCRTT_MN + MaxDLDataRate_SN * (RLCRTT_MN + X2/Xn delay + Queuing in MN) Otherwise it is calculated by MaxDLDataRate * RLC RTT + MaxULDataRate * RLC RTT.

From the underlined equation for the case without DC, we can easily observe that the total layer 2 buffer size is proportional to both the maximum DL/UL Data rate and the RLC RTT values, which are dependent on the SCS of the carriers.

For the RLC RTT values, the RLC RTT value in the total L2 buffer size equation corresponds to the smallest SCS numerology supported in the Band/Feature set combination according to TS 38.306. For the maximum possible L2 buffer estimation, we are going to use the extreme case using asymmetric SCS (i.e., using 15 kHz SCS and 960 kHz SCS together) and thus only RLC RTT for 15 kHz will be needed.

For the maximum data rate, RAN1 already made some agreements regarding the maximum data rate (i.e., # of PRB, available BW for each SCS) and thus RAN2 can estimate the maximum possible DL/UL Data rate based on the agreements in below box.

From RAN1 102e meeting Agreement: For NR system operating in 52.6 GHz to 71 GHz, • NR should be designed with maximum FFT size of 4096 and maximum of 275RBs per carrier; • Candidate supported maximum carrier bandwidth(s) for a cell is between 400 MHz and 2160 MHz; • If subcarrier spacing 240 kHz or below are supported, NR in 52.6 to 71 GHz is expected to use normal CP length only (does not have any implications on whether ECP is supported for the higher subcarrier spacings, if supported). From RAN1 104e meeting Agreement: - From RAN1 perspective, for NR operation in 52.6 GHz to 71 GHz, - The maximum channel bandwidth for 120 kHz SCS is 400 MHz - The maximum channel bandwidth for 480 kHz SCS is 1600 MHz - The maximum channel bandwidth for 960 kHz SCS is one of the following options - 2000 MHz - 2160 MHz - Send LS to RAN4 to inform about RAN1’s agreement of maximum channel bandwidth and ask RAN4 to decide and feedback the exact value of maximum channel bandwidth for 960 kHz SCS, the corresponding numbers of RBs for the maximum channel bandwidth of SCS(s) supported in 52.6 GHz to 71 GHz.

According to the agreements, we assume the maximum channel BW for 960 kHz SCS as 2000 MHz (i.e., maximum available PRB could be about 170) and thus the maximum possible L2 buffer size can be roughly estimated based on this.

Observation 1: RAN1 already made a few agreements regarding the maximum data rate (i.e., # of PRB, available BW) for FR2-2 carriers with higher SCS.

Observation 2: The maximum possible total L2 buffer size can be roughly estimated based on the agreements in RAN1.

For estimating the maximum possible total L2 buffer size, we assume the extreme CA scenario using a FR1 carrier with 15 kHz SCS and 15 FR2-2 carriers with 960 kHz together and calculate the maximum DL/UL Data rate based on the equation in section 4.1.2 in TS 38.306. Finally, the maximum possible L2 buffer size is estimated to be about 14Gbytes as below.

Observation 3: The required total L2 buffer size can be up to about 12.9Gbytes in some extreme case with the use of asymmetric SCS.

Introduction of new UE capability to reduce the burden on too large L2 buffer size

Even though the above extreme case assumed for the estimation is unrealistic, it shows that the required L2 buffer size can be extremely large depending on the operator’s available BW and the baseband processing capability of the UE, which can prevent the UE even with sufficient radio capability from supporting carrier combinations with FR2-2 carriers due to the lack of memory. Also, there could be the UE that wants to use FR2-2 carrier with 960kHz SCS only for URLLC service but cannot support the FR2-2 carriers with 960kHz SCS due to the lack of the memory (Note that the minimum channel bandwidth for 960 kHz SCS could be about 400MHz which is quite large). Thus, RAN2 needs to continue the discussion on how to address the L2 buffer size problem.

If the L2 buffer size relaxation is inevitable, the first possible option could be redefining the current L2 buffer size in TS38.306. However, in the previous RAN2#116-e, RAN2 made the agreement to keep the current definition to prevent any packet overflow at L2 buffer of UE. Then, the remaining option is introducing a new UE capability to reduce the burden on the L2 buffer size for a particular UE. For the details, as already suggested in the previous RAN2-116e meeting, we can just think of allowing scale-down of the required L2 buffer size with some scaling factor (e.g., 0.8, 0.6, 0.4, 0.2). Then, the new UE capability information can be defined in form of ENUMERATOR to indicate one of the scaling factors. Meanwhile, it should be noted that this capability is applied only to combinations using FR2-2 carriers and should not have any impact on the conventional operation without using FR2-2.

By introducing the UE capability, the UE having not enough memory size can report the limitation with a certain scaling factor. When the network receives that information, it can limit the amount of radio resources allocated to the UE to prevent L2 buffer overflow. For example, if the scaling factor is set to 0.8, then the network can skip 1 DL slot scheduling for that UE every 5 slots, which means that the DL data rate of that UE can be reduced by 20% while supporting other UEs without any loss on data rate. Here, it seems quite reasonable for the UE to take the loss in data rate because it can’t afford higher data rate due to its own limitation. Meanwhile, this kind of scheduling operation can increase the complexity at the base station, but it can be expected that the network can handle this with the similar manner in the scheduling problem considering the UE Tx power regulation.

Observation 4: When a certain UE reports its limitation on L2 buffer size with some scaling factor, the network can limit the amount of resource allocation to the UE, which leads to DL/UL data rate loss only for that UE without affecting other UEs.

From the contributions submitted to the last RAN2#116bis-e, we can find some consensus on the use of scaling factor to handle the problem of too large L2 buffer requirement. However, regarding the details on how to implement this, there seem two different options suggested by the companies as below.

- Option 1: Reuse the existing scaling factor that is originally used for max data rate calculation.

- Option 2: Define a new scaling factor (i.e., new UE capability parameter).

Considering the expected spec impact, the option 1 seems to be the simplest way to handle this problem for now. By updating the description of the existing scaling factor in 38.306, we can reuse it also for reducing L2 buffer size requirement. However, we have some concern on this. Since the legacy scaling factor is originally introduced to indicate the limitation of UE’s on baseband processing capability, it seems a bit misleading to use this value also for L2 buffer size relaxation even without actual limitation on the PHY layer max data rate (i.e., baseband processing). In some cases, the network can unnecessarily limit the maximum number of RBs allocated to some UE during a symbol time even without any actual limitation on the UE’s baseband processing capability. Thus, for us, the use of new scaling factor for L2 buffer size calculation seems clearer than reusing the existing one.

Finally, we would like to propose the following.

Proposal 1: RAN2 to introduce a new UE capability to allow scaling L2 buffer size only for band combinations including FR2-2 band and consider the TP attached in Annex as baseline.

Conclusion

Based on the discussion above, we have the following observations and proposals:

Observation 1: RAN1 already made a few agreements regarding the maximum data rate (i.e., # of PRB, available BW) for FR2-2 carriers with higher SCS.

Observation 2: The maximum possible total L2 buffer size can be roughly estimated based on the agreements in RAN1.

Observation 3: The required total L2 buffer size can be up to about 12.9Gbytes in some extreme case with the use of asymmetric SCS.

Observation 4: When a certain UE reports its limitation on L2 buffer size with some scaling factor, the network can limit the amount of resource allocation to the UE, which leads to DL/UL data rate loss only for that UE without affecting other UEs.

Proposal 1: RAN2 to introduce a new UE capability to allow scaling L2 buffer size only for band combinations including FR2-2 band and consider the TP attached in Annex as baseline.

References

- RP-210862, Revised WID on Extending current NR operation to 71GHz, CMCC, RAN #91e, March 16- March 26, 2021.

- RP-193259, New SID: Study on supporting NR from 52.6GHz to 71GHz. Intel, Sitges, Spain, RAN#86, December 9 – 12, 2019

- TS 38.306

- Annex A: Text Proposal

- A.1	TP for TS 38.306

The modification is made against to the running CR to 38.306 for NR operation for up to 71GHz (R2-2202004).

START OF FIRST CHANGE

- 4.1.4	Total layer 2 buffer size for DL/UL

The total layer 2 buffer size is defined as the sum of the number of bytes that the UE is capable of storing in the RLC transmission windows and RLC reception and reassembly windows and also in PDCP reordering windows for all radio bearers.

The required total layer 2 buffer size in MR-DC and NR-DC is the maximum value of the calculated values based on the following equations:

-	[MaxULDataRate_MN * RLCRTT_MN + MaxULDataRate_SN * RLCRTT_SN + MaxDLDataRate_SN * RLCRTT_SN + MaxDLDataRate_MN * (RLCRTT_SN + X2/Xn delay + Queuing in SN)]*f

-	[MaxULDataRate_MN * RLCRTT_MN + MaxULDataRate_SN * RLCRTT_SN + MaxDLDataRate_MN * RLCRTT_MN + MaxDLDataRate_SN * (RLCRTT_MN + X2/Xn delay + Queuing in MN)]*f

Otherwise it is calculated by [MaxDLDataRate * RLC RTT + MaxULDataRate * RLC RTT]*f.

NOTE:	Additional L2 buffer required for preprocessing of data is not taken into account in above formula.

The required total layer 2 buffer size is determined as the maximum total layer 2 buffer size of all the calculated ones for each band combination and the applicable Feature Set combination in the supported MR-DC or NR band combinations. The RLC RTT for NR cell group corresponds to the smallest SCS numerology supported in the band combination and the applicable Feature Set combination.

wherein

X2/Xn delay + Queuing in SN = 25ms if SCG is NR, and 55ms if SCG is EUTRA

X2/Xn delay + Queuing in MN = 25ms if MCG is NR, and 55ms if MCG is EUTRA

RLC RTT for EUTRA cell group = 75ms

RLC RTT for NR cell group is defined in Table 4.1.4-1

f for band combinations not including FR2-2 = 1

f for band combination including FR2-2 is given by high layer parameter scalingFactorL2Buffer

Table 4.1.4-1: RLC RTT for NR cell group per SCS

| SCS (kHz) | RLC RTT (ms) |
| --- | --- |
| 15KHz | 50 |
| 30KHz | 40 |
| 60KHz | 30 |
| 120KHz | 20 |
| 480KHz | 20 |
| 960KHz | 20 |

END OF FIRST CHANGE

START OF SECOND CHANGE

- 4.2.2	General parameters

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| accessStratumRelease Indicates the access stratum release the UE supports as specified in TS 38.331 [9]. | UE | Yes | No | No |
| delayBudgetReporting Indicates whether the UE supports delay budget reporting as specified in TS 38.331 [9]. | UE | No | No | No |
| dl-DedicatedMessageSegmentation-r16 Indicates whether the UE supports reception of segmented DL RRC messages. | UE | No | No | No |
| drx-Preference-r16 Indicates whether the UE supports providing its preference of a cell group on DRX parameters for power saving in RRC_CONNECTED, as specified in TS 38.331 [9]. | UE | No | No | No |
| inactiveState Indicates whether the UE supports RRC_INACTIVE as specified in TS 38.331 [9]. | UE | Yes | No | No |
| inDeviceCoexInd-r16 Indicates whether the UE supports IDC (In-Device Coexistence) assistance information as specified in TS 38.331 [9]. | UE | No | No | No |
| maxBW-Preference-r16, maxBW-Preference-r17 Indicates whether the UE supports providing its preference of a cell group on the maximum aggregated bandwidth for power saving in RRC_CONNECTED, as specified in TS 38.331 [9]. | UE | No | No | Yes (Include FR2-2 diff) |
| maxCC-Preference-r16 Indicates whether the UE supports providing its preference of a cell group on the maximum number of secondary component carriers for power saving in RRC_CONNECTED, as specified in TS 38.331 [9]. | UE | No | No | No |
| maxMIMO-LayerPreference-r16, maxMIMO-LayerPreference-r17 Indicates whether the UE supports providing its preference of a cell group on the maximum number of MIMO layers for power saving in RRC_CONNECTED, as specified in TS 38.331 [9]. | UE | No | No | Yes (Include FR2-2 diff) |
| mcgRLF-RecoveryViaSCG-r16 Indicates whether the UE supports recovery from MCG RLF via split SRB1 (if supported) and via SRB3 (if supported) as specified in TS 38.331[9]. | UE | No | No | No |
| minSchedulingOffsetPreference-r16 Indicates whether the UE supports providing its preference on the minimum scheduling offset for cross-slot scheduling of the cell group for power saving in RRC_CONNECTED, as specified in TS 38.331 [9]. | UE | No | No | No |
| mpsPriorityIndication-r16 Indicates whether the UE supports mpsPriorityIndication on RRC release with redirect as defined in TS 38.331 [9]. | UE | No | No | No |
| onDemandSIB-Connected-r16 Indicates whether the UE supports the on-demand request procedure of SIB(s) or posSIB(s) while in RRC_CONNECTED, as specified in TS 38.331 [9]. | UE | No | No | No |
| overheatingInd Indicates whether the UE supports overheating assistance information. | UE | No | No | No |
| partialFR2-FallbackRX-Req Indicates whether the UE meets only a partial set of the UE minimum receiver requirements for the eligible FR2 fallback band combinations as defined in Clause 4.2 of TS 38.101-2 [3] and Clause 4.2 of TS 38.101-3 [4]. If not indicated, the UE shall meet all the UE minimum receiver requirements for all the FR2 fallback combinations in TS 38.101-2 [3] and TS 38.101-3 [4]. The UE shall support configuration of any of the FR2 fallback band combinations regardless of the presence or the absence of this field. | UE | No | No | No |
| redirectAtResumeByNAS-r16 Indicates whether the UE supports reception of redirectedCarrierInfo in an RRCRelease message in response to an RRCResumeRequest or RRCResumeRequest1 which is triggered by the NAS layer, as specified in TS 38.331 [9]. | UE | No | No | No |
| reducedCP-Latency Indicates whether the UE supports reduced control plane latency as defined in TS 38.331 [9] | UE | No | No | No |
| referenceTimeProvision-r16 Indicates whether the UE supports provision of referenceTimeInfo in DLInformationTransfer message and in SIB9 and reference time information preference indication via assistance information, as specified in TS 38.331 [9]. | UE | No | No | No |
| releasePreference-r16 Indicates whether the UE supports providing its preference assistance information to transition out of RRC_CONNECTED for power saving, as specified in TS 38.331 [9]. | UE | No | No | No |
| resumeWithStoredMCG-SCells-r16 Indicates whether the UE supports not deleting the stored MCG SCell configuration when initiating the resume procedure. | UE | No | No | No |
| resumeWithStoredSCG-r16 Indicates whether the UE supports not deleting the stored SCG configuration when initiating resume. The UE which indicates support for resumeWithStoredSCG-r16 shall also indicate support for resumeWithSCG-Config-r16. | UE | No | No | No |
| resumeWithSCG-Config-r16 Indicates whether the UE supports (re-)configuration of an SCG during the resume procedure. | UE | No | No | No |
| scalingFactorL2Buffer-r17 Indicates the scaling factor to be applied to the total layer 2 buffer size calculation for each band combination and the applicable Feature Set combination including FR2-2 band as defined in 4.1.4. Value f0p8 indicates the scaling factor 0.8, f0p6 indicates 0.6, and so on. If absent, the scaling factor 1 is applied to the total layer 2 buffer size calculation. | UE | No | No | No |
| splitSRB-WithOneUL-Path Indicates whether the UE supports UL transmission via MCG path and DL reception via either MCG path or SCG path, as specified for the split SRB in TS 37.340 [7]. The UE shall not set the FDD/TDD specific fields for this capability (i.e. it shall not include this field in UE-MRDC-CapabilityAddXDD-Mode). | UE | No | No | No |
| splitDRB-withUL-Both-MCG-SCG Indicates whether the UE supports UL transmission via both MCG path and SCG path for the split DRB as specified in TS 37.340 [7]. The UE shall not set the FDD/TDD specific fields for this capability (i.e. it shall not include this field in UE-MRDC-CapabilityAddXDD-Mode). | UE | Yes | No | No |
| srb3 Indicates whether the UE supports direct SRB between the SN and the UE as specified in TS 37.340 [7]. The UE shall not set the FDD/TDD specific fields for this capability (i.e. it shall not include this field in UE-MRDC-CapabilityAddXDD-Mode). This field is not applied to NE-DC. | UE | Yes | No | No |

END OF SECOND CHANGE

- A.2	TP for TS 38.331

The modification is made against to the running CR to 38.331 for UE capability (R2-2201783).

START OF FIRST CHANGE

#### –	UE-NR-Capability

The IE UE-NR-Capability is used to convey the NR UE Radio Access Capability Parameters, see TS 38.306 [26].

UE-NR-Capability information element

-- ASN1START

-- TAG-UE-NR-CAPABILITY-START

UE-NR-Capability ::=            SEQUENCE {

accessStratumRelease            AccessStratumRelease,

pdcp-Parameters                 PDCP-Parameters,

rlc-Parameters                  RLC-Parameters                                                        OPTIONAL,

mac-Parameters                  MAC-Parameters                                                        OPTIONAL,

phy-Parameters                  Phy-Parameters,

rf-Parameters                   RF-Parameters,

measAndMobParameters            MeasAndMobParameters                                                  OPTIONAL,

fdd-Add-UE-NR-Capabilities      UE-NR-CapabilityAddXDD-Mode                                           OPTIONAL,

tdd-Add-UE-NR-Capabilities      UE-NR-CapabilityAddXDD-Mode                                           OPTIONAL,

fr1-Add-UE-NR-Capabilities      UE-NR-CapabilityAddFRX-Mode                                           OPTIONAL,

fr2-Add-UE-NR-Capabilities      UE-NR-CapabilityAddFRX-Mode                                           OPTIONAL,

featureSets                     FeatureSets                                                           OPTIONAL,

featureSetCombinations          SEQUENCE (SIZE (1..maxFeatureSetCombinations)) OF FeatureSetCombination         OPTIONAL,

lateNonCriticalExtension        OCTET STRING (CONTAINING UE-NR-Capability-v15c0)                      OPTIONAL,

nonCriticalExtension            UE-NR-Capability-v1530                                                OPTIONAL

}

-- Regular non-critical extensions:

UE-NR-Capability-v1530 ::=               SEQUENCE {

fdd-Add-UE-NR-Capabilities-v1530         UE-NR-CapabilityAddXDD-Mode-v1530                            OPTIONAL,

tdd-Add-UE-NR-Capabilities-v1530         UE-NR-CapabilityAddXDD-Mode-v1530                            OPTIONAL,

dummy                                    ENUMERATED {supported}                                       OPTIONAL,

interRAT-Parameters                      InterRAT-Parameters                                          OPTIONAL,

inactiveState                            ENUMERATED {supported}                                       OPTIONAL,

delayBudgetReporting                     ENUMERATED {supported}                                       OPTIONAL,

nonCriticalExtension                     UE-NR-Capability-v1540                                       OPTIONAL

}

UE-NR-Capability-v1540 ::=              SEQUENCE {

sdap-Parameters                         SDAP-Parameters                                               OPTIONAL,

overheatingInd                          ENUMERATED {supported}                                        OPTIONAL,

ims-Parameters                          IMS-Parameters                                                OPTIONAL,

fr1-Add-UE-NR-Capabilities-v1540        UE-NR-CapabilityAddFRX-Mode-v1540                             OPTIONAL,

fr2-Add-UE-NR-Capabilities-v1540        UE-NR-CapabilityAddFRX-Mode-v1540                             OPTIONAL,

fr1-fr2-Add-UE-NR-Capabilities          UE-NR-CapabilityAddFRX-Mode                                   OPTIONAL,

nonCriticalExtension                    UE-NR-Capability-v1550                                        OPTIONAL

}

UE-NR-Capability-v1550 ::=               SEQUENCE {

reducedCP-Latency                        ENUMERATED {supported}                                       OPTIONAL,

nonCriticalExtension                     UE-NR-Capability-v1560                                       OPTIONAL

}

UE-NR-Capability-v1560 ::=               SEQUENCE {

nrdc-Parameters                         NRDC-Parameters                                               OPTIONAL,

receivedFilters                         OCTET STRING (CONTAINING UECapabilityEnquiry-v1560-IEs)       OPTIONAL,

nonCriticalExtension                    UE-NR-Capability-v1570                                        OPTIONAL

}

UE-NR-Capability-v1570 ::=               SEQUENCE {

nrdc-Parameters-v1570                   NRDC-Parameters-v1570                                         OPTIONAL,

nonCriticalExtension                    UE-NR-Capability-v1610                                        OPTIONAL

}

-- Late non-critical extensions:

UE-NR-Capability-v15c0 ::=               SEQUENCE {

nrdc-Parameters-v15c0                    NRDC-Parameters-v15c0                                        OPTIONAL,

partialFR2-FallbackRX-Req                ENUMERATED {true}                                            OPTIONAL,

nonCriticalExtension                     UE-NR-Capability-v15g0                                       OPTIONAL

}

UE-NR-Capability-v15g0 ::=               SEQUENCE {

rf-Parameters-v15g0                      RF-Parameters-v15g0                                          OPTIONAL,

nonCriticalExtension                     SEQUENCE {}                                                  OPTIONAL

}

-- Regular non-critical extensions:

UE-NR-Capability-v1610 ::=               SEQUENCE {

inDeviceCoexInd-r16                     ENUMERATED {supported}                                        OPTIONAL,

dl-DedicatedMessageSegmentation-r16     ENUMERATED {supported}                                        OPTIONAL,

nrdc-Parameters-v1610                   NRDC-Parameters-v1610                                         OPTIONAL,

powSav-Parameters-r16                   PowSav-Parameters-r16                                         OPTIONAL,

fr1-Add-UE-NR-Capabilities-v1610        UE-NR-CapabilityAddFRX-Mode-v1610                             OPTIONAL,

fr2-Add-UE-NR-Capabilities-v1610        UE-NR-CapabilityAddFRX-Mode-v1610                             OPTIONAL,

bh-RLF-Indication-r16                   ENUMERATED {supported}                                        OPTIONAL,

directSN-AdditionFirstRRC-IAB-r16       ENUMERATED {supported}                                        OPTIONAL,

bap-Parameters-r16                      BAP-Parameters-r16                                            OPTIONAL,

referenceTimeProvision-r16              ENUMERATED {supported}                                        OPTIONAL,

sidelinkParameters-r16                  SidelinkParameters-r16                                        OPTIONAL,

highSpeedParameters-r16                 HighSpeedParameters-r16                                       OPTIONAL,

mac-Parameters-v1610                    MAC-Parameters-v1610                                          OPTIONAL,

mcgRLF-RecoveryViaSCG-r16               ENUMERATED {supported}                                        OPTIONAL,

resumeWithStoredMCG-SCells-r16          ENUMERATED {supported}                                        OPTIONAL,

resumeWithStoredSCG-r16                 ENUMERATED {supported}                                        OPTIONAL,

resumeWithSCG-Config-r16                ENUMERATED {supported}                                        OPTIONAL,

ue-BasedPerfMeas-Parameters-r16         UE-BasedPerfMeas-Parameters-r16                               OPTIONAL,

son-Parameters-r16                      SON-Parameters-r16                                            OPTIONAL,

onDemandSIB-Connected-r16               ENUMERATED {supported}                                        OPTIONAL,

nonCriticalExtension                    UE-NR-Capability-v1640                                        OPTIONAL

}

UE-NR-Capability-v1640 ::=               SEQUENCE {

redirectAtResumeByNAS-r16               ENUMERATED {supported}                                        OPTIONAL,

phy-ParametersSharedSpectrumChAccess-r16  Phy-ParametersSharedSpectrumChAccess-r16                    OPTIONAL,

nonCriticalExtension                    UE-NR-Capability-v1650                                        OPTIONAL

}

UE-NR-Capability-v1650 ::=               SEQUENCE {

mpsPriorityIndication-r16                ENUMERATED {supported}                                       OPTIONAL,

highSpeedParameters-v1650                HighSpeedParameters-v1650                                    OPTIONAL,

nonCriticalExtension                     UE-NR-Capability-v17xx                                                  OPTIONAL

}

UE-NR-Capability-v17xx ::=               SEQUENCE {

powSav-Parameters-v17xx             PowSav-Parameters-v17xx            OPTIONAL,

mac-Parameters-v17xx                MAC-Parameters-v17xx               OPTIONAL,

ims-Parameters-v17xx				IMS-Parameters-v17xx				OPTIONAL,

measAndMobParameters-v17xx			MeasAndMobParameters-v17xx			OPTIONAL,

scalingFactorL2Buffer-r17			ENUMERATED { f0p2, f0p4, f0p6, f0p8 }	OPTIONAL,

nonCriticalExtension                     SEQUENCE {}                    OPTIONAL

}

*editor’s note: The specific values for scalingFactorL2Buffer can be changed following further discussion.

UE-NR-CapabilityAddXDD-Mode ::=         SEQUENCE {

phy-ParametersXDD-Diff                  Phy-ParametersXDD-Diff                                        OPTIONAL,

mac-ParametersXDD-Diff                  MAC-ParametersXDD-Diff                                        OPTIONAL,

measAndMobParametersXDD-Diff            MeasAndMobParametersXDD-Diff                                  OPTIONAL

}

UE-NR-CapabilityAddXDD-Mode-v1530 ::=    SEQUENCE {

eutra-ParametersXDD-Diff                 EUTRA-ParametersXDD-Diff

}

UE-NR-CapabilityAddFRX-Mode ::= SEQUENCE {

phy-ParametersFRX-Diff              Phy-ParametersFRX-Diff                                            OPTIONAL,

measAndMobParametersFRX-Diff        MeasAndMobParametersFRX-Diff                                      OPTIONAL

}

UE-NR-CapabilityAddFRX-Mode-v1540 ::=    SEQUENCE {

ims-ParametersFRX-Diff                   IMS-ParametersFRX-Diff                                       OPTIONAL

}

UE-NR-CapabilityAddFRX-Mode-v1610 ::=    SEQUENCE {

powSav-ParametersFRX-Diff-r16            PowSav-ParametersFRX-Diff-r16                                OPTIONAL,

mac-ParametersFRX-Diff-r16               MAC-ParametersFRX-Diff-r16                                   OPTIONAL

}

BAP-Parameters-r16 ::=                   SEQUENCE {

flowControlBH-RLC-ChannelBased-r16       ENUMERATED {supported}                                       OPTIONAL,

flowControlRouting-ID-Based-r16          ENUMERATED {supported}                                       OPTIONAL

}

-- TAG-UE-NR-CAPABILITY-STOP

-- ASN1STOP

END OF FIRST CHANGE