3GPP TSG-RAN WG2 #118-e	R2-220xxxx

E-meeting, May 2022

Agenda Item:	x.x.x.x

Source:	OPPO

Title:	Summary of [AT118-e][617][Relay] 38351 relay CR (OPPO)

Document for:	Discussion, Decision

# Introduction

This is for the discussion of CR for SRAP

- [AT118-e][617][Relay] 38351 relay CR (OPPO)

Scope: Update the rapporteur CR, incorporating decisions of this meeting and taking into account related proposals in the related tdocs: R2-2204796, R2-2204797, R2-2205133, R2-2205431.

Intended outcome: Agreed CR (without CB if possible)

Deadline:  Wednesday 2022-05-18 0400 UTC

# Phase-1 Discussion

The related papers are summarized as follows

R2-2204632	Correction on SRAP for L2 U2N Relay	OPPO	CR	Rel-17	38.351	17.0.0	0001	-	F	NR_SL_relay-Core

R2-2204796	Correction on BEARER ID determination	ZTE, Sanechips	CR	Rel-17	38.351	17.0.0	0002	-	F	NR_SL_relay-Core

R2-2204797	Correction on the DL bearer mapping	ZTE, Sanechips	CR	Rel-17	38.351	17.0.0	0003	-	F	NR_SL_relay-Core

R2-2205133	Corrections on SRAP PDU handling and ID field determination	ASUSTeK	CR	Rel-17	38.351	17.0.0	0004	-	F	NR_SL_relay-Core

R2-2205431	Correction on the handling of unknown, unforeseen, and erroneous protocol data and other miscellaneous in SRAP	Huawei, HiSilicon	CR	Rel-17	38.351	17.0.0	0005	-	F	NR_SL_relay-Core

# Change by 4796

4796 propose to remove the “drb-Identity minus 1” operation.

Q1: Do you agree with the issue proposed by 4796?

| Company | Yes/No |  | Comments |
| --- | --- | --- | --- |
| OPPO | No |  | If we do not do minus-1 operation, how to carry DRB ID of 32 (max value) using 5-bit? |
| Nokia | No |  | We agree with OPPO that the offset of minus 1 is needed, as the DRB-entity in 38.331 is defined as an integer from 1-32 and therefore is not compatible with 5 bits otherwise |
| Qualcomm |  | No |  |
| vivo |  | No | Same view as OPPO. |
| Sharp |  | No |  |

# Change by 4797

4797 propose to specify the network behaviour, where especially the behaviour for CU-DU split is proposed to be specified in detail (by referring to SRB Mapping Info IE in 38.473).

Q2: Do you agree with the issue proposed by 4797?

| Company | Yes/No | Comments |
| --- | --- | --- |
| OPPO | No | So the first Q is whether 351 aims at network behavior as well – we do not think so (this is different from IAB where the MT function is at network side). |
| Nokia | No | We agree that this is network behaviour, which should not be specified, at least not in 351 |
| Qualcomm | No | Network behavior need not be specified. |
| vivo | No | Cannot see the reason why to specify NW behaviour in Uu. |
| Sharp | No |  |

# Change-2 by 5133

5133 change-2 propose to clarify further the timing when relay UE to perform discard examination for SRB0 message from remote UE.

Q3: Do you agree with the issue proposed by 5133 change-2?

| Company | Yes/No | Comments |
| --- | --- | --- |
| OPPO | No | Not get the point yet, isn’t that so SRB0 is without SRAP header, so not related to error data handling operation? |
| ASUSTeK | Yes | This change is to address the issue that the condition check in sub-clause 5.3.3.1 could fail because there may be no entry in the current sl-RemoteUE-ToAddModList with a sl-L2Identity-Remote matching the Layer-2 ID of the remote UE when an SRAP Data SDU including the RRCSetupRequest message is received from the remote UE. In short, when receiving the SRAP Data SDU including the RRCSetupRequest message, the relay UE should wait for reception of sl-RemoteUE-ToAddModList and then performs the UE/BEARER ID field determination as specified in sub-clause 5.3.3.1. Thus, a NOTE is added to clarify the UE behaviour for this situation. [OPPO] still fail to get the point, isn’t that so the RRCSetupRequest message is of SRB0 so w/o SRAP header, how we do a check on it? [ASUSTeK] The condition in 5.3.3.1 is: - if there is an entry in sl-RemoteUE-ToAddModList, whose sl-L2Identity-Remote matches the Layer-2 ID of the remote UE from which the SRAP Data packet is received: Our understanding is that the condition checks the sl-L2Identity-Remote in sl-RemoteUE-ToAddModList configured to the Relay UE against the Layer-2 ID of the remote UE from which the SRAP Data packet is received. It does not check the SRAP header of the received RRCSetupRequest message. If the condition is satisfied, the Relay UE then determines the UE ID and BEARER ID to be included in the header of the SRAP Data PDU for forwarding to the gNB. |
| Nokia | No | We agree to OPPOs comments. Furthermore, we already have an immense amount of notes in the current version of the spec, where we should strive to provide procedural changed instead of notes to clarify procedures. |
| Qualcomm | Yes | NOTE to clarify the UE behavior is fine |
| vivo | No | Similar view as OPPO. The current Spec says that the UE shall do the related check only when there is an entry matching the L2 ID of the remote. This means, when there isn’t such entry, no such check needs to be performed until there is. |
| Sharp | Yes | We understand the intention is to avoid discarding examination early performed on a new connecting remote UE. And a Note could be nice. |

# Change-1 by 5133 and Change-5 by 5431

The two changes are both for error data handling.

In 5133 change-1 and 5431 change-5-3), it identifies the case that if a RRC_IDLE / RRC_INACTIVE relay UE receives a RRCReconfigurationComplete message from a remote UE during a direct-to-indirect switching, it cannot perform discard examination.

Q4-1: Do you agree with the issue proposed by 5133 change-1 and 5431 change-5-3)?

| Company | Yes/No | Comments |
| --- | --- | --- |
| OPPO | Yes with comment | In fact, this issue is common for the reception of SRB1 message relying on default configuration (i.e., the first SRB1 message), since there is no SRAP configuration before the reception of the concerned SRB1 message. Yet we can further discuss the solution in Phase-2: we are not sure how 5133 change-1 solves this issue, and 5431 change-5-3) seems overshoot it because it rules out all SRB1 message discard examination. |
| ASUSTeK | Yes | The intention of 5133 change-1 is trying not to limit the timing for performing the discard examination to the time “when a SRAP Data PDU is received” so as to prevent the received SRAP Data PDU from being discarded, considering the case that the relay UE in RRC_IDLE/INACTIVE has not obtained the sl-SRAP-Config-Relay associated with the remote UE yet when the SRAP Data PDU including the RRCReconfigurationComplete message is received from the remote UE. Regarding 5431 change-5-3, we share the same view with OPPO that it would be overkill to rule out all SRB1 message in the discard examination. |
| Nokia | No strong view |  |
| Qualcomm | Yes with comments | Agree with issue raised, but do not agree with the way change is supported in 5431 TP. It is better to separate the conditions for remote UE and relay UE for different cases to make them clear. We can further discuss the way to address the changes in phase 2. |
| vivo | Comments | Share the intention. But same view as OPPO, it is not correct to rule out discard examination for all SRB1 messages. Revised changes are needed. |
| Sharp | Yes | We think the issue regarding to 5133 change-1 should be clarified. |

5431 change-5-1) propose to handle control PDU.

Q4-2: Do you agree with the issue proposed by 5431 change-5-1)?

| Company | Yes/No | Comments |
| --- | --- | --- |
| OPPO | No | There is no control PDU in Rel-17. |
| ASUSTeK | No | Since SRAP Control PDU is not used in Rel-17, we think smart UE implementation will handle this situation. |
| Nokia | No | Agree with the above |
| Qualcomm | No | We need not specify as control PDU is not supported in Rel-17 |
| vivo | Yes, and comments | We somewhat share the intention of this change: how Rel-17 UE co-works with future release UEs over the relay link is unknown. So it is not 100% guaranteed for the time being that a Rel-17 will never receive a SRAP PDU setting D/C field to a reserved value. As the safest solution, perhaps it’s better to cover also SRAP PDU discarding with erroneous D/C value. |
| Sharp | No |  |

5431 change-5-2) propose to clarify that in case relay-UE get a data PDU from remote UE1 using ID for remote UE2, it should be discarded as well..

Q4-3: Do you agree with the issue proposed by 5431 change-5-2)?

| Company | Yes/No | Comments |
| --- | --- | --- |
| OPPO | Yes |  |
| ASUSTeK | No | We are fine with the intention, but we think it would be rare case since the mismatch between local ID and L2 ID of Remote UE would occur only when gNB performs local ID update for the Remote UE. |
| Nokia | Yes | No strong view, but we think that the clarification is beneficial, and can be accepted at this stage of the CR |
| Qualcomm | Yes | Same comments as Q4-1 on the format of the changes. |
| vivo | Yes, and comments | Whereas we share the intention and the identified cases as in change 5-2), there seems to be some overlapping between the proposed change and the 1st sentence in the existing paragraph, e.g. whether “mismatching” case can also cover the “not including” case. Perhaps this can be considered during CR producing phase to simplify the descriptions. |
| Sharp | Yes |  |

# Other Changes in 5431 and 4632

Other changes in 5431 and 4632 are mostly editorial.

Q5: Any comment to the change in drafted CR (as a merged version from 4632 and 5431 (other than change-5))?

| Company | Tdoc, Clause | Comments |
| --- | --- | --- |
| OPPO | 5431, clause 5.2.3 | It is preferred to align the usage of packet vs. PDU. Here the description is for remote UE, while packet should be used for relay UE w.r.t the packet forwarding between the two collocated entites, so not suggest to make this change. |

- xxx.

# Conclusion

We have the following proposals:

Proposal 1	xxx.

# Reference

- R2-2204632	Correction on SRAP for L2 U2N Relay	OPPO	CR	Rel-17	38.351	17.0.0	0001	-	F	NR_SL_relay-Core

- R2-2204796	Correction on BEARER ID determination	ZTE, Sanechips	CR	Rel-17	38.351	17.0.0	0002	-	F	NR_SL_relay-Core

- R2-2204797	Correction on the DL bearer mapping	ZTE, Sanechips	CR	Rel-17	38.351	17.0.0	0003	-	F	NR_SL_relay-Core

- R2-2205133	Corrections on SRAP PDU handling and ID field determination	ASUSTeK	CR	Rel-17	38.351	17.0.0	0004	-	F	NR_SL_relay-Core

- R2-2205431	Correction on the handling of unknown, unforeseen, and erroneous protocol data and other miscellaneous in SRAP	Huawei, HiSilicon	CR	Rel-17	38.351	17.0.0	0005	-	F	NR_SL_relay-Core