3GPP TSG RAN WG2#119-e	R2-2206951

e-Meeting, 17th - 29th August, 2022

3GPP TSG-RAN WG4 Meeting #103-e		R4-2210782

Electronic Meeting, May 09 – May 20, 2022

Title:	LS on DC location for intra-band CA

Response to:	-

Release:	Release 17

Work Item:	NR_RF_FR2_req_enh2

Source:	RAN4

To:	RAN2

Cc:	-

Contact person:	Hao Du

duhao.txyjy@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# 1	Overall description

RAN4 has further discussed the remaining issues in DC location reporting and concludes as follows:

Offset range (mentioned in LS R4-2119965)

The maximum offset range is ±300 MHz for FR1 and ±1200 MHz for FR2, and the granularity is the lowest subcarrier spacing used in the CA configuration.

Applicability update

In addition to >2CC case, the R17 DC location reporting scheme is also applicable for 2CCs and single UL carrier within CA. The single UL carrier within CA at least includes the following cases:

- non-CA case in UL (i.e., UL SCCs not exist or removed by network, only DL CA is configured)

- UL CA are configured, but only one UL CC is activated.

Compatibility with R15/R16 reporting scheme

The new reporting scheme is optional and for cases where multiple reporting scheme are applicable and UE supports these capabilities, it is up to the network to decide which scheme will be used.

For a UL CA configuration in one band combination, if multiple DCs exist there is only one scheme will be used for these DC location reporting, for example, only Rel-17 reporting scheme can be used throughout different UL CC groups on the same band.

DL CC(s) clarification

RAN4 would like to inform that when UE is configured for one UL CC but more than one DL CC, the DC location maybe positioned in DL CC and for FR2 UE needs to be able to signal the location of the DC on any configured DL CC.

In addition, for FR2, TX LO cannot locate in the DL only spectrum, and the DL only spectrum here means the spectrum described in TS 38.101-2 clause 5.3A.4 and defined by Fsd according to Table 5.3A.4-3, and DL CC locate in DL-only frequency spectrum will not be used to calculate the default DC location.

CC group for multiple DCs reporting (mentioned in LS R4-2206602)

RAN4 would like to share more details as follows:

- CC groups do not always consist of multiple CCs, but it may consist of single CC.

- Considering R17 spec restriction, limit the CC groups so that each group contains only one UL CC

- In R17, since there are only two UL CCs in case NW deactivate or deconfigure UL CC, there is no second DC location.

Considering the signalling overhead, RAN4 also have some discussions and in some scenarios, network may not always need the DC locations from all the possible permutations, but no further agreement on this issue.

In addition, RAN4 would like to confirm that shift7dot5kHz is still needed for Rel-17 DC location report signalling.

# 2	Actions

To RAN WG2 group

ACTION: 	RAN4 respectfully asks RAN2 to take RAN4 conclusions into consideration.

# 3	Dates of next TSG RAN WG4 meetings

RAN4#104-e	22 Aug 2022- 26 Aug 2022	Electronic

RAN4#104-bis-e	10 Oct 2022 - 19 Oct 2022	Electronic