3GPP TSG RAN2 Meeting #119-e	R2-2207012

# Electronic, 17th – 26th August, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR |  | rev |  | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for DL-PRS processing window activation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-Core |  |  |  |  |  | Date: |  |  | 2022-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Each BWP has its own list of PPWs (BWP-DownlinkDedicated àdl-PPW-PreConfigToAddModList-r17) Issue 1: The scenario is as follows: - UE’s active DL BWP is DL BWP X for serving cell A. - UE receives activation command (PPW Activation/Deactivation Command MAC CE) activating PPW with PPW ID ‘N’ for serving cell A - UE activates PPW with PPWID ‘N’ for DL BWP X of serving cell A - UE receives BWP switching command to switch active DL BWP to BWP Y for serving cell A - UE’s active BWP is switched to DL BWP Y for serving cell A There are two possible UE behavior w.r.t. PPW on new activated DL BWP for serving cell A: - Option 1: if PPWs are configured for DL BWP Y, UE implicitly activates PPW with PPW ID ‘N’ - Option 2: if PPWs are configured for DL BWP Y, UE assumes that none are activated. UE waits for new activation command. It’s not clear whether UE applies option 1 or option 2. UE behavior w.r.t. PPW on deactivated BWP X for serving cell A is also not clear from spec. Simple approach would be to clarify that 'When a BWP is activated, all the PPWs configured on the BWP are considered deactivated.' Issue 2: The scenario is as follows: - UE’s active DL BWP is BWP X for serving cell A - UE receives activation command (PPW Activation/Deactivation Command MAC CE) activating PPW with PPW ID ‘N’ for serving cell A - UE activates PPW with PPWID ‘N’ for BWP X of serving cell A - UE receives RRCReconfiguration message with updated dl-PPW-PreConfigToAddModList-r17 for BWP X There are two possible UE behavior w.r.t. PPW for serving cell A: - Option 1: UE activates PPW with PPWID ‘N’ in the updated list - Option 2: UE assumes that PPW is not activated. UE waits for new activation command Simple approach would be to clarify that 'Upon reconfiguration of PPW(s) of the active BWP, all the PPWs on the BWP are considered deactivated.' |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarified that when a BWP is activated, all the PPWs configured on the BWP are considered deactivated; Upon reconfiguration of PPW(s) of the active BWP, all the PPWs on the BWP are considered deactivated. Impacted functionality: DL-PRS processing window activation Inter-operability: - If the network is implemented according to the CR and the UE is not, there are no interoperability issues. - If the UE is implemented according to the CR and the network is not, there are no interoperability issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Activation of DL-PRS processing window activation upon BWP switching and reconfiguration is unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.18.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

### 5.18.21	PPW Activation/Deactivation Command

If the UE is configured with pre-configured PPW, the network may send DL MAC CE for PPW Activation/Deactivation Command to the UE as in clause 6.1.3.42. For the activated PPW, the UE shall follow the specified UE behaviour in clause 5.24.

Upon activation of DL BWP, the PPW(s) configured for that BWP are considered deactivated. Upon reconfiguration of PPW(s) of the active DL BWP, all the PPW(s) for that BWP are considered deactivated.

Upon the reception of the MAC CE for PPW Activation/Deactivation Command, the MAC entity shall:

1>	if the DL MAC CE for PPW Activation/Deactivation Command indicates the deactivation of a pre-configured PPW:

2>	deactivate the PPW.

1>	else if the DL MAC CE for PPW Activation/Deactivation Command indicates the activation of a pre-configured PPW:

2>	activate the PPW according to the procedure specified in clause 5.24.

<End of Change 1>