3GPP TSG-RAN WG2 Meeting #119-e	R2-2207025

# Online, August 17th – 29th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | 4828 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on SCG failure information procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ITRI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2022-08-05 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - The general description of the purpose of SCG failure information procedure in clause 5.6.13.1 should cover all cases handled by the procedure. Based on clause 5.6.13.2, NE-DC is one of the cases and, in this case, the UE transmits the SCG failure information message via the NR MCG as specified in TS 38.331 clause 5.7.3a. TS 38.331 clause 5.7.3a.1 also specifies that NR MN is informed about the SCG failure on E-UTRA SN, in case of NE-DC. However, the consistency is missing in the general purpose description in clause 5.6.13.1 of TS 36.331. More specifically, the general description lacks the purpose of informing NR MN about the SCG failure on E-UTRA SN, i.e. the case of NE-DC is not covered by the general description. Without fixing the error, inconsistency and thus ambiguity exist. - Similarly, Figure 5.6.13.1-1 which illustrates signalling between the UE and the network in clause 5.6.13.1 of TS 36.331 should be able to cover the case of NE-DC where NR MN is informed about the SCG failure on E-UTRA SN reported by the UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Fix the above-mentioned inconsistent error in the general description of clause 5.6.13.1, TS 36.331, by adding the missing “or NR MN”. - Change the “EUTRAN” block to “Network” block in Figure 5.6.13.1-1. Impact analysis Impacted 5G architecture options: NE-DC Impacted functionality: SCG failure information reporting Inter-operability: If the UE is implemented according to this CR while the network is not, there is no interoperability issue. If the network is implemented according to this CR while the UE is not, interoperability issue exists as there is ambiguity about which network node should be informed of the SCG failure if the UE is configured with NE-DC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The purpose described in clause 5.6.13.1 is inconsistent with what the procedure actually does, which may cause ambiguity. - The figure shown in clause 5.6.13.1 for general description cannot cover all cases handled in the procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.13.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

5.6.13	SCG failure information

5.6.13.1	General

Figure 5.6.13.1-1: SCG failure information

The purpose of this procedure is to inform E-UTRAN or NR MN about an SCG failure the UE has experienced i.e. SCG radio link failure, SCG change failure.

END OF CHANGE