3GPP TSG-RAN WG2 Meeting #119                                                      R2-2207027

Online, 15 Augest–26 Augest 2022

Source:	Qualcomm Incorporated

Title:	Discussion on Rel-17 leftover issues

Agenda Item:	8.14 QoE

Document for:	Discussion and Decision

# Introduction & Background

Rel-18 QoE includes one objective to continue discuss Rel-17 left-over issues as follow.

- Left-over features from Rel-17, as well as the enhancements of existing features which are not included in Rel-17 normative phase, should be supported in Rel-18 if consensus on benefits are reached [RAN3, RAN2].

- Specify per-slice QoE measurement configuration enhancement.

- Specify RAN visible QoE enhancements for QoE value, RAN visible QoE trigger event, RAN visible QoE Report over F1.

- Specify QoE reporting handling enhancement for overload scenario.

This contribution discusses RAN2 part for Rel-17 left-over issues.

# Discussion

2.1 Per-slice QoE measurement configuration enhancement

Per-slice based QoE configuration is intended to request UE performing QoE measurements only when the slice is matched with the mapped slice in application layer. In Rel-17, PDU Session list is added in the QoE measurement report message. Based on the PDU Session and the associated slicing in gNB, the gNB can derive the slicing information for the received QoE measurements.

In existing specification, gNB only knows the associated slice information for the activated PDU Sessions, but does not know the slice information for each application or each service type of application. Hence, the gNB cannot filter which QoE measurements should be performed by UE for the required slice(s). For example, PDU Session 1 is activated for slice 1, and PDU Session 2 is activated for slice 2. In UE side, the application for VR service type is mapped to slice 2 with PDU Session 2, and application for DASH streaming is mapped to slice 1 with PDU Session 1. If QoE Reference #1 and #2 are configured by MCE and the QoE Reference #1 is required for VR service type for slice 1, and QoE Reference #2 is required for DASH streaming for slice 2, then UE do not need to perform QoE measurements. But in gNB side, there is no application (service type) and slice mapping, so the gNB will provide both QoE Reference #1 and #2 configurations to UE, then if no UE slice filtering, the UE will perform QoE collection for both QoE References.

Observation 1: In current specification, if leveraging on gNB slice filtering, the gNB can only configure QoE configuration for the the slice(s) with activated radio resource.

Observation 2: The gNB does not have slice information for each application or each service type of application. Hence, the gNB cannot filter out which QoE measurements should be performed by UE for the required slice(s).

Proposal 1: UE based slice filtering for QoE measurement is needed to support per-slice based QoE configuration and measurement collection.

Per-slice container QoE configuration

The most easy way is to request application to check whether the associated slice is matched with the QoE required slice. The MCE server includes the required slice information into the QoE configuration container. When application layer receives the QoE configuration container, the application layer will determine whether to perform QoE measurement based on the requested slice for the QoE. If the slice is matched with the slice the application is mapped, then performs QoE measurements.

This alternative can be used in multiple scenarios, e.g. a PDU session is activated for the application, or a PDU session is deactivated for the application. Since the MCE server does not know whether PDU sessions associated with the requested slice(s) are activated for the application or not, this provides the possibility to MCE server configuring QoE meausrements to UE application layer when there are no PDU Sessions activated for the requested slice(s).

Proposal 2: For container based QoE configuration, the required slice information is included in QoE configuration container, and application layer determines whether to perform QoE measurement based on the QoE required slice information.

Per-slice RVQoE configuration

As discussed in container based QoE configuration, gNB needs to configure the required slice(s) to the UE over RRC layer, and UE RRC layer forwards the required slice(s) information to application layer.

Slice information can be S-NSSAI or PDU Session ID, from AS layer and gNB side, both of the S-NSSAI and PDU Session ID is feasible, from application layer, S-NSSAI is more straight-forward information to be used. Therefore, it is proposed to include slice information into RVQoE configuration message.

Proposal 3: For RVQoE configuration, the required slice information is included into RVQoE configuration, and RRC layer forwards to application layer.

Proposal 4: Which slice information (S-NSSAI or PDU Session ID) is used can be confirmed by SA4.

2.2 RAN visible QoE enhancements

This bullet is to specify RAN visible QoE enhancements for QoE value, RAN visible QoE trigger event, RAN visible QoE Report over F1. Within these enhancements, QoE value were discussed in RAN3 in Rel-17, so it is expected to wait for RAN3 discussion. For RAN visible QoE trigger event, RAN2 can discuss the definition of trigger event, the executation layer (i.e. RRC layer or application layer).

Proposal 5: For RVQoE value, RAN2 waits for RAN3 discussion; for RAN visible QoE trigger event, RAN2 should discuss the definition of trigger event, the executation layer (i.e. RRC layer or application layer), how to configure, how to report etc.

2.3 QoE reporting enhancement for overload scenario

During RAN planery discussion, the proponent of this bullet proposed to provide QoE configuration priority to gNB and gNB can select lower priority QoE configuration to be paused. One additional optimization is to provide the priority to UE as well, then UE can buffer the higher priority QoE data when the buffer size is limited.

Proposal 6: RAN2 is proposed to discuss whether the QoE configuration priority can be configured to UE as well, then UE can buffer higher priority QoE measurement when QoE reports are paused.

# Conclusion

This contribution discusses the RAN2 part for Rel-17 left-over issues, and provides the following proposals.

Observation 1: In current specification, if leveraging on gNB slice filtering, the gNB can only configure QoE configuration for the the slice(s) with activated radio resource.

Observation 2: The gNB does not have slice information for each application or each service type of application. Hence, the gNB cannot filter out which QoE measurements should be performed by UE for the required slice(s).

Proposal 1: UE based slice filtering for QoE measurement is needed to support per-slice based QoE configuration and measurement collection.

Proposal 2: For container based QoE configuration, the required slice information is included in QoE configuration container, and application layer determines whether to perform QoE measurement based on the QoE required slice information.

Proposal 3: For RVQoE configuration, the required slice information is included into RVQoE configuration, and RRC layer forwards to application layer.

Proposal 4: Which slice information (S-NSSAI or PDU Session ID) is used can be confirmed by SA4.

Proposal 5: For RVQoE value, RAN2 waits for RAN3 discussion; for RAN visible QoE trigger event, RAN2 should discuss the definition of trigger event, the executation layer (i.e. RRC layer or application layer), how to configure, how to report etc.

Proposal 6: RAN2 is proposed to discuss whether the QoE configuration priority can be configured to UE as well, then UE can buffer higher priority QoE measurement when QoE reports are paused.

# Reference

[1]    RP-221803: Enhancement on NR QoE management and optimizations for diverse services