3GPP TSG-RAN WG2 #119-e	R2-2207029

E-meeting, August 2022

Agenda Item:	6.15.4

Source:	OPPO

Title:	Discussion on left issue on user plane procedure

Document for:	Discussion, Decision

Introduction

This paper will discuss some left issues on user plane for both SL DRX and inter-UE coordination from RAN2 #118 meeting.

Discussion

SL DRX

In R2 #118 meeting, regarding DRX, the following issues have been left for further discussion due to lack of time:

For SL impact to Uu DRX,

(8, 5) Proposal 7: RAN2 should discuss whether to agree or disagree with proposal 2 (“Capture in MAC spec, when the PUCCH resource is configured, the start time of drx-HARQ-RTT-TimerSL for configured sidelink grant reuses that for dynamic sidelink grant.”) of R2-2204865. (5, 7) Proposal 8: RAN2 discuss whether to agree or disagree with proposal 3 (“Capture in MAC spec, when the PUCCH resource is not configured, start the drx-HARQ-RTT-TimerSL for the corresponding HARQ process at the first symbol after end of PSSCH occasion for configured sidelink grant.”) in the R2-2204865.

For SL RTT timer,

Proposal 3b RAN2 to discuss RTT timer for pools without PSFCH in the next meeting, via e.g., 1) configured time length, 2) fixing it to 0 without a configured timer.

For SL inactivity timer,

(6, 5) Proposal 20. RAN2 is not to agree on correction for relocating the down-selection of inactivity timer of groupcast in the R2-2204574.

For SR configuration of SL DRX Command MAC CE,

Proposal 5a RAN2 aims at down-selection of SR configuration for SL DRX Command MAC CE between the following 2 options in next meeting: • Option 1: Reuse the SR configuration for SL logical channel, and which logical channel triggered a Scheduling Request for SL DRX command indication is up to UE implementation. • Option 2: Reuse the SR configuration for CSI report.

For the duplicated text in MAC spec

(7, 5) Proposal 14: RAN2 is not to agree on correction 2 (“In section 5.22.1.1, remove the text “5> if selected resource for initial transmission occasion is not in the SL DRX Active time as specified in clause 5.28.1 of any destination that has data to be sent: 6> use retransmission occasion(s) for initial transmission of PSCCH and PSSCH.”;”) in the R2-2204574 (3, 10) Proposal 16: RAN2 is not to agree on correction 4 (“In section 5.22.1.3.1, remove the text “2> if all PSCCH duration(s) and PSSCH duration(s) for initial transmission …: 3> ignore the sidelink grant.” in 5.22.1.3.1.”) in the R2-2204574

The above left issues will be discussed in the following sub-clauses.

Left issue1: SL impact to Uu DRX

The issue is generally about the handling of drx-RetransmissionTimerSL/drx-HARQ-RTT-TimerSL for CG resources (in case PUCCH is configured or not configured). To answer this question with the observation of companies’ replies in offline [707] of the last meeting,

- Firstly, one should consider whether Uu DRX (drx-RetransmissionTimerSL/drx-HARQ-RTT-TimerSL) for SL CG resource is needed or not;

- Then (if needed), whether the current specification already supports DRX for SL CG resources;

- Last (if not supported yet), is the detailed timer handling.

For the need of Uu DRX for SL CG resources, considering

- RTT timer and retransmission timer are supported for UL CG resource;

- gNB can/may schedule retransmission resources between SL CG resources.

So based on the above reasons, by following the UL principle and considering the actual need for SL, the Uu DRX (drx-RetransmissionTimerSL/drx-HARQ-RTT-TimerSL) for SL CG resource is needed.

By following UL principle and considering the actual need for SL, the drx-RetransmissionTimerSL/drx-HARQ-RTT-TimerSL for SL CG resource is needed.

Then for the current specification, some companies are considering using the following existing text to cover the support of CG resource

1> if a DRX group is in Active Time: <omitted> 2> if the PDCCH indicates an SL transmission: 3> if the PUCCH resource is configured: 4> start the drx-HARQ-RTT-TimerSL for the corresponding HARQ process in the first symbol after the end of the corresponding PUCCH transmission carrying the SL HARQ feedback; or 4> start the drx-HARQ-RTT-TimerSL for the corresponding HARQ process in the first symbol after the end of the corresponding PUCCH resource for the SL HARQ feedback when the PUCCH is not transmitted due to UL/SL prioritization; 4> stop the drx-RetransmissionTimerSL for the corresponding HARQ process. 3> else: 4> start the drx-HARQ-RTT-TimerSL for the corresponding HARQ process at the first symbol after end of PDCCH occasion; 4> stop the drx-RetransmissionTimerSL for the corresponding HARQ process.

In our understanding, the above text cannot be used for CG resource, since

- By following the UL CG resource principle, the use of RTT/Retransmission timer in CG case should not be restricted by “in Active time”;

- Another issue is in CG scheduling,

  - For Type-1 CG, there is no “PDCCH indicates an SL transmission”;

  - For Type-2 CG, although there is a PDCCH indicates the activation of CG resource, the PDCCH is at the beginning of the CG grant, which means gNB can only schedule retransmission at the first SL CG resource, which is an unreasonable restriction to gNB;

Therefore, the current specification (as above text) cannot cover CG cases.

The current specification cannot support the RTT/Retransmission timer for SL CG resources.

Finally, the issue is how to support the RTT/Retransmission timer for SL CG resources, i.e., the detailed timer handling.

Firstly, for the case when PUCCH is configured, it is much more straightforward by following the agreement for the DG resource, i.e., the RTT timer is started after the PUCCH resource.

While for the case when PUCCH is not configured, 1)the agreement for DG is not feasible anymore since there may be no PDCCH resource; 2)In UL, the RTT timer is started after the PUSCH transmission. So to mimic UL timer handling, the feasible solution is that UE starts the RTT timer after PSSCH transmission.

For configured SL grant, when the PUCCH resource is configured, start drx-HARQ-RTT-TimerSL for configured sidelink grant at the first symbol after the end of the corresponding PUCCH resource.

For configured SL grant, when the PUCCH resource is not configured, start the drx-HARQ-RTT-TimerSL for the corresponding HARQ process at the first symbol after the end of PSSCH occasion for configured sidelink grant.

Left issue2: RTT timer value in case no PSFCH

In the last RAN2 meeting, the following agreement leave one issue to be discussed

Proposal 3b RAN2 to discuss RTT timer for pools without PSFCH in the next meeting, via e.g., 1) configured time length, 2) fixing it to 0 without a configured timer.

For the 2 options, both are feasible but the configurable value is preferred since,

- Option 1 (configured value) can cover Option 2 (fixed 0) but is more future proof, in case a non-zero timer is to be introduced later, and can achieve more power-saving gain (e.g., there is a great possibility that the retransmission is not happened immediately after the previous transmission, i.e., there is a time gap between 2 adjacent transmissions, especially in mode-1 scheduling);

- Option 1 is also aligned with the other 2 cases (HARQ feedback enable and HARQ feedback disable W/ PSFCH), i.e., a unified behavior can be adopted in MAC spec, while Option 2 needs additional MAC spec impact to explain the delta case, i.e., fixed (instead of configured) RTT timer value for resource pool without PSFCH.

So we still see the benefits of configured RTT timer in case there is no PSFCH configured in the resource pool.

Rely on configured RTT timer length for resource pools without PSFCH as for the other cases.

Left issue3: Groupcast inactivity-timer

RAN2 has discussed the correction for relocating the down-selection of inactivity timer of groupcast in the last meeting. In the current specification (section 5.28.2 of 38.321), different places are used for down-selection of cycle/on-duration timer length and inactivity timer length,

When one or multiple SL DRX is configured, the MAC entity shall: 1> if multiple SL DRX Cycles that are mapped with multiple SL-QoS-Profiles of a Destination Layer-2 ID and interested cast type is associated to groupcast or broadcast: 2> select sl-drx-Cycle whose length of the sl-drx-cycle is the shortest one among multiple SL DRX Cycles that are mapped with multiple SL-QoS-Profiles associated with the Destination Layer-2 ID: 2> select sl-drx-onDurationTimer whose length of the sl-drx-onDurationTimer is the longest one among multiple SL DRX onDuration timers that are mapped with multiple SL-QoS-Profiles associated with the Destination Layer-2 ID. 1> if an sl-drx-HARQ-RTT-Timer expires: 2> if the data of the corresponding Sidelink process was not successfully decoded or if the HARQ feedback (i.e., negative acknowledgement) is not transmitted for unicast due to UL/SL prioritization: 3> start the sl-drx-RetransmissionTimer for the corresponding Sidelink process in the first slot after the expiry of sl-drx-HARQ-RTT-Timer. When the cast type is groupcast or broadcast as indicated by upper layer, the sl-drx-StartOffset and sl-drx-SlotOffset are derived from the following equations: sl-drx-StartOffset (ms) = Destination Layer-2 ID modulo sl-drx-Cycle (ms). sl-drx-SlotOffset (ms) = Destination Layer-2 ID modulo the number of slots in one subframe (ms). 1> if the SL DRX cycle is used, and [(DFN × 10) + subframe number] modulo (sl-drx-Cycle) = sl-drx-StartOffset: 2> start sl-drx-onDurationTimer after sl-drx-SlotOffset from the beginning of the subframe. 1> if an SL DRX is in Active Time: 2> monitor the SCI (i.e., 1st stage SCI and 2nd stage SCI) in this SL DRX. 2> if the SCI indicates a new SL transmission: 3> if Source Layer-1 ID of the SCI is equal to the 8 LSB of the intended Destination Layer-2 ID and Destination Layer-1 ID of the SCI is equal to the 16 LSB of the intended Source Layer-2 ID and the cast type indicator in the SCI is set to unicast: 4> start or restart sl-drx-InactivityTimer for the corresponding Source Layer-2 ID and Destination Layer-2 ID pair after the first slot of SCI reception. 3> if Destination Layer-1 ID of the SCI (i.e., 2nd stage SCI) is equal to the 16 LSB of the intended Destination Layer-1 ID and the cast type indicator in the SCI is set to groupcast: 4> select sl-drx-InactivityTimer whose length of the sl-drx-InactivityTimer is the largest one among multiple SL DRX Inactivity timers that are mapped to multiple SL-QoS-Profiles of Destination Layer-2 ID associated with the Destination Layer-1 ID of the SCI; and 4> start or restart sl-drx-InactivityTimer for the corresponding Destination Layer-2 ID after the first slot of SCI reception.

In R2-2204574, it is proposed to relocate the selection of inactivity timer to keep the alignment between cycle/on duration timer and inactivity timer (i.e., relocate the down-selection of inactivity timer together with cycle and on_duration timer). More than half of companies are in favor of this correction and one company want to have more time to check the details, so this issue has been left as an issue to be solved. So in this paper, the need for this correction is further elaborated.

For the down-selection of inactivity timer in groupcast, R2 have made the following agreement

For GC, when performing the down-selection of the inactivity timer, select the inactivity timer whose inactivity timer length is the largest one (among multiple ones for the corresponding L2 id) as the selected inactivity timer.

According to the above agreement, the down-selection of the inactivity-timer should be based on L2 ID which is exactly the same as we have agreed for on-duration timer and DRX cycle.

And the down-selection of inactivity-timer does not have to be performed upon SCI reception, i.e., as for the selection of cycle and on-duration timer which is performed before SCI reception, there seems no reason to differentiate between cycle/on-duration timer length setting and inactivity timer length setting.

Therefore, we see no reason to have different UE behaviors on the down-selection of inactivity timer compared with on-duration/cycle, i.e., no reason to couple the down-selection procedure and the SCI reception procedure. So, it proposes to align the down-selection of inactivity timer of groupcast with the down-selection of cycle/on-duration timer.

RAN2 agrees on correction for aligning down-selection of inactivity timer length with the down-selection of cycle/on-duration timer length.

Left issue4: SR for DRX command MAC-CE

Another left issue is the SR configuration for DRX command MAC CE, the following 2 options are agreed for further down-selection in the last RAN2 meeting:

•	Option 1: Reuse the SR configuration for SL logical channel, and which logical channel triggered a Scheduling Request for SL DRX command indication is up to UE implementation.

•	Option 2: Reuse the SR configuration for CSI report.

Both the 2 options are workable, while Option 1 relies on the SR for LCH by UE implementation, which leads to the difficulty by network to figure out whether the received SR is for data or for MAC-CE, especially considering data LCHs have various QoS requirements, may or may not align with QoS requirement (e.g., PDB) of DRX command-CE. So Option 2 is preferred, i.e., at least the two MAC-CE can put similar PDB requirement to network scheduler.

SR configuration for SL DRX Command MAC CE reuses the SR configuration for CSI report.

Left issue5: Duplicated/redundant text in MAC spec

Firstly, one left issue is in R2-2204574, which proposed the deletion of the following text (similar change in multiple places)

Because

- the LCP ensures the MAC PDU can only be assembled when the destination is in SL active time;

- and if there is no MAC PDU obtained for initial transmission (e.g., the destination is not in SL active time), UE can associate a new sidelink process to this grant which allows initial transmission in re-transmission occasions

The main argument from the opponent of this change is even after the MAC PDU generation (after LCP), the initial transmission occasion can be reselected due to pre-emption/re-evaluation which may result in “initial transmission occasion is not in the SL DRX Active time”.

Firstly, we fail to understand the logic of the argument from opponent since the text is in-between the resource selection procedure, which is before LCP procedure, it is meaningless since actually UE can only judge “… in the SL DRX Active time … of any destination that has data to be sent” assuredly during LCP afterwards. I.e., the active time determination at resource selection procedure is not reliable and it may change when LCP is performed.

We understand besides the above logical issue of the argument, even if the above case (initial transmission occasion is reselected due to pre-emption) happens, the text should also be removed as in the following figure:

Figure 1. The transmission occasion to be used for initial transmission in different cases

As seen in the above figure,

- In the upper case, where no MAC PDU is generated (due to the active time restriction in LCP) for the first grant X1 (which was supposed to be used for initial transmission), following the current spec, the UE will use transmission occasion X2 for the initial transmission;

- In the lower case, the MAC PDU has been generated for grant X1, yet it is finally abandoned due to the pre-emption after LCP), the UE will use transmission occasion X1’(assume the re-selected resource X1’ is before X2) for the initial transmission;

Therefore, even if the lower case exists, as raised by the opponent, if the previous “initial” transmission occasion is dropped, no matter due to pre-emption or the LCP restriction, the UE will use the immediately next transmission occasion (the re-selected or the “retransmission” occasion) for the transmission in its buffer, i.e., no additional spec impact is needed.

Based on the current spec, UE is to use the next transmission occasion for the transmission of newly generated MAC PDU in its buffer, i.e., no additional spec impact is needed.

Actually, the definition of “initial/re-” transmission occasion is not very clear between MAC and PHY layer, and even not clear in MAC specification during resource selection procedure (the initial/retransmission occasion is defined after the occasions have been selected).

Therefore,

- The text (if …use retransmission occasion…) does not provide additional clarification since as analyzed above, following the current spec, the UE can already use the immediately next available transmission occasion for the transmission of the MAC PDU in its buffer;

- The text instead introduces more confusion due to the different understanding of the “initial/retransmission” occasion and the logical issue of how the UE judge the “Active time” during resource selection procedure.

Due to the above reasons, the text “5> if selected resource for initial transmission occasion is not in the SL DRX Active time as specified in clause 5.28.1 of any destination that has data to be sent: 6> use retransmission occasion(s) for initial transmission of PSCCH and PSSCH.” should be removed in MAC specification.

RAN2 agrees on removing the text “5> if selected resource for initial transmission occasion is not in the SL DRX Active time as specified in clause 5.28.1 of any destination that has data to be sent: 6> use retransmission occasion(s) for initial transmission of PSCCH and PSSCH. ” in 5.22.1.1 of 38.321.

Secondly, besides the above issue, another change proposed in R2-2204574 is

“In section 5.22.1.3.1, remove the text “2> if all PSCCH duration(s) and PSSCH duration(s) for initial transmission …: 3>	ignore the sidelink grant.” in 5.22.1.3.1.”

1> if the sidelink grant is a dynamic sidelink grant or selected sidelink grant and no MAC PDU has been obtained in the previous sidelink grant when PSCCH duration(s) and 2nd stage SCI on PSSCH of the previous sidelink grant is not in SL DRX Active time as specified in clause 5.28.3 of any destination that has data to be sent: NOTE 1: Void. 2> (re-)associate a Sidelink process to this grant, and for the associated Sidelink process: 2> if all PSCCH duration(s) and PSSCH duration(s) for initial transmission of a MAC PDU of the dynamic sidelink grant or the configured sidelink grant is not in SL DRX Active time as specified in clause 5.28.3 of the destination that has data to be sent: 3> ignore the sidelink grant. NOTE 1A: The Sidelink HARQ Entity will associate the selected sidelink grant to the Sidelink process determined by the MAC entity. 3> obtain the MAC PDU to transmit from the Multiplexing and assembly entity, if any; 3> if a MAC PDU to transmit has been obtained: 4> if a HARQ Process ID has been set for the sidelink grant: 5> (re-)associate the HARQ Process ID corresponding to the sidelink grant to the Sidelink process. NOTE 1a: There is one-to-one mapping between a HARQ Process ID and a Sidelink process in the MAC entity configured with Sidelink resource allocation mode 1. 4> determines Sidelink transmission information of the TB for the source and destination pair of the MAC PDU as follows: 5> … 4> deliver the MAC PDU, the sidelink grant and the Sidelink transmission information of the TB to the associated Sidelink process; 4> instruct the associated Sidelink process to trigger a new transmission. 3> else: 4> flush the HARQ buffer of the associated Sidelink process.

According to the above text from the current specification,

- As we explained in the last RAN2 meeting, this is a duplicated text, i.e., the ignoring behavior is just the result of the LCP procedure, i.e., when LCP procedure fails to find an available destination of which the DRX is at active time, no PDU would be obtained from the Multiplexing and assembly entity, and the following UE behavior (associate HARQ Process ID, determine the sidelink transmission information…) will not be performed, the UE would flush the HARQ buffer, which already means the grant is ignored,

- in R16, we also specified the case (other than due to SL DRX) where the MAC PDU may not be obtained, and we use this “flush the HARQ buffer” behavior for the “ignoring SL grant” purpose. Our understanding is “flush the HARQ buffer” = “ignore the SL grant” for the initial transmission. So in R17, the “ignore” behavior is not aligned with the R16 design, duplicate the buffer flushing step, and does not bring additional information;

- if we keep the text (ignore SL grant) as it is, yet considering the following level-3 steps are to be performed subsequently after the grant-ignoring, it means the UE will still obtain MAC PDU although the SL grant is ignored, and the UE still need to determine whether the transmission should be performed based on the LCP filtering, which means the “ignore SL grant” behavior is useless.

The “ignore SL grant” in 5.22.1.3.1 is not only duplicated but also harmful, i.e., the UE behavior is to generate the MAC PDU for an ignored grant.

So to keep the specification more readable and refined, we propose to remove the related text in the specification.

RAN2 agrees on the correction of removing the text “2>	if all PSCCH duration(s) and PSSCH duration(s) for initial transmission of a MAC PDU of the dynamic sidelink grant or the configured sidelink grant is not in SL DRX Active time as specified in clause 5.28.3 of the destination that has data to be sent: 3>	ignore the sidelink grant.” in 5.22.1.3.1 of 38.321.

IUC

In R2 #118 meeting, regarding IUC, the following issues have been left for further discussion due to lack of time:

Resource pool selection for IUC-info/IUC request message,

Proposal 8(18/18): Tx resource pool selection shall take the transmission of request MAC CE/IUC MAC CE into consideration. FFS to implement this as normative text or NOTE in the MAC spec. Proposal 9(12/16): It is up to PHY layer of UE B to ensure IUC scheme 2 occurs in the right resource pool . FFS a LS to RAN1 is needed to confirm this.

UE-B’s behaviour for multiple preferred resource sets,

Proposal 4: Destination selection step within the LCP needs to consider the IUC information used for sensing/resource selection. For example, PHY of UE-B indicates to MAC layer, that IUC information from UE-A was taking into account during the sensing/resource selection procedure. Correspondingly MAC would set the destination to the destination ID of UE-A while performing the LCP procedure.

The above left issues will be discussed in the following sub-clauses.

Left issue1: resource pool selection for IUC/ IUC REQ message

For the resource pool selection for IUC-info/IUC REQ message, both scheme 1 and scheme 2 have been discussed in last meeting.

Firstly, some clarification is needed regarding the resource pool selection for IUC-info and IUC-request message since this is related to whether the UE performs resource pool selection first or decides on sending IUC-info/IUC REQ message first. There are some RAN1 agreements related to this issue:

Agreement A resource pool level (pre-)configuration can enable one of the following options: - Option 1: - For Condition 2-A-1 of Scheme 2, support following additional criteria to determine resource(s) where expected/potential resource conflict occurs - For the case when UE-A is a destination UE of a TB transmitted by UE-B - The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) whose RSRP measurement is larger than a RSRP threshold according to the priorities included in the SCI: - prio_TX and prio_RX are the priorities indicated in the SCI making the overlapping reservations for UE-B and other UE respectively - For the case when UE-A is a destination UE of a TB transmitted by another UE - The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) when RSRP measurement of UE-B’s reserved resource is larger than a RSRP threshold according to the priorities included in the SCI: - prio_TX and prio_RX are the priorities indicated in the SCI making the overlapping reservations for other UE and UE-B respectively - Option 4: - For Condition 2-A-1 of Scheme 2, support following additional criteria to determine resource(s) where expected/potential resource conflict occurs - For the case when UE-A is a destination UE of a TB transmitted by UE-B - The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) whose RSRP measurement is larger than a (pre)configured RSRP threshold compared to the RSRP measurement of UE-B’s reserved resource. - For the case when UE-A is a destination UE of a TB transmitted by another UE - The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) when RSRP measurement of UE-B’s reserved resource is larger than a (pre)configured RSRP threshold compared to the RSRP measurement of the resource(s). - Support of Option 4 is subject to UE capability - FFS: Whether/how RSRP threshold depends on priority, MCS, overlap Agreement For inter-UE coordination triggered by a condition rather than request reception in Scheme 1, - A resource pool level (pre-)configuration can enable one of the following alternatives: - Alt 1: it is up to UE-A’s implementation whether or not to trigger the inter-UE coordination information generation. - Alt 2: the inter-UE coordination information generation can be triggered only when UE-A has data to be transmitted together with the inter-UE coordination information to UE-B - Note: Rel-16 procedure of UL/SL prioritization, LTE SL/NR SL prioritization, and congestion control is applied to the transmission of the inter-UE coordination information triggered by a condition. Agreement For inter-UE coordination triggered by UE-B’s explicit request in Scheme 1, - A resource pool level (pre-)configuration can enable one of the following alternatives: - Alt 1: it is up to UE-B’s implementation whether or not to trigger the request generation - Alt 2: the request generation can be triggered only when UE-B has data to be transmitted to UE-A - Note: Rel-16 procedure of UL/SL prioritization, LTE SL/NR SL prioritization, and congestion control is applied to the transmission of the request transmission.

As seen in the above agreements, no matter in scheme 1 or scheme 2, the UE determines whether to send IUC-info/IUC-REQ messages based on the resource pool level configuration, which means UE performs resource pool selection first and then decides to send IUC-info /IUC-REQ if the IUC-info /IUC-REQ trigger condition configured in the resource pool is fulfilled. R2 should follow R1 agreements on this issue.

According to RAN1 agreements, for both scheme 1 and scheme 2, UE determines whether to send IUC-info /IUC REQ message based on the resource pool level configuration.

Then for scheme 1, we have agreed on Tx resource pool selection shall take the transmission of request MAC CE/IUC-info MAC CE into consideration, but leave the way for capturing this (i.e., whether NOTE or normative text) as FFS. Firstly, as above observation from the RAN1 agreement, we still think the resource pool selection procedure should not be impacted by IUC-info /IUC-REQ MAC CE transmission since it happens before the IUC-info /IUC REQ is triggered. And we can try to find some way to capture the related RAN2 agreement as a NOTE which is not against RAN1 agreement for compromise.

RAN2 not pursue normative text for Scheme-1 IUC-based TX resource pool selection.

For the detailed wording of the NOTE, by following the similar wording of the existing NOTE1 in 5.22.1.1 and trying to express the spirit of both RAN1/RAN2 agreements, we propose the following suggestion:

NOTE x: The MAC entity can create a selected sidelink grant for IUC-information or IUC-request MAC CE based on the corresponding IUC-information and/or IUC-request MAC CE trigger condition configuration, if any, in the resource pool.

RAN2 discuss capturing the NOTE as “The MAC entity can create a selected sidelink grant for IUC-information or IUC-request MAC CE based on the corresponding IUC-information and/or IUC-request MAC CE trigger condition configuration, if any, in the resource pool”.

For scheme 2, RAN2 has a majority view on it is a RAN1 issue, i.e., up to PHY layer to ensure IUC scheme 2 occurs in the right resource pool, but cannot make a consensus on whether to send LS to RAN1. As the above RAN1 agreement clearly states that “A resource pool level (pre-)configuration” is used for the UE to determine whether the IUC is needed or not, we don’t see the need in RAN2 to discuss this issue again, and also the need for the LS to RAN1.

No RAN2 discussion on resource pool selection for scheme 2 IUC is needed.

Left issue2: UE-B’s behavior for multiple preferred resource sets

For UE-B’s behavior to multiple preferred resource sets from different UE-As, RAN1 has the following agreement

Agreement - For UE-B’s behavior when UE-B receives multiple preferred resource sets from the different UE-As, - UE-B uses each received preferred resource set for its resource selection for each TB to be transmitted to each UE-A providing the preferred resource set. - Conclusion: UE-B’s behavior when UE-B receives multiple non-preferred resource sets from the different UE-As. - No RAN1 specification change to TS38.214 is deemed necessary in RAN1#108-e (except for the processing timeline) - For UE-B’s behavior when UE-B receives both a single preferred resource set and a single non-preferred resource set from the different UE-As, - FFS: It is up to UE-B implementation to use one or multiple of them in its resource (re)selection

RAN2 has discussed how to capture this agreement in last RAN2 meeting but no conclusion has been made.

There are 3 solutions for handling the above agreement which has been discussed,

- Change LCP restriction, i.e., add IUC-info as an additional LCP restriction;

- Send LS to RAN1 to reconsider the above agreement;

- Use a similar solution as what was decided for SL DRX.

Firstly, using LCP restriction to capture the RAN1 agreement is not preferred since,

- The RAN1 agreement is UE-B’s behavior for preferred resource set during resource selection, not LCP, and the other bullet of this agreement have been captured in the resource selection section already;

- The feasibility of using LCP to restrict the mapping between the selected resources and destination with IUC information requires further discussion considering the complexity of cross-layer interaction and various cases (single/multiple preferred/non-preferred resource set from same/different UE-As);

Secondly, it is not appreciable to send LS to RAN1 to revert the RAN1 agreement since it is not just reverting a single agreement, it is about the whole spirit behind IUC design. I.e., it is not easy for R1 to solve it solely.

So considering the feasibility and R17 is already freezing, a similar solution as what R2 decided for SL DRX is preferred. In fact, the key issue for IUC and SL DRX is the same, i.e., the parameters/inputs for resource selection may not be accurate since the destination is selected after the LCP procedure. As we have discussed in SL DRX, 1)this issue is due to the gap between resource selection and LCP since R16; 2)it can be handled by UE implementation. To look back at the solution in SL DRX,

- In the resource selection procedure, up to UE implementation to decide a destination for “Active time” determination;

- In LCP, select any candidate destination which is in active time and fulfill the R16 principle.

To mimic the solution, in IUC:

- In the resource selection procedure, rely on UE implementation to decide a destination for “IUC information” usage considering the destination may be selected during the LCP procedure (based on the priority of LCH associated with destinations);

- In the LCP procedure, there is already some UE flexibility on destination selection for multiple destinations satisfying conditions in LCP. (NOTE 1 in 38.321, clause 5.22.1.4.1.2);

Therefore, the preferred solution for the agreement on multiple preferred resource sets from different UE-As is,

- Capture the agreement in the section for resource selection, which is align with what we did for other related agreements and other bullets in this agreement;

- On the detailed way to capture, NOTE is preferred since it is just a guidance to UE implementation.

RAN2 not pursue IUC-aware LCP change at maintenance stage now. And leave the IUC-info utilization at resource selection to UE implementation, i.e., not pursue normative text change.

For the wording, the following text is suggested,

NOTE X: When UE-B receives multiple preferred resource sets from the different UE-A, it is up to UE-B implementation to decide preferred resource set from which UE-A to be used during resource selection.

RAN2 discuss to capture the NOTE as “When UE-B receives multiple preferred resource sets from the different UE-A, it is up to UE-B implementation to decide preferred resource set from which UE-A to be used during resource selection.”

Conclusion

We have the following observations:

Observation 1	By following UL principle and considering the actual need for SL, the drx-RetransmissionTimerSL/drx-HARQ-RTT-TimerSL for SL CG resource is needed.

Observation 2	The current specification cannot support the RTT/Retransmission timer for SL CG resources.

Observation 3	Based on the current spec, UE is to use the next transmission occasion for the transmission of newly generated MAC PDU in its buffer, i.e., no additional spec impact is needed.

Observation 4	The “ignore SL grant” in 5.22.1.3.1 is not only duplicated but also harmful, i.e., the UE behavior is to generate the MAC PDU for an ignored grant.

Observation 5	According to RAN1 agreements, for both scheme 1 and scheme 2, UE determines whether to send IUC-info /IUC REQ message based on the resource pool level configuration.

We have the following proposals:

Proposal 1	For configured SL grant, when the PUCCH resource is configured, start drx-HARQ-RTT-TimerSL for configured sidelink grant at the first symbol after the end of the corresponding PUCCH resource.

Proposal 2	For configured SL grant, when the PUCCH resource is not configured, start the drx-HARQ-RTT-TimerSL for the corresponding HARQ process at the first symbol after the end of PSSCH occasion for configured sidelink grant.

Proposal 3	Rely on configured RTT timer length for resource pools without PSFCH as for the other cases.

Proposal 4	RAN2 agrees on correction for aligning down-selection of inactivity timer length with the down-selection of cycle/on-duration timer length.

Proposal 5	SR configuration for SL DRX Command MAC CE reuses the SR configuration for CSI report.

Proposal 6	RAN2 agrees on removing the text “5> if selected resource for initial transmission occasion is not in the SL DRX Active time as specified in clause 5.28.1 of any destination that has data to be sent: 6> use retransmission occasion(s) for initial transmission of PSCCH and PSSCH. ” in 5.22.1.1 of 38.321.

Proposal 7	RAN2 agrees on the correction of removing the text “2> if all PSCCH duration(s) and PSSCH duration(s) for initial transmission of a MAC PDU of the dynamic sidelink grant or the configured sidelink grant is not in SL DRX Active time as specified in clause 5.28.3 of the destination that has data to be sent: 3> ignore the sidelink grant.” in 5.22.1.3.1 of 38.321.

Proposal 8	RAN2 not pursue normative text for Scheme-1 IUC-based TX resource pool selection.

Proposal 9	RAN2 discuss capturing the NOTE as “The MAC entity can create a selected sidelink grant for IUC-information or IUC-request MAC CE based on the corresponding IUC-information and/or IUC-request MAC CE trigger condition configuration, if any, in the resource pool”.

Proposal 10	No RAN2 discussion on resource pool selection for scheme 2 IUC is needed.

Proposal 11	RAN2 not pursue IUC-aware LCP change at maintenance stage now. And leave the IUC-info utilization at resource selection to UE implementation, i.e., not pursue normative text change.

Proposal 12	RAN2 discuss to capture the NOTE as “When UE-B receives multiple preferred resource sets from the different UE-A, it is up to UE-B implementation to decide preferred resource set from which UE-A to be used during resource selection.”

Reference

[1] R2-2204574 Correction on user plane aspects for SL DRX

[2] R2-2206302 [AT118-e][707][V2X/SL] MAC corrections (LG)