3GPP TSG-RAN WG2 Meeting #119-e	R2-2207032

# Electronic meeting, 17th Aug – 29th Aug 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 3208 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections related to MBS Interest Indication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS-Core |  |  |  |  |  | Date: |  |  | 2022-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1) It is agreed in last meeting that a change is made from “The cell reselected by the UE due to frequency prioritization for MBS is providing SIB20” to “SIB1 scheduling information of the cell reselected by the UE due to frequency prioritization for MBS contains SIB20” for avoiding the misunderstanding of the scheduling/broadcasting/providing of SIB20 in TS38.304. In current TS 38.331, the ambiguous wording of “SIB1 broadcasting SIB21” and “SIB21 is provided by the PCell” are still used. Therefore, it is better to uniform the wording in both specifications of TS 38.331 and TS 38.304 for clearity. 2) The wording may make ambiguous that which PCell the UE has connected to but did not provide SIB20/21. 3) The field name supportedBandCombination does not align with that used in the ASN.1 of TS 38.331. 4) Only in case that all bullet 2>s in clause 5.9.4.3 are satisfied for a MBS frequency, this frequency can be called as a MBS frequency of interest. 5) If UE reports an interested broadcast frequency based on the USD-only mode, it is unreasonable that UE is not allowed to report the interested service provided on this frequency even SIB21 does not provide the mapping between the interested frequency and the associated FSAIs. 6) The IE MBS-ServiceList contains at least one entry according to the ASN1 of TS 38.331, which means the reported interested broadcast service in MII is not empty. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) Uniform the wording such as “SIB1 scheduling information contains SIB20/SIB21”; 2) Add “prior to the current PCell” to clarify that the MII may not be exchanged from the last PCell. 3) Change the field name “supportedBandCombination” into “supportedBandCombinationList”. 4) Change the description of “the concerned MBS frequency of interest” into “the concerned MBS frequency” in the third bullet 2> of clause 5.9.4.3. 5) Add “SIB21 acquired from the PCell does not provide the frequency mapping for the concerned service and that frequency is included in the USD of this service” to clarify that UE can report the interested broadcast service provided on the determined interested frequency in MII even if SIB21 does not contain the mapping between this interested frequency and the associated FSAIs in USD. 6) Add “the set of services, determined in accordance with 5.9.4.4, is not empty” to ensure that the reported MBS-ServiceList is not empty. Impact analysis Impacted 5G architecture options: NR standalone Impacted functionality: MBS Inter-operability: There are no inter-operatbility issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE may not send MII if the SIB21 is not broadcasting but is in the SIB1 scheduling information of the PCell. UE will not include the interested broadcast service in MII if the interested frequency of MII is determined only based on USD. UE may include in MII message the IE MBS-ServiceList but the determined interested broadcast service in accordance with 5.9.4.4 is empty. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.9.4.2, 5.9.4.3, 5.9.4.4, 5.9.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of change

### 5.9.4	MBS Interest Indication

#### 5.9.4.1	General

Figure 5.9.4.1-1: MBS Interest Indication

The purpose of this procedure is to inform the network that the UE in RRC_CONNECTED state is receiving or is interested to receive MBS broadcast service(s) and to inform the network about the priority of MBS broadcast versus unicast and multicast MRB reception. MBS Interest Indication can only be sent after AS security activation.

#### 5.9.4.2	Initiation

An MBS capable UE in RRC_CONNECTED may initiate the procedure in several cases including upon successful connection establishment/resume, upon entering or leaving the broadcast service area, upon MBS broadcast session start or stop, upon change of interest, upon change of priority between MBS broadcast reception and unicast/multicast reception, upon change to a PCell in which the SIB1 scheduling information contains SIB21, upon receiving SIB20 of an SCell via dedicated signalling, upon handover.

Upon initiating the procedure, the UE shall:

1>	if SIB21 is in the SIB1 scheduling information of the PCell:

2>	ensure having a valid version of SIB21 for the PCell;

2>	if the UE did not transmit MBS Interest Indication since last entering RRC_CONNECTED state; or

2>	if since the last time the UE transmitted an MBS Interest Indication, prior to the current PCell, the UE connected to a PCell in which the SIB1 scheduling information does not contain SIB21:

3>	if the set of MBS broadcast frequencies of interest, determined in accordance with 5.9.4.3, is not empty:

4>	set the contents of MBS Interest Indication according to 5.9.4.5 and initiate transmission of the MBSInterestIndication message;

2>	else:

3>	if the set of MBS broadcast frequencies of interest, determined in accordance with 5.9.4.3, is different from mbs-FreqList included in the last transmission of the MBS Interest Indication; or

3>	if the prioritisation of reception of all indicated MBS broadcast frequencies compared to reception of any of the established unicast bearers and multicast MRBs has changed since the last transmission of the MBS Interest Indication:

4>	set the contents of MBS Interest Indication according to 5.9.4.5 and initiate transmission of the MBSInterestIndication message;

NOTE:	The UE may send MBS Interest Indication even when it is able to receive the MBS services it is interested in i.e. to avoid that the network allocates a configuration inhibiting MBS broadcast reception.

3>	else if SIB20 is in the SIB1 scheduling information of the PCell or is provided for the SCell:

4>	if since the last time the UE transmitted the MBS Interest Indication, prior to the current PCell, the UE connected to a PCell in which the SIB1 scheduling information does not contain SIB20 and the UE was not provided with SIB20 for an SCell; or

4>	if the set of MBS broadcast services of interest determined in accordance with 5.9.4.4 is different from mbs-ServiceList included in the last transmission of the MBS Interest Indication:

5>	set the contents of MBS Interest Indication according to 5.9.4.5 and initiate the transmission of MBSInterestIndication message.

#### 5.9.4.3	MBS frequencies of interest determination

The UE shall:

1>	consider a frequency to be part of the MBS frequencies of interest if the following conditions are met:

2>	at least one MBS session the UE is receiving or interested to receive via a broadcast MRB is ongoing or about to start; and

NOTE 1:	The UE may determine whether the session is ongoing from the start and stop time indicated in the User Service Description (USD), see TS 38.300 [2] or TS 23.247 [67].

2>	for at least one of these MBS sessions, SIB21 acquired from the PCell includes mapping between the concerned frequency and one or more MBS FSAIs indicated in the USD for this session, or for at least one of these MBS sessions, the concerned frequency is not included in SIB21 but is indicated in the USD for this session; and

NOTE 2:	The UE considers a frequency to be part of the MBS frequencies of interest even though NG-RAN may (temporarily) not employ a broadcast MRB for the concerned session, i.e., the UE does not verify if the session is indicated on MCCH.

2>	the supportedBandCombinationList the UE included in UE-NR-Capability contains at least one band combination including the concerned MBS frequency.

NOTE 3:	When evaluating which frequencies the UE is capable of receiving, the UE does not take into account whether they are currently configured as serving frequencies.

#### 5.9.4.4	MBS services of interest determination

The UE shall:

1>	consider an MBS service to be part of the MBS services of interest if the following conditions are met:

2>	the UE is receiving or interested to receive this service via a broadcast MRB; and

2>	the session of this service is ongoing or about to start; and

2>	one or more MBS FSAIs in the USD for this service is included in SIB21 acquired from the PCell for a frequency belonging to the set of MBS frequencies of interest, determined according to 5.9.4.3 or SIB21 acquired from the PCell does not provide the frequency mapping for the concerned service and that frequency is included in the USD of this service.

NOTE:	The UE may determine whether the session is ongoing from the start and stop time indicated in the User Service Description (USD), see TS 38.300 [2] or TS 23.247 [67].

#### 5.9.4.5	Setting of the contents of MBS Interest Indication

The UE shall set the contents of the MBS Interest Indication as follows:

1>	if the set of MBS frequencies of interest, determined in accordance with 5.9.4.3, is not empty:

2>	include mbs-FreqList and set it to include the MBS frequencies of interest sorted by decreasing order of interest, using the absoluteFrequencySSB for serving frequency, if applicable, and the ARFCN-ValueNR(s) as included in SIB21 or in USD (for neighbouring frequencies);

2>	include mbs-Priority if the UE prioritises reception of all indicated MBS frequencies above reception of any of the unicast bearers and multicast MRBs;

NOTE:	If the UE prioritises MBS broadcast reception and unicast/multicast data cannot be supported because of congestion on the MBS carrier(s), NG-RAN may for example initiate release of unicast bearers/multicast MRBs.

2>	if SIB20 is in the SIB1 scheduling information of the PCell or is provided for the SCell, and the set of services, determined in accordance with 5.9.4.4, is not empty:

3>	include mbs-ServiceList and set it to indicate the set of MBS services of interest sorted by decreasing order of interest determined in accordance with 5.9.4.4.

End of change