3GPP TSG-RAN WG2 #119 electronic                                   R2-2207089

Online, August, 2022

Source: 			OPPO

Title: 	Consideration on LPHAP

Agenda item:		8.2.4

Document for: 	Discussion and Decision

- Introduction

In the last RAN#96 meeting, the study item on expanded and improved NR positioning was approved. Specifically, regarding LPHAP, the task for the RAN2 is highlighted as follows [1]:

- mproved accuracy, integrity, and power efficiency:

  - Study solutions for Integrity for RAT dependent positioning techniques [RAN2, RAN1]:

    - Identify the error sources, [RAN1, RAN2].

    - Study methodologies, procedures, signalling, etc for determination of positioning integrity for both UE-based and UE-assisted positioning [RAN2]

    - Focus on reuse of concepts and principles being developed for RAT-Independent GNSS positioning integrity, where possible.

  - Study solutions for accuracy improvement based on PRS/SRS bandwidth aggregation for intra-band carriers considering e.g. timing errors, phase coherency, frequency errors, power imbalance, etc [RAN4]:

  - Study solutions for accuracy improvement based on NR carrier phase measurements [RAN1, RAN4]

    - Reference signals, physical layer measurements, physical layer procedures to enable positioning based on NR carrier phase measurements for both UE-based and UE-assisted positioning [RAN1]

    - Focus on reuse of existing PRS and SRS, with new reference signals only considered if found necessary

  - Study the requirements on LPHAP as developed by SA1 and evaluate whether existing RAN functionality can support these power consumption and positioning requirements. Based on the evaluation, and, if found beneficial, study potential enhancements to help address any limitations [RAN2, RAN1]

    - Study is limited to a single representative use case (use case 6 as defined TS 22.104). The choice of selected use case can be reviewed at the start of the study.

    - Study is limited to enhancements to RRC_INACTIVE and/or RRC_IDLE state.

In this paper, we would like to present our views on supporting the LPHAP in the R18 NR.

- Discussion

In the TS 22.104 A 7.2 section, the low power high accuracy positioning use cases are indicated, as follows:

| Use Case # | Horizontal accuracy | Corresponding service level (22.261) | Positioning interval/duty cycle | Battery life time/minimum operation time |
| --- | --- | --- | --- | --- |
| 1 | 10m | Service Level 1 | On request | 24 months |
| 2 | 2m to 3m | Service Level 2 | <4 seconds | >6months |
| 3 | < 1m | Service Level 3 | no indication | 1work shift 8 hours (up to 3 days, 1month for inventory purposes) |
| 4 | < 1m | Service Level 3 | 1 second | 6 ~ 8 years |
| 5 | < 1m | Service Level 3 | 5 seconds – 15 minutes | 18 months |
| 6 | < 1m | Service Level 3 | 15 s to 30 s | 6 ~ 12 months |
| 7 | 30 cm | Service Level 5 | 250 ms | 18 months |
| 8 | 30 cm | Service Level 5 | 1 second | 6 ~ 8 years (no strong limitation in battery size) |
| 9 | 10m | Service Level 1 | 20 minutes | 12 years (@20mj/positioning fix) |

From the SID, we can find that only the use case #6 is in the scope of the study item, which exhibits sub-meter horizontal accuracy, 15s to 30s positioning duty cycle and 6~12 months battery life. Such positioning accuracy requirement is equivalent to the R16/R17 NR positioning accuracy requirement. As a result, up to 100MHz bandwidth for RS allocation is needed. On the other hand, 6~12 months battery life requires the UE to save the power in a high level for performing positioning

Observation 1: sub-meter positioning accuracy requirement for the use case #6 needs up to 100MHz bandwidth for SRS/DL-PRS allocation.

Observation2: 6~12 months battery life requirement enforces the UE to save the power in a high level for performing positioning.

Conventionally, the IOT devices does not need up to 100MHz bandwidth for data transmission services, not to mention the high battery life drain rate. Hence, the LPHAP devices may support up to 100MHz bandwidth for positioning, but only limited bandwidth, e.g., 20 MHz, for data services simultaneously. RAN2 should study the impact of such LPHAP devices on the specification, for instance, how to locate the DL-PRS distribution bandwidth via reading SI.

Proposal 1: RAN2 to study the impact of LPHAP devices simultaneously supporting up to 100MHz bandwidth for positioning but only limited bandwidth, e.g., 20MHz for data services on the specification, for instance, how to locate the DL-PRS distribution bandwidth via reading SI.

Moreover, 6~12months battery life requires reducing the amount of power consumption as much as possible. In R17, positioning in RRC_Inactive state was introduced. Specifically, positioning in RRC_Inactive state depends on the small data transmission. If the condition of using SDT resource is met, e.g., by the RSRP measurement result, the UE could just apply the SDT procedure for UL transmission of LPP or LCS msg. In addition, if the UE has one on-going SDT service, the network could send the DL LPP or LCS msg towards the UE directly. Otherwise, either the UE needs to trigger the RRC Resume procedure, or the network needs to page and transits the UE to RRC_connected state for transmission of the UL or DL LPP or LCS msg, respectively, which is considered to be power consuming (several additional signaling msg interaction needs to be involved).

Observation 3: positioning in RRC_Inactive state depends on small data transmission, which eliminates the necessity of UE transiting to RRC_Connected state and therefore saves the power of reception and transmission of signaling msg.

In addition, the UE could measure the DL-PRS when the UE is in the RRC_Inactive state, only if the DL-PRS has been configured towards the UE in the RRC Release with suspend msg. SRS transmission is also possible for the UEs in the RRC_Inactive state, if the SRS configuration has been sent towards the UE in the RRC Release with suspend msg, though SRS transmission needs to be stopped when the Timing advance or the spatial relation for the configured RS becomes invalid. In all, positioning procedure in RRC_Inactive state could be taken as the baseline for study of LPHAP, for the sake of power efficiency.

Proposal 2: RAN2 to agree that positioning procedure in RRC_Inactive state could be taken as a baseline for study of LPHAP, for the sake of power efficiency.

Note that pre-configuration of multiple of DL-PRS configuration corresponding to different areas has already been supported in the R16 positioning WI. UE does not need to enter the RRC_Connected state to update the DL-PRS configuration. On the other hand, SRS transmission is still constrained by the timing advance and spatial relation for the configured RS validity, which means that currently UE cannot transmit SRS within a relatively large area without reconfiguration of the SRS from the network. In R18, to support LPHAP, RAN2 might need to study the mechanism to let the UE perform SRS transmission without further reconfiguration from the network in a relatively large area.

Proposal 3: In R18, to support LPHAP, RAN2 to study the mechanism to let the UE perform SRS transmission without further reconfiguration from the network in a relatively large area.

In the RRC_Idle state, the UE cannot not transmit SRS. Hence, the network cannot know the UE position by making UL measurement on the UE transmitted SRS. On the other hand, in the RRC_Idle state, the UE needs to perform SS-RSRP measurement on the SS signals or the PBCH-DMRS for the purpose of cell selection/reselection, etc. So similarly, it may be possible for the UE to perform DL-PRS measurement for DL positioning in the RRC_Idle state. The issue is how to let UE transmit the location measurement result/positioning result towards the network without taking too much signaling msg exchange, or the UE can transit to the RRC_Connected state to send the DL-PRS measurement towards the network periodically (with a relatively long period configured). So, we propose RAN2 to study the possibility of employing DL-PRS measurement for positioning in the RRC_Idle state.

Observation 4: In the RRC_Idle state, the UE cannot not transmit SRS. Hence, the network cannot know the UE position by making UL measurement on the UE transmitted SRS.

Proposal 4: RAN2 to study the possibility of employing DL-PRS measurement for positioning in the RRC_Idle state.

- Conclusion and proposals

RAN2 is kindly asked to discuss and adopt the following observations and proposals:

Observation 1: sub-meter positioning accuracy requirement for the use case #6 needs up to 100MHz bandwidth for SRS/DL-PRS allocation.

Observation2: 6~12 months battery life requirement enforces the UE to save the power in a high level for performing positioning.

Proposal 1: RAN2 to study the impact of LPHAP devices simultaneously supporting up to 100MHz bandwidth for positioning but only limited bandwidth, e.g., 20MHz for data services on the specification.

Observation 3: positioning in RRC_Inactive state depends on small data transmission, which eliminate the necessity of UE transiting to RRC_Connected state and therefore saves the power of reception and transmission of signaling msg.

Proposal 2: RAN2 to agree that the positioning in RRC_Inactive state could be taken as a baseline for study of LPHAP, for the sake of power efficiency.

Proposal 3: In R18, to support LPHAP, RAN2 to study the mechanism to let the UE perform SRS transmission without further reconfiguration from the network in a relatively large area.

Observation 4: In the RRC_Idle state, the UE cannot not transmit SRS. Hence, the network cannot know the UE position by making UL measurement on the UE transmitted SRS.

Proposal 4: RAN2 to study the possibility of employing DL-PRS measurement for positioning in the RRC_Idle state.

- Reference

[1] RP-221814, Revised SID on Study on expanded and improved NR positioning, Intel, CATT, Ericsson