3GPP TSG-RAN WG2 Meeting #119-e                                                   R2-2207111

Electronic, 17th - 26th August, 2022

Source:	CATT

Title:	Discussion on LPHAP

Agenda Item:	8.2.4

Document for:	Discussion and Decision

Introduction

According to the SID [1], one objective of the R18 expanded and improved NR positioning is:

- Improved accuracy, integrity, and power efficiency: - Study the requirements on LPHAP as developed by SA1 and evaluate whether existing RAN functionality can support these power consumption and positioning requirements. Based on the evaluation, and, if found beneficial, study potential enhancements to help address any limitations [RAN2, RAN1] - Study is limited to a single representative use case (use case 6 as defined TS 22.104). The choice of selected use case can be reviewed at the start of the study. - Study is limited to enhancements to RRC_INACTIVE and/or RRC_IDLE state

The required operation time of the 5G enabled IoT device and duty cycle of the updated position information for use case 6 defined in TS 22.104 [2] is shown as follow.

Table A.7.2-1: Low power high accuracy positioning use cases

| Use Case # | Horizontal accuracy | Corresponding service level (22.261) | Positioning interval/ duty cycle | battery life time/ minimum operation time |
| --- | --- | --- | --- | --- |
| 1 | 10 m | Service Level 1 | on request | 24 months |
| 2 | 2 m to 3 m | Service Level 2 | < 4 seconds | > 6 months |
| 3 | < 1 m | Service Level 3 | no indication | 1 work shift - 8 hours (up to 3 days, 1 month for inventory purposes) |
| 4 | < 1 m | Service Level 3 | 1 second | 6 - 8 years |
| 5 | < 1 m | Service Level 3 | 5 seconds - 15 minutes | 18 months |
| 6 | < 1 m | Service Level 3 | 15 s to 30 s | 6 - 12 months |
| 7 | 30 cm | Service Level 5 | 250 ms | 18 months |
| 8 | 30 cm | Service Level 5 | 1 second | 6 - 8 years (no strong limitation in battery size) |
| 9 | 10 m | Service Level 1 | 20 minutes | 12 years (@20mJ/position fix) |

In this contribution, some scenario analysis and discussion are provided on potential solutions to improve accuracy and power efficiency.

Discussion

The LPHAP is raised mainly for industrial applications, which require battery-constrained UE to sustain a long lifetime. Focus on use case 6, the required horizontal accuracy is less than 1 meter, positioning interval is between 15s to 30s and the wanted battery life time is up to 6-12 months.

According to the enhanced positioning technology in R17, 1 meter horizontal accuracy is already supported, that no further enhancement is needed at present. What may need to be further studied is how to reduce the power consumption, for 6-12 months battery life time is actually a high standard. The specific power consumption level is under evaluating by RAN1, but some potential solutions can be discussed in advance.

In the following part, the service type will be discussed firstly, and then some possible improving solutions will be analyzed.

  - Service Type Analysis

In TS 23.273 [3], there already defined some location services, which can be divided into the following categories:

- NI-LR

- MT-LR

- MO-LR

- Deferred 5GC-MT-LR

- Low Power Periodic and Triggered 5GC-MT-LR

From the aspect of location request initiate units, these location service types can be divided as NI-LR, MT-LR and MO-LR. Further, considering the request time characteristics, immediate and deferred location request are distinguished. Immediate location request refer to positioning service which expect to receive location response immediately. Deferred location request expect to receive response when the pre-defined condition is fulfilled in the future, like when UE entry/leave the defined area or periodic report. Far from now, only the deferred MT-LR is supported. Further, the Low Power Periodic and Triggered 5GC-MT-LR is studied, and the RRC_INACTIVE positioning is supported using low power event reporting.

Considering the requirements of use case 6 present cycle characteristics and low power consumption requirement, and the objective limit the study is on RRC_INACTIVE and/or RRC_IDLE state. The deferred 5GC-MT-LR procedure for Periodic or Triggered Location Events can describe these requirements very well. Therefore the Low Power Periodic and Triggered 5GC-MT-LR is suggested to be discussed as the basic service type.

Proposal 1: RAN2 take the Low Power Periodic and Triggered 5GC-MT-LR as the basic service type.

  - Enhancements on positioning in RRC_INACTIVE

Low power and high accuracy are the mainly requirements of LPHAP. Since the high accuracy requirement is already fulfilled, that the low power requirement is focused in this section. In view of the deferred 5GC-MT-LR procedure for periodic or triggered location events when a UE is in RRC INACTIVE state [4], there are serval aspects which might be optimized.

SRS configuration optimized by valid area

For use case 6, the scenario is mainly in factory or industrial park, the IoT equipment may have stable and periodic positioning requirement. Usually these IoT equipment moving in limited range and relatively fixed route. Figure 1 shows an intelligent logistics industrial park. The mobile platform is carrying some containers between two warehouses round and round, its route may span serval cells. To get the real-time position of these logistic containers, LCS server can request the mobile platform to report its location every 10 seconds.

Figure 1. Intelligent logistics scene

According to INACTIVE positioning in R17, the validity of SRS configuration is based on the TA validation, which is judged by the TA timer mechanism and the RSRP change solution. That is, when the inactivePosSRS-TimeAlignmentTimer is expired or the RSRP increased/decreased more than inactivePosSRS-RSRP-ChangeThreshold, the SRS configuration becomes invalid. Additionally, after cell reselection, UE will also release the SRS configuration [5]. The above mentioned mechanisms strictly limit the valid scope of the configured SRS. For the mobile platform in figure 1, NW needs to update the SRS configuration for UE very frequently, it may even happens after each event report.

Considering this issue, one potential solution is to configure area specific SRS configuration along with valid area. The valid area can cover serval cells. If UE is still in the valid area, LMF will not trigger gNB to assign SRS configuration after receiving event report. For example, in figure 1, the valid area contains cell#1, cell#2 and cell#3, the mobile platform can use the received SRS configuration during the whole location service as long as UE is within the valid area. By this way, some signaling overhead can be reduced together with some energy consumption.

Therefore, it is proposed for RAN2 to further study enhancement on the area specific SRS configuration along with the corresponding valid area.

Proposal 2: RAN2 to study the enhancement of area specific SRS configuration.

Low efficiency to contain SRS configuration in RRCRelease message

Figure 2. Signaling between UE and gNB

In procedure of “Low Power Periodic and Triggered 5GC-MT-LR Procedure with SDT - UL-only positioning”, the SRS configuration is sent to UE within the RRCRelease message. Figure 2 shows part of the corresponding signaling between UE and gNB. Considering the overhead of the SRS configuration is big, it may consume a lot of energy to receive the RRCRelease message.

One potential solution is to preconfigure some candidate SRS configurations for UE, and when needed, NW only send an index to indicate which one to choose. Preconfigure candidate SRS configurations can be realized by broadcasting in system information. Similar with the scene in figure 1, UE keeps moving between three cells. If the network preconfigured serval SRS for UE, when UE moves in range of a cell and send an event report, the gNB will only need to indicate the corresponding SRS index to UE. This enhancement may bring some energy saving under specific scenarios.

Therefore, it is proposed for RAN2 to further study and identify potential enhancement options on the low efficiency issue of containing SRS configuration in RRCRelease message from high layer.

Proposal 3: RAN2 to study the low efficiency issue brought by containing SRS configuration in RRCRelease message.

Paging optimize during the deferred MT-LR period

For some IoT terminals, apart from the positioning service, there may be no other communication requirements. So it may be possible to relax Paging Occasions (POs) monitoring according to the time characteristic of positioning service. Refer to the deferred 5GC-MT-LR procedure, only when received the deferred location request from LCS client or AF, AMF wait for UE to become reachable and initiate a network triggered service request procedure to establish signaling connection with UE. After that, during the whole deferred MT-LR period, NW may not page UE due to positioning requirement.

Therefore, UE could stop monitoring the POs during the deferred MT-LR according to its capability or UE type (like, IIOT). As shown in figure 3, UE is monitoring the POs based on DRX or eDRX mechanism, once UE received a request of deferred 5GC-MT-LR procedure for periodic event, UE can stop monitoring the PO until the end of the whole location event. By this way, the power consumption can be further reduced.

Figure 3. Paging optimize method

Proposal 4: RAN2 to consider optimizing the paging during the deferred MT-LR period.

DL-PRS/SRS configuration optimized by (e)DRX information

Far from now, the (e)DRX information applied by UE and the DL-PRS/SRS configured by LMF are independent, which may result in misalignment between PRS or SRS configuration and the DRX configuration. The scenario shown in Figure 4 may occur, i.e., the transmission occasion of PRS is in DRX off duration of UE. In such scenario, if UE wakes up to perform PRS measurement, additional power consumption will be introduced. Instead, if UE does not wake up, UE cannot perform the PRS measurement at the corresponding PRS transmission occasion, which may lead to positioning failure. Similar problem is present for SRS transmission in UL positioning. This issue makes the positioning procedure at low efficient and accuracy, it also brings some unnecessary energy consumption.

Figure 4. DL-PRS configuration and DRX period

Proposal 5: RAN2 to further discuss the enhancement on PRS/SRS configuration alignment with the (e)DRX configuration.

  - Positioning in RRC_IDLE state

In R17, positioning in RRC_INACTIVE was studied. Based on the small data transmission (SDT) technology, UE reporting event report and receiving the SRS configuration from gNB with keeping in RRC_INACTIVE state were realized. To further reduce the power consumption, positioning in RRC_IDLE state is proposed. Firstly, the benefit and the difficulty need to be analyzed.

On the one hand, in RRC_IDLE state, UE do not need to keep connected with the core network, which may bring some energy consumption reduction, but the exactly energy consumption reduction effect need to be evaluated by RAN1.

On the other hand, to realize positioning in RRC_IDLE, the following key points should be considered:

- How to report event report when the positioning event is detected;

- How to receive the event report acknowledgement;

- How to receive the SRS configuration in UL positioning or multi-RTT positioning procedure;

- If the NAS connection and security establishment procedure can be avoided.

All in all, RAN2 need to evaluate the degree of energy consumption reduction as well as the difficulty in solving the above mentioned key points, and then decide whether to support RRC_IDLE positioning in R18.

Proposal 6: RAN2 to further analyze the benefit as well as the difficulty of positioning in RRC_IDLE state, and discuss whether to deprioritize the positioning in RRC_IDLE stage for LPHAP in R18.

Conclusion

According to the analysis in section 2, the following proposals are provided:

Proposal 1: RAN2 take the Low Power Periodic and Triggered 5GC-MT-LR as the basic service type.

Proposal 2: RAN2 to study the enhancement of area specific SRS configuration.

Proposal 3: RAN2 to study the low efficiency issue brought by containing SRS configuration in RRCRelease message.

Proposal 4: RAN2 to consider optimizing the paging during the deferred MT-LR period.

Proposal 5: RAN2 to further discuss the enhancement on PRS/SRS configuration alignment with the (e)DRX configuration.

Proposal 6: RAN2 to further analyze the benefit as well as the difficulty of positioning in RRC_IDLE state, and discuss whether to deprioritize the positioning in RRC_IDLE stage for LPHAP in R18.

Reference

- RP-213588 Revised SID on Study on expanded and improved NR positioning.

- TS 22.104 Service requirements for cyber-physical control applications in vertical domains.

- TS 23.273 5G System (5GS) Location Services (LCS).

- R2-2203949 LS on Positioning in RRC_INACTIVE State.

- 3GPP TS 38.331 Radio Resource Control (RRC) protocol specification.