3GPP TSG RAN WG2 Meeting #119-e                                                        R2-2207119

# Electronic meeting, 17th-29th August 2022

Agenda item:	8.5.3

Source:	Intel Corporation

Title:	Study of C-DRX enhancements for XR traffic

Document for:	Discussion and decision

# Introduction

This document discusses RAN2 centric solutions for the following objective of Rel-18 XR SID [1] considering the related RAN1 agreements and discussions, e.g. as in RAN1#109-e summary email discussion [2].

Study XR specific power saving techniques to accommodate XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc...). Focus is on the following techniques: C-DRX enhancement and PDCCH monitoring enhancement.

# Discussion

## Background on RAN1 progress

During RAN1#109-e, RAN1 already agreed on the areas of interest on C-DRX enhancements to study XR specific power saving techniques:

- High priority Issue 1-1: Alignment between CDRX and XR traffic for resolving the mismatch between CDRX cycle and XR traffic periodicity for each flow

- High priority Issue 1-2: C-DRX enhancements to handle jitter

- Medium priority Issue 1-3: CDRX enhancements for multiple XR traffic flows [Note 2]

- Low priority Issue 1-4: CDRX enhancements to adjust to variable burst sizes and frame rate

  - Note: Some companies think the adjustment for variable burst sizes can be realized by existing spec already

- Low priority Issue 1-5: low latency handling

- Low priority Issue 1-6: SFN wraparound mismatch (if handled in RAN1)

To minimize unnecessary/overlapping discussions across WGs, our suggestion is for RAN2 to also focus on the prioritized areas by RAN1 (i.e. those marked as high and medium priority) and discuss RAN2 centric solutions or potential RAN2 impacts for the solutions already identified in RAN1. It is important to remember that RAN1 also look into related issue during Rel-17 XR study as captured in TR

RAN1 already discussed potential areas of interest to enhance C-DRX mechanism for XR traffic, therefore RAN2 should avoid redundant discussions. RAN2 should take RAN1 discussions as a starting point for the study phase in RAN2 side.

- RAN2 initiates its study of C-DRX enhancements for XR traffic on the key issues or areas of interest already prioritized by RAN1: Issue (1-1) Alignment between C-DRX and XR traffic for resolving the mismatch between C-DRX cycle and XR traffic periodicity for each XR traffic flow, Issue (1-2) C-DRX enhancements to handle jitter and Issue (1-3) C-DRX enhancements for multiple XR traffic flows.

RAN1 is working on the evaluation of the candidate solutions but in parallel we suggest for RAN2 to discuss RAN2 centric solution details to share some conclusions on the study side from RAN2 point of view. Next sub-sections provide corresponding details considering the proposed list of solutions identified during RAN1#109-e summary email discussion [2].

## [Issue 1-1] Alignment between C-DRX and XR traffic for resolving the mismatch between C-DRX cycle and XR traffic periodicity for each flow

For this issue (1-1), RAN1 captured the following proposed solutions as part of the summary email discussion [2]:

- Non-uniform CDRX cycle pattern

- Uniform non-integer CDRX cycle

- Dynamic signaling indicated adjustment of CDRX start offset/duration [Note 1]

- Semi-static configurations of drx_startoffset to match the traffic periodicity

- Periodic XR-specific PDCCH monitoring in both DRX ON and OFF based on XR packet generation cycle

RAN1 is discussing how possible values of some C-DRX configurations might not be optimum for given XR traffic characteristics. For example, typical XR frame rates are 60 and 120 frames per seconds (fps), which result in non-integer frame periodicities of 16.67ms and 8.33ms, respectively. Current values of the C-DRX cycle length which is defined in multiples of 1ms can therefore not align with XR traffic periodicity. If RAN1’s evaluation confirms the benefit to use non-integer (in ms) values of C-DRX cycle or to adapt C-DRX start offset, RAN2 impact would be minimal. In addition, XR data traffic might arrive on data burst, i.e. XR traffic periodicity could be associated with the start of each data burst, but the periodicity of the actual data/PDUs sent within a given burst of data could be different. Therefore, it may be beneficial to use C-DRX configurations that better suit the current/ongoing XR traffic.

Moreover, RAN2 could discuss how different C-DRX configurations or some of the parameters could efficiently use different values depending on the type of XR data stream (e.g video, audio), as well as, its current/ongoing traffic characteristics. One solution may be for the gNB to provide the multiple C-DRX configurations (or only those different parameters) in advance and later only send a fast indication (e.g. via L1 or MAC CE) to trigger the switch. Another solution may be for the UE using long C-DRX being indicated to switch to short C-DRX without triggering the initiation of DRX Inactivity Timer. This may help when data is expected in the near future (i.e. when a data burst is about to arrive). Moreover, it could be associated the new concept under discussion by SA2, i.e. PDU set. For example, this data burst might be related to the arrival of a given “PDU set” which is expected to have x number of PDUs (denoting size of PDU set).

- RAN2 to wait for RAN1’s conclusion on the following proposed solutions as RAN2 foreseen impact is minimal: non-uniform CDRX cycle pattern and uniform non-integer CDRX cycle.

- RAN2 to study how to enable a mechanism that allows dynamic switching between C-DRX configurations (or at least for part of the parameters of the CDRX configuration).

  - This new mechanism of Proposal 3 may allow gNB to provide multiple C-DRX related configurations in advance, and later only provide a fast indication to trigger the switch. FFS which parameters of the C-DRX configuration could benefit from this mechanism.

  - This new mechanism of Proposal 3 may allow switching from long C-DRX to short C-DRX without triggering the DRX Inactivity Timer. FFS whether/how to relate this new switching mechanism/feature to the “PDU set” concept under discussion in SA2.

## [Issue 1-2] C-DRX enhancements to handle jitter

For this issue (1-2), RAN1 captured the following proposed solutions as part of the summary email discussion [2]:

- Jitter handling by dynamic indication of DRX On Duration starting time and/or another DRX On Duration time by DCI or WUS

- PDCCH monitoring triggered by WUS within DRX on duration

- Adjustment of DRX On Duration or Active Time conditioned on data reception

    - This solution triggers additional On Duration for PDCCH monitoring or early switching to DRX OFF

- Non-uniform PDCCH monitoring occasions within On Duration matched to jitter distribution

- XR-specific PDCCH monitoring window at each cycle in both DRX ON and OFF based on network delay jitter

- Two stage/level CDRX configuration

Different solutions are proposed on how to handle the jitter considering the ON duration configuration associated with a given CDRX configuration. For example, UE may rely on the network to take jitter into account when configuring the desirable value of the ON duration, SPS/CG periodicity or even when triggering skip of PDCCH monitoring. For example, after start of ON duration, UE could sparsely monitor PDCCH and when PDCCH is detected, UE may switch to frequent PDCCH monitoring. To this end, RAN1 is also discussing the study of Rel-17 PDCCH monitoring adaptation solution to understand whether it can adequately handle jitter in XR traffic.

- RAN2 to ask RAN1 whether adaptation of Rel-17 PDCCH monitoring solutions is sufficient to handle jitter associated with XR traffic.

Our understanding is that Application (in UE and/or XR Server) would only be able to expose a maximum or at most a range of the expected jitter. If so, RAN would need to assume with the maximum possible value to account for the worse case scenario. This might not be optimum from power saving point of view i.e. if this means that UE needs to be awake longer for PDCCH monitoring.

We suggest for RAN2 to study new mechanisms that could mitigate this while allowing a dynamic adjustment of UE’s active time (e.g. ON duration) to better accommodate jitter associated with XR traffic. One solution may be for gNB to share the expected jitter to be considered in relation to UE’s configured ON duration. Another solution may be to efficiently allow changes to the ON duration and/or DRX offset values. If so, this may be based on the mechanism discussed in previous section.

- To study how to enable a mechanism that allows dynamic adjustment of UE’s active time (e.g. ON duration) to better accommodate the jitter associated with XR traffic.

  - This new mechanism of Proposal 5 may be for gNB to share expected jitter information to be accounted by the UE in relation to UE’s configured ON duration.

  - This new mechanism of Proposal 5 may be to efficiently allow changes to the ON duration and/or DRX offset values e.g. via the new mechanism discussed in Proposal 3.

## [Issue 1-3] C-DRX enhancements for multiple XR traffic flows

For this issue (1-3), RAN1 captured the following proposed solutions as part of the summary email discussion [2]:

- Multiple active CDRX configurations

- Dynamic activation of CDRX configuration(s) is not precluded

- Multi-period CDRX cycle or On Duration sequence of a CDRX cycle or non-uniform CDRX cycle pattern

This issue aims to enhance UE’s power saving operation when multiple XR traffic flows/streams with different requirements/characteristics (e.g. audio, video, pose or even I vs P frames) are simultaneously transmitted.

An alternative is that gNB provides different C-DRX configurations (or part of the configurations) that could better suit the data characteristics for each ongoing XR traffic flow. We suggest for RAN2 to study new mechanisms that allow a UE to use multiple C-DRX configurations in order to better fit the current/ongoing XR data traffic. The operation of multiple C-DRX configurations at the same time might not be as complex considering that some parameters of the C-DRX configuration are fully independent of PDSCH/PUSCH scheduling e.g. the start of drx-onDurationTimer, drx-ShortCycle or drx-LongCycle. However, there are other C-DRX timers that are related to PDSCH/PUSCH scheduling: drx-InactivityTimer, drx-RetransmissionTimerDL, and drx-RetransmissionTimerUL. Therefore, for a given UE, PDCCH scheduling PDSCH and/or PUSCH can be used to differentiate the C-DRX configuration to be impacted (of the multiple configurations being used). RAN1 input/coordination would indeed be needed to determine how the PDCCH can be used to differentiate the C-DRX configuration being used. For example, multiple C-RNTIs may be used/configured for a given MAC entity, new DRX related field may be provided via the DCI, etc.

- RAN2 to study how to enable a mechanism to allow simultaneous operation of multiple C-DRX configurations.

  - The mechanism of Proposal 6 allows the UE to be configured with multiple DRX configurations in one DRX group of a MAC entity to be active simultaneously; i.e., one or multiple C-DRX timers (e.g. drx-InactivityTimer, drx-RetransmissionTimerDL, and drx-RetransmissionTimerUL) are configured independently per DRX configuration and are (re)started based on received PDCCH.

# Conclusion

The observations captured are the following:

Observation 1.	RAN1 already discussed potential areas of interest to enhance C-DRX mechanism for XR traffic, therefore RAN2 should avoid redundant discussions. RAN2 should take RAN1 discussions as a starting point for the study phase in RAN2 side.

The proposals captured are the following:

Proposal 1.	RAN2 initiates its study of C-DRX enhancements for XR traffic on the key issues or areas of interest already prioritized by RAN1: Issue (1-1) Alignment between C-DRX and XR traffic for resolving the mismatch between C-DRX cycle and XR traffic periodicity for each XR traffic flow, Issue (1-2) C-DRX enhancements to handle jitter and Issue (1-3) C-DRX enhancements for multiple XR traffic flows.

Proposal 2.	RAN2 to wait for RAN1’s conclusion on the following proposed solutions as RAN2 foreseen impact is minimal: non-uniform CDRX cycle pattern and uniform non-integer CDRX cycle.

Proposal 3.	RAN2 to study how to enable a mechanism that allows dynamic switching between C-DRX configurations (or at least for part of the parameters of the CDRX configuration).

Proposal 3.1.	This new mechanism of Proposal 3 may allow gNB to provide multiple C-DRX related configurations in advance, and later only provide a fast indication to trigger the switch. FFS which parameters of the C-DRX configuration could benefit from this mechanism.

Proposal 3.2.	This new mechanism of Proposal 3 may allow switching from long C-DRX to short C-DRX without triggering the DRX Inactivity Timer. FFS whether/how to relate this new switching mechanism/feature to the “PDU set” concept under discussion in SA2.

Proposal 4.	RAN2 to ask RAN1 whether adaptation of Rel-17 PDCCH monitoring solutions is sufficient to handle jitter associated with XR traffic.

Proposal 5.	To study how to enable a mechanism that allows dynamic adjustment of UE’s active time (e.g. ON duration) to better accommodate the jitter associated with XR traffic.

Proposal 5.1.	This new mechanism of Proposal 5 may be for gNB to share expected jitter information to be accounted by the UE in relation to UE’s configured ON duration.

Proposal 5.2.	This new mechanism of Proposal 5 may be to efficiently allow changes to the ON duration and/or DRX offset values e.g. via the new mechanism discussed in Proposal 3.

Proposal 6.	RAN2 to study how to enable a mechanism to allow simultaneous operation of multiple C-DRX configurations.

Proposal 6.1.	The mechanism of Proposal 6 allows the UE to be configured with multiple DRX configurations in one DRX group of a MAC entity to be active simultaneously; i.e., one or multiple C-DRX timers (e.g. drx-InactivityTimer, drx-RetransmissionTimerDL, and drx-RetransmissionTimerUL) are configured independently per DRX configuration and are (re)started based on received PDCCH.

# Reference

- RP-220285, RAN Revised SID on Study on XR Enhancements for NR, March 2022.

- R1-2205411, Moderator Summary#2 on XR specific power saving techniques, May 2022

- TR 38.838, Study on XR (Extended Reality) Evaluations for NR, RAN1, Rel-17, December 2021.