3GPP TSG-RAN2 Meeting #119-e	R2-2207151

# Online, 17 – 19 Aug, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | 4831 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to 36.331 on neighbour cell ephemeris |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN |  |  |  |  |  | Date: |  |  | 2022-08-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In NTN, the ephemeris information of neighbour cell is needed for measurement and mobility purposes. With the ephemeris information, the UE can predict the delay drifting and Doppler shifts. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the satellite information of the neighbour cells Impact analysis Impacted 4G architecture options: IoT NTN Impacted functionality: SIB31/SIB31-NB Inter-operability: If the UE is implemented according to this CR while the network is not, or vice versa, the UE can not correctly decode the SIB31/SIB31-NB. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE cannot predict the delay drifting and Doppler shifts of the neighbour cells. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1, 6.7.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==================================CHANGE BEGINS===================================

### 6.3.1	System information blocks

[Unchanged parts omitted]

#### –	SystemInformationBlockType31

The IE SystemInformationBlockType31 contains satellite assistance information for the serving cell and the neighbour cells. SystemInformationBlockType31 is only signalled in a NTN cell.

SystemInformationBlockType31 information element

-- ASN1START

SystemInformationBlockType31-r17 ::= SEQUENCE {

servingSatelliteInfo-r17		NTN-SatelliteInfo-r17,

neighSatelliteInfoList-r17		NeighSatelliteInfoList-r17		OPTIONAL, 	-- Need OR	lateNonCriticalExtension		OCTET STRING					OPTIONAL,

...

}

SatelliteParameters-r17 ::= 	SEQUENCE {

ephemerisInfo-r17			CHOICE {

stateVectors				EphemerisStateVectors-r17,

orbitalParameters			EphemerisOrbitalParameters-r17

},

nta-CommonParameters-17			SEQUENCE {

nta-Common-r17					INTEGER (0..8316827)		OPTIONAL,	-- Need OP

nta-CommonDrift-r17				INTEGER (-261935..261935)	OPTIONAL,	-- Need OP

nta-CommonDriftVariation-r17	INTEGER (0..29479)			OPTIONAL	-- Need OP

},

ul-SyncValidityDuration-r17		ENUMERATED {s5, s10, s15, s20, s25, s30, s35, s40,

s45, s50, s55, s60, s120, s180, s240, s900},

epochTime-r17					SEQUENCE {

startSFN-r17					INTEGER (0..1023),

startSubFrame-r17				INTEGER (0..9)

}																OPTIONAL,	-- Need OP

k-Offset-r17					INTEGER (0..1023),

k-Mac-r17						INTEGER (1..512) 				OPTIONAL,	-- Need OP

...

}

NeighSatelliteInfoList-r17 ::=      SEQUENCE (SIZE(1..maxSat-r17))  OF NTN-SatelliteInfo-r17

NTN-SatelliteInfo-r17 ::=          SEQUENCE {

ntn-SatelliteInfo-r17             SatelliteParameters-r17,

dl-CarrierFreqList-r17            SEQUENCE (SIZE(1.. maxFreq))  OF FreqInfoNTN-r17

}

FreqInfoNTN-r17 ::=          		SEQUENCE {

dl-CarrierFreq-r17                ARFCN-ValueEUTRA,

cellListNTN-r17                   SEQUENCE (SIZE(1.. maxCellIntra))  OF PhysCellId

}

-- ASN1STOP

==================================NEXT CHANGES==================================

#### 6.7.3.1	NB-IoT System information blocks

[Unchanged parts omitted]

#### –	SystemInformationBlockType31-NB

The IE SystemInformationBlockType31-NB contains satellite assistance information. SystemInformationBlockType31-NB is only signalled in a NTN cell.

SystemInformationBlockType31-NB information element

-- ASN1START

SystemInformationBlockType31-NB-r17 ::= SEQUENCE {

servingSatelliteInfo-r17		NTN-SatelliteInfo-r17,

neighSatelliteInfoList-r17		NeighSatelliteInfoList-r17		OPTIONAL, 	-- Need OR	lateNonCriticalExtension		OCTET STRING					OPTIONAL,

...

}

-- ASN1STOP