3GPP TSG RAN WG2 #119e												R2-2207154

Electronic meeting, August 17th –26th, 2022

Agenda Item:	8.8.2

Source: 	NEC

Title:	Considerations on Measurement Reports Enhancements

Document for:	Discussion

- Introduction

In RAN#94, a work item on NR Support for UAV was approved. Objectives for enhancements on measurement reports is included in this work item as follows [1].

1. Specify the following enhancements on measurement reports [RAN2]: - UE-triggered measurement report based on configured height thresholds - Reporting of height, location and speed in measurement report - Flight path reporting - Measurement reporting based on a configured number of cells (i.e. larger than one) fulfilling the triggering criteria simultaneously Note: Work done in LTE is a starting point for this objective. NR-specific enhancements can be considered, if needed, while overall the LTE and NR solutions should be harmonized as much as possible.

This contribution provides our initial considerations on enhancements on measurement reports.

- Discussion

2.1 RRM enhancement

UAV UEs have different communications characteristic from normal terrestrial UEs as the radio characteristic will vary with altitudes. If its height is below eNB, its radio characteristic is more like a UE on the ground, i.e. multipath transmission and limited neighbour interference. If its height is above eNB, its radio characteristic is different from normal terrestrial UE, e.g. Line of Sight transmission and strong neighbour interference. Thus, following RRM enhancements have been done in LTE Rel-15.

As eNB should track altitude change of a drone to adjust communication mode, height triggered measurement event H1 and H2 are introduced to inform network of entering or leaving flying status. If configured, UAV UE will triggered measurement report to deliver measurement results as well as UE height when the flying altitude of the UAV UE is higher or lower than a height threshold. We observe such a mechanism can be reused for NR case.

Moreover, LTE Rel-15 introduced UE location reporting based on the existing location information IE and reporting mechanism, which can also be reused for NR case.

Support RRM enhancements on measurement report triggering based on H1/H2 events as well as UE location reporting introduced in LTE Rel-15 for NR UAVs.

2.2 Interference control

A UAV UE would see more cells with similar signal strength and would see more far away cells than a terrestrial UE. Thus, neighbouring cell quality events may be triggered by UAV UE more frequently than terrestrial UE. To mitigate interferences to both aerial UEs and terrestrial UEs, LTE Rel-15 introduced measurement reporting based on a configured number of cells.

numberOfTriggeringCells indicates the number of cells detected that are required to fulfill an event for a measurement report to be triggered. This field is set only for the events concerning neighbor cells’ quality, i.e. eventA3, eventA4, eventA5. We observed such a mechanism can also be reused for NR UAVs to control the amount of measurement reporting.

Support LTE Rel-15 measurement reporting based on a configured number of cells for NR UAVs.

As the number of UAS operations in all classes of airspace increases rapidly, the interference caused by over signalling will be more serious. For example, a same height event H1 or H2 may be triggered again after leaving. Thus, further reducing the amount of measurement reporting may be considered for NR case.

RAN2 to consider further controlling the amount of measurement reporting.

2.2 Mobility enhancement

To support improved mobility performance and interference detection in following intended areas, LTE Rel-15 introduced a mechanism to deliver flight path report. For example, upon reception of RRCConnectionSetup, UE inform the network “flight path info is available” by including flightPathInfoAvailable in RRCConnectionSetupComplete. In addition, informing the network the availability of flight path information can also be done during RRC connection resume, RRC connection reconfiguration and RRC connection reestablishment procedures. Then, upon reception of the UEInformationRequest message with flightPathInfoReq field present, UE includes flightPathInfoReport and set it to include the list of waypoints along the flight path.

Support LTE Rel-15 flight path report mechanism for NR UAVs.

Flight path report in RRCInactive

With the above mechanism, for reporting flight path, the aerial UE needs to enter RRC_CONNECTED state, inform the network “flight path info is available” and then wait for the network to initiate the procedure by sending the UEInformationRequest after successful security activation. However, some UAV UEs may be in RRC_INACTIVE state. Therefore, flight path report in Rel-15 may have a longer delay and introduce signaling overheads for such UEs.

In LTE Rel-15, UAV flight path report may have a longer delay and introduce signaling overhead for RRC_INACTIVE UEs.

In order to reduce the delay and signaling overheads, a straightforward way is to support flight path report in RRCInactive. In Rel-17, a procedure allowing data and/or signalling transmission while remaining in RRC_INACTIVE state (i.e., without transitioning to RRC_CONNECTED state), i.e., small data transmission (SDT) has been introduced. It can be considered as a start point for supporting flight path report in RRC_INACTIVE.

Support flight path report in RRC_INACTIVE for NR UAVs. SDT can be considered as a start point for supporting flight path report in RRC_INACTIVE.

Conditional mobility

In NR Rel-16, conditional mobility (e.g. CHO) was introduced. This can reduce a possible mobility failure. During the Rel-18 scope discussions in RAN plenaries, the CHO specific enhancements (e.g. new UAV specific condition) were dropped. On the other hand, we consider that the CHO itself can be applied to the UAV. For the candidate cell selection by the target gNB (i.e. decision on acceptance for requested candidate), the flight path information will be useful. For example, if some candidate cells are overlapped, the target NB may select candidate cell(s) which is more suitable for the expected flight path (i.e., way point).

RAN2 to discuss whether the flight path information is also sent to the target gNB in HandoverPreparationInformation.

- Conclusion

Observation 1:	In LTE Rel-15, UAV flight path report may have a longer delay and introduce signaling overhead for RRC_INACTIVE UEs.

Proposal 1:	Support RRM enhancements on measurement report triggering based on H1/H2 events as well as UE location reporting introduced in LTE Rel-15 for NR UAVs.

Proposal 2:	Support LTE Rel-15 measurement reporting based on a configured number of cells for NR UAVs.

Proposal 3:	RAN2 to consider further controlling the amount of measurement reporting.

Proposal 4:	Support LTE Rel-15 flight path report mechanism for NR UAVs.

Proposal 5:	Support flight path report in RRC_INACTIVE for NR UAVs. SDT can be considered as a start point for supporting flight path report in RRC_INACTIVE.

Proposal 6:	RAN2 to discuss whether the flight path information is also sent to the target gNB in HandoverPreparationInformation.

- References

[1] RP-213600, WI on NR Support for UAV, Nokia.