3GPP TSG-RAN2 Meeting #119 electronic	R2-2207164

Online, Aug 17th – 26th, 2022

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | 0500 | rev |  | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on the Gap numbers restriction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_MUSIM-Core |  |  |  |  |  | Date: |  |  | 2022-7-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) |  |
| Reason for change: |  | In the last meeting, it has been agreed to extend signalling to allow UEs to optionally support 3 periodic gaps in Rel-17.Meanwhile, in the last meeting it has also been agreed that =>The following statement should be captured in 38.300: "Network should always provide at least one of the requested gap pattern or no gaps. Network providing an alternative gap pattern instead of the one requested by the UE is not supported in this release" However, these modification hasn’t been included in the current state 2 spec and the current description only restrict the network configured MUSIM gaps. The similar restriction should also be adopted for the Gap preference indication. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add gap number restriction for the MUSIM gap. - Include the above agreements to the chapter 20.3 Impact analysis Impacted 5G architecture options: NR-DC, NE-DC, NR SA Impacted functionality: MUSIM Gap preference indication and configuration Inter-operability: If the network implements this CR, but the UE doesn’t, the UE may set the incorrect MUSIM gap preference and finally leads to MUSIM gap configuration fail. If the UE implements this CR, but the network doesn’t, the network may set the incorrect MUSIM gap configuration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It would cause confusion for the MUSIM gap preference indication and configuration. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 20.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR .. |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Start of change

20.3	UE notification on Network Switching

For MUSIM operation, a MUSIM device in RRC_CONNECTED state in Network A may have to switch from Network A to Network B. Network A is NR and Network B can either be E-UTRA or NR. Before switching from Network A, a MUSIM device should notify Network A to either leave RRC_CONNECTED state, or be kept in RRC_CONNECTED state in Network A while temporarily switching to Network B.

When configured to do so, a MUSIM device can signal the network A a preference to leave RRC_CONNECTED state by using RRC or NAS signaling. After sending a preference to leave RRC_CONNECTED state by using RRC signaling, if the MUSIM device does not receive an RRCRelease message from the Network A within a certain time period (configured by the Network A), the MUSIM device can enter RRC_IDLE state in Network A.

When configured to do so, a MUSIM device can signal the network A a preference to be kept in RRC_CONNECTED state in Network A while temporarily switching to network B. This is indicated by scheduling gaps preference.This preference can include information for setup or release of gap(s). At most 4 gap patterns preference can be indicated for MUSIM purpose: 3 periodic gaps and a single aperiodic gap. The network A should always provide at least one of the requested gap patterns or no gaps. Network providing an alternative gap pattern instead of the one requested by the UE is not supported in this release.

# End of change