3GPP TSG-RAN2 Meeting #119 electronic	R2-2207165

Online, Aug 17th – 26th, 2022

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 3225 | rev |  | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on the MUSIM-GapInfo |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_MUSIM-Core |  |  |  |  |  | Date: |  |  | 2022-7-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) |  |
| Reason for change: |  | - MUSIM-GapInfo is used for both the MUSIM gap preference Indication and the MUSIM gap configuration, however, in the field description of musim-GapLength/ starting-SFN and also other elements, it only indicates MUSIM gap preference; - The “cond’ is only for the downlink message, the description for the –cond aperiodic is correct when the MUSIM-GapInfo is included in the downlink message. However, the MUSIM-GapInfo is also used for the MUSIM gap preference Indication and for the MUSIM gap preference Indication, the musim-Starting-SFN-AndSubframe-r17 optional; - The need S is for the filed with a default value, to specify the UE behavior upon receiving a message with the field absent. For the musim-GapLength-r17, there is no default value, thus the need S is incorrect. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Delete “preference” from the field description of MUSIM-GapInfo. - Clarify that for the MUSIM gap preference Indication, the musim-Starting-SFN-AndSubframe-r17 is optional. For the MUSIM gap configuration, it shall refer to the ----cond description table. - Delete the need S for the musim-GapLength-r17. For the UL (i.e. included in the UAI), clarify that the musim-GapLength-r17 is mandatory present for both aperiodic and aperiodic Gap. For the DL (i.e. included in the Reconfiguration), add “—cond setup’ to indicate that it’s mandatory for gap setting up, including both periodic and aperiodic. Impact analysis Impacted 5G architecture options: NR-DC, NE-DC, NR SA Impacted functionality: MUSIM Gap preference indication and configuration Inter-operability: If the network implements this CR, but the UE doesn’t, the UE may set the incorrect MUSIM gap preference and finally leads to MUSIM gap configuration fail. If the UE implements this CR, but the network doesn’t, the network may set the incorrect MUSIM gap configuration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It would cause confusion for the MUSIM gap configuration and MUSIM gap preference indication. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR .. |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Start of change

### 6.3.2	Radio resource control information elements

/********************************************Omit the unchanged part************************************************/

#### –	MUSIM-GapInfo

The IE MUSIM-GapInfo is used to indicate MUSIM gap parameters.

MUSIM-GapInfo information element

-- ASN1START

-- TAG-MUSIM-GAPINFO-START

MUSIM-GapInfo-r17 ::=               SEQUENCE {

musim-Starting-SFN-AndSubframe-r17  MUSIM-Starting-SFN-AndSubframe-r17             OPTIONAL, -- Cond aperiodic

musim-GapLength-r17                 ENUMERATED {ms3, ms4, ms6, ms10, ms20}         OPTIONAL, -- Cond setup

musim-GapRepetitionAndOffset-r17    CHOICE {

ms20-r17                            INTEGER (0..19),

ms40-r17                            INTEGER (0..39),

ms80-r17                            INTEGER (0..79),

ms160-r17                           INTEGER (0..159),

ms320-r17                           INTEGER (0..319),

ms640-r17                           INTEGER (0..639),

ms1280-r17                          INTEGER (0..1279),

ms2560-r17                          INTEGER (0..2559),

ms5120-r17                          INTEGER (0..5119),

...

}                                                                                  OPTIONAL -- Cond periodic

}

MUSIM-Starting-SFN-AndSubframe-r17 ::= SEQUENCE {

starting-SFN-r17                       INTEGER (0..1023),

startingSubframe-r17                   INTEGER (0..9)

}

-- TAG-MUSIM-GAPINFO-STOP

-- ASN1STOP

MUSIM-GapInfo field descriptions

musim-GapLength Indicates the length of the UE's MUSIM gap as specified in TS 38.133 [14] section 9.1.2D. This field is mandatory present for both aperiodic and aperiodic Gap preference indication.

musim-GapRepetitionAndOffset Indicates the gap repetition period in ms and gap offset in number of subframes for the periodic MUSIM gap as specified in TS 38.133 [14] section 9.1.2D.

musim-Starting-SFN-AndSubframe Indicates gap starting position for the aperiodic MUSIM gap. This field is optional present for the aperiodic MUSIM Gap preference indication.

starting-SFN indicates gap starting SFN number for the aperiodic MUSIM gap.

startingSubframe indicates gap starting subframe number for the aperiodic MUSIM gap.

| Conditional Presence | Explanation |
| --- | --- |
| aperiodic | This field is mandatory present in case of aperiodic MUSIM gap configuration. Otherwise it is absent. |
| periodic | This field is mandatory present in case of periodic MUSIM gap configuration. Otherwise it is absent. |
| setup | This field is mandatory present in case of setting up periodic/aperiodic MUSIM gap. Otherwise it is absent. |

# End of change