3GPP TSG-RAN WG2 Meeting #119	 R2- 22XXXX

Electronic 17 – 26 Aug, 2022

Agenda Item:	6.15.4

Source:	Xiaomi

Title:	Discussion on retransmission issue

Document for:	Discussion

Introduction

In R17, SL DRX is introduced to save UE power. Sidelink retransmission timer is introduced to extend active time to monitor retransmission from peer UE. In this contribution, we discuss the issue of retransmission and propose solution.

Discussion

Sidelink retransmission timer is started after sidelink RTT timer expiry. Following part is specified to start sidelink RTT timer in [1].

3> if PSFCH resource is not configured for the SL grant associated to the SCI: 4> start the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the slot following the end of PSSCH transmission (i.e., currently received PSSCH). 3> if PSFCH resource is configured for the SL grant associated to the SCI: 4> if HARQ feedback is enabled by the SCI and the cast type indicator in the SCI is set to unicast; or 4> if HARQ feedback is enabled by the SCI and the cast type indicator in the SCI is set to groupcast and positive-negative acknowledgement is selected; 5> start the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the first slot after the end of the corresponding PSFCH transmission carrying the SL HARQ feedback; or 5> start the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the first slot after the end of the corresponding PSFCH resource for the SL HARQ feedback when the SL HARQ feedback is not transmitted due to UL/SL prioritization; 4> if HARQ feedback is enabled by the SCI and the cast type indicator in the SCI is set to groupcast and negative-only acknowledgement is selected; 5> start the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the first slot after the end of the corresponding PSFCH transmission carrying the SL HARQ feedback; or 5> start the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the first slot after the end of the corresponding PSFCH resource for the SL HARQ feedback when the SL HARQ feedback is not transmitted due to UL/SL prioritization; or 5> start the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the first slot after the end of the corresponding PSFCH resource for the SL HARQ feedback when the SL HARQ feedback is a positive acknowledgement. 4> if HARQ feedback is disabled by the SCI and the resource(s) for one or more retransmission opportunities is not scheduled in the SCI: 5> start the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the slot following the end of PSFCH resource. 4> if HARQ feedback is disabled by the SCI and the resource(s) for one or more retransmission opportunities is scheduled in the SCI: 5> start the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the slot following the end of PSSCH transmission (i.e., currently received PSSCH).

Sidelink RTT timer may be started in following cases,

- PSFCH is not configured for the received SCI

- PSFCH transmission to the received SCI, including unicast and groupcast

- PSFCH to the received SCI is not transmitted due to UL/SL prioritization

- HARQ disabled by the received SCI

Note the RTT timer can start in different positions, e.g. after PSFCH or after PSSCH. The common part is the RX UE shall receive the SCI of initial transmission or retransmission to start RTT timer. In the retransmission case, TX UE may not be able to perform retransmission during sidelink retransmission timer running, due to SL/UL prioritization or NR/LTE sidelink prioritization. In such case, RTT timer would not be restarted due to no SCI is received, and no more retransmission timer would be started. TX UE has to wait for the next active time to perform retransmission. Data loss may occur if the next active time is beyond the PDB of the retransmission data.

Observation 1: Data loss may occur if tx UE is not able to perform retransmission during retransmission timer running due to SL/UL prioritization or NR/LTE sidelink prioritization.

Sidelink RTT timer and retransmission timer can be restarted multiple times, as long as the data is not successfully decoded. Furthermore, according to the agreements, the sidelink DRX timers are defined by physical slots not logical slots. However, the sidelink transmission resource may not be available on all slots. There may be the case that no transmission resource is available during retransmission timer running. TX UE is not able to perform retransmission during retransmission timer running. Data loss may occur.

Observation 2: Data loss may occur if tx UE is not able to perform retransmission during retransmission timer running due to no transmission resource is available during retransmission timer running.

One simple solution is to extend the active time if no SCI is received during the retransmission timer running. To keep the legacy logistic of RTT timer and retransmission timer handling, a new RTT timer start event can be introduced, i.e. no SCI is received for the corresponding Sidelink process during associated sl-drx-RetransmissionTimer running. After RTT timer expires, retransmission timer would be started again following legacy procedure. Corresponding TP is given in section 5.

Proposal 1: Start RTT timer if no SCI is received for the corresponding Sidelink process during associated sl-drx-RetransmissionTimer running.

Proposal 2: Agree the TP in section 5.

Conclusion

Based on the discussion in section 2, we have following proposals:

Observation 1: Data loss may occur if tx UE is not able to perform retransmission during retransmission timer running due to SL/UL prioritization or NR/LTE sidelink prioritization.

Observation 2: Data loss may occur if tx UE is not able to perform retransmission during retransmission timer running due to no transmission resource is available during retransmission timer running.

Proposal 1: Start RTT timer if no SCI is received for the corresponding Sidelink process during associated sl-drx-RetransmissionTimer running.

Proposal 2: Agree the TP in section 5.

Reference

[1] 38.321

TP

- 5.28.2	Behaviour of UE receiving SL-SCH Data

When SL DRX is configured, the Active Time includes the time while:

-	sl-drx-onDurationTimer or sl-drx-InactivityTimer is running; or

-	sl-drx-RetransmissionTimer is running; or

-	period of sl-LatencyBoundCSI-Report configured by RRC in case SL-CSI reporting MAC CE is not received; or

-	the time between the transmission of the request of SL-CSI reporting and the reception of the SL-CSI reporting MAC CE in case SL-CSI reporting MAC CE is received; or

-	Slot associated with the announced periodic transmissions by the UE transmitting SL-SCH Data.

When one or multiple SL DRX is configured, the MAC entity shall:

1>	if multiple SL DRX Cycles that are mapped with multiple SL-QoS-Profiles of a Destination Layer-2 ID and interested cast type is associated to groupcast or broadcast:

2>	select sl-drx-Cycle whose length of the sl-drx-cycle is the shortest one among multiple SL DRX Cycles that are mapped with multiple SL-QoS-Profiles associated with the Destination Layer-2 ID:

2>	select sl-drx-onDurationTimer whose length of the sl-drx-onDurationTimer is the longest one among multiple SL DRX onDuration timers that are mapped with multiple SL-QoS-Profiles associated with the Destination Layer-2 ID.

1>	if an sl-drx-HARQ-RTT-Timer expires:

2>	if the data of the corresponding Sidelink process was not successfully decoded or if the HARQ feedback (i.e., negative acknowledgement) is not transmitted for unicast due to UL/SL prioritization:

3>	start the sl-drx-RetransmissionTimer for the corresponding Sidelink process in the first slot after the expiry of sl-drx-HARQ-RTT-Timer.

When the cast type is groupcast or broadcast as indicated by upper layer, the sl-drx-StartOffset and sl-drx-SlotOffset are derived from the following equations:

sl-drx-StartOffset (ms) = Destination Layer-2 ID modulo sl-drx-Cycle (ms).

sl-drx-SlotOffset (ms) = Destination Layer-2 ID modulo the number of slots in one subframe (ms).

1>	if the SL DRX cycle is used, and [(DFN × 10) + subframe number] modulo (sl-drx-Cycle) = sl-drx-StartOffset:

2>	start sl-drx-onDurationTimer after sl-drx-SlotOffset from the beginning of the subframe.

1>	if an SL DRX is in Active Time:

2>	monitor the SCI (i.e., 1st stage SCI and 2nd stage SCI) in this SL DRX.

2>	if the SCI indicates a new SL transmission:

3>	if Source Layer-1 ID of the SCI is equal to the 8 LSB of the intended Destination Layer-2 ID and Destination Layer-1 ID of the SCI is equal to the 16 LSB of the intended Source Layer-2 ID and the cast type indicator in the SCI is set to unicast:

4>	start or restart sl-drx-InactivityTimer for the corresponding Source Layer-2 ID and Destination Layer-2 ID pair after the first slot of SCI reception.

3>	if Destination Layer-1 ID of the SCI (i.e., 2nd stage SCI) is equal to the 16 LSB of the intended Destination Layer-1 ID and the cast type indicator in the SCI is set to groupcast:

4>	select sl-drx-InactivityTimer whose length of the sl-drx-InactivityTimer is the largest one among multiple SL DRX Inactivity timers that are mapped to multiple SL-QoS-Profiles of Destination Layer-2 ID associated with the Destination Layer-1 ID of the SCI; and

4>	start or restart sl-drx-InactivityTimer for the corresponding Destination Layer-2 ID after the first slot of SCI reception.

2>	if the SCI indicates an SL transmission:

3>	if a next retransmission opportunity is scheduled in the SCI:

4>	derive the sl-drx-HARQ-RTT-Timer from the retransmission resource timing of the next retransmission resource in the SCI.

3>	else:

4>	use the sl-drx-HARQ-RTT-Timer configured by upper layers.

3>	if PSFCH resource is not configured for the SL grant associated to the SCI:

4>	start the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the slot following the end of PSSCH transmission (i.e., currently received PSSCH).

3>	if PSFCH resource is configured for the SL grant associated to the SCI:

4>	if HARQ feedback is enabled by the SCI and the cast type indicator in the SCI is set to unicast; or

4>	if HARQ feedback is enabled by the SCI and the cast type indicator in the SCI is set to groupcast and positive-negative acknowledgement is selected;

5>	start the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the first slot after the end of the corresponding PSFCH transmission carrying the SL HARQ feedback; or

5>	start the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the first slot after the end of the corresponding PSFCH resource for the SL HARQ feedback when the SL HARQ feedback is not transmitted due to UL/SL prioritization;

4>	if HARQ feedback is enabled by the SCI and the cast type indicator in the SCI is set to groupcast and negative-only acknowledgement is selected;

5>	start the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the first slot after the end of the corresponding PSFCH transmission carrying the SL HARQ feedback; or

5>	start the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the first slot after the end of the corresponding PSFCH resource for the SL HARQ feedback when the SL HARQ feedback is not transmitted due to UL/SL prioritization; or

5>	start the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the first slot after the end of the corresponding PSFCH resource for the SL HARQ feedback when the SL HARQ feedback is a positive acknowledgement.

4>	if HARQ feedback is disabled by the SCI and the resource(s) for one or more retransmission opportunities is not scheduled in the SCI:

5>	start the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the slot following the end of PSFCH resource.

4>	if HARQ feedback is disabled by the SCI and the resource(s) for one or more retransmission opportunities is scheduled in the SCI:

5>	start the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the slot following the end of PSSCH transmission (i.e., currently received PSSCH).

NOTE:	Void.

3>	stop the sl-drx-RetransmissionTimer for the corresponding Sidelink process.

2> if no SCI is received for the corresponding Sidelink process during associated sl-drx-RetransmissionTimer running:

3> start the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the slot following the end of sl-drx-RetransmissionTimer expiry.

1>	if an SL DRX Command MAC CE is received for the Source Layer-2 ID and Destination Layer-2 ID pair of a unicast:

2>	stop sl-drx-onDurationTimer for the Source Layer-2 ID and Destination Layer-2 ID pair of a unicast;

2>	stop sl-drx-InactivityTimer for the Source Layer-2 ID and Destination Layer-2 ID pair of a unicast.