3GPP TSG-RAN WG2 Meeting #119	 R2-2207182

Electronic 17 – 26 Aug, 2022

Agenda Item:	8.9.2

Source:	Xiaomi

Title:	Discussion on U2U relay discovery and relay selection

Document for:	Discussion

Introduction

L2 and L3 U2U relay is to be supported. In this contribution, we discuss the common parts of L2/L3 U2U relay, i.e. relay discovery and (re)selection.

Discussion

In general the baseline for U2U NR sidelink relaying should be based on U2N NR sidelink relaying principles and functionality, where required functionality is sufficiently similar or the same. Changes for the implementation of U2U NR sidelink relaying could be either by way of a delta to the existing procedure, or may be new functionality i.e. where the pursuit of a delta solution creates unnecessary complexity.

Proposal 1: the baseline for U2U NR sidelink relaying should be based on U2N NR sidelink relaying principles and functionality, where said functionality is sufficiently similar or the same.

Relay discovery

Firstly, as discovery in itself does not necessarily lead to link establishment, as a baseline sending discovery messages using legacy Model A and Model B discovery procedures (regardless of their potential modification of the messages for U2U relaying).

Proposal 2: Discovery Model A and Model B are considered as a baseline solution for discovery in the U2U NR sidelink relaying procedure.

However, it is noted that a new discovery message is expected to be defined by SA2 for U2U relay discovery. One question for RAN2 is whether to introduce new resources or if legacy resources can be reused for transmission of this U2U relay discovery message. In R17, dedicated or shared resource pool for discovery can be configured by NW. Resource pool selection is also defined when both pools are configured. We think there is no need to introduce new dedicated resource pool for U2U relay discovery. In addition, resource pool selection is based on available resource pool indicated by the serving cell for the frequency(s) chosen by the UE to transmit the discovery message on. In addition to being broadcast resource pools are available via dedicated signalling. We see no need to overturn these legacy behaviours for U2U relaying.

Legacy resource pools and pool selection mechanisms can be reused for U2U relay discovery transmission.

Proposal 3: Legacy resource pool, resource pool signalling and resource pool selection mechanisms can be reused for U2U relay discovery transmission.

Also, discovery messages in legacy are sent on a dedicated radio bearer SL-SRB4 which can also assist the UE in managing priority of discovery messages.

Proposal 4: Reuse dedicated radio bearer SL-SRB4 for U2U relay discovery transmission.

It is noted that some solutions for U2U discovery in SA2 [1] propose to employ DCR transmission as an optimisation for discovery and link establishment. It is not clear that use of existing sidelink communication messages for discovery would be easily re-usable within the legacy RAN discovery framework and hence could unintentionally add levels of complexity.

It is recommended that RAN2 ask SA2 that when selecting their final solution they take the fore mentioned legacy aspects into account in order to not unnecessarily create additional complexity and impact possible work item completion.

Proposal 5: Send SA2 an LS to ask that when selecting their final solution(s) that they take into account the legacy RAN discovery message transmission framework and avoid unnecessary complexity impact in RAN.

Then next question is how to trigger the relay discovery. If there is no existing PC5 connection between source and target UE, the trigger is up to upper layer. If source and target UE has established PC5 connection, RAN2 should discuss how a remote UE triggers the U2U discovery.

In U2N relay, the remote UE can trigger discovery based on Uu channel quality, since the poor Uu channel quality indicates potential RLF. In U2U relay, the remote UE should also trigger the relay discovery if existing PC5 connection fails or is about to fail. One straightforward factor to indicate potential PC5 connection failure is the channel quality below certain threshold.

For example, if the existing PC5 connection is established via relay UE, the PC5 connection channel quality is between the remote UE and the serving relay UE, separately for both hops. Also, in SA2 TR [1], multiple solutions propose to trigger discovery based on signal quality.

Proposal 6: U2U relay discovery should be triggered if the PC5 channel quality with target UE without relay or with selected U2U relay is below certain threshold.

Moreover, unlike Uu, reference signal may not always available on sidelink, so UE may not obtain the measurement result on sidelink. T400 expiry and maximum number of consecutive HARQ DTX were introduced to trigger sidelink RLF. Therefore, sidelink RLF may occur before UE can detect PC5 channel quality becomes lower than the threshold. U2U relay discovery should be triggered in advance in case the sidelink measurement is not available. Otherwise, UE can only trigger U2U relay discovery after sidelink RLF occurs, which may delay the relay selection and extend the service interruption.

Proposal 7: U2U relay discovery should be triggered in advance of sidelink RLF in case the sidelink measurement result is not available.

Regarding the content carried in U2U relay discovery. In SA2 TR [1], it is proposed to consider traffic load of the relay UE during relay selection in solution#1 as following.

When a UE-to-UE relay receives a Direct Communication Request or a Solicitation message with the relay_indication set, then it shall decide whether to forward the message (i.e. modify the message and broadcast it in its proximity), according to e.g. Relay Service Code if there is any, Application ID, authorization policy (e.g. relay for specific ProSe Service), the current traffic load of the relay, the radio conditions between the source UE and the relay UE, etc. It may exist a situation where multiple UE-to-UE relays can be used to reach the target UE or the target UE may also directly receive the Direct Communication Request or Solicitation message from the source UE. The target UE may choose which one to reply according to e.g. signal strength, local policy (e.g. traffic load of the UE-to-UE relays), Relay Service Code if there is any or operator policies (e.g. always prefer direct communication or only use some specific UE-to-UE relays).

Therefore, traffic load should be included in the discovery message. RAN2 can further discuss how to define the load of the relay UE. One straight way to define the load is based on relay UE’s CBR result.

Proposal 8: Load information of U2U relay UE can be included in the U2U relay discovery message, FFS on how to determine the load information.

Relay (re)selection

UE may discover multiple U2U relay UEs through use of the discovery procedure. SA2 has discussed NAS criteria of the candidate relay UE from service perspective. Furthermore, it’s also proposed the signal strength could be considered during relay selection in solution#1 [1], as following.

It may exist a situation where multiple UE-to-UE relays can be used to reach the target UE or the target UE may also directly receive the Direct Communication Request or Solicitation message from the source UE. The target UE may choose which one to reply according to e.g. signal strength, local policy (e.g. traffic load of the UE-to-UE relays), Relay Service Code if there is any or operator policies (e.g. always prefer direct communication or only use some specific UE-to-UE relays).

In U2N relay, only the U2N relay UE whose signal strength is above the threshold is considered for relay (re)selection. Similar procedure could be introduced during U2U relay (re)selection.

Proposal 9: Candidate U2U relay UE is considered to meet the AS criteria if its measurement result is above a configured threshold.

When multiple candidate Relay UEs detecting a target Remote UE it may be beneficial that multiple Relay UEs indicate the availability of the target Remote UE to the source Remote UE. In particular as in the case where only one Relay UE is selected (as per example in #1 [1]), this may not be the most optimum Relay UE to connect to for the source Remote UE, in particular if other Relay UEs are available, and hence may not lead to the best U2U relay configuration for all parties in the final U2U relay. As such the Relay UE may indicate to the source Remote UE, additional AS Information during discovery such as the PC5 connection signal strength for target Remote UE.

Proposal 10: Candidate U2U Relay UE may indicate the PC5 connection signal strength for target Remote UE to the source Remote UE, via the AS Information during discovery.

As discussed in 2.1, load is also considered during the relay selection. If multiple U2U relay UE fulfils AS and NAS requirement, a simple solution is to select the U2U relay UE with the lowest load. If there is multiple lowest load relay, it’s up to remote UE to select one.

Proposal 11: If multiple U2U relay UE fulfils AS and NAS requirement, UE select the U2U relay UE with the lowest load.

Conclusion

Based on the discussion in section 2, we have following proposals

Proposal 1: the baseline for U2U NR sidelink relaying should be based on U2N NR sidelink relaying principles and functionality, where said functionality is sufficiently similar or the same.

Proposal 2: Discovery Model A and Model B are a baseline solution for discovery in the U2U NR sidelink relaying procedure.

Proposal 3: Legacy resource pool, resource pool signalling and resource pool selection mechanisms can be reused for U2U relay discovery transmission.

Proposal 4: Reuse dedicated radio bearer SL-SRB4 as baseline.

Proposal 5: Send SA2 an LS to ask that when selecting their final solution(s) that they take into account the legacy RAN discovery message transmission framework and avoid unnecessary complexity impact in RAN.

Proposal 6: U2U relay discovery should be triggered if the PC5 channel quality with target UE or selected U2U relay is below certain threshold.

Proposal 7: U2U relay discovery should be triggered in advance of sidelink RLF in case the sidelink measurement result is not available.

Proposal 8: Load information of U2U relay UE can be included in the U2U relay discovery message, FFS on how to determine the load information.

Proposal 9: Candidate U2U relay UE is considered to meet the AS criteria if its measurement result is above a configured threshold.

Proposal 10: Candidate U2U Relay UE may indicate the PC5 connection signal strength for target Remote UE to the source Remote UE, via the AS Information during discovery.

Proposal 11: If multiple U2U relay UE fulfils both AS and NAS requirement, UE select the U2U relay UE with the lowest load.

Reference

[1] 	TR 23.700-33 V0.3.0 (2022-05)  Study on system enhancement for Proximity based Services (ProSe) in the 5G System (5GS); Phase 2 (Release 18)

[2]