3GPP TSG-RAN WG2 Meeting 119 electronic	R2-2207217

Online, August 17th – 29th  2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 3234 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on null security algorithm |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2022-8-4 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the CT1 and SA3’s reply’s LS(C1-223972, S3- 221590), it is not appropriate to use “is protected” for Sidelink RB, since null security algorithm can be considered as security is activated, but can not be considered as the message is protected, as shown in following: SA3 would like to thank RAN5 for their LS on V2X PC5 link for unicast communication with null security algorithms. SA3 has reviewed the content of the LS and relevant specification and would like to provide clarification on the usage of the NULL security algorithms. The wording of “no security” is misleading in the current specification of TS 33.536, i.e. the NULL security algorithms for encryption and integrity are still security algorithms, but without any protection of the content of the messages. Put in another way, the selection of NULL algorithms means that the PC5 messages are considered protected for the purposes of being allowed to be sent or received (even if in effect there is no protection of the messages). Furthermore SA3 would like to confirm that the key establishment procedures in clause 5.3.3.1.3 of TS 33.536 can be skipped if the receiving UE chooses NULL PC5 integrity protection algorithm for the PC5 signalling integrity protection.. CT1 has reviewed the content of the LS and relevant specifications and would like to indicate that according to CT1 specification the use of null security protection algorithm (i.e., in 5GS NEA0 and NIA0) are in fact security protection algorithms used for integrity protecting and ciphering of the PC5 messages. However, when applied they cannot provide any integrity protection or ciphering for the PC5 messages. Therefore if the null security protection algorithms, i.e. integrity protection and ciphering using null security protection, are activated as a result of successful security control mode procedure, the PC5 messages are transmitted unprotected.. Therefore, from RAN2’s perspective, since security is out of RAN2’s scope, it is suggested to follow SA3 and CT1’s understanding that null security algorithm can be considered as security is activated, but in this case, the PC5 message is not protected. |
| Summary of change: |  | Modify the description of security procedure for SL-SRB and SL-DRB. Impact analysis Impacted 5G architecture options: Standalone/Non-standalone Impacted functionality: NR sidelink communication configuration Inter-operability: - If the UE is implemented according to this CR but the network is not, no inter-operability issue foreseen. - If the network is implemented according to this CR but the UE is not, no inter-operability issue foreseen. - If one UE is implemented according to this CR but the other UE is not, no inter-operability issue foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Description of sidelink security is incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of the change

5.8	Sidelink

5.8.1	General

NR sidelink communication consists of unicast, groupcast and broadcast. For unicast, the PC5-RRC connection is a logical connection between a pair of a Source Layer-2 ID and a Destination Layer-2 ID in the AS. The PC5-RRC signalling, as specified in clause 5.8.9, can be initiated after its corresponding PC5 unicast link establishment (TS 23.287 [55]). The PC5-RRC connection and the corresponding sidelink SRBs and sidelink DRB(s) are released when the PC5 unicast link is released as indicated by upper layers.

For each PC5-RRC connection of unicast, one sidelink SRB (i.e. SL-SRB0) is used to transmit the PC5-S message(s) before the PC5-S security has been established. One sidelink SRB (i.e. SL-SRB1) is used to transmit the PC5-S messages to establish the PC5-S security. One sidelink SRB (i.e. SL-SRB2) is used to transmit the PC5-S messages after the PC5-S security has been established. One sidelink SRB (i.e. SL-SRB3) is used to transmit the PC5-RRC signalling and only sent after the PC5-S security has been established. One sidelink SRB (i.e. SL-SRB4) is used to transmit/receive the NR sidelink discovery messages.

For unicast of NR sidelink communication, AS security comprises of integrity protection of PC5 signalling (SL-SRB1, SL-SRB2 and SL-SRB3) and user data (SL-DRBs), and it further comprises of ciphering of PC5 signaling (SL-SRB1 only for the Direct Link Security Mode Complete message as specified in TS 24.587 [57] for V2X service or TS 24.554 [72] for Proximity-services, SL-SRB2 and SL-SRB3) and user data (SL-DRBs). The ciphering and integrity protection algorithms and parameters for a PC5 unicast link are exchanged by PC5-S messages in the upper layers as specified in TS 33.536 [60], and apply to the corresponding PC5-RRC connection in the AS. Once AS security is activated for a PC5 unicast link in the upper layers as specified in TS 33.536 [60], the ciphering method configured by upper layer shall be applied all messages on SL-SRB2 and SL-SRB3 and/or user data on SL-DRBs of the corresponding PC5-RRC connection.

For unicast of NR sidelink communication, if the change of the key is indicated by the upper layers as specified in TS 24.587 [57], UE re-establishes the PDCP entity of the SL-SRB1, SL-SRB2, SL-SRB3 and SL-DRBs on the corresponding PC5-RRC connection.

NOTE 1:	In case the configurations for NR sidelink communication are acquired via the E-UTRA, the configurations for NR sidelink communication in SIB12 and sl-ConfigDedicatedNR within RRCReconfiguration used in clause 5.8 are provided by the configurations in SystemInformationBlockType28 and sl-ConfigDedicatedForNR within RRCConnectionReconfiguration as specified in TS 36.331 [10], respectively.

NOTE 2:	In this release, there is one-to-one correspondence between the PC5-RRC connection and the PC5 unicast link as specified in TS 38.300[2].

NOTE 3:	All SL-DRBs related to the same PC5-RRC connection have the same activation/deactivation setting for ciphering and the same activation/deactivation setting for integrity protection as specified in TS 33.536 [60].

NOTE 4:	When integrity check failure concerning SL-SRB1 for a specific destination is detected, the UE sends an indication to the upper layers [57].

End of the change