3GPP TSG-RAN WG2 #119-e	R2-2207220

Electronic meeting, 17 August – 29 August 2022

Agenda Item:	8.9.3

Source:	Ericsson

Title:	Service Continuity Enhancements for Layer-2 UE-to-Network Relays

Document for:	Discussion, Decision

# 1	Introduction

In the WID [1], one of the objectives is to specify mechanisms to enhance service continuity for the single-hop Layer 2 (L2) UE-to-Network relays detailed as follows:

- Specify mechanisms to enhance service continuity for single-hop Layer-2 UE-to-Network relay for the following scenarios [RAN2, RAN3]:

- Inter-gNB indirect-to-direct path switching (i.e., “remote UE <-> relay UE A <-> gNB X” to “remote UE <-> gNB Y”)

- Inter-gNB direct-to-indirect path switching (i.e., “remote UE <-> gNB X” to “remote UE <-> relay UE A <-> gNB Y”)

- Intra-gNB indirect-to-indirect path switching (i.e., “remote UE <-> relay UE A <-> gNB X” to “remote UE <-> relay UE B <-> gNB X”)

- Inter-gNB indirect-to-indirect path switching (i.e., “remote UE<-> relay UE A <-> gNB X” to “remote UE <-> relay UE B <-> gNB Y”)

Note 2A: Scenario D is to be supported by reusing solutions for the other scenarios without specific optimizations

We discuss the objectives and express our views accordingly in this contribution.

# 2	Discussion

The Rel-17 work item on NR Sidelink Relay studied and specified mechanisms to support service continuity during a path switch procedure (i.e., indirect to direct path and vice-versa) in L2 UE-to-Network (U2N) relay. But service continuity during a path switch procedure in R17 was limited to the case of an intra-gNB scenario. However, the Rel-18 work item on NR sidelink relay enhancements details the objectives for service continuity in inter-gNB path switch scenarios.

In Rel-17, it was also specified that a gNB can select a U2N relay UE in any RRC state (i.e., RRC_IDLE, RRC_INACTIVE, or RRC_CONNECTED) as the target U2N relay UE. As a result, the same should also be applicable to the inter-gNB path switch procedure.

For inter-gNB path switching in Rel-18, the gNB can select a target U2N relay UE in any RRC state, i.e., RRC_CONNECTED/IDLE/INACTIVE following the path switch scenario agreed in Rel-17.

In general, we believe the procedures for intra-gNB based path switching developed in Rel.17 should be used as the baseline with necessary additions/modifications to accommodate for the inter-gNB scenario. This includes the solutions to handle the case when the target U2N relay UE is in any RRC state. In the following, we discuss the different cases based on the RRC state of the U2N relay UE.

## 2.1 Target U2N Relay UE in RRC_CONNECTED state

- For the target U2N relay UE in an RRC_CONNECTED state, most of the steps from the Rel-17 intra-gNB path switch procedures can be re-used with the addition of inter-gNB signaling over the Xn interface. The inter-gNB signaling over the Xn interface is under the RAN3 purview and the corresponding details should be left to RAN3.

For the target U2N relay UE in RRC_CONNECTED state, the Rel-17 procedures for intra-gNB path switching are used as a baseline for inter-gNB path switching with the addition of inter-gNB signaling over the Xn interface. The details of the Xn signaling are upto RAN3.

## 2.2 Target U2N Relay UE in RRC_IDLE/INACTIVE state

For the target U2N relay UE in an RRC_IDLE or RRC_INACTIVE state, steps from the U2N relay UE in RRC_CONNECTED state should be re-used with the additional step of transitioning the target U2N relay UE. The outcome of the previous discussions in Rel-17 can be consolidated into the following solutions:

  - Remote UE oriented: Remote UE using PC5 signaling triggers the U2N relay UE in IDLE/INACTIVE to transit to the CONNECTED state

  - Paging-based: U2N relay UE in IDLE/INACTIVE can be paged by the serving gNB to transition to the CONNECTED state to enable the relaying of remote UE data

- After much deliberation, it was agreed the remote UE oriented solution can be used to transition the target U2N relay UE to the CONNECTED state from IDLE or INACTIVE state and the same should also be applicable for the Rel-18 inter/intra-gNB scenarios.

The Rel-17 remote UE oriented solution to trigger the target U2N relay UE to the CONNECTED state should also be applicable to the Rel-18 inter/intra-gNB scenarios.

- Although, the paging-based solution can provide some benefits in terms of reducing the latency of the path switch procedure by potentially transitioning the target U2N relay UE to the CONNECTED state before remote UE initiates the PC5 communication. The following is our analysis:

- For the target U2N relay UE in the INACTIVE state, the paging-based solution is already feasible based on Rel-17 specifications and with no changes required to the legacy paging mechanism. That is, if the gNB can retrieve the target U2N relay UE’s context, it can page (using RAN-paging) the target U2N relay UE to initiate the random-access procedure. This can also be done before the path switch command is sent to the remote UE and as a result, potentially, the target U2N relay UE is already in the CONNECTED state when the remote UE initiates PC5 communication for relaying.

- For the target U2N relay UE in the IDLE state, additional specification work is required to support this state as this would involve cross-WG interactions. Given the current workload, such solutions should be down prioritized.

## 2.3 Measurement framework

- In Rel.17, the following measurement report triggering events were introduced to support switching between a direct path and indirect path, and vice-versa [2]:

- Event X1: Serving U2N Relay UE becomes worse than threshold1 and NR Cell becomes better than threshold2

- Event X2: Serving L2 U2N Relay UE becomes worse than threshold

- Event Y1: PCell becomes worse than threshold1 and candidate L2 U2N Relay UE becomes better than threshold2

- Event Y2: Candidate L2 U2N Relay UE becomes better than threshold

In Rel-18, new measurement report triggering events are needed to support the indirect-to-indirect path switch scenario. The new event can be based on the above Events X, Y with the serving U2N relay becoming worse than a threshold1 and the candidate U2N relay UE becomes better than a threshold2.

New measurement report triggering events are to be supported for the indirect-to-indirect path switch scenario based on the Events X and Y.

## 2.4 Inter-gNB Path Switch Scenario, Target U2N Relay UE

Traditionally, in inter-gNB mobility scenarios, based on the handover request message from the source gNB, it is upto the target gNB to accept a UE handover (via admission control) and provide the corresponding configuration to the source gNB. In this case, the source gNB pre-selects a particular target gNB based on UE measurements. As a result, both source and target gNB are involved in the decision-making process.

Figure 1: Inter-gNB path switch scenario

A similar principle should be applied in the inter-gNB path switch scenario, where the source gNB pre-selects a list of candidate U2N relay UEs under target gNB coverage and sends this list (over Xn signaling) to the target gNB. Based on this information, it would also be appropriate for the target gNB to decide on the target U2N relay UE because only the target gNB has knowledge about the candidate U2N relay’s context in terms of for e.g., Uu signal strength, capability, and various other aspects.

For Rel-18 inter-gNB path switch procedure, only the target gNB has knowledge about the candidate U2N relays in terms of for e.g., the Uu signal strength, capability, and various other aspects.

By having the target gNB decide on the target U2N relay UE, we can also avoid the situation where the source and target gNB would need to exchange messages over the Xn interface for the source gNB to obtain the candidate U2N relay UE’s context information (for e.g., Uu signal strength, capability) from the target gNB.

In an inter-gNB path switch procedure, unnecessary exchange of information about the candidate relay UEs over the Xn interface should be avoided.

As a result, the source gNB pre-selects the candidate U2N relay UEs and the target gNB should be the entity to decide on the target U2N relay (within its coverage) with the details of the Xn signaling pending on RAN3. In addition, the target gNB should provide the remote UE (via source gNB) with the configuration to setup the indirect path.

In an inter-gNB path switch procedure, based on the preselected candidate U2N relays UEs by the source gNB, the target gNB is to decide on the target U2N relay UE. The Xn signaling details are pending on RAN3.

# 3 Conclusion

In the previous sections we made the following observations:

- For Rel-18 inter-gNB path switch procedure, only the target gNB has knowledge about the candidate U2N relays in terms of for e.g., the Uu signal strength, capability, and various other aspects.

In an inter-gNB path switch procedure, unnecessary exchange of information about the candidate relay UEs over the Xn interface should be avoided.

Based on the discussion in the previous sections we propose the following:

- For inter-gNB path switching in Rel-18, the gNB can select a target U2N relay UE in any RRC state, i.e., RRC_CONNECTED/IDLE/INACTIVE following the path switch scenario agreed in Rel-17.

For the target U2N relay UE in RRC_CONNECTED state, the Rel-17 procedures for intra-gNB path switching are used as a baseline for inter-gNB path switching with the addition of inter-gNB signaling over the Xn interface. The details of the Xn signaling are upto RAN3.

The Rel-17 remote UE oriented solution to trigger the target U2N relay UE to the CONNECTED state should also be applicable to the Rel-18 inter/intra-gNB scenarios.

New measurement report triggering events are to be supported for the indirect-to-indirect path switch scenario based on the Events X and Y.

In an inter-gNB path switch procedure, based on the preselected candidate U2N relays UEs by the source gNB, the target gNB is to decide on the target U2N relay UE. The Xn signaling details are pending on RAN3.

# 4 References

- RP- 221262, Revised WID on NR sidelink relay enhancements, RAN #96, Budapest, Hungary, June 6-9, 2022.

- TS 38.331, V17.0.0, “Radio Resource Control (RRC) protocol specification, (Release 17)”