3GPP TSG-RAN WG2 Meeting #119-electronic	R2-2207226

# Online, August 17th - August 29th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 1310 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on pdsch-AggregationFactor in NR MBS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS-Core |  |  |  |  |  | Date: |  |  | 2022-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In Rel-17, to support PDSCH transmission across more than one consecutive slots, pdsch-AggregationFactor can be configured in pdsch-config (for unicast), or pdsch-AggregationFactorMulticast in pdsch-Config-Multicast can be configured (for multicast), or pdsch-AggregationFactor can be configured in pdsch-ConfigPTM (for broadcast). However, in the latest MAC spec, it doesn’t capture the case where the parameter pdsch-AggregationFactorMulticast provides the number of transmissions of a multicast TB within a bundle of the downlink assignment. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that the number of transmissions of a TB within a bundle of the downlink assignment is provided by AGGREGATION_NUMBER set by the lower layers. Impact analysis Impacted 5G architecture options: Standalone Impacted functionality: NR MBS multicast Inter-operability: If the UE is implemented according to this CR but the network is not, or vice versa, there is no inter-operability issue foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PDSCH aggregation is not supported for dynamic scheduling for multicast. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF THE CHANGE

5.3.2.1	HARQ Entity

The MAC entity includes a HARQ entity for each Serving Cell, which maintains a number of parallel HARQ processes. Each HARQ process is associated with a HARQ process identifier. The HARQ entity directs HARQ information and associated TBs received on the DL-SCH to the corresponding HARQ processes (see clause 5.3.2.2).

The number of parallel DL HARQ processes per HARQ entity is specified in TS 38.214 [7]. The dedicated broadcast HARQ process is used for BCCH.

The HARQ process supports one TB when the physical layer is not configured for downlink spatial multiplexing. The HARQ process supports one or two TBs when the physical layer is configured for downlink spatial multiplexing.

The number of transmissions of a TB within a bundle of the downlink assignment is given by AGGREGATION_NUMBER, which is set to a value provided by lower layers as specified in clause 5.1.2.1 of TS 38.214 [7]. Bundling operation relies on the HARQ entity for invoking the same HARQ process for each transmission that is part of the same bundle. If AGGREGATION_NUMBER > 1, after the initial transmission, AGGREGATION_NUMBER – 1 HARQ retransmissions follow within a bundle.

The MAC entity shall:

1>	if a downlink assignment has been indicated:

2>	allocate the TB(s) received from the physical layer and the associated HARQ information to the HARQ process indicated by the associated HARQ information.

1>	if a downlink assignment has been indicated for the broadcast HARQ process:

2>	allocate the received TB to the broadcast HARQ process.

NOTE:	It is up to UE implementation to allocate the received TB for MCCH or broadcast MTCH to one HARQ process.

END OF THE CHANGE