3GPP TSG-RAN WG2 Meeting #119-electronic	   R2-2207227

Online, August 17th - August 29th, 2022

Agenda item:		8.11.2

Source:	vivo

Title:	Supporting Multicast Reception in RRC_INACTIVE

Document for:		Discussion and Decision

1 Introduction

For Rel-18 NR, a WI on enhancements of NR Multicast and Broadcast Services had been approved [1]. According to the WID, RAN2 needs to specify the general procedures to support multicast reception for RRC_INACTIVE UEs, from the perspective of configuration, mobility, and state transition.

- Specify support of multicast reception by UEs in RRC_INACTIVE state [RAN2, RAN3] - PTM configuration for UEs receiving multicast in RRC_INACTIVE state [RAN2] - Study the impact of mobility and state transition for UEs receiving multicast in RRC_INACTIVE. (Seamless/lossless mobility is not required) [RAN2, RAN3]

In this contribution, we would like to provide our understanding on the general procedures for multicast reception in RRC_INACTIVE.

2 Discussion

2.1 Configuration delivery and use cases in RRC_INACTIVE

Based on TS 23.247 [2], the following definition and authorization requirements are specified for NR multicast service.

Multicast communication service: A 5GS communication service in which the same service and the same specific content data are provided simultaneously to a dedicated set of UEs (i.e., not all UEs in the multicast coverage are authorized to receive the data). The following authorizations are defined: a) Whether the UE is authorized to use the Multicast service in the PLMN. b) The authorization for a UE of receiving the content of a specific multicast MBS session. A Multicast MBS session may be "open to any UEs". NOTE 1: UE authorization for a specific Multicast MBS session can be implicitly performed when UE is configured for a specific Multicast MBS session, e.g. via Service Announcement for public safety use case. NOTE 2: The authorization mentioned by a) is required even if an authorization according to b) is available. If the UE is not authorized to use the Multicast service by the PLMN, the UE is not authorized to join any multicast MBS Session even if the Multicast MBS session is "open to any UEs". For a Multicast MBS session, it is required that the 5GC authorizes the UE based on the MBS subscription data and whether the Multicast MBS session is "open to any UEs", which are preconfigured, or provided by the AF (see clause 7.1.1).

For NR multicast, it is known that only the authorized UEs are permitted to receive the multicast data. In other words, the UEs without authorization to access NR multicast are forbidden to receive any multicast content even though they are already in the multicast coverage.

Observation 1: The UEs without authorization to access NR multicast are forbidden to receive any content of specific multicast service.

In our understanding, RAN2 needs to take the authorizations for NR multicast into account, when deciding the mechanism to deliver the configuration used for multicast reception in RRC_INACTIVE. Specifically, to get rid of multicast configuration exposure, RRC dedicated message, such as the existing RRC Release message, (for which both ciphering and integrity protection can be provided) is supposed to be used for the configuration delivery. Thus, we propose,

Proposal 1: Configurations for UEs receiving multicast in RRC_INACTIVE state are indicated by suspendConfig in the RRC Release message.

Further, we need to consider the configuration validation cell. Basically, there are three possible opinions,

- Option 1: The configuration is only valid in a cell that is the serving cell in which the UE received RRC Release message with suspendConfig.

- Option 2: The configuration is only valid in a cell (indicated by NW) that is different than the serving cell in which the UE received RRC Release message with suspendConfig.

- Option 3: The configuration is valid within an area.

Firstly, we think Option 3 can be excluded in Rel-18 as RAN2 had discussed the area-specific NR MBS support in Rel-17. Finally, due to cross-layer complexity and limited deployment scenarios, the concept of area-specific NR MBS is not specified. For Option 2, the network needs to associate the configuration with a serving cell. From UE point of view, only the UE with the capability of receiving multicast from a non-serving cell can realize the intended multicast reception. It may not be a common case in Rel-18. Regarding Option 3, it is quite straightforward and should be supported in Rel-18 as a baseline.

Proposal 2: As a baseline, configurations for UEs receiving multicast in RRC_INACTIVE state are valid in a cell that is the serving cell in which the UE received RRC Release message with suspendConfig.

Further, in the WI, it also mentions that “In Rel-17, RAN only specifies multicast for UEs in RRC_CONNECTED state, which may not fully fulfill the requirements of, e.g., Mission Critical Services, especially for cells with a large number of UEs according to TR 23.774. Also, to always keep UEs in RRC_CONNECTED state is not power efficient. It is ‎therefore important to support multicast for UEs in RRC_INACTIVE”. In our understanding, on one hand, multicast reception in RRC_INACTIVE generally is intended to help to relieve the heavy load on the scarce PUCCH resources (i.e. the NW may release the UEs that are only interested in multicast service to RRC_INACTIVE and reserve the PUCCH resources for the other UEs with frequent unicast transmission, accommodating more UEs to communicate within the same serving cell). On the other hand, it further helps the UEs in the cell center edge with low mobility to keep multicast reception without performing RRM related measurement (e.g. RLM, BFD) and SRS transmission. In this sense, to maximize the benefit brought by multicast reception in RRC_INACTIVE, we think PUCCH and SRS transmission are not required. Meanwhile, RLM, BFD, and BFR procedures are also not required.

Observation 2: Multicast reception in RRC_INACTIVE is intended for UL resources offload and UE power saving.

Proposal 3: PUCCH and SRS transmission is not supported for multicast reception in RRC_INACTIVE.

Proposal 4: RLM, BFD, and BFR procedures are not required for multicast reception in RRC_INACTIVE.

Besides, for Rel-15/16/17 RRC_INACTIVE UEs, it always assumes the bandwidth and location of the initial DL BWP are the same as those of CORESET 0. If we reuse this legacy principle for multicast reception in Rel-18, without doubt, the bandwidth might be not large enough to support some multicast services requiring high throughput. To as fully as possible fulfill the requirement of multicast reception, similarly to the Rel-17 NR MBS broadcast, we think the common frequency resource (CFR) for group-common PDCCH/PDSCH of multicast reception will cover the following cases:

    - [Case E] the case where a CFR is defined with the following properties:

      - The CFR is different than the initial BWP, and is not larger than the carrier bandwidth.

      - The CFR needs to fully contain the initial BWP in frequency domain and has the same SCS and CP as the initial BWP.

    - [Case A] A CFR with the same size as the initial BWP, where the initial BWP has the same frequency resources as CORESET0. In this case the CFR has the same frequency resources and same SCS and CP as the initial BWP.

    - [Case C] A CFR with same size as the initial BWP, where the initial BWP has the frequency resources configured by SIB1. In this case the CFR has the same frequency resources and same SCS and CP as the initial BWP.

Proposal 5: CFR used for multicast reception in RRC_INACTIVE can be configured to support the following cases:

    - [Case E] the case where a CFR is defined with the following properties:

      - The CFR is different than the initial BWP, and is not larger than the carrier bandwidth.

      - The CFR needs to fully contain the initial BWP in frequency domain and has the same SCS and CP as the initial BWP.

    - [Case A] A CFR with the same size as the initial BWP, where the initial BWP has the same frequency resources as CORESET0. In this case the CFR has the same frequency resources and same SCS and CP as the initial BWP.

    - [Case C] A CFR with same size as the initial BWP, where the initial BWP has the frequency resources configured by SIB1. In this case the CFR has the same frequency resources and same SCS and CP as the initial BWP.

Proposal 6: Only initial BWP is used for multicast reception in RRC_INACTIVE.

2.2 UE behavior upon the reception of configuration

Based on the discussion above, we hypothesize that the RRC Release message with suspendConfig is used to acquire multicast configuration. Then we have to further discuss the corresponding UE behavior (e.g. multicast MRB handling) upon the reception of configuration.

In Rel-17 NR MBS, the multicast MRB establishment is performed when mrb-ToAddModList is included in the RRC Reconfiguration message and the broadcast MRB establishment may be started when becoming interested in the MBS broadcast service. For the multicast reception in RRC_INACTIVE in Rel-18, obviously, the UE had already joined a specific multicast service with interest when it is in RRC_CONNECTED. And the network is feasible to establish or modify the corresponding multicast MRB before releasing the UE into RRC_INACTIVE. In other words, the UE, which is capable to perform multicast reception in RRC_INACTIVE, can acquire the multicast MRB configuration (e.g. RLC bearer configuration) during the state in RRC_CONNECTED.

Based on the considerations above, it seems a spontaneous logic that the UE shall restore the multicast MRB configuration, keep the multicast MRB (associated with an RLC entity used for PTM reception) resumed, and configure the physical layer and MAC entity in accordance with the L1/L2-level multicast PTM configuration (e.g. search space configuration, multicast DRX configuration).

Proposal 7: For multicast reception in RRC_INACTIVE, when receiving RRC release with suspendConfig (potentially including multicast-related configuration), the UE can at least perform the following actions:

  - MAC is reset except multicast DRX or GC-SPS.

  - SRBs and DRBs are suspended except SRB0 and multicast MRB(s) associated with an RLC entity used for PTM reception.

  - Configure the PHY layer in accordance with multicast PTM configuration.

2.3 Failure handling

If Proposal 7 above can be agreeable, then the UE will keep the necessary multicast configurations upon the reception of the RRC Release message. And then, after transiting into RRC_INACTIVE, the UE will continuously monitor the PDCCH for multicast reception. However, also assuming Proposals 3 and 4 are agreeable, the UE may fail to receive the multicast data while keeping monitoring PDCCH for a time due to poor radio quality(e.g. beam failure). In such a case, as RLM/BFR mechanisms are not available for the INACTIVE UE and the NW can neither know the channel state of the UEs nor freely indicate the UE-specific reconfiguration, the UE has no choice but to monitor the PDCCH in vain. As a result, unnecessary power consumption is inevitable until cell reselection is implemented.

To avoid such embarrassment, RAN2 should consider early recovery for the RRC_INACTIVE UEs with multicast reception. For instance, upon moving out of the coverage area of multicast service, the UE should be allowed to trigger RRC resumption, so that the network can access the UE and reconfigure it appropriately.

Proposal 8: For the UE receiving multicast in RRC_INACTIVE, RRC resumption is initiated upon moving out of the coverage of multicast service.

2.4 Handling of non-MRB data

During the multicast reception in RRC_INACTIVE, we think the following cases need to be considered,

- DL message is sent by the network to further reconfigure or release the multicast configuration (e.g. the network wants to make RRC_INACTIVE UEs go back to RRC_CONNECTED and informs of multicast session stop, along with configuration release).

- UL message is sent by the UE as interest change (e.g. the UE generates the NAS message to report session join/leave and initiates RRC resumption).

Figure 1: non-MRB data arrival during multicast reception in RRC_INACTIVE

For the DL message case, we think the Rel-17 MR MBS group paging mechanism can be reused. Specifically, the UE can simultaneously monitor group paging on its own PO. If the network would like to change the multicast configuration, it can include the TMGI associated with a multicast service in the paging message. As a response, the RRC_INACTIVE UE will initiate the RRC resumption procedure and go to RRC_CONNECTED. After that, we think the network may keep the UEs in RRC_CONNECTED to finish the multicast reception, or reconfigure the multicast configuration and send the UEs back to RRC_INACTIVE again for subsequent multicast reception.

Proposal 9: For the UE receiving multicast in RRC_INACTIVE, Rel-17 MBS group paging mechanism can be reused.

For the UL message case, it is beneficial to let the network be aware of the UE’s interests in multicast services in time. Based on the newly reported session join/leave, the network can determine whether to reconfigure or release the already configured multicast configuration or transit the UE back to RRC_CONNECTED. So, we propose the following proposal,

Proposal 10: During the multicast reception in RRC_INACTIVE, UE session join/leave procedure is supported.

2.5 Mobility support

Next, we investigate the multicast reception in RRC_INACTIVE when cell reselection occurs, assuming that the configurations are only valid in a cell that is the PCell in which the UE received RRC Release message with suspendConfig. As long as the configuration is not valid anymore due to the leave of the current serving cell, the UE cannot continue to receive multicast service. To this end, we think failure handling is also needed for this case. For example, cell reselection will lead to the termination of ongoing multicast reception in RRC_INACTIVE and also triggers the UE to initiate the RRC resumption procedure.

Proposal 11: RRC resumption should be initiated when cell reselection occurs during the multicast reception in RRC_INACTIVE.

Besides, for Rel-15/16/17 RRC_INACTIVE UEs, configured measurement gap by the network is not supported for itner-frequency measurement. For Rel-18 multicast reception in RRC_INACTIVE, although data transmission misalignment between the gNB and UEs in the time domain can be avoided with an explicit measurement gap configuration, for simplicity, we prefer to not introduce optimization. If high QoS requirements are expected, the network anyway can use group paging to transit the UEs back to RRRC_CONNECTED. Thus, here are the last proposal,

Proposal 12: Measurement gap configuration is not supported for multicast reception in RRC_INACTIVE.

3 Conclusion

In this contribution, we have provided our considerations on the support of multicast reception in RRC_INACTIVE. All our observations and proposals are summarized as follows,

Observation 1: The UEs without authorization to access NR multicast are forbidden to receive any content of specific multicast service.

Observation 2: Multicast reception in RRC_INACTIVE is intended for UL resources offload and UE power saving.

Proposal 1: Configurations for UEs receiving multicast in RRC_INACTIVE state are indicated by suspendConfig in the RRC Release message.

Proposal 2: As a baseline, configurations for UEs receiving multicast in RRC_INACTIVE state are valid in a cell that is the serving cell in which the UE received RRC Release message with suspendConfig.

Proposal 3: PUCCH and SRS transmission is not supported for multicast reception in RRC_INACTIVE.

Proposal 4: RLM, BFD, and BFR procedures are not required for multicast reception in RRC_INACTIVE.

Proposal 5: CFR used for multicast reception in RRC_INACTIVE can be configured to support the following cases:

    - [Case E] the case where a CFR is defined with the following properties:

      - The CFR is different than the initial BWP, and is not larger than the carrier bandwidth.

      - The CFR needs to fully contain the initial BWP in frequency domain and has the same SCS and CP as the initial BWP.

    - [Case A] A CFR with the same size as the initial BWP, where the initial BWP has the same frequency resources as CORESET0. In this case the CFR has the same frequency resources and same SCS and CP as the initial BWP.

    - [Case C] A CFR with same size as the initial BWP, where the initial BWP has the frequency resources configured by SIB1. In this case the CFR has the same frequency resources and same SCS and CP as the initial BWP.

Proposal 6: Only initial BWP is used for multicast reception in RRC_INACTIVE.

Proposal 7: For multicast reception in RRC_INACTIVE, when receiving RRC release with suspendConfig (potentially including multicast-related configuration), the UE can at least perform the following actions:

  - MAC is reset except multicast DRX or GC-SPS.

  - SRBs and DRBs are suspended except SRB0 and multicast MRB(s) associated with an RLC entity used for PTM reception.

  - Configure the PHY layer in accordance with multicast PTM configuration.

Proposal 8: For the UE receiving multicast in RRC_INACTIVE, RRC resumption is initiated upon moving out of the coverage of multicast service.

Proposal 9: For the UE receiving multicast in RRC_INACTIVE, Rel-17 MBS group paging mechanism can be reused.

Proposal 10: During the multicast reception in RRC_INACTIVE, UE session join/leave procedure is supported.

Proposal 11: RRC resumption should be initiated when cell reselection occurs during the multicast reception in RRC_INACTIVE.

Proposal 12: Measurement gap configuration is not supported for multicast reception in RRC_INACTIVE.

4 References

[1] RP-221458, Revised WID: Enhancements of NR Multicast and Broadcast Services.

[2] TS 23.247, Architectural Enhancements for 5G Multicast-broadcast Services; Stage 2.